3GPP TSG-RAN WG5 Meeting #93-e	R5-216759

# Online, , 8th November 2021 - 19th November 2021

Agenda Item:	6.6.9.9

Source:	MCC TF160

Title:	Contact parameters in SIP SUBSCRIBE for MCX

Document for:	Discussion

# 1	Introduction

TS 36.579-1 clause 5.5.2.14 specifies the SIB SUBSCRIBE expected from the UE for various subscriptions.

It seems that as working assumption the same Contact parameters have been specified as for the SIP REGISTER. Nevertheless it is not clear whether this assumption is valid according to the core specs.

# 2	Issues

## 2.1	Main questions

Issue 2.1-1:	There seems to be no core spec requesting the SIP SUBSCRIBE's Contact parameters (feature tags) to be the same as in the SIP REGISTER. Nevertheless if such requirement exists, then it shall be made clear in table 5.5.2.14-1 of TS 36.579-1.

Assumption 2.1-1:	In general there is no common restriction or requirement that the SIP SUBSCRIBE's Contact parameters have to be the same as in the SIP REGISTER.

Conclusion 2.1-1:	Based on the assumption 2.1-1 it needs to be sorted out in which case there are mandatory Contact parameters which have to be contained in the SIP SUBSCRIBE.

Issue 2.1-2:	Assuming that the UE does not need to provide all (mandatory) Contact parameters of the SIP REGISTER, there is the question whether it may do so and whether there might be even further Contact parameters.

Assumption 2.1-2:	The UE shall provide all Contact parameters as specified by the core specs for a particular kind of subscription and in general it may provide additional parameters (e.g. a sub-set of the SIP REGISTER's Contact parameters).

Conclusion 2.1-2:	Based on assumption 2.1-2 a clarification would need to be added to table 5.5.2.14-1 of TS 36.579-1 so that there may be additional Contact parameters.
NOTE: This may be needed for other SIP messages too and for other header fields (e.g. Accept header field of the SIP INVITE).

## 2.2	Explicit core spec requirements regarding SIP SUBSCRIBE's Contact parameters

Based on assumption 2.1-1 different kinds of subscription need to be distinguished regarding requirements for mandatory Contact parameters.

### 2.2.1	Determination of service settings

According to TS 24.379 clause 7.2.4, TS 24.281 clause 7.2.4, TS 24.282 clause 7.2.4 there are no requirements regarding the SIP SUBSCRIBE's Contact parameters.

### 2.2.2	Affiliation status determination procedure

According to TS 24.379 clauses 9.2.1.3, 9.2.1.6, TS 24.281 clause 8.2.1.3, TS 24.282 clause 8.2.3 there are no requirements regarding the SIP SUBSCRIBE's Contact parameters.

### 2.2.3	Functional alias status determination procedure

According to TS 24.379 clause 9A.2.1.2, TS 24.282 clause 22.2.1.3 there are no requirements regarding the SIP SUBSCRIBE's Contact parameters.

### 2.2.4	Group management client (GMC) procedures

According to TS 24.481 clause 6.3.13.2.1 the GMC shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field. TS 24.481 does not distinguish MCPTT, MCVideo and MCData for this case, what might be wrong. 
At least for MCPTT the UE shall provide media feature tag +g.3gpp.icsi-ref="urn:urn-7:3gpp-service.ims.icsi.mcptt" but there is no requirement regarding +g.3gpp.mcptt.

NOTE: Similar as for the media feature tag TS 24.481 clause 6.3.13.2.1 hard-codes the P-Preferred-Service header field to MCPTT, what might be wrong too.

### 2.2.4	Configuration management client procedures

Similar to TS 24.481 according to TS 24.484 clause 6.3.13.2.2 the media feature tag +g.3gpp.icsi-ref="urn:urn-7:3gpp-service.ims.icsi.mcptt" is expected to be contained in the Contact parameters and the P-Preferred-Service header field is hard-coded to MCPTT too.

# 3	Conclusions

If assumptions 2.1-1 and 2.1-2 are applicable, then at least Table 5.5.2.14-1 needs to be changed, so that only the mandatory Contact parameter according to TS 24.481 and TS 24.484 shall be there, but additional Contact parameters may be there.

CT1 may need to be informed about hard-coding of the Contact feature parameters to MCPTT in TS 24.481 clause 6.3.13.2.1 and TS 24.484 clause 6.3.13.2.2.