3GPP TSG-RAN WG5 Meeting #93-e	R5-216760

Electronic Meeting, 8th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-2 | CR | 0154 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of the conditions for Multi-RTT and Dl-AoD assistance data elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_pos-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The present conditions for some assistance data IE fields used by Multi-RTT and DL-AoD are incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specify the present condition for sub-test 20 and sub-test 21 in some IE fields. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The assistance data elements used for protocol conformance test cases for NR positioning in clause 9 are wrong. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.1.6, 8.4.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

8.4	Default LPP message and information elements contents

The default values of LPP messages and information elements used, unless indicated otherwise in specific clauses of this specification are as defined in clause 5.4 with the following exceptions:

-	LPP PROVIDE ASSISTANCE DATA

Table 8.4-1: ProvideAssistanceData

| Derivation Path: 37.355 clause 6.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| LPP-Message ::= SEQUENCE { |  |  |  |
| transactionID SEQUENCE { | Dependent on test case. |  |  |
| initiator |  |  |  |
| transactionNumber |  |  |  |
| } |  |  |  |
| endTransaction | TRUE |  |  |
| sequenceNumber | Not present |  |  |
| acknowledgement | Not present |  |  |
| lpp-MessageBody CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| provideAssistanceData-r9 SEQUENCE { |  |  |  |
| commonIEsProvideAssistanceData | Not present |  |  |
| a-gnss-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 7 and 15 only; and as defined in Table 5.4.1.1-1. |
| gnss-CommonAssistData SEQUENCE { |  |  |  |
| gnss-ReferenceTime | As defined in 37.571-5 [12] |  |  |
| gnss-ReferenceLocation | As defined in 37.571-5 [12] |  |  |
| gnss-IonosphericModel | As defined in 37.571-5 [12] |  |  |
| gnss-EarthOrientationParameters | Not present |  |  |
| } |  |  |  |
| gnss-GenericAssistData(SIZE(1..4))OF{ | SIZE is dependent on the number of GNSSs supported by the UE. If one GNSS supported by the UE, SIZE = 1 If two GNSSs supported by the UE, SIZE = 2 If three GNSSs supported by the UE, SIZE = 3 If four GNSSs supported by the UE, SIZE = 4 |  |  |
| gnss-ID | For each GNSS supported by the UE. |  |  |
| sbas-ID | Not present |  |  |
| gnss-TimeModels | As defined in 37.571-5 [12] |  |  |
| gnss-DifferentialCorrections | Not present |  |  |
| gnss-NavigationModel | As defined in 37.571-5 [12] |  |  |
| gnss-RealTimeIntegrity | Not present |  |  |
| gnss-DataBitAssistance | Not present |  |  |
| gnss-AcquisitionAssistance | As defined in 37.571-5 [12] |  |  |
| gnss-Almanac | As defined in 37.571-5 [12] |  |  |
| gnss-UTC-Model | As defined in 37.571-5 [12] |  |  |
| gnss-AuxiliaryInformation | As defined in 37.571-5 [12] |  |  |
| } |  |  |  |
| gnss-Error | Not present |  |  |
| } |  |  |  |
| otdoa-ProvideAssistanceData SEQUENCE { |  |  | Sub-tests 5 and 7 only |
| otdoa-ReferenceCellInfo | As defined in Table 8.4.1.2-1 |  |  |
| otdoa-NeighbourCellInfo | As defined in Table 8.4.1.2-2 |  |  |
| otdoa-Error | Not present |  |  |
| } |  |  |  |
| epdu-Provide-AssistanceData | Not present |  |  |
| sensor-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 18 only as defined in clause 5.4.1.5 |
| sensor-AssistanceDataList-r14 | As defined in Table 5.4.1.5-2 |  |  |
| sensor-Error-r14 | Not present |  |  |
| } |  |  |  |
| tbs-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 16 only as defined in clause5.4.1.3 |
| tbs-AssistanceDataList-r14 SEQUENCE { |  |  |  |
| mbs-AssistanceDataList-r14 SEQUENCE (SIZE(1..n)) OF SEQUENCE{ |  |  |  |
| mbs-AlmanacAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| mbs-AcquisitonAssistance-r14 | As defined in Table 5.4.1.3-2 |  |  |
| } |  |  |  |
| } |  |  |  |
| tbs-Error-r14 | Not present |  |  |
| } |  |  |  |
| wlan-ProvideAssistanceData-r14 SEQUENCE { |  | Rel-14 onwards | Sub-test 17 only as defined in clause 5.4.1.4 |
| wlan-DataSet-r14 | As defined in Table 5.4.1.4-2 |  |  |
| wlan-Error-r14 | Not present |  |  |
| } |  |  |  |
| nr-Multi-RTT-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 19 only |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-Multi-RTT-Error-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-AoD-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 20 only |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-PositionCalculationAssistance-r16 | Present or not present dependent on pc_UEB_DL_AoD. As defined in Table 8.4.1.7-1 |  |  |
| nr-DL-AoD-Error-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-TDOA-ProvideAssistanceData-r16 SEQUENCE { |  | Rel-16 onwards | Sub-test 21 only |
| nr-DL-PRS-AssistanceData-r16 | As defined in Table 8.4.1.6-1 |  |  |
| nr-SelectedDL-PRS-IndexList-r16 | Not present |  |  |
| nr-PositionCalculationAssistance-r16 | Present or not present dependent on pc_UEB_DL_TDOA. As defined in Table 8.4.1.7-1 |  |  |
| nr-DL-TDOA-Error-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# < Unchanged sections omitted >

8.4.1.6	Multi-RTT Assistance Data Elements

This clause defines the Multi-RTT assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages.

-	NR DL-PRS ASSISTANCE DATA

Table 8.4.1.6-1: NR-DL-PRS-AssistanceData

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-AssistanceData-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ReferenceInfo-r16 SEQUENCE { |  |  |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-DL-PRS-ResourceID-List-r16 | Not present |  |  |
| nr-DL-PRS-ResourceSetID-r16 | Not present |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataList-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-DL-PRS-AssistanceDataPerFreq-r16 { | 2 entries |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 1 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 1 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 3 entries |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | 1 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 1 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 1 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 1 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 1 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[2] SEQUENCE { |  | entry 2 | In case of sub-test 20 UE-based DL-AoD or sub-test 21 DL-TDOA method supported by the UE as defined in clause 8.2.10 and clause 8.2.11. |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | 2 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 2 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 2 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 2 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 2 |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[3] SEQUENCE { |  | entry 3 | In case of sub-test 21 UE-based DL-TDOA method supported by the UE as defined in clause 8.2.11 |
| dl-PRS-ID-r16 | 3 |  |  |
| nr-PhysCellID-r16 | 4 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 4 |  |  |
| mnc-r15 | As defined TS 38.508-1 table 4.4.2-3 for NR Cell 4 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 4 Identifier defined in TS 38.508-1 Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 4 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-DL-PRS-AssistanceDataPerFreq-r16[2] SEQUENCE { |  | entry 2 | In case of sub-test 21 UE-based DL-TDOA method supported by the UE as defined in clause 8.2.11 |
| nr-DL-PRS-PositioningFrequencyLayer-r16 SEQUENCE { |  |  |  |
| dl-PRS-SubcarrierSpacing-r16 | SubcarrierSpacing | 38.508-1 [30] Table 4.6.3-188 |  |
| dl-PRS-ResourceBandwidth-r16 | 1 | 24 PRBs |  |
| dl-PRS-StartPRB-r16 | same value as ‘offsetToCarrier’ as defined for the DL frequency of NR Cell 3 |  |  |
| dl-PRS-PointA-r16 | absoluteFrequencyPointA as defined for the DL frequency of the NR Cell 3 |  |  |
| dl-PRS-CombSizeN-r16 | n2 |  |  |
| dl-PRS-CyclicPrefix-r16 | normal |  |  |
| } |  |  |  |
| nr-DL-PRS-AssistanceDataPerFreq-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF NR-DL-PRS-AssistanceDataPerTRP-r16{ | 1 entry |  |  |
| NR-DL-PRS-AssistanceDataPerTRP-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | 3 |  |  |
| nr-CellGlobalID-r16 SEQUENCE { |  |  |  |
| mcc-r15 | As defined in TS 38.508-1 [30] Table 4.4.2-3 for NR Cell 3 |  |  |
| mnc-r15 | As defined in TS 38.508-1 [30] Table 4.4.2-3 for NR Cell 3 |  |  |
| nr-cellidentity-r15 | Set to NR Cell 3 Identifier defined in TS 38.508-1 [30] Table 4.4.2-2 |  |  |
| } |  |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 3 frequency |  |  |
| nr-DL-PRS-SFN0-Offset-r16 SEQUENCE { |  |  |  |
| sfn-Offset-r16 | 0 |  |  |
| integerSubframeOffset-r16 | 0 |  |  |
| } |  |  |  |
| nr-DL-PRS-ExpectedRSTD-r16 | 0 |  |  |
| nr-DL-PRS-ExpectedRSTD-Uncertainty-r16 | 4 | About 1 s |  |
| nr-DL-PRS-Info-r16 | NR-DL-PRS-Info-r16 as specified in Table 8.4.1.6-3 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-SSB-Config-r16 SEQUENCE (SIZE (1..nrMaxTRPs-r16)) OF NR-SSB-Config-r16 { | 2 entries |  | In case of sub-test 21 UE-based DL-TDOA method supported by the UE as defined in clause 8.2.11 |
| NR-SSB-Config-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-PhysCellID-r16 | 3 |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 3 frequency | As defined in 38.508 -1 [30] subclause 6.2.3. |  |
| ss-PBCH-BlockPower-r16 | 0 |  |  |
| halfFrameIndex-r16 | 0 |  |  |
| ssb-periodicity-r16 | ms20 |  |  |
| ssb-PositionsInBurst-r16 | Not present |  |  |
| ssb-SubcarrierSpacing-r16 | Subcarrier spacing of SSB for NR Cell 3 |  |  |
| sfn-SSB-Offset-r16 | 0 |  |  |
| } |  |  |  |
| NR-SSB-Config-r16[2] SEQUENCE { |  | entry 2 |  |
| nr-PhysCellID-r16 | 4 |  |  |
| nr-ARFCN-r16 | ARFCN-ValueNR for NR Cell 4 frequency | As defined in 38.508 -1 [30] subclause 6.2.3. |  |
| ss-PBCH-BlockPower-r16 | 0 |  |  |
| halfFrameIndex-r16 | 0 |  |  |
| ssb-periodicity-r16 | ms20 |  |  |
| ssb-PositionsInBurst-r16 | Not presnet |  |  |
| ssb-SubcarrierSpacing-r16 | Subcarrier spacing of SSB for NR Cell 4 |  |  |
| sfn-SSB-Offset-r16 | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.4.1.6-2: Void

-	NR-DL-PRS-Info

Table 8.4.1.6-3: NR-DL-PRS-Info

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-DL-PRS-Info-r16 ::= SEQUENCE { |  |  |  |
| nr-DL-PRS-ResourceSetList-r16 SEQUENCE (SIZE (1..nrMaxSetsPerTrp-r16)) OF NR-DL-PRS-ResourceSet-r16 { | 1 entry |  |  |
| NR-DL-PRS-ResourceSet-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceSetID-r16 | 0 |  |  |
| dl-PRS-Periodicity-and-ResourceSetSlotOffset-r16 | The periodicity is 160ms and the resource set slot offset is 11 ms for any SCS configuration |  |  |
| dl-PRS-ResourceRepetitionFactor-r16 | Not present |  |  |
| dl-PRS-ResourceTimeGap-r16 | Not present |  |  |
| dl-PRS-NumSymbols-r16 | n2 |  |  |
| dl-PRS-MutingOption1-r16 | Not present |  |  |
| dl-PRS-MutingOption2-r16 | Not present |  |  |
| dl-PRS-ResourcePower-r16 | 0 |  |  |
| dl-PRS-ResourceList-r16 SEQUENCE (SIZE (1..nrMaxResourcesPerSet-r16)) OF NR-DL-PRS-Resource-r16 { | 1 entry |  |  |
| NR-DL-PRS-Resource-r16[1] SEQUENCE { |  | entry 1 |  |
| nr-DL-PRS-ResourceID-r16 | 0 |  |  |
| dl-PRS-SequenceID-r16 | 0 |  |  |
| dl-PRS-CombSizeN-AndReOffset-r16 CHOICE { |  |  |  |
| n2-r16 | 0 |  |  |
| } |  |  |  |
| dl-PRS-ResourceSlotOffset-r16 | 0 |  |  |
| dl-PRS-ResourceSymbolOffset-r16 | 0 |  |  |
| dl-PRS-QCL-Info-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

# < Unchanged sections omitted >

8.4.1.7	DL-AoD Assistance Data Elements

This clause defines the DL-AoD assistance data elements which shall be provided to the UE in the tests in LPP Provide Assistance Data messages in addition to those defined in Table 8.4.1.6-1 and Table 8.4.1.6-2 and Table 8.4.1.6-3.

-	NR-PositionCalculationAssistance

Table 8.4.1.7-1: NR-PositionCalculationAssistance

| Derivation Path: 37.355 clause 6.4.3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| NR-PositionCalculationAssistance-r16 ::= SEQUENCE { |  |  |  |
| nr-TRP-LocationInfo-r16 SEQUENCE (SIZE (1..nrMaxFreqLayers-r16)) OF NR-TRP-LocationInfoPerFreqLayer-r16 { | 2 entries |  |  |
| NR-TRP-LocationInfoPerFreqLayer-r16[1] SEQUENCE { |  | entry1 |  |
| referencePoint-r16 |  |  |  |
| trp-LocationInfoList-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF TRP-LocationInfoElement-r16 { | 3 entries |  |  |
| TRP-LocationInfoElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 0 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | Not present | Same as the reference point location |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| TRP-LocationInfoElement-r16[2] SEQUENCE { |  | entry 2 | In case of sub-test 20 UE-based DL-AoD or sub-test 21 DL-TDOA method supported by the UE as defined in clause 8.2.10 and clause 8.2.11. |
| dl-PRS-ID-r16 | 1 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | trp-Location-r16 for NR cell 2 as defined in 37.571-5 [12] |  |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| TRP-LocationInfoElement-r16[3] SEQUENCE { |  | entry 3 | In case of sub-test 21 UE-based DL-TDOA method supported by the UE as defined in clause 8.2.11 |
| dl-PRS-ID-r16 | 3 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | trp-Location-r16 for NR cell 4 as defined in TS 37.571-5 [12] |  |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| NR-TRP-LocationInfoPerFreqLayer-r16[2] SEQUENCE { |  | entry 2 |  |
| referencePoint-r16 | Not present | Same as in the previous entry of the NR-TRP-LocationInfoPerFreqLayer list. |  |
| trp-LocationInfoList-r16 SEQUENCE (SIZE (1..nrMaxTRPsPerFreq-r16)) OF TRP-LocationInfoElement-r16 { | 1 entry |  |  |
| TRP-LocationInfoElement-r16[1] SEQUENCE { |  | entry 1 |  |
| dl-PRS-ID-r16 | 2 |  |  |
| nr-PhysCellID-r16 | Not present |  |  |
| nr-CellGlobalID-r16 | Not present |  |  |
| nr-ARFCN-r16 | Not present |  |  |
| associated-DL-PRS-ID-r16 | Not present |  |  |
| trp-Location-r16 | trp-Location-r16 for NR cell 3 as defined in TS 37.571-5 [12] |  |  |
| trp-DL-PRS-ResourceSets-r16 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| nr-DL-PRS-BeamInfo-r16 | Not present |  |  |
| nr-RTD-Info-r16 | Not present |  |  |
| } |  |  |  |