3GPP TSG-RAN5 Meeting #93-e	R5-216780

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2506 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to 5GMM RACS TC 9.1.9.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ANRITSU LTD |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | [TS 23.003, cl.29.2] The UE radio capability ID is an identifier used to represent a set of UE radio capabilities, defined in 3GPP TS 23.501 [119] and in 3GPP TS 23.401 [72], composed as shown in figure 29.2-1. Figure 29.2-1: Structure of UE radio capability ID The UE radio capability ID is composed of the following elements (each element shall consist of hexadecimal digits only): 1) Type Field (TF): identifies the type of UE radio capability ID. The following values are defined: - 0: manufacturer-assigned UE radio capability ID; - 1: network-assigned UE radio capability ID; and - 2 to F: spare values for future use. 2) The Vendor ID is an identifier of UE manufacturer. This is defined by a value of Private Enterprise Number issued by Internet Assigned Numbers Authority (IANA) in its capacity as the private enterprise number administrator, as maintained at https://www.iana.org/assignments/enterprise-numbers/enterprise-numbers. Its length is 8 hexadecimal digits. This field is present only if the Type Field is set to 0; NOTE: The private enterprise number issued by IANA is a decimal number in the range between 0 and 4294967295 that needs to be converted to a fixed length 8 digit hexadecimal number when used within the UE Radio Capability ID. E.g. 32473 is converted to 00007ED9. 3) The Version ID is the current Version ID configured in the UCMF. This field is present only if the Type Field is set to 1. Its length is 2 hexadecimal digits. 4) Radio Configuration Identifier (RCI): identifies the UE radio configuration. Its length is 11 hexadecimal digits. If it is Type Field set to ‘1’, then, VersionID shall be included as highlighted above. Currently, UE-Radio-Capability ID does not include VersionID. - GMM Capabilities in REGISTRATION REQUEST are cheked in the preamble which can be avoided since these IEs are checked in the test body. - Since UE radio capability ID needs to be provided in the preamble, state 0N-B is required. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - VersionID set to 00 - Removed message specific contents(Table 9.1.9.1.3.3-1) for preamble - Preamble updated with 0N-B |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Conformant UE will fail the test case |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.9.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | R5s211170 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start Of change>

#### 9.1.9.1	RACS / Network assigned UE radio capability ID

9.1.9.1.1	Test Purpose (TP)

(1)

with { UE is switched on and the UE has an applicable UE radio capability ID for the current UE radio configuration for the selected network }

ensure that {

when { the UE performs an initial registration procedure }

then { the UE includes the UE radio capability ID in the REGISTRATION REQUEST message }

}

(2)

with { UE in state 5GMM-REGISTERED and 5GMM-IDLE mode and having assigned a new UE Radio Capability ID in the Registration Accept message }

ensure that {

when { UE enters a new tracking area that is not in the list of tracking areas that the UE previously registered in the AMF }

then { UE sends REGISTRATION REQUEST message including the new UE radio capability ID }

}

9.1.9.1.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.501, clauses 4.16, 5.4.4.1, 5.4.4.2, 5.4.4.3, 5.4.4.4, 5.5.1.3.2

[TS 24.501, clause 4.16]

UE radio capability signalling optimisation (RACS) is a feature that is optional at both the UE and the network and which aims to optimise the transmission of UE radio capability over the radio interface (see 3GPP TS 23.501 [8]). RACS works by assigning an identifier to represent a set of UE radio capabilities. This identifier is called the UE radio capability ID. A UE radio capability ID can be either manufacturer-assigned or network-assigned. The UE radio capability ID is an alternative to the signalling of the radio capabilities container over the radio interface.

In this release of the specification, RACS is applicable to neither NB-N1 mode nor non-3GPP access.

If the UE supports RACS:

a)	the UE shall indicate support for RACS by setting the RACS bit to "RACS supported" in the 5GMM capability IE of the REGISTRATION REQUEST message;

b)	if the UE performs a registration procedure for initial registration and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID in the UE radio capability ID IE as a non-cleartext IE in the REGISTRATION REQUEST message. If both a network-assigned UE radio capability ID and a manufacturer-assigned UE Radio Capability ID are applicable, the UE shall include the network-assigned UE radio capability ID in the REGISTRATION REQUEST message;

…

g)	if the UE performs a registration procedure for mobility and periodic registration update due to entering a tracking area that is not in the list of tracking areas that the UE previously registered in the AMF and the UE has an applicable UE radio capability ID for the current UE radio configuration in the selected network, the UE shall include the UE radio capability ID in the UE radio capability ID IE as a non-cleartext IE in the REGISTRATION REQUEST message. If both a network-assigned UE radio capability ID and a manufacturer-assigned UE Radio Capability ID are applicable, the UE shall include the network-assigned UE radio capability ID in the REGISTRATION REQUEST message.

If the network supports RACS:

a)	the network may assign a network-assigned UE radio capability ID to a UE which supports RACS by including a UE radio capability ID IE in the REGISTRATION ACCEPT message or in the CONFIGURATION UPDATE COMMAND message;

[TS 24.501, clause 5.4.4.1]

The purpose of this procedure is to:

a)	allow the AMF to update the UE configuration for access and mobility management-related parameters decided and provided by the AMF by providing new parameter information within the command; or

b)	request the UE to perform a registration procedure for mobility and periodic registration update towards the network to update access and mobility management-related parameters decided and provided by the AMF (see subclause 5.5.1.3).

…

9.1.9.1.3	Test description

9.1.9.1.3.1	Pre-test conditions

System Simulator:

-	NGC Cell A configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [4] belongs to Home PLMN and set as serving cell;

-	NGC Cell B configured according to Table 6.3.2.2-1 and Table 6.3.2.2-3 in 38.508-1 [4] and set as Non-Suitable “Off” cell.

-	System information combination NR-2 as defined in TS 38.508[4] clause 4.4.3.1.2 is used.

UE:

None

Preamble:

-	The UE is registered on NGC Cell A using the procedure described in TS 38.508-1 [4] clause 4.5.2.2 and the REGISTRATION ACCEPT message includes the UE radio capability ID 1.

-	The UE is in state Switched OFF (State 0N-B) as per TS 38.508-1 [4] Table 4.4A.2-0.

9.1.9.1.3.2	Test procedure sequence

Table 9.1.9.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2 | Check: Does the UE transmit REGISTRATION REQUEST message including the UE radio capability ID 1? | --> | REGISTRATION REQUEST | 1 | P |
| 3-11 | Steps 5-13 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 12 | SS transmits a REGISTRATION ACCEPT message with a new UE radio capability ID 2. | <-- | REGISTRATION ACCEPT | - | - |
| 13-18 | Steps 15-20a1 of Table 4.5.2.2-2 of the generic procedure in TS 38.508-1 [4] are performed. | - | - | - | - |
| 19 | The SS configures NGC Cell A as the "Suitable Neighbour intra-frequency cell" and NGC Cell B as the "Serving cell". | - | - | - | - |
| 20 | Check: Does the UE perform on NGC Cell B the Registration procedure for mobility registration update by executing the Test procedure to check that UE is camped on a new cell belonging to a new TA as specified in TS 38.508-1 [4] subclause 4.9.5 including the new UE radio capability ID 2 in the REGISTRATION REQUEST message? | - | - | 2 | P |

9.1.9.1.3.3	Specific message contents

Table 9.1.9.1.3.3-1: Void

Table 9.1.9.1.3.3-2: REGISTRATION ACCEPT (Preamble)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘100000000001’H | Type Field (TF): 1 Radio Configuration Identifier (RCI): 000000000001 |  |

Table 9.1.9.1.3.3-3: REGISTRATION REQUEST (Step 2, Table 9.1.9.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '0000 0001'B | Initial registration |  |
| 5GMM capability |  |  |  |
| RACS | ‘1’B | RACS supported |  |
| UE radio capability ID | ‘10000000000001’H | Type Field (TF): 1 Radio Configuration Identifier (RCI): 00000000001 |  |

Table 9.1.9.1.3.3-4: REGISTRATION ACCEPT (Step 12, Table 9.1.9.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UE radio capability ID | ‘10000000000002’H | Type Field (TF): 1 Radio Configuration Identifier (RCI): 000000000002 |  |

Table 9.1.9.1.3.3-5: REGISTRATION REQUEST (Step 20, Table 9.1.9.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.1-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| 5GS registration type | '00000010'B | mobility registration updating |  |
| 5GMM capability |  |  |  |
| RACS | ‘1’B | RACS supported |  |
| UE radio capability ID | ‘10000000000002’H | Type Field (TF): 1 Radio Configuration Identifier (RCI): 000000000002 |  |

<End of Change>