3GPP TSG RAN Meeting #93-e	                   R5-216790

Electronic Meeting, 8th – 19th November 2021

Agenda item:	5.3.1.4.1

Source:	Apple, Keysight and Ericsson

Title:     Discussion on A-MPR values for NS_27

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

The purpose of this discussion paper is to address several issues observed for AMPR testing for NS_27 (band n48):

- Test parameters to set proper test frequencies are undefined.

- In 38.521-1 Table 6.2.3.4.1-13 (Test Configuration table for NS_27), for several Test IDs listed, Fc values don’t have ARFCN assignment, as test frequencies are not part of the frequency channel raster for band n48.  Such ambiguity may lead to choose a Fc which is not in the same range of AMPR core requirement values defined in Table 6.2.3.3.16-1. Due to lack of clarification of current tables for multiple test points, test case 6.2.3 may unfairly fail.

# 2	Discussion

Minimum conformance requirements for AMPR for NS_27 values are defined as follows:

Table 6.2.3.3.16-1: A-MPR for NS_27 Channel Bandwidth, MHz; Carrier Centre Frequency, Fc, MHz; Region A; Region B. RBstart*12*SCS; RBend*12*SCS; LCRB*12*SCS; A-MPR; LCRB*12*SCS; A-MPR. 15 MHz; 3557.5 ≤ FC < 3562.5; <1.8 MHz; A3; ≥10.8 MHz; A3. 3687.5 < FC ≤ 3692.5; >11.52 MHz. 15 MHz; 3562.5 ≤ FC < 3567.5; ≤1.08 MHz; <1.44 MHz; A4; ≥11.52 MHz; [2]. 3682.5 < FC ≤ 3687.5; ≥13.22 MHz. 20 MHz; 3560 ≤ FC < 3570; <3.6 MHz; A5; ≥10.8 MHz; A5. 3680 < FC ≤ 3690; >12.96 MHz. 20 MHz; 3570 ≤ FC < 3580; ≤2.16 MHz; <1.44 MHz; A6; ≥14.4 MHz; [2]. 3670 < FC ≤ 3680; ≥16.92. 40 MHz; 3570 ≤ FC < 3600; <11.34 MHz; A7. ≥11.34 MHz, ≤31.0 MHz; ≥18 MHz; A2. <18 MHz; A1. >31.0 MHz; <1.8 MHz; A7. 3650 < FC ≤ 3680; >24.48 MHz; A7. ≤24.48 MHz, ≥6.48 MHz; ≥18 MHz; A2. <18 MHz; A1. <6.48 MHz; <1.8 MHz; A7. 40 MHz; 3600 ≤ FC ≤ 3650; ≤6.12 MHz; <1.44 MHz; A8; >[20] MHz; [4.5]. ≥ 32.76. NOTE 1: Void NOTE 2: Void. Table 6.2.3.3.16-2: A-MPR for modulation and waveform type Modulation/Waveform; A1; A2; A3; A4; A5; A6; A7; A8. Outer; Outer; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner; Outer/Inner. DFT-s-OFDM; PI/2 BPSK; [4.5]; [6]; 4; 4; 4; 4; 10.5; 4. QPSK; [4.5]; [6]; 4; 4; 4; 4; 10.5; 4. 16 QAM; [4.5]; [6]; 5; 4; 5; 4; 11; 4. 64 QAM; [4.5]; [6]; 5; 4; 5; 4; 11; 4. 256 QAM; [6]; 11. CP-OFDM; QPSK; [5.5]; [7]; 6; 4; 6; 4; 11.5; 4. 16 QAM; [5.5]; [7]; 6; 4; 6; 4; 11.5; 4. 64 QAM; [5.5]; [7]; 6; 4; 6; 4; 11.5; 4. 256 QAM; [7]; 11.5. NOTE 1: The backoff applied is max (MPR, A-MPR) where MPR is defined in Table 6.2.2-1 NOTE 2: Outer and inner allocations are defined in clause 6.2.2.

## 2.1 Issue 1: Test parameters to set proper test frequencies are undefined

AMPR test case 6.2.3 of TS 38.521-1 ([1]) goes through total 252 different test points (test IDs) for band n48 (NS-27) (refer to Table 6.2.3.4.1-13).

According to 38.905 ([2], [3]), test frequencies for this test and NS value are defined different from Low, Mid and High channels defined in 38.508-1 section 4.3.1.1.1.48:

Test frequencies are defined instead as [3]:

Hence test parameters related to test frequencies for AMPR testing for NS_27 should be added as part of the initial conditions.

Proposal 1: Add test parameters related to test frequencies for AMPR testing for NS_27 as part of the initial conditions.

## 2.2 Issue 2: Test frequencies not fitting channel raster

As described in section 2.1, test frequencies are defined as lowest and highest for the AMPR frequency ranges defined for each channel bandwidth. However, those frequency range edges might not fit the frequency band channel raster.

In TS 38.521-1 Table 6.2.3.4.1-13, Test Configuration table for NS_27 for several Test IDs listed, Fc (e.g. 3557.5 MHz, 3560 MHz, 3650 MHz, 3680 MHz, 3687.5 MHz, 3692.5 MHz) doesn’t have ARFCN assignment, as they don’t fit the frequency band channel raster.

Proposal 2: Test frequencies shall be adjusted to fit channel raster.

In the following table, an analysis of Test IDs 28, 72, 156, 184 and 252 is provided as an example to show the impact on test results depending on the shift direction used to fit test frequency to the channel raster: these test IDs will fail if we pick Fc 3650.01 MHz and 3680.01 MHz instead of Fc= 3649.98 MHz and Fc= 3679.98MHz.

| Test_ID | Current A-MPR (dB) | Modulation/Fc/SCS | Expected A-MPR based on minimum conformance requirements (dB) |
| --- | --- | --- | --- |
| 28 | 4.5 | BW=40MHz, SCS= 30kHz , RB= 100, Fc= 3650.01MHz | 10.5 (A7 DFT-s-OFDM BPSK, Outer_Full) |
| 28 | 4.5 | BW=40MHz, SCS= 30kHz , RB= 100, Fc= 3649.98MHz | 4.5 (4.5, DFT-s-OFDM DPSK, Outer_Full) |
| 72 | 2 | BW=20MHz, SCS= 30kHz , RB= 50, Fc= 3680.01MHz | 5 (A5 DFT-s-OFDM QAM_16, Outer_Full) |
| 72 | 2 | BW=20MHz, SCS= 30kHz , RB= 50, Fc= 3679.98MHz | 2 (2 DFT-s-OFDM QAM_16, Outer_Full) |
| 156 | 2 | BW=20MHz, SCS= 30kHz , RB= 51, Fc= 3680.01MHz | 6 (A5 CP-OFDM QPSK, Outer_Full) |
| 156 | 2 | BW=20MHz, SCS= 30kHz , RB= 51, Fc= 3679.98MHz | 2 (2 CP-OFDM QPSK, Outer_Full) |
| 184 | 2 | BW=20MHz, SCS= 30kHz , RB= 51, Fc= 3680.01MHz | 6 (A5 CP-OFDM QAM_16, Outer_Full) |
| 184 | 2 | BW=20MHz, SCS= 30kHz , RB= 51, Fc= 3679.98MHz | 2 (2 CP-OFDM QAM_16, Outer_Full) |
| 252 | 4.5 | BW=40MHz, SCS= 30kHz , RB= 106, Fc= 3650.01MHz | 11.5 (A7 CP-OFDM QAM_256, Outer_Full) |
| 252 | 4.5 | BW=40MHz, SCS= 30kHz , RB= 106, Fc= 3649.98MHz | 4.5 (4.5 CP-OFDM QAM_256, Outer_Full) |

Proposal 3: When adjusting test frequencies to make them fit to channel raster, shift direction shall be the one that makes such test frequency be within the frequency range such test ID was designed for (AMPR shown in UL RB allocation in test configuration table 6.2.3.4.1-13).

Once test frequencies are properly defined, test requirements need to be revisited to ensure the alignment with minimum conformance requirements.

### Channel raster as per TS 38.521-1

TS 38.521-1 Section 5.4.2.3 explains channel raster for operating bands as following

Based on the text above in case of band n48:

- For SCS=15kHz, ΔFRaster = 15kHz

- For SCS≥30kHz, ΔFRaster = 30kHz

When adjusting test frequencies to fit the channel raster, there are 2 options:

- Option 1: Maximize the coverage for the frequency ranges defined for AMPR, what implies that two different test frequencies might be required for each Test ID (one for SCS=15kHz and ΔFRaster = 15kHz and one for SCS≥30kHz and ΔFRaster = 30kHz).

- Option 2: Use common test frequencies for all SCSs and ΔFRaster (the ones for SCS≥30kHz and ΔFRaster = 30kHz) in order to ease test specification maintenance and readability considering that test coverage maximization with Option 1 is negligible.

Proposal 4: When adjusting test frequencies to make them fit to channel raster, use common test frequencies for all SCSs and ΔFRaster (the ones for SCS≥30kHz and ΔFRaster = 30kHz) in order to ease test specification maintenance and readability (Option 2).

# 3	Conclusions

The goal of this discussion paper is to address several issues observed for AMPR testing  for NS_27 (band n48). The following proposals are made:

Proposal 1: Add test parameters related to test frequencies for AMPR testing for NS_27 as part of the initial conditions.

Proposal 2: Test frequencies shall be adjusted to fit channel raster.

Proposal 3: When adjusting test frequencies to make them fit to channel raster, shift direction shall be the one that makes such test frequency be within the frequency range such test ID was designed for (AMPR shown in UL RB allocation in test configuration table 6.2.3.4.1-13).

Proposal 4: When adjusting test frequencies to make them fit to channel raster, Use common test frequencies for all SCSs and ΔFRaster (the ones for SCS≥30kHz and ΔFRaster = 30kHz) in order to ease test specification maintenance and readability (Option 2).

Sample CRs implementing proposed changes can be found in [4] and [5].

# 4	References

TS 38.521-1 v17.2.0, “NR, User equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone”

TR 38.905 v17.2.0 “NR; Derivation of test points for radio transmission and reception User Equipment (UE) conformance test cases”

R5-201765,” Test points analysis for NS_27 A_MPR FR1 test case”, Ericsson, TS 38.905 v16.3.0, May 2020, 3GPP TSG-RAN5 Meeting #87-e.

CR to 38.521-1: R5-216515, “Corrections for NS_27 in A-MPR FR1 test 6.2.3”, RAN5#93-e, Keysight, Ericsson, Apple

CR to 38.905: R5-216513, “Test point selection update for NS_27 A-MPR in FR1”, RAN5#93-e, Keysight, Ericsson, Apple