# Title:	Test tolerance methodology for Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion tests

# Source:	Huawei, Hisilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion tests.

The test cases in question, from TS 38.533, are:

- 6.1.1.3 Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

This document describes the process to derive the test tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Prototype test case in Annex A of TS 38.133

The test conditions are defined in the following extract from TS 38.133 v16.9.0.

#### A.6.1.1.3	Cell reselection to FR1 intra-frequency NR case for UE fulfilling low mobility relaxed measurement criterion

##### A.6.1.1.3.1	Test Purpose and Environment

This test is to verify the requirement for the intra frequency NR cell reselection requirements for UE fulfilling low mobility criterion specified in clause 4.2.2.9.2

##### A.6.1.1.3.2	Test Parameters

The test scenario comprises of 1 NR carrier and 2 cells as given in tables A.6.1.1.3.2-1, A.6.1.1.3.2-2 and A.6.1.1.3.2-3. The test consists of two successive time periods, with time duration of T1 and T2 respectively. Both cell 1 and cell 2 are already identified by the UE prior to the start of the test. Cell 1 and cell 2 belong to different tracking areas. Furthermore, UE has not registered with network for the tracking area containing cell 2.

Table A.6.1.1.3.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations. |  |

Table A.6.1.1.3.2-2: General test parameters for FR1 intra frequency NR cell re-selection test case for UE fulfilling low mobility criterion

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1, 2, 3 | Cell1 | The UE camps on cell 1 in the initial phase |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| T1 end condition | Active cell |  | 1, 2, 3 | Cell2 | The UE reselects to cell 2 during T1 period |
|  | Neighbour cells |  | 1, 2, 3 | Cell1 |  |
| Final condition | Active cell |  | 1, 2, 3 | Cell1 | The UE reselects to cell 1 during T2 period |
|  | Neighbour cells |  | 1, 2, 3 | Cell2 |  |
| RF Channel Number |  |  | 1, 2, 3 | 1 |  |
| Time offset between cells |  |  | 1 | 3 ms | Asynchronous cells |
|  |  |  | 2 | 3 s | Synchronous cells |
|  |  |  | 3 | 3 s | Synchronous cells |
| Access Barring Information |  | - | 1, 2, 3 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR1 |  |
|  |  |  | 2 | SSB.1 FR1 |  |
|  |  |  | 3 | SSB.2 FR1 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 2 | Configured in SIB2 of Cell 1 |
|  |  |  |  | SMTC pattern 6 | Configured in SIB2 of Cell 2 |
|  |  |  | 2 | SMTC pattern 1 |  |
|  |  |  | 3 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1, 2, 3 | 0.64 | The value shall be used for all cells in the test. |
| PRACH configuration index |  |  | 1, 2, 3 | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2 |
| rangeToBestCell |  |  | 1, 2, 3 | Not configured |  |
| T1 |  | s | 1, 2, 3 | 25 | T1 needs to be defined so that cell re-selection reaction time is taken into account. |
| T2 |  | s | 1, 2, 3 | 25 | T2 needs to be defined so that cell re-selection reaction time is taken into account. |

Table A.6.1.1.3.2-3: Cell specific test parameters for FR1 intra frequency NR cell re-selection test case in AWGN for UE fulfilling low mobility criterion

| Parameter | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1 | N/A |  | N/A |  |
|  |  | 2 | TDDConf.1.1 |  | TDDConf.1.1 |  |
|  |  | 3 | TDDConf.2.1 |  | TDDConf.2.1 |  |
| PDSCH RMC configuration |  | 1 | SR.1.1 FDD |  | N/A |  |
|  |  | 2 | SR.1.1 TDD |  |  |  |
|  |  | 3 | SR.2.1 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.1.1 FDD |  | CR.1.1 FDD |  |
|  |  | 2 | CR.1.1 TDD |  | CR.1.1 TDD |  |
|  |  | 3 | CR.2.1 TDD |  | CR.2.1 TDD |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |
|  |  | 2 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |
|  |  | 3 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |
| OCNG Pattern |  | 1, 2, 3 | OP.1 defined in A.3.2.1 |  | OP.1 defined in A.3.2.1 |  |
| Initial DL BWP configuration |  | 1, 2, 3 | DLBWP.0.1 |  | DLBWP.0.1 |  |
| Initial UL BWP configuration |  | 1, 2, 3 | ULBWP.0.1 |  | ULBWP.0.1 |  |
| RLM-RS |  | 1, 2, 3 | SSB |  | SSB |  |
| Qrxlevmin | dBm/SCS | 1, 2 | -140 |  | -140 |  |
|  |  | 3 | -137 |  | -137 |  |
| Pcompensation | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qhysts | dB | 1, 2, 3 | 0 |  | 0 |  |
| Qoffsets, n | dB | 1, 2, 3 | 0 |  | 0 |  |
| SSearchDeltaP | dB | 1, 2, 3 | 3 |  | 3 |  |
| TSearchDeltaP | s | 1, 2, 3 | 5 |  | 5 |  |
| Cell_selection_and_ reselection_quality_ measurement |  | 1, 2, 3 | SS-RSRP |  | SS-RSRP |  |
|  | dB | 1, 2, 3 | -3.11 | 2.79 | 2.79 | -3.11 |
| Note2 | dBm/SCS | 1 | -98 |  |  |  |
|  |  | 2 | -98 |  |  |  |
|  |  | 3 | -95 |  |  |  |
| Note2 | dBm/15 kHz | 1, 2, 3 | -98 |  |  |  |
|  | dB | 1, 2, 3 | 13 | 16 | 16 | 13 |
| SS-RSRP Note3 | dBm/SCS | 1 | -85 | -82 | -82 | -85 |
|  |  | 2 | -85 | -82 | -82 | -85 |
|  |  | 3 | -82 | -79 | -79 | -82 |
| Io | dBm/9.36 MHz | 1 | -52.21 | -52.21 | specified in Cell 1 columns- |  |
|  | dBm/9.36 MHz | 2 | -52.21 | -52.21 |  |  |
|  | dBm/38.16 MHz | 3 | -46.12 | -46.12 |  |  |
| Treselection | s | 1, 2, 3 | 0 | 0 | 0 | 0 |
| SintrasearchP | dB | 1, 2, 3 | 60 |  | 60 |  |
| Propagation Condition |  | 1, 2, 3 | AWGN |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

##### A.6.1.1.3.3	Test Requirements

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T1, to the moment when the UE camps on Cell 2, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on Cell 2.

The cell re-selection delay to an already detected cell shall be less than 17 s.

The cell reselection delay to an already detected cell is defined as the time from the beginning of time period T2, to the moment when the UE camps on cell 1, and starts to send preambles on the PRACH for sending the RRCSetupRequest message to perform a Tracking Area Update procedure on cell 1.

The cell re-selection delay to an already detected cell shall be less than 17 s.

The rate of correct cell reselections observed during repeated tests shall be at least 90%.

NOTE:	The cell re-selection delay to an already detected cell can be expressed as: Tevaluate,NR_Intra + TSI-NR,

Where:

Tevaluate,NR_Intra	See Table 4.2.2.9.2-1 in clause 4.2.2.9.2 for reselection to Cell 2 during T1 with UE fulfilling low mobility criterion. 15.36s.

TSI-NR	Maximum repetition period of relevant system info blocks that needs to be received by the UE to camp on a cell; 1280 ms is assumed in this test case.

This gives a total of 16.64 s, allow 17 s for the cell re-selection delay to an already detected cell for UE fulfilling low mobility criterion in the test case.

# 3. 	Discussion

The test case consists of 3 test configurations. Each test configuration has 2 time periods: T1 and T2. The distribution of power is identical for all 3 test configurations.

During T1 the nominal Ês/Iot value for NR Cell 1 and Cell 2 are -3.11dB and 2.79dB respectively. SSB_RP value for NR Cell 1 and Cell 2 are -85dBm/15kHz (-82dBm/30kHz) and -82dBm/15kHz (-79dBm/30kHz) respectively. During this time interval UE camps on Cell 1 and is expected to reselect Cell 2. So following conditions needs to be satisfied:

- Cell 2 shall be detectable;

- Cell 1 Srxlev < SintrasearchP to ensure UE performing intra-frequency measurement;

- SrxlevRef - Cell 1 Srxlev < SSearchDeltaP to ensure low-mobility criterion is satisifed;

- Rn - Rs > 3dB to ensure UE performing intra-frequency reselection;

During T2 the nominal Ês/Iot value for NR Cell 1 and Cell 2 are 2.79 dB and -3.11 dB respectively. SSB_RP value for NR Cell 1 and Cell 2 are -82dBm/15kHz (-79dBm/30kHz) and -85dBm/15kHz (-82dBm/30kHz) respectively. During this time interval UE camps on Cell 2 and is expected to reselect Cell 1. So following conditions needs to be satisfied:

- Cell 1 shall be detectable;

- Cell 2 Srxlev < SintrasearchP to ensure UE performing intra-frequency measurement;

- SrxlevRef - Cell 2 Srxlev < SSearchDeltaP to ensure low-mobility criterion is satisifed;

- Rn - Rs > 3dB to ensure UE performing intra-frequency reselection;

With nominal signal levels conditions above are easily met, but the effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

In the accompanying spreadsheet only configurations 1 and 3 are considered. For other configurations, critical test parameters are identical with test configuration 1 and 3, thus the same level uncertainties and method of deciding test tolerances can be used.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.6.1.1.3.1-1, Table A.6.1.1.3.1-2, Table A.6.1.1.3.1-3 in TS 38.133. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty which may affect the verdict of the test.

The key parameters are:

- Noc (in dBm): The noise EPRE on RF channel 1;

- Es1/Noc (in dB): The ratio of signal EPRE of NR Cell 1 over noise EPRE ;

- Es2/Noc (in dB): The ratio of signal EPRE of NR Cell 2 over noise EPRE ;

b) Derived parameters

Several derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

During T1 and T2 UE is expected to perform cell reselection. UE shall perform intra-frequency measurement on camped cell and intra-frequency neighbour cell and evaluate S-criterion on both Cells .So Srxlev for Cell 1 and Cell 2 are calculated.

To evaluate whether measurement restriction rule is satisfied we needs to compare Srxlev with SintrasearchP. So we also calculate:

(Srxlev - SintrasearchP)

To evaluate whether low-mobility criterion is satisfied we need to compare Srxlev with SrxlevRef, where SrxlevRef is the reference Srxlev specified in 38.304 cl.5.2.4.9.1. So we also calculate:

(SrxlevRef – Srxlev)

To determine whether rank-based cell reselection shall occur we also need to calculate:

(Rn - Rs)

Furthermore, SSB Es/Iot and SSB_RP of NR Cell 1 and Cell 2 are calculated, as these are side conditions for the measurement accuracy.

c) Uncertainties

The SS provides AWGN, 2 NR cells to the UE. We propose to control the following parameters:

- Average Noc absolute level uncertainty, Noc ±1.5 dB;

- Noc level uncertainty over SSB PRBs #RBJ-J+19, Noc ±1.5 dB.

- Es/Noc uncertainty for NR Cell 1, Ês1/Noc ±0.3 dB

- Es/Noc uncertainty for NR Cell 2, Ês2/Noc ±0.3 dB

This choice forms a minimum set, so the superposition principle can be applied if necessary.

The Test system uncertainties appear in section c) of the accompanying spreadsheet.

In addition, Cell 1 and Cell 2 shall be detectable to ensure intra-frequency cell-reselection requirements defined in TS 38.133 cl.4.2.2.3 apply.

#### 4.2.2.3	Measurements of intra-frequency NR cells

The UE shall be able to identify new intra-frequency cells and perform SS-RSRP and SS-RSRQ measurements of the identified intra-frequency cells without an explicit intra-frequency neighbour list containing physical layer cell identities.

The UE shall be able to evaluate whether a newly detectable intra-frequency cell meets the reselection criteria defined in TS38.304 [1] within Tdetect,NR_Intra when that Treselection= 0 . An intra frequency cell is considered to be detectable according to the conditions defined in Annex B.1.2 for a corresponding Band.

## B.1.2	Conditions for measurements on NR intra-frequency cells for cell re-selection

This clause defines the following conditions for NR intra-frequency measurements performed based on SSBs for cell re-selection: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.1.2-1 for FR1 NR cells.

The conditions are defined in Table B.1.2-2 for FR2 NR cells.

Table B.1.2-1: Conditions for intra-frequency cell re-selection in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A | -124 | -121 | -4 |
|  | NR_FDD_FR1_B | -123.5 | -120.5 |  |
|  | NR_TDD_FR1_C | -123 | -120 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -122.5 | -119.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -122 | -119 |  |
|  | NR_FDD_FR1_F | -121.5 | -118.5 |  |
|  | NR_FDD_FR1_G | -121 | -118 |  |
|  | NR_FDD_FR1_H | -120.5 | -117.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

In summary,

- SSB_RP: SSB_RP requirements for NR_FDD_FR1_H is the tightest. SSB_RP shall be no less than -120.5 dBm/15kHz (-117.5 dBm/30kHz);

- SSB Es/Iot: SSB Es/Iot shall be no less than -4dB

d) Controlled parameters critical to verdict

In many RRM test cases there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. In this test case there are two aspects to consider:

- The uncertainties in downlink signal: The uncertainties in downlink signal should be within the SSB_RP and SSB Es/Iot range to ensure that intra-frequency cell reselection requirements apply.

- The uncertainties in UE intra-frequency measurements: The R-value difference between Cells should be constrained to ensure rank-based cell-reselection can be correctly triggered.

The controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given in the “Requirement” column of section g) in the spreadsheet.

Analysis is performed for configuration 1 (BW 10MHz, SCS 15kHz, 52 RBs) and configuration 3 (BW 40MHz, SCS 30kHz, 106 RBs), the shaded cells, in the spreadsheet, show what changes with configuration.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x), and is usually in dB/dB. Denoting critical parameter y as a function of test equipment uncertainties x1, x2, ..., i.e. y=fx1,x2,⋯. Then sensitivity factors of xi on y can be expressed as:

sensitivity factors of xi on y=∂y∂xi

Then we have:

| controlled parameters | Key parameters |  |  |
| --- | --- | --- | --- |
|  | Noc | Ês1/Noc | Ês2/Noc |
| Es/Iot, Cell 1 | 0 | 1 | 0.975 for T1 0.952 for T2 |
| SSB_RP, Cell 1 | 1 | 1 | 0 |
| Es/Iot, Cell 2 | 0 | 0.952 for T1 0.975 for T2 | 1 |
| SSB_RP, Cell 2 | 1 | 0 | 1 |
| SrxlevRef, Cell 1 | 1 | 1 | 0 |
| SrxlevRef, Cell 2 | 1 | 0 | 1 |
| (Rn - Rs), Cell 1 | 0 | 1 | 1 |
| (Rn - Rs), Cell 2 | 0 | 1 | 1 |

Table 1. sensitivity factors of key parameters on controlled parameters

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for T1 and T2.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

e) Determine parameters to offset

During T1 and T2, All Es/Iot and SSB_RP side conditions are still satisfied after taking overall uncertainty into account. So both cells can be guaranteed to be detectable.

During T1 and T2. To ensure UE perform measurement on intra-frequency neighbour cells, following measurement restriction rule specified in 38.304 cl. 5.2.4.2 need to be satisfied for Cell 1 and Cell 2 respectively:

Srxlev ≤ SIntraSearchP

The values in this inequality are derived as follows:

- Srxlev = Qrxlevmeas – (Qrxlevmin + Qrxlevminoffset )– Pcompensation - Qoffsettemp according to 38.304 cl.5.2.3.2;

- Qrxlevmeas is the SS-RSRP value of camped cell. Its nominal value is -85 dBm/15kHz (-82 dBm/30kHz);

- Qrxlevmin is the minimum required RX level in the cell. It's -140dBm/15kHz (-137 dBm/30kHz);

- Qrxlevminoffset is Offset to the signalled Qrxlevmin, It's 0 dB according to 38.508-1 Table 4.6.1-28.

- Pcompensation is 0 dB according to Table A.6.1.1.3.2-3;

- Qoffsettemp is 0 dB according to 38.508-1 Table 4.6.1-28.

- SIntraSearchP is 60 dB according to 38.133 Table A.6.1.1.3.2-3.

Then the inequality therefore reduces and rearranges to:

Qrxlevmeas (-85 dBm) – Qrxlevmin (-140dBm) – SIntraSearchP (60 dB) = -5 dB ≤ 0

measurement restriction rule is satisfied under nominal conditions. The overall uncertainty for Srxlev, taking into account test system uncertainties, is ±1.53 dB as calculated in step d) of the accompanying spreadsheet. The inequality is therefore also satisfied when uncertainties are included.

During T1 and T2. low mobility criterion should be satisfied for Cell 1 and Cell 2 respectively to verify relaxed requirements for intra-frequency cell reselection. Following low mobility criterion evaluation specified in 38.304 cl. 5.2.4.9.1 need to be satisfied:

(SrxlevRef – Srxlev) < SSearchDeltaP

The values in this equation are derived as follows:

- Srxlev = 55dB as already discussed above.

- SrxlevRef is the reference Srxlev. According 38.304 cl.5.2.4.9.1 UE shall set SrxlevRef to the current Srxlev when it re-selects a new cell. So for Cell 1, SrxlevRef = 58dB (Srxlev value of Cell 1in T2 since UE re-selects to Cell 1 in T2), and for Cell 2, SrxlevRef = 58dB (Srxlev value of Cell 2 in T1 since UE re-selects to Cell 2 in T1).

- SSearchDeltaP is 6dB according to 38.133 Table A.6.1.1.3.2-3;

Then the inequality therefore reduces and rearranges to:

SrxlevRef (58dB) – Srxlev (55dB)– SSearchDeltaP (6 dB) < 0

Low mobility criterion is satisfied under nominal conditions. The overall uncertainty for Srxlev, taking into account test system uncertainties, is ±1.53 dB as calculated in step d) of the accompanying spreadsheet. The inequality is therefore also satisfied when uncertainties are included.

During T1 and T2. To apply intra-frequency cell reselection requirements, Ranking value of neighbour cell and camped cell shall satisfy following inequality for Cell 1 and Cell 2 respectively:

Rn – Rs ≥ 3dB

The values in this equation are derived as follows:

- Rs = Qmeas,s +Qhyst - Qoffsettemp according to 38.304 cl.5.2.4.6;

- Rn = Qmeas,n -Qoffset - Qoffsettemp according to 38.304 cl.5.2.4.6;

- Qmeas,s is the SS-RSRP value of camped cell. Its nominal value is -85 dBm/15kHz (-82 dBm/30kHz);

- Qmeas,n is the SS-RSRP value of neighbour cell. Its nominal value is -82 dBm/15kHz (-79 dBm/30kHz);

- Qhyst is 0 dB according to 38.133 Table A.6.1.1.3.2-3;

- Qoffset is 0dB according to 38.133 Table A.6.1.1.3.2-3;

- Qoffsettemp is 0 dB according to 38.508-1 Table 4.6.1-28.

Then the inequality therefore reduces and rearranges to:

Qmeas,n (58dB)- Qmeas,s (55dB) ≥ 3dB

Rank criterion is satisfied under nominal conditions. However, the overall uncertainty for Rn – Rs, taking into account test system uncertainties, is ±0.42 dB as calculated in step d) of the accompanying spreadsheet. So uncertainties would make the test verdict unreliable. Test Tolerances therefore need to be applied.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b).

For Cell 1 during T1, to ensure that a conformant UE is always in a condition where it must reselect, the Es/Noc of cell 1 is decreased by the (Rn – Rs) uncertainty. The actual offset required is 0.45 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

For Cell 2 during T2, to ensure that a conformant UE is always in a condition where it must reselect, the Es/Noc of cell 2 is decreased by the (Rn – Rs) uncertainty. The actual offset required is 0.45 dB. This value is found empirically by observing the minimum/maximum parameter values in step g).

To ensure low mobility criterion is satisfied during T1 and T2, SSearchDeltaP is increased by 3dB since SSearchDeltaP can only be chosen from {3,6,9,12,15} according to 38.331

g) Check controlled parameters Min/Max

Using a format like that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements. It is not necessary to calculate all parameters during each time interval T1 and T2, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

With the uncertainty values and test tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.1.1.3 also contains configurations with different subcarrier spacing and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- Qrxlevmin, section a)

- SSB_RP minimum requirement, section g) and associated Source/Reference

# 6. Treatment of other test cases

N/A