3GPP TSG-RAN5 Meeting #93-e	R5-216896

# Electronic Meeting, 08th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2064 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to IE Table 4.6.3-79 - MeasResults |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | SRVCC_NR_to_UMTS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, To add default configuration for IE Table 4.6.3-79 - MeasResults |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Default configuration for IE Table 4.6.3-79 - MeasResults is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

–	MeasResults

Table 4.6.3-79: MeasResults

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MeasResults ::= SEQUENCE { |  |  |  |
| measId | MeasId |  |  |
| measResultServingMOList | Not checked |  | SFTD_NEIGHBOUR or SFTD_PSCELL |
| measResultServingMOList SEQUENCE (SIZE (1.. maxNrofServingCells)) OF MeasResultServMO { | 1 entry |  |  |
| MeasResultServMO[1] SEQUENCE { |  | entry 1 |  |
| servCellId | ServCellIndex |  |  |
| measResultServingCell SEQUENCE { |  |  |  |
| physCellId | PhysCellId |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell SEQUENCE { |  |  |  |
| rsrp | Not checked |  |  |
| rsrq | Not checked |  |  |
| sinr | Not checked |  |  |
| } |  |  |  |
| resultsCSI-RS-Cell | Not present |  |  |
| } |  |  |  |
| rsIndexResults | Not present |  |  |
| } |  |  |  |
| cgi-Info | Not present |  |  |
| } |  |  |  |
| measResultBestNeighCell | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultNeighCells | Not present |  | A1, A2, SFTD_NEIGHBOUR or SFTD_PSCELL |
| measResultNeighCells CHOICE { |  |  |  |
| measResultListNR | Set according to specific message content |  | A3, A4, A5, A6 |
| measResultListEUTRA | Set according to specific message content |  | B1, B2 |
| measResultListUTRA-FDD-r16 | Set according to specific message content |  | B1_UTRA, B2_UTRA |
| } |  |  |  |
| measResultServFreqListEUTRA-SCG | Not present |  |  |
| measResultServFreqListNR-SCG | Not checked |  |  |
| measResultSFTD-EUTRA | Not present |  |  |
| measResultSFTD-NR | Not present |  |  |
| measResultSFTD-NR SEQUENCE { |  |  | SFTD_PSCELL |
| physCellId | PhysCellId of PSCell |  |  |
| sfn-OffsetResult | Not checked |  |  |
| frameBoundaryOffsetResult | Not checked |  |  |
| rsrp-Result | Not checked |  |  |
| } |  |  |  |
| measResultCellListSFTD-NR | Not present |  |  |
| measResultCellListSFTD-NR SEQUENCE (SIZE (1..maxCellSFTD)) OF MeasResultCellSFTD-NR { | 1 entry |  | SFTD_NEIGHBOUR |
| MeasResultCellSFTD-NR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | PhysCellId |  |  |
| sfn-OffsetResult | Not checked |  |  |
| frameBoundaryOffsetResult | Not checked |  |  |
| rsrp-Result | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| measResultForRSSI-r16 | Not present |  |  |
| locationInfo-r16 | Not present |  |  |
| ul-PDCP-DelayValueResultList-r16 | Not present |  |  |
| measResultsSL-r16 | Not present |  |  |
| measResultCLI-r16 | Not present |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| A1 | If event trigger Id in corresponding Measurement Configuration was Event A1 |
| A2 | If event trigger Id in corresponding Measurement Configuration was Event A2 |
| A3 | If event trigger Id in corresponding Measurement Configuration was Event A3 |
| A4 | If event trigger Id in corresponding Measurement Configuration was Event A4 |
| A5 | If event trigger Id in corresponding Measurement Configuration was Event A5 |
| A6 | If event trigger Id in corresponding Measurement Configuration was Event A6 |
| B1 | If event trigger Id in corresponding Measurement Configuration was Event B1 |
| B2 | If event trigger Id in corresponding Measurement Configuration was Event B2 |
| B1_UTRA | If event trigger Id in corresponding Measurement Configuration was Event B1-UTRA-FDD |
| B2_UTRA | If event trigger Id in corresponding Measurement Configuration was Event B2-UTRA-FDD |
| SFTD_NEIGHBOUR | Measurement reporting triggered by SFTD measurement on NR neighbour |
| SFTD_PSCELL | Measurement reporting triggered by SFTD measurement on NR PSCell |

<End of modified section 1>