3GPP TSG-RAN5 Meeting #93-e	R5-216912

# Electronic Meeting, 08th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1500 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to HST RRM TC 4.6.1.7 - intra-freq meas |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_HST-UEConTest |  |  |  |  |  |  | Date: |  | 2021-10-29 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, To complete NR HST TC 4.6.1.7 - EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16 |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1. Message contents are added. 2. Editor notes is removed. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | X |  | Test specifications |  |  | TS 38.508-1 CR 2068, 2069, 2070 TS 38.903 CR 0267 TS 38.533 CR 1503 |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.6.1.7	EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

4.6.1.7.1	Test purpose

This test is to verify that the UE makes correct reporting of an event for UE configured with highSpeedMeasFlag-r16. This test will partly verify the intra-frequency cell search requirements in TS 38.133 [6] clause 9.2.5.1 and 9.2.5.2.

4.6.1.7.2	Test applicability

This test applies to all types of E-UTRA UE release 16 and forward, supporting EN-DC FR1 and long DRX cycle.

4.6.1.7.3	Minimum conformance requirements

The minimum conformance requirements are defined in clause 4.6.1.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.6.1.7.

4.6.1.7.4	Test description

4.6.1.7.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.6.1.7.4.1-1.

Table 4.6.1.7.4.1-1: supported test configurations for EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Configuration | Description |
| --- | --- |
| 4.6.1.7-1 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4.6.1.7-2 | LTE FDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.6.1.7-3 | LTE FDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| 4.6.1.7-4 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 4.6.1.7-5 | LTE TDD, NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 4.6.1.7-6 | LTE TDD, NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note 1: The UE is only required to be tested in one of the supported test configurations. Note 2: Target NR Cell 3 has the same SCS, BW and duplex mode as NR serving Cell 2 |  |

Configure the test equipment and the DUT according to the parameters in Table 4.6.1.7.4.1-2 and Table 4.6.1.7.4.1-3.

Table 4.6.1.7.4.1-2: Initial conditions for EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.1-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.1.1.2-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | A.3.1.8.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | A.3.2.3.4 |  |
| Exceptions to connection diagram | N/A |  |  |

Table 4.6.1.7.4.1-3: General test parameters for EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| highSpeedMeasFlag-r16 |  | 1,2,3,4,5,6 | Present | To enable high speed measurement enhancements |
| Active cell |  | 1, 2, 3,4,5,6 | E-UTRAN Cell 1 and NR Cell 2 |  |
| Neighbour cell |  | 1, 2, 3,4,5,6 | NR Cell 3 | Cell to be identified. |
| RF Channel Number |  | 1, 2, 3,4,5,6 | 1: Cell 1 2: Cell 2 and Cell 3 |  |
| SSB configuration |  | 1,4 | SSB.1 FR1 |  |
|  |  | 2,5 | SSB.1 FR1 |  |
|  |  | 3,6 | SSB.2 FR1 |  |
| SMTC configuration |  | 1,4 | SMTC.2 |  |
|  |  | 2,5 | SMTC.1 |  |
|  |  | 3,6 | SMTC.1 |  |
| A3-Offset | dB | 1, 2, 3,4,5,6 | -4.5 |  |
| CP length |  | 1, 2, 3,4,5,6 | Normal |  |
| Hysteresis | dB | 1, 2, 3,4,5,6 | 0 |  |
| Time To Trigger | s | 1, 2, 3,4,5,6 | 0 |  |
| Filter coefficient |  | 1, 2, 3,4,5,6 | 0 | L3 filtering is not used |
| DRX |  | 1, 2, 3,4,5,6 | DRX.7. | 640ms DRX cycle |
| Time offset between PCell and PSCell |  | 1, 2, 3,4,5,6 | 3 s | Synchronous EN-DC |
| Time offset between serving and neighbour cells |  | 1,4 | 3 ms | Asynchronous cells. The timing of Cell 3 is 3ms later than the timing of Cell 2. |
|  |  | 2,5 | 3 s | Synchronous cells |
|  |  | 3,6 | 3 s | Synchronous cells |
| T1 | s | 1, 2, 3,4,5,6 | 5 |  |
| T2 | s | 1, 2, 3,4,5,6 | 6 |  |

1.	Message contents are defined in clause 4.6.1.7.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 and Cell 3 are NR FR1 cells in the same frequency. Cell 2 is the PSCell and Cell 3 is the neighbour NR Cell.

4.6.1.7.4.2	Test procedure

Same test procedure as in subclause 4.6.1.1.4.2 with Step 1 and 8 are replaced by following:

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. UE is configured with highSpeedMeasFlag-r16.

8.	UE shall transmit a MeasurementReport message triggered by Event A3 for Cell 3 on PCell (Cell 1). If the overall delays measured from the beginning of time period T2 is less than 5122 ms then the number of successful tests is increased by one. If the UE fails to report the event within the overall delays measured requirement then the number of failure tests is increased by one.

4.6.1.7.4.3	Message contents

In addition to the same message contents as clause 4.6.1.2.4.3, the following exceptions are also needed:

Table 4.6.1.7.4.3-1: SIB1 (Cell 2)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| servingCellConfigCommon |  | ServingCellConfigCommonSIB | Table 4.6.1.7.4.3-2 |  |
| } |  |  |  |  |

Table 4.6.1.7.4.3-2: ServingCellConfigCommonSIB (Table 4.6.1.7.4.3-1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 with condition HST |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| highSpeedConfig-r16 SEQUENCE { |  |  |  |
| highSpeedMeasFlag-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.6.1.7.4.3-3: CellGroupConfig (Step 1)

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-19 with condition EN-DC |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CellGroupConfig ::= SEQUENCE { |  |  |  |
| spCellConfig SEQUENCE { |  |  |  |
| servCellIndex | ServCellIndex for Cell 2 |  |  |
| reconfigurationWithSync SEQUENCE { |  |  |  |
| spCellConfigCommon | ServingCellConfigCommon | Table 4.6.1.7.4.3-4 |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 4.6.1.7.4.3-4: ServingCellConfigCommon (Table 4.6.1.7.4.3-3)

| Derivation Path: TS 38.508-1[14], Table 4.6.3-168 with condition HST |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| highSpeedConfig-r16 SEQUENCE { |  |  |  |
| highSpeedMeasFlag-r16 | true |  |  |
| } |  |  |  |
| } |  |  |  |

4.6.1.7.5	Test requirement

Table 4.6.1.7.4.1-2 and Table 4.6.1.7.5-1 defines the primary level settings including test tolerances for all tests.

Table 4.6.1.7.5-1: NR Cell specific test parameters for EN-DC FR1 event-triggered reporting without gap in DRX for UE configured with highSpeedMeasFlag-r16

| Parameter | Unit | Test configuration | Cell 2 |  | Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| TDD configuration |  | 1,4 | N/A |  | N/A |  |  |
|  |  | 2,5 | TDDConf.1.1 |  | TDDConf.1.1 |  |  |
|  |  | 3,6 | TDDConf.2.1 |  | TDDConf.2.1 |  |  |
| PDSCH RMC configuration |  | 1,4 | SR.1.1 FDD |  | N/A |  |  |
|  |  | 2,5 | SR.1.1 TDD |  |  |  |  |
|  |  | 3,6 | SR.2.1 TDD |  |  |  |  |
| RMSI CORESET RMC configuration |  | 1,4 | CR.1.1 FDD |  | CR.1.1 FDD |  |  |
|  |  | 2,5 | CR.1.1 TDD |  | CR.1.1 TDD |  |  |
|  |  | 3,6 | CR.2.1 TDD |  | CR.2.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1,4 | CCR.1.1 FDD |  | CCR.1.1 FDD |  |  |
|  |  | 2,5 | CCR.1.1 TDD |  | CCR.1.1 TDD |  |  |
|  |  | 3,6 | CCR.2.1 TDD |  | CCR.2.1 TDD |  |  |
| OCNG Patterns |  | 1, 2, 3,4,5,6 | OP.1 |  | OP.1 |  |  |
| TRS configuration |  | 1,4 | TRS.1.1 FDD |  | N/A |  |  |
|  |  | 2,5 | TRS.1.1 TDD |  | N/A |  |  |
|  |  | 3,6 | TRS.1.2 TDD |  | N/A |  |  |
| Initial BWP configuration |  | 1, 2, 3,4,5,6 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |  |
| Active DL BWP configuration |  | 1, 2, 3,4,5,6 | DLBWP.1.1 |  | DLBWP.1.1 |  |  |
| Active UL BWP configuration |  | 1, 2, 3,4,5,6 | ULBWP.1.1 |  | ULBWP.1.1 |  |  |
| RLM-RS |  | 1, 2, 3,4,5,6 | SSB |  | SSB |  |  |
| Note 2 | dBm/SCS | 1,4 | -98 |  |  |  |  |
|  |  | 2,5 | -98 |  |  |  |  |
|  |  | 3,6 | -95 |  |  |  |  |
| Note 2 | dBm/15 kHz | 1,4 | -98 |  |  |  |  |
|  |  | 2,5 |  |  |  |  |  |
|  |  | 3,6 |  |  |  |  |  |
|  | dB | 1,4 | 4 | -1.46 | -Infinity |  | -1.46 |
|  |  | 2,5 |  |  |  |  |  |
|  |  | 3,6 |  |  |  |  |  |
|  | dB | 1,4 | 4 | 4 | -Infinity |  | 4 |
|  |  | 2,5 |  |  |  |  |  |
|  |  | 3,6 |  |  |  |  |  |
| SS-RSRP Note 3 | dBm/SCS kHz | 1,4 | -94 | -94 | -Infinity |  | -94 |
|  |  | 2,5 | -94 | -94 | -Infinity |  | -94 |
|  |  | 3,6 | -91 | -91 | -Infinity |  | -91 |
| Io | dBm/9.36 MHz | 1,4 | -64.60 | -62.25 | -64.60 |  | -62.25 |
|  | dBm/9.36 MHz | 2,5 | -64.60 | -62.25 | -64.60 |  | -62.25 |
|  | dBm/38.16 MHz | 3,6 | -58.50 | -56.16 | -58.50 |  | -56.16 |
| Propagation Condition |  | 1, 2,4,5 | AWGN |  |  | AWGN 1944 Hz Note 4 |  |
|  |  | 3,6 | AWGN |  |  | AWGN 3334 Hz Note 5 |  |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: The AWGN 1944 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 1944Hz. Note 5: The AWGN 3334 Hz condition is a non-fading propagation channel with one tap. Doppler shift is a constant 3334Hz. |  |  |  |  |  |  |  |

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than Tidentify_intra_without_index + 2 × TTIDCCH ms from the beginning of time period T2. The UE is not required to read the neighbour cell SSB index in this test.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays because of TTI insertion uncertainty of the measurement report in DCCH.

The overall delays measured test requirement is expressed as:

Tidentify_intra_without_index = TPSS/SSS_sync_intra + T SSB_measurement_period_intra, is the measurement reporting delay.

TPSS/SSS_sync_intra = ceil(5 × Kp) × DRX cycle × CSSFintra = 3200 ms, is the time period used in PSS/SSS detection defined in 38.133 [6] clause 9.2.5.1

TSSB_measurement_period_intra = ceil(Y × Kp ) × DRX cycle × CSSFintra = 1920 ms, equal to a measurement period of SSB based measurement defined in 38.133 [6] clause 9.2.5.2.

Where:

Kp = 1 for intra-frequency SMTC which is fully non-overlapping with measurement gaps;

DRX cycle = 640 as given in Table 4.6.1.7.4.1-3;

CSSFintra = 1 when only 1 intra-frequency layer is configured.

Y = 3 for DRX cycle =640 ms and SMTC period = 20 ms as given in Table 4.6.1.7.4.1-3.

TTIDCCH = 1 ms, is the TTI insertion uncertainty.

That gives a total of 5122 ms.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90% with confidence level of 95%.

<End of modified section 1>