3GPP TSG-RAN5 Meeting #93-e	R5-217050

# Electronic Meeting, 08th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2574 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to MDT NR TC 8.1.6.1.4.6-CEF Intra-Freq measurements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-09-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is an “FFS” in the specific content of the Table 8.1.6.1.4.6.3.3-3: UEInformationResponse (step 17). |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Revised the specific content of the “UEInformationResponse” messge. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.1.4.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.1.6.1.4.6	Connection Establishment Failure / Logging and reporting / Reporting of Intra-frequency measurements

8.1.6.1.4.6.1	Test Purpose (TP)

(1)

with { UE has connection establishment failure information available with the intra-frequency measurement result }

ensure that {

when { UE receives a UEInformationRequest message with connEstFailReportReq set to true }

then { UE sends a UEInformationResponse message containing the measurement result for intra-frequency neighbouring cell }

}

8.1.6.1.4.6.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 38.331, clauses 5.3.3.7, 5.7.10.3 and 6.2.2. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if connEstFailReportReq is set to true and the UE has connection establishment failure or connection resume failure information in VarConnEstFailReport and if the RPLMN is equal to plmn-Identity stored in VarConnEstFailReport:

2>	set timeSinceFailure in VarConnEstFailReport to the time that elapsed since the last connection establishment failure or connection resume failure in NR;

2>	set the connEstFailReport in the UEInformationResponse message to the value of connEstFailReport in VarConnEstFailReport;

2>	discard the connEstFailReport from VarConnEstFailReport upon successful delivery of the UEInformationResponse message confirmed by lower layers;

[TS 38.331, clause 5.3.3.7]

The UE shall:

1>	if timer T300 expires:

…

2>	store the following connection establishment failure information in the VarConnEstFailReport by setting its fields as follows:

…

3>	if available, set the measResultNeighCells, in order of decreasing ranking-criterion as used for cell re-selection, to include neighbouring cell measurements for at most the following number of neighbouring cells: 6 intra-frequency and 3 inter-frequency neighbours per frequency as well as 3 inter-RAT neighbours, per frequency/ set of frequencies per RAT and according to the following:

4>	for each neighbour cell included, include the optional fields that are available;

NOTE 2:	The UE includes the latest results of the available measurements as used for cell reselection evaluation, which are performed in accordance with the performance requirements as specified in TS 38.133 [14].

[TS 38.331, clause 6.2.2]

–	UEInformationResponse

The UEInformationResponse message is used by the UE to transfer information requested by the network.

…

ConnEstFailReport field descriptions

measResultFailedCell This field refers to the last measurement results taken in the cell, where connection establishment failure or connection resume failure happened.

measResultNeighCells This field refers to the neighbour cell measurements when connection establishment failure or connection resume failure happened.

numberOfConnFail This field is used to indicate the latest number of consecutive failed RRCSetup or RRCResume procedures in the same cell independent of RRC state transition.

numberOfPreamblesSent This field is used to indicate the number of random access preambles that were transmitted.

timeSinceFailure This field is used to indicate the time that elapsed since the connection (establishment or resume) failure. Value in seconds. The maximum value 172800 means 172800s or longer.

8.1.6.1.4.6.3	Test description

8.1.6.1.4.6.3.1	Pre-test conditions

System Simulator:

- NR Cell 1 and Cell 2

UE:

- None.

Preamble:

- The UE is in state 1N-A as defined in TS 38.508-1 [4], subclause 4.4A, on cell 1.

- System information combination NR-2 as defined in TS 38.508-1 [4] clause 4.4.3.1.2.

8.1.6.1.4.6.3.2	Test procedure sequence

Table 8.1.6.1.4.6.3.2-1 illustrates the downlink power levels and other changing parameters to be applied for the cells at various time instants of the test execution. Row marked "T0" denotes the initial conditions after preamble, while columns marked "T1" is to be applied subsequently. The exact instants on which these values shall be applied are described in the texts in this clause.

Table 8.1.6.1.4.6.3.2-1: Time instances of cell power level and parameter changes (FR1)

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -85 | ”Off” | Only Cell 1 is available. (NOTE 1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -85 | -91 | The power level values are assigned to satisfy RCell 1 > RCell 2. |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 Table 6.2.2.1-3. |  |  |  |  |  |

Table 8.1.6.1.4.6.3.2-2: Time instances of cell power level and parameter changes (FR2)

|  | Parameter | Unit | Cell 1 | Cell 2 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | "Off" | Only Cell 1 is available. (NOTE 1). |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | The power level values are assigned to satisfy RCell 1 > RCell 2. |
| NOTE 1: Power level “Off” is defined in TS 38.508-1 Table 6.2.2.2-3. |  |  |  |  |  |

Table 8.1.6.1.4.6.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 1 and NR Cell 2 parameter according to the row "T1" in Table 8.1.6.1.4.6.3.2-1/2. | - | - | - | - |
| 2 | The SS waits for 40s to ensure that the UE detects intra-frequency cell. | - | - | - | - |
| 3 | Void | - | - | - | - |
| 4 | The SS transmits a Paging message. | <-- | NR RRC: Paging | - | - |
| 5 | The UE transmits an RRCSetupRequest message. | --> | NR RRC: RRCSetupRequest | - | - |
| 6 | The SS waits for 2sec to ensure that the UE detects T300 expiry. | - | - | - | - |
| 7 | Void | - | - | - | - |
| 8-10 | Steps 1 to 3 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 11 | The UE transmits an RRCSetupComplete message and a SERVICE REQUEST message on NR Cell 1. | --> | NR RRC: RRCSetupComplete 5GMM: SERVICE REQUEST | - | - |
| 12-15 | Steps 5 to 8 of the generic radio bearer establishment procedure in TS 38.508-1 table 4.5.4.2-3 on NR Cell 1. | - | - | - | - |
| 16 | The SS sends a UEInformationRequest message to get connEstFailReportReq on NR Cell 1. | <-- | UEInformationRequest | - | - |
| 17 | Check: Does the UE send a UEInformationResponse message with connEstFailReport on NR Cell 1? | --> | UEInformationResponse | 1 | P |

8.1.6.1.4.6.3.3	Specific message contents

Table 8.1.6.1.4.6.3.3-1: RRCSetupComplete (step 11, Table 8.1.6.1.4.6.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RRCSetupComplete ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| rrcSetupComplete SEQUENCE { |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |
| ue-MeasurementsAvailable-r16 SEQUENCE { |  |  |  |
| connEstFailInfoAvailable-r16 | true |  |  |
| } |  |  |  |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.6.3.3-2: UEInformationRequest (step 16, Table 8.1.6.1.4.6.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-32A, condition FAIL |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationRequest-r16 ::= SEQUENCE { |  |  |  |
| rrc-TransactionIdentifier | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationRequest-r16 SEQUENCE { |  |  |  |
| connEstFailReportReq-r16 SEQUENCE { |  |  |  |
| idleModeMeasurementReq-r16 | Not present |  |  |
| logMeasReportReq-r16 | Not present |  |  |
| connEstFailReportReq-r16 | True |  |  |
| ra-ReportReq-r16 | Not present |  |  |
| rlf-ReportReq-r16 | Not present |  |  |
| mobilityHistoryReportReq-r16 | Not present |  |  |
| lateNonCriticalExtension | Not present |  |  |
| nonCriticalExtension | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| criticalExtensionsFuture | Not present |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.1.6.1.4.6.3.3-3: UEInformationResponse (step 17, Table 8.1.6.1.4.6.3.2-3)

| Derivation Path: TS 38.508-1 [4] Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| connEstFailReport-r16 SEQUENCE { |  |  |  |
| measResultFailedCell-r16 SEQUENCE { |  |  |  |
| CGI-Info-Logging-r16 SEQUENCE { |  | Information of NR Cell 2 |  |
| plmn-Identity-r16 | the same MCC and MNC stored in EFIMSI on the test USIM card |  |  |
| cellIdentity-r16 | NR Cell 1 |  |  |
| trackingAreaCode-r16 | 1 |  |  |
| } |  |  |  |
| measResult-r16 SEQUENCE { |  |  |  |
| cellResults-r16 SEQUENCE { |  | Measurement result of NR Cell 2 |  |
| resultsSSB-Cell-r16 SEQUENCE { |  |  |  |
| rsrp | as specified in TS 38.508-1 Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in TS 38.508-1 Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in TS 38.508-1 Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| rsIndexResults-r16 SEQUENCE { |  |  |  |
| resultsSSB-Indexes-r16 SEQUENCE (SIZE (1..maxNrofIndexesToReport2)) OF ResultsPerSSB-Index { | 1 entry |  |  |
| ResultsPerSSB-Index[1] SEQUENCE { |  | entry 1 |  |
| ssb-Index | SSB-Index | an SS-Block within an SS-Burst |  |
| ssb-Results SEQUENCE { |  | MeasQuantityResults |  |
| rsrp | as specified in TS 38.508-1 Table 4.6.3-152 | Integer value for RSRP measurements |  |
| rsrq | as specified in TS 38.508-1 Table 4.6.3-153 | Integer value for RSRQ measurements |  |
| sinr | as specified in TS 38.508-1 Table 4.6.3-172 | Integer value for SINR measurements |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| locationInfo-r16 | Not checked |  |  |
| measResultNeighCells-r16 SEQUENCE { |  |  |  |
| measResultNeighCellListNR SEQUENCE (SIZE (1..maxFreq)) OF MeasResult2NR-r16 { | 1 entry |  |  |
| MeasResult2NR-r16[1] SEQUENCE { |  | entry 1 |  |
| ssbFrequency-r16 | ARFCN-ValueNR |  |  |
| refFreqCSI-RS-r16 | ARFCN-ValueNR |  |  |
| measResultList-r16 SEQUENCE (SIZE (1..maxCellReport)) OF MeasResultNR SEQUENCE { | 1 entry | the log measurement results taken in the PCell upon detecting radio link failure or the source PCell upon handover failure. |  |
| MeasResultNR[1] SEQUENCE { |  | entry 1 |  |
| physCellId | Not checked |  |  |
| measResult SEQUENCE { |  |  |  |
| cellResults SEQUENCE { |  |  |  |
| resultsSSB-Cell | Not checked |  |  |
| resultsCSI-RS-Cell | Not checked |  |  |
| } |  |  |  |
| rsIndexResults SEQUENCE { |  |  |  |
| resultsSSB-Indexes | Not checked |  |  |
| resultsCSI-RS-Indexes | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| cgi-Info | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| numberOfConnFail-r16 | Integer between 1-8 |  |  |
| perRAInfoList-r16 | Not checked |  |  |
| timeSinceFailure-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<End of modified section 1>