3GPP TSG-RAN5 Meeting #93-e	R5-217076

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2584 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Editorial update of Idle mode TC 6.1.2.x |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - The referred table numbers are incorrect. - There is aready verdict in generic procedure Table 4.9.4.2.2-1, no need to set it again in the case procedure. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Referred tables were updated with editorial corrections. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect tables will be used in this case. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.2.4.3.2, 6.1.2.5.3.2, 6.1.2.9.3.2, 6.1.2.18.3.2, 6.1.2.21.3.2 and 6.1.2.22.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

[Start of change]

6.1.2.4.3	Test description

6.1.2.4.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 10.

-	System information combination NR-4 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.4.3.2	Test procedure sequence

Table 6.1.2.4.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.1.2.4.3.2-3.

Table 6.1.2.4.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -78 | NR Cell 10 become stronger than ThreshNR Cell 10, high. |
| Note: Power level “Off” is defined in TS38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |

Table 6.1.2.4.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | NR Cell 10 become stronger than ThreshNR Cell 10, high. |
| Note: The uncertain downlink signal level is specified in TS 38.508-1 [4] clause FFS. |  |  |  |  |  |

Table 6.1.2.4.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 10 SS/PBCH EPRE level according to the row "T1" in table 6.1.2. 4.3.2-1/2. | - | - | - | - |
| 2 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 10? | - | - | 1 | - |

[Skip unchanged text]

6.1.2.5.3	Test description

6.1.2.5.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 is set to NR FDD mode and NR Cell 10 is set to NR TDD mode.

-	System information combination NR-4 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.5.3.2	Test procedure sequence

Table 6.1.2.5.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configuration marked "T1" is applied at the points indicated in the Main behaviour description in Table 6.1.2.5.3.2-3.

Table 6.1.2.5.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -80 | NR Cell 1 become lower than Threshserving, low and NR Cell 10 become stronger than ThreshNR Cell 10, low. |
| Note: Power level “Off” is defined in TS38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |

Table 6.1.2.5.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 10 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | NR Cell 1 become lower than Threshserving, low and NR Cell 10 become stronger than ThreshNR Cell 10, low. |
| Note: The uncertain downlink signal level is specified in TS 38.508-1 [4] section FFS. |  |  |  |  |  |

Table 6.1.2.5.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS changes NR Cell 10 SS/PBCH EPRE level according to the row "T1" in table 6.1.2.5.3.2-1/2. | - | - | - | - |
| 2 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 10? | - | - | 1 | - |

[Skip unchanged text]

6.1.2.9.3	Test description

6.1.2.9.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 11.

-	System information combination NR-3 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.9.3.2	Test procedure sequence

Table 6.1.2.9.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", "T2" and so on are applied at the points indicated in the Main behaviour description in Table 6.1.2.9.3.2-3.

Table 6.1.2.9.3.2-1: Time instances of cell power level and parameter changes For FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
|  | Qhysts | dB | 24 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | Off | NR Cell 2 becomes stronger than NR Cell 1 but NR Cell 1 remains the highest ranked one due to Qhysts NR Cell 1 |
|  | Qhysts | dB | 24 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | Off | Qhysts NR Cell 1 change causes NR Cell 2 to become highest ranked cell |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | Off | NR Cell 1 becomes the strongest and highest ranked one due to power adjustment |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | Off | Qoffsets,n NR Cell1 change to 24dB, Qoffsets,n NR Cell 2 remains zero |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 24 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T5 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | Off | NR Cell 1 becomes weaker but it remains the highest ranked one due to Qoffsets,n NR Cell 1 |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 24 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T6 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -88 | Off | NR Cell 2 becomes the highest ranked one due to Qoffsets,n NR Cell 1 change |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T7 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | Off | NR Cell 1 becomes the highest ranked one |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T8 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | -94 | TreselectionNR of NR Cell 1 change to 7S |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 7 | 0 | 0 |  |
| T9 | SS/PBCH SSS EPRE | dBm/SCS | -94 | Off | -88 | NR Cell 2 becomes the highest ranked one |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 7 | 0 | 0 |  |
| Note: Power level “Off” is defined in TS38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |  |

Table 6.1.2.9.3.2-2: Time instances of cell power level and parameter changes For FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 11 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Off | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
|  | Qhysts | dB | 24 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Off | NR Cell 2 becomes stronger than NR Cell 1 but NR Cell 1 remains the highest ranked one due to Qhysts NR Cell 1 |
|  | Qhysts | dB | 24 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Off | Qhysts NR Cell 1 change causes NR Cell 2 to become highest ranked cell |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | Off | NR Cell 1 becomes the strongest and highest ranked one due to power adjustment |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | Off | Qoffsets,n NR Cell1 change to 24dB, Qoffsets,n NR Cell 2 remains zero |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 24 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T5 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Off | NR Cell 1 becomes weaker but it remains the highest ranked one due to Qoffsets,n NR Cell 1 |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 24 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T6 | SS/PBCH SSS EPRE | dBm/SCS | -91 | -82 | Off | NR Cell 2 becomes the highest ranked one due to Qoffsets,n NR Cell 1 change |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T7 | SS/PBCH SSS EPRE | dBm/SCS | -82 | -91 | Off | NR Cell 1 becomes the highest ranked one |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 0 | 0 | 0 |  |
| T8 | SS/PBCH SSS EPRE | dBm/SCS | -82 | Off | -94 | TreselectionNR of NR Cell 1 change to 7S |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 7 | 0 | 0 |  |
| T9 | SS/PBCH SSS EPRE | dBm/SCS | -91 | Off | -82 | NR Cell 2 becomes the highest ranked one |
|  | Qhysts | dB | 0 | 0 | 0 |  |
|  | Qoffsets,n | dB | 0 | 0 | 0 |  |
|  | TreselectionNR | S | 7 | 0 | 0 |  |

Table 6.1.2.9.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | SS re-adjusts the SSS levels according to row "T1" in table 6.1.2.9.3.2-1/2. | - | - | - | - |
| 1A | Wait for 34[FR1]/130[FR2] seconds, the SS transmits a Paging message on NR Cell 2. | <-- | NR RRC: Paging | - | - |
| 2 | Check: Does the UE send an NR: RRCSetupRequest on NR Cell 2 within the next 10 seconds? | --> | NR RRC: RRCSetupRequest | 1 | F |
| 3 | SS notifies UE of the system information change on NR Cell 1 by send Short Message on PDCCH using P-RNTI. | <-- | NR RRC: Paging | - | - |
| 4 | SS resets Qhysts NR Cell 1 according to row "T2" in table 6.1.2.9.3.2-1/2, The ValueTag of SIB2 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 1. | - | - | - | - |
| 5 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 5A | Wait for 8[FR1]/27[FR2] seconds. | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 2? | - | - | 1 | - |
| 7 | SS re-adjusts SSS levels according to rows "T3" in table 6.1.2.9.3.2-1/2. | - | - | - | - |
| 8 | Wait for 8[FR1]/27[FR2] seconds to allow UE to recognise the change. | - | - | - | - |
| 9 | The test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 1. | - | - | - | - |
| 10 | SS notifies UE of the system information change on NR Cell 1 by send Short Message on PDCCH using P-RNTI. | <-- | NR RRC: Paging | - | - |
| 11 | SS changes Qoffsets,n NR Cell 1 according to rows "T4" in table 6.1.2.9.3.2-1/2. The ValueTag of SIB3 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 1. | - | - | - | - |
| 12 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 13 | SS re-adjusts SSS levels according to row "T5" in table 6.1.2.9.3.2-1/2. | - | - | - | - |
| 14 | Wait for 8[FR1]/27[FR2] seconds to allow UE to recognise the change. | - | - | - | - |
| 15 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is still camped on NR Cell 1? | - | - | 3 | - |
| 16 | SS notifies UE of the system information change on NR Cell 1 by send Short Message on PDCCH using P-RNTI. | <-- | NR RRC: Paging | - | - |
| 17 | SS resets Qoffsets,n NR Cell1 according to row "T6" in table 6.1.2.9.3.2-1/2, The ValueTag of SIB3 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 1. | - | - | - | - |
| 18 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 18A | Wait for 8[FR1]/27[FR2] seconds. | - | - | - | - |
| 19 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 2? | - | - | 3 | - |
| 20 | SS re-adjusts SSS levels according to rows "T7" in table 6.1.2.9.3.2-1/2. | - | - | - | - |
| 21 | Wait for 8[FR1]/27[FR2] seconds to allow UE to recognise the change. | - | - | - | - |
| 22 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 1? | - | - | 3 | - |
| 23 | SS sends notification of the system information change on NR Cell 1 by send Short Message on PDCCH using P-RNTI. | <-- | NR RRC: Paging | - | - |
| 24 | SS changes TreselectionNR Cell 1 according to rows "T8" in table 6.1.2.9.3.2-1/2, The ValueTag of SIB2 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 1. | - | - | - | - |
| 25 | SS re-adjusts cell power levels according to rows "T8" in table 6.1.2.9.3.2-1/2. | - | - | - | - |
| 26 | Wait for 34[FR1]/130[FR2] seconds to allow UE to detect Cell 11. | - | - | - | - |
| 27 | SS re-adjusts SSS levels according to rows "T9" in table 6.1.2.9.3.2-1/2. | - | - | - | - |
| 28 | Check: Does the UE send an NR: RRCSetupRequest on NR Cell 11 within Timer=TreselectionNR from step 27? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 29 | SS waits for Timer=TreselectionNR Cell 1 expires after step 27 |  |  |  |  |
| 30 | Check: Does the UE send an NR: RRCsetupRequest on NR Cell 11? | --> | NR RRC: RRCSetupRequest | 2 | P |
| 31-34 | Steps 2 to 5 of the generic test procedure in TS 38.508-1 [4] Table 4.9.5.2.2-1 are performed on NR Cell 11. | - | - | - | - |
| Note: The wait time at steps 2 and 26 is Tdetect,NR_Intra and at Steps 8,14 and 21 it is Tevaluate,NR_Intra (as per TS 38.133 [30], clause 4.2.2.3) plus the time to read the system information TSI-NR. |  |  |  |  |  |

[Skip unchanged text]

6.1.2.18.3	Test description

6.1.2.18.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 3 as defined in TS 38.508-1 [4] Table 4.4.2-2 is used.

-	NR Cell 1 uses System information combination NR-3 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cells.

-	NR Cell 2 and NR Cell 3 uses System information combination NR-4 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1 is used in NR cells.

UE:

-	None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.18.3.2	Test procedure sequence

Table 6.1.2.18.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. Configurations marked "T1", "T2" and "T3" are applied at the points indicated in the Main behaviour description in Table 6.1.2.13.3.2-2.

Table 6.1.2.18.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -102 | -94 | -118 | Srxlev of NR Cell 1 is less than Sintrasearch. |
|  | SIntraSearch | dB | 20 | 20 | 20 |  |
|  | Srxlev | dB | 8 | 16 | -8 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -118 | -102 | -94 | Srxlev of NR Cell 2 is less than Snonintrasearch. |
|  | SnonIntraSearch | dB | 20 | 20 | 20 |  |
|  | Srxlev | dB | -8 | 8 | 16 |  |
| T3 | SnonIntraSearch | dB | 20 | 20 | 2 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -118 | -88 | Srxlev of NR Cell 3 is greater than Snonintrasearch but Cell 1 is high priority cell. |
|  | Srxlev | dB | 22 | 22 | 22 |  |
| Note: Power level “Off” is defined in TS 38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |  |

Table 6.1.2.18.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Off | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | Srxlev of NR Cell 1 is less than Sintrasearch. |
|  | SIntraSearch | dB | 20 | 20 | 20 |  |
|  | Srxlev | dB | FFS | FFS | FFS |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | Srxlev of NR Cell 2 is less than Snonintrasearch. |
|  | SnonIntraSearch | dB | 20 | 20 | 20 |  |
|  | Srxlev | dB | FFS | FFS | FFS |  |
| T3 | SnonIntraSearch | dB | 20 | 20 | 2 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | Srxlev of NR Cell 3 is greater than Snonintrasearch but Cell 1 is high priority cell. |
|  | Srxlev | dB | FFS | FFS | FFS |  |
| Note: The uncertain downlink signal level is specified in TS 38.508-1 [4] section FFS |  |  |  |  |  |  |

Table 6.1.2.18.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.18.3.2-1/2. | - | - | - | - |
| 2 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 2? | - | - | 1 | - |
| 4 | The SS re-adjusts the SSS levels according to row "T2" in table 6.1.2.18.3.2-1/2. | - | - | - | - |
| 5 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 3? | - | - | 2 | - |
| 7 | The SS notifies the UE of change of System Information on NR Cell 3 by sending Short Message on PDCCH using P-RNTI. | <-- | NR RRC: Paging | - | - |
| 8 | The SS changes the SnonIntraSearch for NR Cell 3 according to row "T3" in table 6.1.2.18.3.2-1/2, The ValueTag of SIB2 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 3. | - | - | - | - |
| 9 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 10 | The SS re-adjusts the SSS levels according to row "T4" in table 6.1.2.18.3.2-1/2. | - | - | - | - |
| 11 | Wait for 120 seconds to allow UE to recognise the change. | - | - | - | - |
| 12 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 1? | - | - | 3 | - |

6.1.2.18.3.3	Specific message contents

Table 6.1.2.18.3.3-1: Conditions for specific message contents in Tables below

| Condition | Explanation |
| --- | --- |
| NR Cell 1 | This condition applies to system information transmitted on NR Cell 1. |
| NR Cell 2 | This condition applies to system information transmitted on NR Cell 2. |
| NR Cell 3 | This condition applies to system information transmitted on NR Cell 3. |

Table 6.1.2.18.3.3-2: SIB2 of NR Cell 1, NR Cell 2 and NR Cell 3 (preabmle and all steps, Table 6.1.2.18.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearch | 10 | 20 dB |  |
| cellReselectionPriority | 5 |  | NR Cell 1 |
|  | 1 |  | NR Cell 2 and Cell 3 |
| } |  |  |  |
| intraFreqCellReselectionInfo SEQUENCE { |  |  |  |
| s-IntraSearchP | 10 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-3: SIB4 of NR Cell 2 and NR Cell 3 (preabmle and all steps, Table 6.1.2.18.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF InterFreqCarrierFreqInfo { | 1 entry |  |  |
| InterFreqCarrierFreqInfo[1] SEQUENCE { |  | entry 1 |  |
| dl-CarrierFreq | Same downlink ARFCN as used for NR Cell 3 |  | NR Cell 2 |
|  | Same downlink ARFCN as used for NR Cell 1 |  | NR Cell 3 |
| t-ReselectionNR | 1 |  |  |
| threshX-HighP | 7 | 14 dBm | NR Cell 3 |
| cellReselectionPriority | 5 |  | NR Cell 3 |
|  | 1 |  | NR Cell 2 |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-4: SIB1 of NR Cell 3 (step 8, Table 6.1.2.18.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |
| si-SchedulingInfo | SI-SchedulingInfo |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-5: SI-SchedulingInfo (si-SchedulingInfo in Table 6.1.2.18.3.3-4)

| Derivation Path: TS 38.508-1 [4], Table 4.6.3-173 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SI-SchedulingInfo ::= SEQUENCE { |  |  |  |
| schedulingInfoList SEQUENCE (SIZE(1..maxSI-Message)) OF SchedulingInfo { | 1 entry |  |  |
| SchedulingInfo[1] SEQUENCE { |  | entry 1 |  |
| sib-MappingInfo SEQUENCE (SIZE (1..maxSIB)) OF SIB-TypeInfo { | 1 entry |  |  |
| SIB-TypeInfo[1] SEQUENCE { |  | entry 1 |  |
| type | sibType2 |  |  |
| valueTag | 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.1.2.18.3.3-6: SIB2 of NR Cell 3 (step 8, Table 6.1.2.18.3.2-3)

| Derivation Path: TS 38.508-1 [4], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 1 | 20 dB |  |
| } |  |  |  |
| } |  |  |  |

[Skip unchanged text]

6.1.2.21.3	Test description

6.1.2.21.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1, NR Cell 2 and NR Cell 3

-	System information combination NR-4 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1, and message contents defined in clause 4.6.1 and clause 4.6.2 with QBASED condition is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.21.3.2	Test procedure sequence

Table 6.1.2.21.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1", "T2", "T3" and "T4" are applied at the points indicated in the Main behaviour description in Table 6.1.2.21.3.2-3.

Table 6.1.2.21.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -94 | -80 | -118 | Squal of NR Cell 1 is less than SIntraSearchQ. |
|  | RSRQ | dB | -13.78 | -10.96 | -34.81 |  |
|  | Qrxlevmin | dBm | -110 | -110 | -110 |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SIntraSearchQ | dB | 20 | 20 | 20 |  |
|  | Noc | dBm/SCS | -94 | -94 | -94 |  |
|  | Srxlev | dB | 16 | 30 | -8 |  |
|  | Squal | dB | 6.22 | 9.04 | -14.81 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -118 | -94 | -80 | Squal of NR Cell 2 is less than SnonIntraSearchQ. |
|  | RSRQ | dB | -34.81 | -13.78 | -10.96 |  |
|  | Qrxlevmin | dBm | -110 | -110 | -110 |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SnonIntraSearchQ | dB | 20 | 20 | 20 |  |
|  | Noc | dBm/SCS | -94 | -94 | -94 |  |
|  | Srxlev | dB | -8 | 16 | 30 |  |
|  | Squal | dB | -14.81 | 6.22 | 9.04 |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | -118 | -118 | -80 |  |
|  | RSRQ | dB | -34.81 | -34.81 | -10.96 |  |
|  | Qrxlevmin | dBm | -110 | -110 | -110 |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SnonIntraSearchQ | dB | 20 | 20 | 2 |  |
|  | Noc | dBm/SCS | -94 | -94 | -94 |  |
|  | Srxlev | dB | -8 | -8 | 30 |  |
|  | Squal | dB | -14.81 | -14.81 | 9.04 |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | -80 | -118 | -80 | Squal of NR Cell 3 is greater than SnonIntraSearchQ but NR Cell 1 is high priority cell. |
|  | RSRQ | dB | -10.96 | -34.81 | -10.96 |  |
|  | Qrxlevmin | dBm | -110 | -110 | -110 |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SnonIntraSearchQ | dB | 20 | 20 | 2 |  |
|  | Noc | dBm/SCS | -94 | -94 | -94 |  |
|  | Srxlev | dB | 30 | -8 | 30 |  |
|  | Squal | dB | 9.04 | -14.81 | 9.04 |  |
| Note: Power level “Off” is defined in TS38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |  |

Table 6.1.2.21.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 2 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Off | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | Squal of NR Cell 1 is less than SIntraSearchQ. |
|  | RSRQ | dB | FFS | FFS | FFS |  |
|  | Qrxlevmin | dBm | -110+Delta(NRf1) | -110+Delta(NRf1) | -110+Delta(NRf1) |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SIntraSearchQ | dB | 20 | 20 | 20 |  |
|  | Noc | dBm/SCS | FFS | FFS | FFS |  |
|  | Srxlev | dB | FFS | FFS | FFS |  |
|  | Squal | dB | FFS | FFS | FFS |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | Squal of NR Cell 2 is less than SnonIntraSearchQ. |
|  | RSRQ | dB | FFS | FFS | FFS |  |
|  | Qrxlevmin | dBm | -110+Delta(NRf1) | -110+Delta(NRf1) | -110+Delta(NRf1) |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SnonIntraSearchQ | dB | 20 | 20 | 20 |  |
|  | Noc | dBm/SCS | FFS | FFS | FFS |  |
|  | Srxlev | dB | FFS | FFS | FFS |  |
|  | Squal | dB | FFS | FFS | FFS |  |
| T3 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS |  |
|  | RSRQ | dB | FFS | FFS | FFS |  |
|  | Qrxlevmin | dBm | -110+Delta(NRf1) | -110+Delta(NRf1) | -110+Delta(NRf1) |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SnonIntraSearchQ | dB | 20 | 20 | 2 |  |
|  | Noc | dBm/SCS | FFS | FFS | FFS |  |
|  | Srxlev | dB | FFS | FFS | FFS |  |
|  | Squal | dB | FFS | FFS | FFS |  |
| T4 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | FFS | Squal of NR Cell 3 is greater than SnonIntraSearchQ but NR Cell 1 is high priority cell. |
|  | RSRQ | dB | FFS | FFS | FFS |  |
|  | Srxlev | dB | FFS | FFS | FFS |  |
|  | Qrxlevmin | dBm | -110+Delta(NRf1) | -110+Delta(NRf1) | -110+Delta(NRf1) |  |
|  | Qqualmin | dB | -20 | -20 | -20 |  |
|  | SnonIntraSearchQ | dB | 20 | 20 | 2 |  |
|  | Noc | dBm/SCS | FFS | FFS | FFS |  |
|  | Squal | dB | FFS | FFS | FFS |  |
| Note: The uncertain downlink signal level is specified in TS 38.508-1 [4] section FFS |  |  |  |  |  |  |

Table 6.1.2.21.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS re-adjusts the SS/PBCH EPRE levels according to row "T1" in table 6.1.2.21.3.2-1/2. | - | - | - | - |
| 2 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 2? | - | - | 1 | - |
| 4 | The SS re-adjusts the SSS levels according to row "T2" in table 6.1.2.21.3.2-1/2. | - | - | - | - |
| 5 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 6 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 3? | - | - | 2 | - |
| 6A | The SS re-adjusts the SSS levels according to row "T3" in table 6.1.2.21.3.2-1/2. | - | - | - | - |
| 7 | Void. |  |  |  |  |
| 8 | The SS changes the SnonIntraSearchQ for NR Cell 3 according to row "T3" in table 6.1.2.21.3.2-1/2, The ValueTag of SIB2 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 3. | - | - | - | - |
| 8A | The SS notifies the UE of change of System Information on NR Cell 3 by send Short Message on PDCCH using P-RNTI. | <-- | NR RRC: Paging | - | - |
| 9 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 10 | The SS re-adjusts the SSS levels according to row "T4" in table 6.1.2.21.3.2-1/2. | - | - | - | - |
| 11 | Wait for 120 seconds to allow UE to recognise the change. | - | - | - | - |
| 12 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 1? | - | - | 3 | - |

[Skip unchanged text]

6.1.2.22.3	Test description

6.1.2.22.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1 and NR Cell 3.

-	System information combination NR-4 as defined in TS 38.508-1 [4] Table 4.4.3.1.2-1, and message contents defined in clause 4.6.1 and clause 4.6.2 with QBASED condition is used in NR cells.

UE:

None.

Preamble:

-	The UE is in state 1N-A on NR Cell 1(serving cell) according to TS 38.508-1 [4] Table 4.4A.2-1.

6.1.2.22.3.2	Test procedure sequence

Table 6.1.2.22.3.2-1/2 illustrates the downlink power levels and other changing parameters to be applied for the NR cells at various time instants of the test execution. The exact instants on which these values shall be applied are described in the texts in this clause. The configuration "T0" indicates the initial conditions. Subsequent configurations marked "T1" and "T2" are applied at the points indicated in the Main behaviour description in Table 6.1.2.22.3.2-3.

Table 6.1.2.22.3.2-1: Time instances of cell power level and parameter changes for FR1

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | -88 | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -88 | The power level values are assigned to satisfy SrxlevNR Cell 1 > 0, SrxlevNR Cell 3 < ThreshNR Cell 3, highP, SqualNR Cell 1 > 0 and SqualNR Cell 3 > ThreshNR Cell 3, highQ. |
|  | RSRQ | dB | -10.76 | -10.76 |  |
|  | Qqualmin | dB | -20 | -20 |  |
|  | Qrxlevmin | dBm | -106 | -106 |  |
|  | Noc | dBm/SCS | -145 | -145 |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | -88 | -94 | The power level values are assigned to satisfy SrxlevNR Cell 3 > Threshserving, lowP and SqualNR Cell 1 > ThreshNR Cell 1, lowQ. But SqualNR Cell 3 is larger than Threshserving, lowQ |
|  | RSRQ | dB | -10.76 | -10.76 |  |
|  | Qqualmin | dB | -20 | -20 |  |
|  | Qrxlevmin | dBm | -106 | -106 |  |
|  | Noc | dBm/SCS | -145 | -145 |  |
| T3 | threshServingLowQ | dB | 4 | 26 | The values are assigned to satisfy SqualNR Cell 3 < Threshserving, lowQ and SqualNR Cell 3 < ThreshNR Cell 3, highQ. |
|  | threshX-HighQ | dB | 20 | - |  |
| Note: Power level “Off” is defined in TS38.508-1 [4] Table 6.2.2.1-3. |  |  |  |  |  |

Table 6.1.2.22.3.2-2: Time instances of cell power level and parameter changes for FR2

|  | Parameter | Unit | NR Cell 1 | NR Cell 3 | Remark |
| --- | --- | --- | --- | --- | --- |
| T0 | SS/PBCH SSS EPRE | dBm/SCS | FFS | Off | The power level values are assigned to ensure the UE registered on NR Cell 1. |
| T1 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | The power level values are assigned to satisfy SrxlevNR Cell 1 > 0, SrxlevNR Cell 3 < ThreshNR Cell 3, highP, SqualNR Cell 1 > 0 and SqualNR Cell 3 > ThreshNR Cell 3, highQ. |
|  | RSRQ | dB | FFS | FFS |  |
|  | Qqualmin | dB | -20 | -20 |  |
|  | Qrxlevmin | dBm | -106 | -106 |  |
|  | Noc | dBm/SCS | FFS | FFS |  |
| T2 | SS/PBCH SSS EPRE | dBm/SCS | FFS | FFS | The power level values are assigned to satisfy SrxlevNR Cell 3 > Threshserving, lowP and SqualNR Cell 1 > ThreshNR Cell 1, lowQ. But SqualNR Cell 3 is larger than Threshserving, lowQ |
|  | RSRQ | dB | FFS | FFS |  |
|  | Qqualmin | dB | -20 | -20 |  |
|  | Qrxlevmin | dBm | -106 | -106 |  |
|  | Noc | dBm/SCS | FFS | FFS |  |
| T3 | threshServingLowQ | dB | 4 | 26 | The values are assigned to satisfy SqualNR Cell 3 < Threshserving, lowQ and SqualNR Cell 3 < ThreshNR Cell 3, highQ. |
|  | threshX-HighQ | dB | 20 | - |  |
| Note: The uncertain downlink signal level is specified in TS 38.508-1 [4] section FFS. |  |  |  |  |  |

Table 6.1.2.22.3.2-3: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U – S | Message |  |  |
| 1 | The SS changes NR Cell 3 SS/PBCH EPRE level according to the row "T1" in table 6.1.2.22.3.2-1/2. | - | - | - | - |
| 2 | Wait for 34 seconds to allow UE to recognise the change. | - | - | - | - |
| 3 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 3? | - | - | 1 | - |
| 4 | The SS changes the SSS level according to the row "T2" in table 6.1.2.22.3.2-1/2. | - | - | - | - |
| 5 | Wait for 8 seconds to allow UE to recognise the change. | - | - | - | - |
| 6 | The SS transmits a Paging message on NR Cell 1. | <-- | NR RRC: Paging | - | - |
| 6A | Check: Does the UE send an RRCSetupRequest on NR Cell 1 within the next 10s? | --> | NR RRC: RRCSetupRequest | 2 | F |
| 7 | The SS changes threshServingLowQ of NR Cell 3 according to the row "T3" in table 6.1.2.22.3.2-1/2. The ValueTag of SIB2 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 3. The SS changes threshX-HighQ of NR Cell 1 according to the row "T3" in table 6.1.2.22.3.2-1/2. The ValueTag of SIB4 in the SI-SchedulingInfo of SIB1 is increased on NR Cell 1. The SS notifies the UE of change of System Information on NR Cell 3 and Cell 1 by send SM on PDCCH using P-RNTI. | - | - | - | - |
| 8 | Wait for 2.1* modification period to allow the new system information to take effect. | - | - | - | - |
| 9 | Check: Does the test result of generic test procedure in TS 38.508-1 [4] Table 4.9.4.2.2-1 indicate that the UE is camped on NR Cell 1? | - | - | 2 | - |

[End of change]