3GPP TSG-RAN5 Meeting #93-e	R5-217119

# Electronic Meeting, 8 – 19 August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.521-1 | CR | 5371 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Test case 6.2.4EC and 6.6.3EC.3 updated with subPRB allocation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-20 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | NS vaules for subPRB allocation is missing in 6.2.4EC and 6.6.3EC.3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added NS_05 for subPRB alloctaion to 6.2.4EC and and 6.6.3EC.3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The work item cannot be finalized |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.4EC, 6.6.3EC.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Beginning of changes>>

### 6.2.4EC	Additional Maximum Power Reduction (A-MPR) for UE category M2

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_03 (B2, B4), NS_04 (B41), NS_07 (B13), NS_12 (B26), NS_13 (B26), NS_14 (B26), NS_15 (B26), NS_16 (B27), NS_17 (B28) and NS_18 (B28): Minimum requirements in 36.101 are incomplete. Test configuration table and test requirements are missing

#### 6.2.4EC.1	Test purpose

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction A-MPR is allowed for the output power as specified in Table 6.2.2EC-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

#### 6.2.4EC.2	Test applicability

The requirements of this test apply in test case 6.6.2.2EC Additional Spectrum Emission Mask for network signalled values NS_03, NS_04, NS_06, NS_07 and NS_35 to all types of E-UTRA UE release 14 and forward of UE category M2.

The requirements of this test apply in test case 6.6.3EC.3 Additional Spurious Emissions for network signalled values NS_05, NS_07, NS_08, NS_09, NS_12, NS_13, NS_14, NS_15, NS_16, NS_17, NS_18, NS_22, NS_23 to all types of E-UTRA UE release 14 and forward of UE category M2.

NOTE:	As a result TC 6.2.4EC has not been included in the test case applicability table 4.1-1, TS 36.521-2. This does not preclude the test from being used for R&D or other purposes if deemed useful.

#### 6.2.4EC.3	Minimum conformance requirements

Additional ACLR and spectrum emission requirements can be signalled by the network to indicate that the UE shall also meet additional requirements in a specific deployment scenario. To meet these additional requirements, Additional Maximum Power Reduction (A-MPR) is allowed for the output power as specified in Table 6.2.2EA-1. Unless stated otherwise, an A-MPR of 0 dB shall be used.

For UE Power Class 3 and 5 the specific requirements and identified subclauses are specified in Table 6.2.4EC.3-1 along with the allowed A-MPR values that may be used to meet these requirements. The allowed A-MPR values specified below in Table 6.2.4EC.3-1 are in addition to the allowed MPR requirements specified in subclause 6.2.3EC.3.

Table 6.2.4EC.3-1: Additional Maximum Power Reduction (A-MPR) for category M2 UE

| Network Signalling value | Requirements (subclause) | E-UTRA Band | Narrowband bandwidth | Resources Blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 | 6.6.2.1.3 | Table 5.2-1 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_03 | 6.6.2.2.3.1 | 2, 4 | [TBD] | [TBD] | [TBD] |
| NS_04 | 6.6.2.2.3.2 | 41 | [TBD] | [TBD] | [TBD] |
| NS_05 | 6.6.3.3.3.2 | 1 | 1.4, 3, 5 | Table 6.2.4EC.3-2 |  |
| NS_06 | 6.6.2.2.3.3 | 12, 13, 14, 85 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_07 | 6.6.2.2.3.3 6.6.3.3.3.3 | 13 |  | [TBD] |  |
| NS_08 | 6.6.3.3.3.4 | 19 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_09 | 6.6.3.3.3.5 | 21 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_10 |  | 20 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |
| NS_12 | 6.6.3.3.3.7 | 26 | [TBD] | [TBD] |  |
| NS_13 | 6.6.3.3.3.8 | 26 | [TBD] | [TBD] | [TBD] |
| NS_14 | 6.6.3.3.3.9 | 26 | N/A | Table 5.4.2-1 | N/A |
| NS_15 | 6.6.3.3.3.10 | 26 | [TBD] | [TBD] |  |
| NS_16 | 6.6.3.3.3.11 | 27 | [TBD] | [TBD] | [TBD] |
| NS_17 | 6.6.3.3.3.12 | 28 | [TBD] | [TBD] | [TBD] |
| NS_18 | 6.6.3.3.3.13 | 28 | [TBD] | [TBD] | [TBD] |
| NS_32 | - | - |  | - | - |
| NS_22 |  |  |  |  |  |
| NS_23 |  |  |  |  |  |
| NS_35 | 6.6.2.2.3.5 | 71 | 1.4, 3, 5 | Table 5.4.2-1 | N/A |

No other A-MPR requirement than those specified in table 6.2.4EC.3-1 applies to category M2 UE.

…

Table 6.2.4EC.3-2: A-MPR requirements for "NS_05" for Cat-M2 power class 3 UE

| BW [MHz] | 20 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Fc [MHz] | 1930 |  |  |  |  |
| (NBindex,RBstart) | (0,<6) | (0,<6) | (1,<6) | (2,<6) | (2,<6) |
| LCRB | 9, 12 | ≥15 | ≥15 | 18 | 24 |
| A-MPR [dB] | ≤1 | ≤4 | ≤3 | ≤1 | ≤2 |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [8]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [9]. |  |  |  |  |  |

Table 6.2.4E-7: Additional Maximum Power Reduction (A-MPR) for category M2 UE for subPRB allocation

| Network Signalling value | Requirements (subclause) | E-UTRA Band | A-MPR (dB) |
| --- | --- | --- | --- |
| NS_01 | 6.6.2.1.1 | Table 5.5-1 | N/A |
| NS_03 | 6.6.2.2.1 | 2, 4 | Table 6.2.4E-10 |
| NS_04 | 6.6.2.2.2 | 41 | Table 6.2.4E-11 |
| NS_05 | 6.6.3.3.1 | 1 | Table 6.2.4E-17 |
| NS_06 | 6.6.2.2.3 | 12, 13,14 | Table 6.2.4E-18 |
| NS_07 | 6.6.2.2.3 6.6.3.3.2 | 13 | Table 6.2.4E-12 |
| NS_08 | 6.6.3.3.3 | 19 | N/A |
| NS_09 | 6.6.3.3.4 | 21 | N/A |
| NS_10 |  | 20 | N/A |
| NS_12 | 6.6.3.3.5 | 26 | Table 6.2.4E-19 |
| NS_13 | 6.6.3.3.6 | 26 | Table 6.2.4E-20 |
| NS_14 | 6.6.3.3.7 | 26 | N/A |
| NS_15 | 6.6.3.3.8 | 26 | Table 6.2.4E-21 |
| NS_16 | 6.6.3.3.9 | 27 | Table 6.2.4E-22 |
| NS_17 | 6.6.3.3.10 | 28 | N/A |
| NS_18 | 6.6.3.3.11 | 28 | N/A |
| NS_32 | - | - | - |
| NS_35 | 6.6.2.2.7 | 71 | N/A |
| NS_38 | 6.6.3.3.29 | 74 | Table 6.2.4E-24 |
| NS_39 | 6.6.3.3.30 | 74 | N/A |

Table 6.2.4E-8: A-MPR for "NS_03" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (NBindex,SCstart) | (0,≤72) | (0, ≥216) |  |  |  |  |
| Lcsc | 3,6 | 3,6 |  |  |  |  |
| AMPR [dB] | ≤[1] | ≤[1] |  |  |  |  |
|  | 10 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (0,≤60) | (1,≤24) | (3, ≥264) | (4, ≥228) |  |  |
| Lcsc | 3,6 | 3,6 | 3,6 | 3,6 |  |  |
| AMPR [dB] | ≤[1] | ≤[1] | ≤[1] | ≤[1] |  |  |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |  |  |

Table 6.2.4E-9: A-MPR for "NS_04" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 5MHz |  |
| --- | --- | --- |
| (NBindex,SCstart) | (0,≤72) | (0, ≥216) |
| Lcsc | ≥2 |  |
| AMPR [dB] | ≤[2] |  |

Table 6.2.4E-10: A-MPR for “NS_07" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 10 MHz |  |  |
| --- | --- | --- | --- |
| (NBindex,SCstart) | (0, <48) | (0,48-84) | (0, ≥240) |
| Lcsc | ≥2 | ≥2 | ≥2 |
| AMPR [dB] | ≤[12] | ≤[6] | ≤[14.5] |
| (NBindex,SCstart) | (1, <24) | (1,24-72) | (1, ≥264) |
| Lcsc | ≥2 | ≥2 | ≥2 |
| AMPR [dB] | ≤[12] | ≤[6] | ≤[14.5] |
| (NBindex,SCstart) | (2, <60) |  | (2, ≥252) |
| Lcsc | ≥2 |  | ≥2 |
| AMPR [dB] | ≤[6] |  | ≤[1.5] |
| (NBindex,SCstart) | (3, <48) |  | (3, ≥216) |
| Lcsc | ≥2 |  | ≥2 |
| AMPR [dB] | ≤[6] |  | ≤[1.5] |
| (NBindex,SCstart) | (4, <36) |  | (4, ≥192) |
| Lcsc | ≥2 |  | ≥2 |
| AMPR [dB] | ≤[3] |  | ≤[1.5] |

Table 6.2.4E-11: A-MPR for "NS_05" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 5 |  | 10 |  | 15 |  | 20 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| (WBindex,SCstart) | (0,≤36) | (0, ≤6) | (0,≤30) | (0,≤12) | (0,≤24) | (0,≤12) | (0,0) | (0,0) |
| Lcsc | 2 | 6 | 2 | 6 | 2 | 6 | 2 | 3 |
| AMPR [dB] | ≤[1.5] | ≤ [0.5] | ≤[1.5] | ≤[0.5] | ≤[1.5] | ≤[0.5] | ≤[1.5] | ≤[0.5] |
| NOTE 1: WBindex is the wideband index that is defined in [4]. NOTE2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with WBindex |  |  |  |  |  |  |  |  |

Table 6.2.4E-12: A-MPR for "NS_06" for Cat-M2 with sub-PRB allocation

| BW [MHz] | 5 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (NBindex,SCstart) | (0,≤72) | (0, ≥216) |  |  |  |  |
| Lcsc | 3,6 | 3,6 |  |  |  |  |
| AMPR [dB] | ≤[1] | ≤[1] |  |  |  |  |
|  | 10 MHz |  |  |  |  |  |
| (NBindex,SCstart) | (0,≤60) | (1,≤24) | (3, ≥264) | (4, ≥228) |  |  |
| Lcsc | 3,6 | 3,6 | 3,6 | 3,6 |  |  |
| AMPR [dB] | ≤[1] | ≤[1] | ≤[1] | ≤[1] |  |  |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB of NB indicated with NBindex. |  |  |  |  |  |  |

Table 6.2.4E-13: A-MPR for "NS_12" for Cat-M2 with sub-PRB allocation for E-UTRA lower channel edge >= 814.2 MHz and ≤ 829.2 MHz

| BW [MHz] | 5 MHz |
| --- | --- |
| (WBindex,SCstart) | (0, <84) |
| Lcsc | ≥2 |
| AMPR [dB] | ≤[5] |
| BW [MHz] | 10 MHz |
| (WBindex,SCstart) | (0, <72) |
| Lcsc | ≥2 |
| AMPR [dB] | ≤[5] |
| BW [MHz] | 15 MHz |
| (WBindex,SCstart) | (0, <66) |
| Lcsc | ≥2 |
| AMPR [dB] | ≤[5] |
| NOTE 1: NBindex is the narrowband index that is defined in 6.2.7 in [4]. The resource block assignment is defined within the narrowband as defined in 5.3.3.1.12 and 5.3.3.1.13 in [5]. NOTE 2: WBindex is the wideband index that is defined in [4]. NOTE 3: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with NBindex or WBindex |  |

Table 6.2.4E-14: A-MPR for "NS_13" for Cat-M2 with sub-PRB allocation for E-UTRA lower channel edge >= 819 MHz and ≤ 824 MHz

| BW [MHz] | 5 |  |
| --- | --- | --- |
| (WBindex,SCstart) | (0, <30) | (0, <30) |
| Lcsc | 2 | 3,6 |
| AMPR [dB] | ≤[3] | ≤[5] |
| NOTE 1: WBindex is the wideband index that is defined in [4]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with WBindex |  |  |

Table 6.2.4E-15: A-MPR for "NS_15" for Cat-M2 with sub-PRB allocation for E-UTRA highest channel edge > 834 MHz and ≤ 849 MHz

| BW [MHz] | 5 MHz |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| (WBindex,SCstart) | (0, <54) | (0, <54) | (0, <54) | (0, 200-234) | (0, >234) | (0, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤[10.5] | ≤[8.5] | ≤[5.5] | ≤[3.5] | ≤[10] | ≤[9] |
| BW [MHz] | 10 MHz |  |  |  |  |  |
| (WBindex,SCstart) | (1, <48) | (1, <48) | (1, <48) | (1, 200-234) | (1, >234) | (1, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤[10.5] | ≤[8.5] | ≤[5.5] | ≤[3.5] | ≤[10] | ≤[9] |
| BW [MHz] | 15 MHz |  |  |  |  |  |
| (WBindex,SCstart) | (2, <48) | (2, <48) | (2, <48) | (2, 200-234) | (2, >234) | (2, >234) |
| Lcsc | 2 | 3 | 6 | ≥2 | 2,3 | 6 |
| AMPR [dB] | ≤[10.5] | ≤[8.5] | ≤[5.5] | ≤[3.5] | ≤[10] | ≤[9] |
| NOTE 1: WBindex is the wideband index that is defined in [4]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with WBindex |  |  |  |  |  |  |

Table 6.2.4E-16: A-MPR for "NS_16" for Cat-M2 with sub-PRB allocation for E-UTRA lowest channel edge > 807 MHz and ≤ 812 MHz

| BW [MHz] | 5 MHz |
| --- | --- |
| (WBindex,SCstart) | (0, <6) |
| Lcsc | 2 |
| AMPR [dB] | ≤[7] |
| NOTE 1: WBindex is the wideband index that is defined in [4]. NOTE 2: Lcsc is the length of the continuous subcarrier, SCstart is the subcarrier offset relative to the first subcarrier of the first PRB indicated with WBindex |  |

Table 6.2.4E-17: A-MPR for "NS_38" for Cat-M2 with sub-PRB allocation for E-UTRA lowest channel edge > 1427 MHz and ≤ 1447 MHz

| Channel bandwidth [MHz] | Parameters | Region |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| 3 | (NBindex,SCstart ) | (0, 0-27) | (0, 30-42) | (0, 45-51) | (0, 96-120) | (0, 123-168) |
|  | Lcsc | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 | 2,3 ,6 |
|  | A-MPR [dB] | ≤ [16] | ≤ [13] | ≤ [10] | ≤ [7] | ≤ [3] |
| 5 | (WBindex,SCstart ) | (0, < 96) | (0, > 219) |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |
| 10 | (WBindex,SCstart ) | (0, < 93) | (0, > 219) |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |
| 15 | (WBindex,SCstart ) | (0, < 87) | (0, > 234) |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |
| 20 | (WBindex,SCstart ) | (0, < 81) | (0, > 234) |  |  |  |
|  | Lcsc | 2,3,6 | 2,3,6 |  |  |  |
|  | A-MPR [dB] | ≤ [7] | ≤ [1] |  |  |  |

No other A-MPR requirement than those specified in table 6.2.4E-1, table 6.2.4E-2, table 6.2.4E-6 and table 6.2.4E-7 applies to category M1 and M2 UE.

The normative reference for this requirement is TS 36.101 clause 6.2.4 and clause 6.2.4E.

#### 6.2.4EC.4	Test description

##### 6.2.4EC.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA operating bands specified in sub-clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.2.4EC.4.1-1 to 6.2.4EC.4.1-16. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.2.4EC.4.1-1: Test Configuration Table (network signalled value "NS_03")

TBD

Table 6.2.4EC.4.1-2: Test Configuration Table (network signalled value "NS_04")

TBD

Table 6.2.4EC.4.1-3: Test Configuration Table (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| 3 (Note 4) | 20MHz |  | QPSK | 6 | 0 |
| 4 (Note 4) | 20MHz |  | QPSK | 9 | 0 |
| 5 (Note 4) | 20MHz |  | QPSK | 12 | 0 |
| 6 (Note 4) | 20MHz |  | QPSK | 18 | 0 |
| 7 (Note 4) | 20MHz |  | QPSK | 12 | 1 |
| 8 (Note 4) | 20MHz |  | QPSK | 18 | 1 |
| 9 (Note 4) | 20MHz |  | QPSK | 12 | 2 |
| 10 (Note 4) | 20MHz |  | QPSK | 18 | 2 |
| 11 (Note 4) | 20MHz |  | QPSK | 24 | 2 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0. Note 4: Only for power class 3 UE |  |  |  |  |  |

Table 6.2.4EC.4.1-3: Test Configuration Table, subPRB allocation (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration |  | Uplink Configuration |  |  |  |  |  |  |  |
| Configuration ID |  | Ch BW |  | N/A |  | Mod'n |  | RB allocation |  |  |  |  |  |
|  |  |  |  |  |  |  |  | FDD and HD-FDD |  | Narrowband index (Note 1) |  | SCstart |  |
| 1 |  | 5MHz |  |  |  | π/2-BPSK |  | ¼ |  | 0 |  | 0 |  |
| 2 |  | 5MHz |  |  |  | π/2-BPSK |  | ¼ |  | 0 |  | 36 |  |
| 3 | 5MHz |  | QPSK | ½ | 0 | 0 |
| 4 |  | 5MHz |  |  |  | QPSK |  | ½ |  | 0 |  | 6 |  |
| 5 |  | 10MHz |  |  |  | π/2-BPSK |  | ¼ |  | 0 |  | 3 |  |
| 6 |  | 10MHz |  |  |  | QPSK |  | ½ |  | 0 |  | 12 |  |
| 7 |  | 15MHz |  |  |  | π/2-BPSK |  | ¼ |  | 0 |  | 24 |  |
| 8 |  | 15MHz |  |  |  | QPSK |  | ½ |  | 0 |  | 12 |  |
| 9 |  | 20MHz |  |  |  | π/2-BPSK |  | ¼ |  | 0 |  | 0 |  |
| 10 |  | 20MHz |  |  |  | QPSK |  | ¼ |  | 0 |  | 0 |  |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: This table applies only to UE supporting subPRB allocation. |  |  |  |  |  |  |

Table 6.2.4EC.4.1-4: Test Configuration Table (network signalled value "NS_06")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 7 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 8 | 5MHz |  | QPSK | 24 | 0 |
| 9 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 10 | 5MHz |  | 16QAM | 3 | 0 |
| 11 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 12 | 5MHz |  | 16QAM | 24 | 0 |
| 13 (Note 4) | 10MHz |  | QPSK | 4 | 0 |
| 14 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 15 | 10MHz |  | QPSK | 24 | 0 |
| 16 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 17 | 10MHz |  | 16QAM | 5 | 0 |
| 18 | 10MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 | 1.4MHz |  | QPSK | 2 | 0 |
| 2 | 1.4MHz |  | QPSK | 5 | 0 |
| 3 | 1.4MHz |  | QPSK | 6 | 0 |
| 4 | 1.4MHz |  | 16QAM | 2 | 0 |
| 5 | 1.4MHz |  | 16QAM | 6 | 0 |
| 6 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 7 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 8 | 5MHz |  | QPSK | 24 | 0 |
| 9 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 10 | 5MHz |  | 16QAM | 3 | 3 |
| 11 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 12 | 5MHz |  | 16QAM | 24 | 0 |
| 13 (Note 4) | 10MHz |  | QPSK | 4 | 7 |
| 14 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 15 | 10MHz |  | QPSK | 24 | 4 |
| 16 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 17 | 10MHz |  | 16QAM | 5 | 7 |
| 18 | 10MHz |  | 16QAM | 24 | 4 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 UE |  |  |  |  |  |

Table 6.2.4EC.4.1-5: Test Configuration Table (network signalled value "NS_07")

TBD

Table 6.2.4EC.4.1-6: Test Configuration Table (network signalled value "NS_08")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 3 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB# (6 - (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

Table 6.2.4EC.4.1-7: Test Configuration Table (network signalled value "NS_09")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 3 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB# (6 - (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

Table 6.2.4EC.4.1-8: Test Configuration Table (network signalled value "NS_10")

Void, not tested

Table 6.2.4EC.4.1-9: Test Configuration Table (network signalled value "NS_12")

TBD

Table 6.2.4EC.4.1-10: Test Configuration Table (network signalled value "NS_13")

TBD

Table 6.2.4EC.4.1-11: Test Configuration Table (network signalled value "NS_14")

TBD

Table 6.2.4EC.4.1-12: Test Configuration Table (network signalled value "NS_15")

TBD

Table 6.2.4EC.4.1-13: Test Configuration Table (network signalled value "NS_16")

TBD

Table 6.2.4EC.4.1-14: Test Configuration Table (network signalled value "NS_22")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5 MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5 MHz |  | QPSK | 1 | 3 |  |
| 2 | 5 MHz |  | QPSK | 24 | 0 |  |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 - RB allocation) of the narrowband. |  |  |  |  |  |  |

Table 6.2.4EC.4.1-15: Test Configuration Table (network signalled value "NS_18")

TBD

Table 6.2.4EC.4.1-16: Test Configuration Table (network signalled value "NS_35")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range, Mid range, High range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Lowest, 5MHz, 10MHz, Highest |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| Low range, Mid range |  |  |  |  |  |
| 1 (Note 4) | 5MHz |  | QPSK | 1 | 0 |
| 2 (Note 5) | 5MHz |  | QPSK | 3 | 0 |
| 3 | 5MHz |  | QPSK | 24 | 0 |
| 4 (Note 4) | 5MHz |  | 16QAM | 1 | 0 |
| 5 | 5MHz |  | 16QAM | 3 | 0 |
| 6 (Note 5) | 5MHz |  | 16QAM | 5 | 0 |
| 7 | 5MHz |  | 16QAM | 24 | 0 |
| 8 | 10MHz |  | QPSK | 4 | 0 |
| 9 (Note 5) | 10MHz |  | QPSK | 5 | 0 |
| 10 | 10MHz |  | QPSK | 24 | 0 |
| 11 (Note 4) | 10MHz |  | 16QAM | 3 | 0 |
| 12 | 10MHz |  | 16QAM | 5 | 0 |
| 13 | 10MHz |  | 16QAM | 24 | 0 |
| 14 | 20MHz |  | QPSK | 18 | 0 |
| 15 | 20MHz |  | QPSK | 24 | 0 |
| 16 | 20MHz |  | 16QAM | 18 | 0 |
| 17 | 20MHz |  | 16QAM | 24 | 0 |
| High range |  |  |  |  |  |
| 1 (Note 4) | 5MHz |  | QPSK | 1 | 3 |
| 2 (Note 5) | 5MHz |  | QPSK | 3 | 3 |
| 3 | 5MHz |  | QPSK | 24 | 0 |
| 4 (Note 4) | 5MHz |  | 16QAM | 1 | 3 |
| 5 | 5MHz |  | 16QAM | 3 | 3 |
| 6 (Note 5) | 5MHz |  | 16QAM | 5 | 3 |
| 7 | 5MHz |  | 16QAM | 24 | 0 |
| 8 | 10MHz |  | QPSK | 4 | 7 |
| 9 (Note 5) | 10MHz |  | QPSK | 5 | 7 |
| 10 | 10MHz |  | QPSK | 24 | 4 |
| 11 (Note 4) | 10MHz |  | 16QAM | 3 | 7 |
| 12 | 10MHz |  | 16QAM | 5 | 7 |
| 13 | 10MHz |  | 16QAM | 24 | 4 |
| 14 | 20MHz |  | QPSK | 18 | 13 |
| 15 | 20MHz |  | QPSK | 24 | 12 |
| 16 | 20MHz |  | 16QAM | 18 | 13 |
| 17 | 20MHz |  | 16QAM | 24 | 12 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 and RB# (6 – (RB allocation mod 6)) of the narrowband. Note 4: Only for power class 3 UE Note 5: Only for power class 5 UE |  |  |  |  |  |

1.	Connect the SS to the UE antenna connectors as shown in TS 36.508 [7] Annex A Figure A.3 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channel is set according to Table 6.2.4EC.4.1-1 to Table 6.2.4EC.4.1-16.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.2.4EC.4.3.

##### 6.2.4EC.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via M-PDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to the applicable table from Table 6.2.4EC.4.1-1 to Table 6.2.4EC.4.1-16. Since the UE has no payload data to send, the UE transmits uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE until the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

##### 6.2.4EC.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and with the same exceptions as in clauses 6.2.4.4.3.1 to 6.2.4.4.3.30 for each network signalled value.

#### 6.2.4EC.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from Table 6.2.4EC.5-1 to Table 6.2.4EC.5-16. The allowed A-MPR values specified in Table 6.2.4EC.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.3EC. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in Table 6.2.5EA.3-1 and 6.2.5EA.3-2 apply.

Table 6.2.4EC.5-1: UE Power Class test requirements (network signalled value "NS_03")
(for band 2 and band 4)

FFS

Table 6.2.4EC.5-2: UE Power Class test requirements (network signalled value "NS_04")
(for band 41)

FFS

Table 6.2.4EC.5-3: UE Power Class test requirements (network signalled value "NS_05")
(for band 1)

| Configuration ID | A-MPR (dB) | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 2 | 0 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 3 | 0 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 4 | 1 | 0 | 22.0 | 2.0 | 25.7 | 19.3 | N/A |  |  |  |  |
| 5 | 1 | 0 | 22.0 | 2.0 | 25.7 | 19.3 | N/A |  |  |  |  |
| 6 | 4 | 0 | 19.0 | 3.5 | 25.7 | 14.8 | N/A |  |  |  |  |
| 7 | 0 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 8 | 3 | 0 | 20.0 | 2.5 | 25.7 | 16.8 | N/A |  |  |  |  |
| 9 | 0 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 10 | 1 | 0 | 22.0 | 2.0 | 25.7 | 19.3 | N/A |  |  |  |  |
| 11 | 2 | 1 | 20.0 | 2.5 | 25.7 | 16.8 | N/A |  |  |  |  |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |  |

| Configuration ID | A-MPR (dB) | Power class 3 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 |  | 1.5 |  | 0.5 |  | 21.0 |  | 2.0 |  | 25.7 |  | 18.3 |  |
| 2 |  | 1.5 |  | 0.5 |  | 21.0 |  | 2.0 |  | 25.7 |  | 18.3 |  |
| 3 |  | 0.5 |  | 1,0 |  | 21.5 |  | 2.0 |  | 25.7 |  | 18.8 |  |
| 4 |  | 0.5 |  | 1.0 |  | 21.5 |  | 2.0 |  | 25.7 |  | 18.8 |  |
| 5 |  | 1.5 |  | 0.5 |  | 21.0 |  | 2.0 |  | 25.7 |  | 18.3 |  |
| 6 |  | 0.5 |  | 1.0 |  | 21.5 |  | 2.0 |  | 25.7 |  | 18.8 |  |
| 7 |  | 1.5 |  | 0.5 |  | 21.0 |  | 2.0 |  | 25.7 |  | 18.3 |  |
| 8 |  | 0.5 |  | 1.0 |  | 21.5 |  | 2.0 |  | 25.7 |  | 18.8 |  |
| 9 |  | 1.5 |  | 0 |  | 21.5 |  | 2.0 |  | 25.7 |  | 18.8 |  |
| 10 |  | 0.5 |  | 0 |  | 22.5 |  | 2.0 |  | 25.7 |  | 19.8 |  |

Table 6.2.4EC.5-3a: UE Power Class test requirements, subPRB allocation (network signalled value "NS_05")
(for band 1)

Table 6.2.4EC.5-4A: UE Power Class test requirements without TC,c (network signalled value "NS_06")
(for band 13, band 14 and for Mid range in band 12, band 85)

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 2 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 3 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 4 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 5 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 6 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 7 | N/A |  |  |  |  | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 8 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 9 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 10 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 11 | N/A |  |  |  |  | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 12 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 13 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 14 | N/A |  |  |  |  | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 15 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 16 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 17 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 18 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-4B: UE Power Class test requirements with TC,c (network signalled value "NS_06")
(for Low/High range in band 12 and band 85)

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 2 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 3 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5.0 | 22.7 | 10.8 |
| 4 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 5 | 2 | 19.5 | 3.5 | 25.7 | 15.3 | 2 | 16.5 | 5.0 | 22.7 | 10.8 |
| 6 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 7 | N/A |  |  |  |  | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 8 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 9 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 10 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 11 | N/A |  |  |  |  | 1 | 17.5 | 5.0 | 22.7 | 11.8 |
| 12 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 13 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 14 | N/A |  |  |  |  | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 15 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 16 | 0 | 21.5 | 2.0 | 25.7 | 18.8 | N/A |  |  |  |  |
| 17 | 1 | 20.5 | 2.5 | 25.7 | 17.3 | 0 | 18.5 | 4.0 | 22.7 | 13.8 |
| 18 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-5: UE Power Class test requirements (network signalled value "NS_07")
(for band 13)

FFS

Table 6.2.4EC.5-6: UE Power Class test requirements (network signalled value "NS_08")
(for band 19)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-7: UE Power Class test requirements (network signalled value "NS_09")
(for band 21)

| Configuration ID | MPR (dB) | Power class 3 |  |  |  | Power class 5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23 | 2 | 25.7 | 20.3 | 20 | 2 | 22.7 | 17.3 |
| 2 | 1 | 22 | 2 | 25.7 | 19.3 | 19 | 3.5 | 22.7 | 14.8 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |

Table 6.2.4EC.5-8: UE Power Class test requirements (network signalled value "NS_10")
(for band 20)

Void, not tested

Table 6.2.4EC.5-9: UE Power Class test requirements (network signalled value "NS_12")
(for band 26)

FFS

Table 6.2.4EC.5-10: UE Power Class test requirements (network signalled value "NS_13")
(for band 26)

FFS

Table 6.2.4EC.5-11: UE Power Class test requirements (network signalled value "NS_14")
(for band 26)

FFS

Table 6.2.4EC.5-12: UE Power Class test requirements (network signalled value "NS_15")
(for band 26)

FFS

Table 6.2.4EC.5-13: UE Power Class test requirements (network signalled value "NS_16")
(for band 27)

FFS

Table 6.2.4EC.5-14: UE Power Class test requirements (network signalled value "NS_22")
(for band band 42 and 43)

| Configuration ID | MPR (dB) | PCMAX_L,c (dBm) |  | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- |
| Power class 3 |  |  |  |  |  |  |
| 1 | 0 | 23 | 2 |  | 26 | 20 |
| 2 | 1 | 22 | 2 |  | 26 | 19 |
| Power class 5 |  |  |  |  |  |  |
| 1 | 0 | 20 | 2 |  | 23 | 17 |
| 2 | 1 | 19 | 3.5 |  | 24.5 | 14.5 |
| Power class 6 |  |  |  |  |  |  |
| 1 | 0 | 14 | 2.5 |  | 17.5 | 10.5 |
| 2 | 1 | 13 | 2.5 |  | 17.5 | 9.5 |
| NOTE 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |

Table 6.2.4EC.5-15: UE Power Class test requirements (network signalled value "NS_18")
(for band 28)

FFS

Table 6.2.4EC.5-16: UE Power Class test requirements (network signalled value "NS_35")
(for band 71)

| Configuration ID | Power class 3 |  |  |  |  | Power class 5 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) | MPR (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | Upper limit (dBm) | Lower limit (dBm) |
| 1 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 2 | 1 | N/A |  |  |  | 0 | 19.0 | 3.5 | 0 | 17.3 |
| 3 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 4 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 5 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 0 | 19.0 | 3.5 | 22.7 | 17.3 |
| 6 | 2 | N/A |  |  |  | 1 | 18.0 | 4.0 | 1 | 13.3 |
| 7 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| 8 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 9 | 1 | N/A |  |  |  | 0 | 19.0 | 3.5 | 0 | 14.8 |
| 10 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 11 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | N/A |  |  |  |  |
| 12 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 0 | 19.0 | 3.5 | 22.7 | 17.3 |
| 13 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 1 | 18.0 | 4.0 | 22.7 | 14.8 |
| 14 | 0 | 23.0 | 2.0 | 25.7 | 20.3 | 0 | 20.0 | 2.0 | 22.7 | 17.3 |
| 15 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 16 | 1 | 22.0 | 2.0 | 25.7 | 19.3 | 1 | 19.0 | 3.5 | 22.7 | 14.8 |
| 17 | 2 | 21.0 | 2.0 | 25.7 | 18.3 | 2 | 18.0 | 4.0 | 22.7 | 13.3 |
| Note 1: PCMAX,c and T(PCMAX_L,c) are defined in TS 36.101 [2] clause 6.2.5 |  |  |  |  |  |  |  |  |  |  |

<< Clauses skipped here>>

#### 6.6.3EC.3	Additional spurious emissions for UE category M2

Editor’s note: This clause is incomplete. The following aspects are either missing or not yet determined:

- NS_07 (B13), NS_12 (B26), NS_13 (B26), NS_14 (B26), NS_15 (B26), NS_16 (B27), NS_17 (B28) and NS_18 (B28): Test configuration table missing due to minimum requirements for A-MPR in 36.101 are incomplete.

6.6.3EC.3.1	Test purpose

To verify that UE transmitter does not cause unacceptable interference to other channels or other systems in terms of transmitter spurious emissions under the deployment scenarios where additional requirements are specified.

6.6.3EC.3.2	Test applicability

This test case applies to all types of E-UTRA UE release 14 and forward of UE category M2.

6.6.3EC.3.3	Minimum conformance requirements

Same minimum conformance requirements as in clause 6.6.3EA.3.

6.6.3EC.3.4	Test description

6.6.3EC.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on E-UTRA bands specified in clause 5.2E. All of these configurations shall be tested with applicable test parameters for each channel bandwidth, and are shown in Table 6.6.3EC.3.4.1-1 to Table 6.6.3EC.3.4.1-11. The details of the uplink reference measurement channels (RMCs) are specified in Annexe A.2. Configurations of PDSCH and MPDCCH before measurement are specified in Annex C.2.

Table 6.6.3EC.3.4.1-1: Test Configuration Table (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, 20MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 0 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| 3 (Note 4) | 20MHz |  | QPSK | 6 | 0 |
| 4 (Note 4) | 20MHz |  | QPSK | 9 | 0 |
| 5 (Note 4) | 20MHz |  | QPSK | 12 | 0 |
| 6 (Note 4) | 20MHz |  | QPSK | 18 | 0 |
| 7 (Note 4) | 20MHz |  | QPSK | 12 | 1 |
| 8 (Note 4) | 20MHz |  | QPSK | 18 | 1 |
| 9 (Note 4) | 20MHz |  | QPSK | 12 | 2 |
| 10 (Note 4) | 20MHz |  | QPSK | 18 | 2 |
| 11 (Note 4) | 20MHz |  | QPSK | 24 | 2 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB#0 of the narrowband. |  |  |  |  |  |

Table 6.6.3EC.3.4.1-1a: Test Configuration Table, subPRB allocation (network signalled value "NS_05")

| Initial Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | Low range |  |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz, 10MHz, 15MHz, 20MHz |  |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |  |
| Configuration ID | Ch BW | N/A | Mod'n | RB allocation |  |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) | SCstart |
| 1 | 5MHz |  | π/2-BPSK | ¼ | 0 | 0 |
| 2 | 5MHz |  | π/2-BPSK | ¼ | 0 | 36 |
| 3 | 5MHz |  | QPSK | ½ | 0 | 0 |
| 4 | 5MHz |  | QPSK | ½ | 0 | 6 |
| 5 | 10MHz |  | π/2-BPSK | ¼ | 0 | 3 |
| 6 | 10MHz |  | QPSK | ½ | 0 | 12 |
| 7 | 15MHz |  | π/2-BPSK | ¼ | 0 | 24 |
| 8 | 15MHz |  | QPSK | ½ | 0 | 12 |
| 9 | 20MHz |  | π/2-BPSK | ¼ | 0 | 0 |
| 10 | 20MHz |  | QPSK | ¼ | 0 | 0 |
| Note 1: Denotes where in the channel Bandwidth the narrowband shall be placed. Narrowband and Narrowband index are defined in TS36.211, 5.2.4. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: This table applies only to UE supporting subPRB allocation. |  |  |  |  |  |  |

Table 6.6.3EC.3.4.1-2: Test Configuration Table (network signalled value "NS_07")

TBD

Table 6.6.3EC.3.4.1-3: Test Configuration Table (network signalled value "NS_08")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 3 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB# (6 - (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

Table 6.6.3EC.3.4.1-4: Test Configuration Table (network signalled value "NS_09")

| Initial Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 36.508 [7] subclause 4.1 |  |  | Normal |  |  |
| Test Frequencies as specified in TS 36.508 [7] subclause 4.3.1 |  |  | High Range |  |  |
| Test Channel Bandwidths as specified in TS 36.508 [7] subclause 4.3.1 |  |  | 5MHz |  |  |
| Test Parameters for Channel Bandwidths and Narrowband positions |  |  |  |  |  |
|  |  | Downlink Configuration | Uplink Configuration |  |  |
| Configuration ID | Ch BW | N/A for Maximum Power Reduction (MPR) test case | Mod'n | RB allocation |  |
|  |  |  |  | FDD and HD-FDD | Narrowband index (Note 1) |
| 1 | 5MHz |  | QPSK | 1 | 3 |
| 2 | 5MHz |  | QPSK | 24 | 0 |
| Note 1: Denotes the lowest narrowband index in the channel bandwidth where the wideband shall be placed. The allocation is contiguous, starting from the lowest narrowband index. Narrowband, Narrowband index and Wideband are defined in TS 36.211 [8], 5.2.7. Note 2: Test Channel Bandwidths are checked separately for each E-UTRA band, the applicable channel bandwidths are specified in Table 5.4.2.1-1. Note 3: The RBstart of partial RB allocation shall be RB# (6 - (RB allocation mod 6)) of the narrowband. |  |  |  |  |  |

Table 6.6.3EC.3.4.1-5: Test Configuration Table (network signalled value "NS_12")

TBD

Table 6.6.3EC.3.4.1-6: Test Configuration Table (network signalled value "NS_13")

TBD

Table 6.6.3EC.3.4.1-7: Test Configuration Table (network signalled value "NS_14")

TBD

Table 6.6.3EC.3.4.1-8: Test Configuration Table (network signalled value "NS_15")

TBD

Table 6.6.3EC.3.4.1-9: Test Configuration Table (network signalled value "NS_16")

TBD

Table 6.6.3EC.3.4.1-10: Test Configuration Table (network signalled value "NS_17")

TBD

Table 6.6.3EC.3.4.1-11: Test Configuration Table (network signalled value "NS_18")

TBD

1.	Connect SS to the UE antenna connectors as shown in TS 36.508[7] Annex A Figure A.7 using only main UE Tx/Rx antenna.

2.	The parameter settings for the cell are set up according to TS 36.508 [7] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C0, C.1 and C.3.0, and uplink signals according to Annex H.1 and H.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.6.3EC.3.4.1-1 to Table 6.6.3EC.3.4.1-11.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State 3A-RF-CE according to TS 36.508 [7] clause 5.2A.2AA. Message contents are defined in clause 6.6.3EC.3.4.3.

6.6.3EC.3.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via MPDCCH DCI format 6-0A for C_RNTI to schedule the UL RMC according to Table 6.6.3EC.3.4.1-1 to Table 6.6.3EC.3.4.1-11 depending on NS-value. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC

2.	Send continuously uplink power control "up" commands in the uplink scheduling information to the UE to ensure that the UE transmits at PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode according to the test configuration, which shall meet the requirements described in Tables 6.2.4EC.5-1 to 6.2.4EC.5-16 as appropriate. The period of measurement shall be at least the continuous duration one sub-frame (1ms). For HD-FDD slots with transient periods and Half-duplex guard subframe are not under test.

4.	Measure the power of the transmitted signal with a measurement filter of bandwidths according to Table 6.6.3EA.3.3.1-1, 6.6.3EA.3.3.3-1 to 6.6.3EA.3.3.11-1, and 6.6.3EC.3.5.1-1 as appropriate. The centre frequency of the filter shall be stepped in contiguous steps according to the same table. For NS_07 measurements made in a bandwidth of 6.25kHz, measurement parameter settings defined in table 6.6.3EC.3.4.2-1 shall be used. The measured power shall be verified for each step. The measurement period shall capture the active time slots.

Table 6.6.3EC.3.4.2-1: Measurement setup for NS_07

|  | Option 1: Measurement with No RMS VBW available | Option2 Measurement with VBW Filtering on Power scale |
| --- | --- | --- |
| VBW | >=62.5 kHz (10 times or more the RBW) | <=43Hz |
| RBW | <=6.25kHz | <=6.25kHz |
| Detector type | Averages signal envelope during each measurement point, such as "RMS detector" | Peak |
| Averaging mode (Trace averaging) | Power (RMS voltage) | Power (RMS voltage), as controlled by "Average Type" |
| Average Type (applies to detector) | Power (RMS voltage) (automatically occurs with "RMS detector") | Not applicable |
| Average Type (applies to VBW filter) | Not applicable | Power (RMS voltage) |
| Number of averages | 30, to reduce variance as required, or use an even longer sweep time | 1 or use an even narrower VBW filter, thus a longer sweep time |
| Sweep time | [Don't specify] | Sweep rate (span divided by sweep time) <= 0.8 * RBW*VBW |

6.6.3EC.3.4.3	Message contents

Message contents are according to TS 36.508 [7] subclause 4.6 with the condition CEModeA and with the same exceptions as in clauses 6.2.4.4.3.1 to 6.2.4.4.3.30 for each network signalled value.

6.6.3EC.3.5	Test requirement

The measured UE mean power in the channel bandwidth, derived in step 3, shall fulfil requirements in Tables 6.2.4EC.5-1 to 6.2.4EC.5-16 as appropriate,

and

With the exception of NS_07, test requirements for Additional Spurious Emissions are the same as the minimum requirements and are not repeated in this section.

6.6.3EC.3.5.1	Test requirement (network signalled value "NS_07")

When "NS_07" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EC.3.5.1-1. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EC.3.5.1-1: Additional requirements

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) | Measurement bandwidth |
| --- | --- | --- |
|  | 10 MHz |  |
| 769 ≤ f ≤ 775 | -55.5 | 6.25 kHz |
| NOTE: The emissions measurement shall be sufficiently power averaged to ensure standard deviation < 0.5 dB. |  |  |

6.6.3EC.3.5.1	Test requirement (network signalled value "NS_05")

When "NS_05" is indicated in the cell, the power of any UE emission shall not exceed the levels specified in Table 6.6.3EC.3.5.1-2. This requirement also applies for the frequency ranges that are less than FOOB (MHz) in Table 6.6.3.1.3-1 from the edge of the channel bandwidth.

Table 6.6.3EC.3.5.1-2: Additional requirements (PHS)

| Frequency band (MHz) | Channel bandwidth / Spectrum emission limit (dBm) |  |  |  | Measurement bandwidth | NOTE |
| --- | --- | --- | --- | --- | --- | --- |
|  | 5 MHz | 10 MHz | 15 MHz | 20 MHz |  |  |
| 1884.5 f 1915.7 | -41 | -41 | -41 | -41 | 300 KHz | 1 |

<< End of changes>>