3GPP TSG-RAN5 Meeting #93-e	R5-217126

# Electronic Meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-5 | CR | 0211 | rev | - | Current version: | 16.6.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Include satellite SVIDs for 2012 GNSS scenario |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI16_Test |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The R5-215711 from 3GPP TS 37.571-5 introduces new GNSS scenarios with date in 2020. Such CR contemplates to use either new GNSS scenarios or old scenarios (with date in 2012) until September 2023. Instead of including tables with the satellite SVIDs to be simulated for the old scenario from 16.5.0, current specification refers to 16.5.0 to get that information. Therefore, this change proposes to include the satellite SVIDs for the old scenario in two tables in the current specification. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include new tables with satellites SVIDs from 3GPP TS 37-571-2 16.5.0 for GNSS Signalling tests |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Current reference to old specification could mislead in lack of information |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1, 6.1.2, B.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Comment for the editor: Please add “GNSS_Data_Sig_V16.zip” to the attachments of TS 37.571-5 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Comment for the editor: Please add “GNSS_Data_Sig_V16.zip” to the attachments of TS 37.571-5

# < Unchanged sections omitted >

## 4.1	GPS and GNSS orbital model information, assistance data and assistance data files

The following subclauses 5 and 6 define the GPS and GNSS orbital model information, the assistance data and the assistance data files (subclause 5 only) for the test cases as follows:

Subclause 5.1: data for UTRA A-GPS Signalling test cases defined in TS 37.571-2 [7] subclauses 6.1.1 to 6.1.3.

Subclause 5.2: data for UTRA A-GPS Minimum Performance test cases defined in TS 37.571-1 [6] subclause 5.

Subclause 6.1: data for UTRA, E-UTRA and NR A-GNSS Signalling test cases defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9.

Subclause 6.2: data for UTRA, E-UTRA and NR A-GNSS Minimum Performance test cases defined in TS 37.571-1 [6] subclauses 6, 7 and 13.

For subclause 5 the orbital model information is defined and where appropriate is given in Yuma format in .txt files for each scenario in the appropriate data file specified in Annex A.

For subclause 6 the orbital model information is defined and where appropriate is given in Rinex navigation data file format or Yuma format in .txt files for each scenario in the appropriate data file specified in Annex B.

For subclause 5, where the assistance data is fixed or is not required on a per-satellite basis, then it is defined in the following subclauses. Where assistance data is required on a per-satellite basis, or where the values of the data also vary with time then it is specified in comma-separated-variable files in the appropriate data file specified in Annex A. These files specify the values to be used for each satellite, indexed by satellite PRN or SV ID, and, where applicable, the values to be used indexed by both time and satellite PRN or SV ID.

For subclause 6, the assistance data is defined in the following subclauses.

For the aerial GNSS scenarios defined in TS 36.508 [20], the orbital model information is defined and where appropriate is given in Yuma format in .txt files for each scenario in the appropriate data file specified in Annex B.

# < Unchanged sections omitted >

### 6.1.2 GNSS Scenario

Table 6.1.2-0: GNSS Scenarios to be used

| Scenarios | Condition |
| --- | --- |
| New GNSS scenarios from 2020 with the navigation data files defined in Table 6.1.2-1 and Table 6.1.2-2 and the SV IDs defined in Tables 6.1.2-2A to 6.1.2-4 | px_GnssScenario2012 = FALSE |
| GNSS scenarios from 2012 with the navigation data files defined in Table 6.1.2-5 and Table 6.1.2-6 and the SV IDs defined in Table 6.1.2-7 and Table 6.1.2-8 | px_GnssScenario2012 = TRUE |

If px_GnssScenario2012 = FALSE, the following GNSS scenario shall be used. The assistance data specified in the following subclauses is consistent with this GNSS scenario:

-	Rinex navigation data files: the required file(s) in the GNSS orbital data sig zip file specified in Annex B are given below.

Table 6.1.2-1: Rinex navigation data files for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | Rinex navigation datafile(s) |
| --- | --- |
| 1 | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| 2 | Sig GNSS Galileo 2020_9_17 Rinex.txt |
| 3 | Sig GNSS GPS 2020_9_17 Rinex.txt |
| 4 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS GLONASS 2020_9_17 Rinex.txt |
| 8 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS Galileo 2020_9_17 Rinex.txt |
| 9 | Sig GNSS BDS 2020_9_17 Rinex.txt |
| 10 | Sig GNSS GPS 2020_9_17 Rinex.txt and Sig GNSS BDS 2020_9_17 Rinex.txt |

Table 6.1.2-2: Rinex navigation data files for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | Rinex navigation datafile(s) (1) |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15 | GPS | Sig GNSS GPS 2020_9_17 Rinex.txt |
|  | GLONASS | Sig GNSS GLONASS 2020_9_17 Rinex.txt |
|  | Galileo | Sig GNSS Galileo 2020_9_17 Rinex.txt |
|  | BDS | Sig GNSS BDS 2020_9_17 Rinex.txt |
| Note 1: Where the UE supports more than one GNSS then all the relevant Rinex navigation data files are used |  |  |

-	UE location and Reference location: Static at latitude: 35 degrees 44 minutes 39.432 seconds north, longitude: 139 degrees 40 minutes 48.633 seconds east, (Tokyo Japan), height: = 300m.

-	Nominal start time: 17th September 2020 23:40:00  (GPS time).

-	The visible satellites available for simulation and for which Assistance Data (other than Almanac) shall be generated are given in Table 6.1.2-2A.

Table 6.1.2-2A: SV IDs of Visible satellites

| GNSS | SV IDs of Visible satellites |
| --- | --- |
| GPS | 3, 4, 6, 17, 19, 28 |
| GLONASS | 3, 4, 5, 10, 18, 19 |
| Galileo | 3, 5, 13, 15, 21, 27 |
| BDS | 38, 40, 42, 43, 59, 60 |

-	For BDS, the satellite types are given in Table 6.1.2-2B

Table 6.1.2-2B: BDS satellite types

| BDS Satellite type | SV IDs of Satellites |
| --- | --- |
| GEO | 59, 60 |
| IGSO | 38, 40 |
| MEO | 42, 43 |

-	The visible satellites to be simulated in each sub-test case are given  in Table 6.1.2-3 and Table 6.1.2-4

Table 6.1.2-3: Satellites to be simulated for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 3, 4, 5, 10, 18, 19 (GLONASS) |
| 2 | 3, 5, 13, 15, 21, 27 (Galileo) |
| 3 | 3, 4, 6, 17, 19, 28 (GPS) (Note) |
| 4 | GPS: 3, 4, 6, 28. GLONASS: 5, 16, 18, 19. |
| 8 | GPS: 3, 4, 6, 28. Galileo: 3, 5, 13, 21. |
| 9 | 38, 40, 42, 43, 59, 60 (BDS) |
| 10 | GPS: 3, 4, 6, 28. BDS: 38, 40, 59, 60. |
| Note: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. |  |

Table 6.1.2-4: Satellites to be simulated for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | SV IDs of Satellites to be simulated |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15 (Note) | GPS | 3, 4, 6, 17, 19, 28 |
|  | GLONASS | 3, 4, 5, 10, 18, 19 |
|  | Galileo | 3, 5, 13, 15, 21, 27 |
|  | BDS | 38, 40, 42, 43, 59, 60 |
| Note: For this sub-test the satellite simulator shall generate all the GNSS signals supported by the UE for all the simulated satellites. |  |  |

If px_GnssScenario2012 = TRUE, the following GNSS scenario shall be used. The assistance data specified in the following subclauses is consistent with this GNSS scenario:

-	Yuma Almanac data: the required file(s) in the GNSS data sig zip file specified in Annex B are given below.

Table 6.1.2-5: Yuma / AGL Almanac data files for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | Yuma / AGL file(s) |
| --- | --- |
| 1 | Sig GNSS 1-1 AGL.txt |
| 2 | Sig GNSS 1-2 Yuma.txt |
| 3 | Sig GNSS 1-3 Yuma.txt |
| 4 | Sig GNSS 1-3 Yuma.txt and Sig GNSS 1-1 AGL.txt |
| 8 | Sig GNSS 1-3 Yuma.txt and Sig GNSS 1-2 Yuma.txt |
| 9 | Sig GNSS 1-9 Yuma.txt |
| 10 | Sig GNSS 1-3 Yuma.txt and Sig GNSS 1-9 Yuma.txt |

Table 6.1.2-6: Yuma / AGL Almanac data files for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | Yuma / AGL file(s) (1) |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15 | GPS | Sig GNSS 1-3 Yuma.txt |
|  | GLONASS | Sig GNSS 1-1 AGL.txt |
|  | Galileo | Sig GNSS 1-2 Yuma.txt |
|  | BDS | Sig GNSS 1-9 Yuma.txt |
| Note 1: Where the UE supports more than one GNSS then all the relevant Yuma / AGL data files are used |  |  |

-	UE location and Reference location:

Static at latitude: 35 degrees 44 minutes 39.432 seconds north, longitude: 139 degrees 40 minutes 48.633 seconds east, (Tokyo Japan 2012), height: = 300m.

-	Nominal start time:

1st January 2012 00:31:00 (GPS time).

-	Visible satellites simulated are given below

Table 6.1.2-7: Satellites to be simulated for TS 37.571-2 subclauses 6.2.1 to 6.2.3

| Sub-Test Case Number | SV IDs of Satellites to be simulated |
| --- | --- |
| 1 | 3, 4, 9, 10, 18, 20 |
| 2 | 5, 10, 11, 18, 19, 20 |
| 3 | 1, 11, 17, 20, 23, 28 (Note) |
| 4 | GPS: 1, 17, 20, 28. GLONASS: 3, 10, 18, 20 |
| 8 | GPS: 1, 17, 20, 28. Galileo: 5, 10, 11, 18. |
| 9 | 1, 2, 7, 18, 21, 27 |
| 10 | GPS: 1, 17, 20, 28. BDS: 1, 7, 18, 21. |
| Note: For this sub-test the satellite simulator shall generate all the GPS signals supported by the UE for all the simulated satellites. |  |

Table 6.1.2-8: Satellites to be simulated for TS 37.571-2 subclauses 7 and 9

| Sub-Test Case Number | GNSS supported by the UE | SV IDs of Satellites to be simulated |
| --- | --- | --- |
| 7 | [FFS] | [FFS] |
| 15 (Note) | GPS | 1, 11, 17, 20, 23, 28 |
|  | GLONASS | 3,4,9,10,18,20 |
|  | Galileo | 5,10,11,18,19,20 |
|  | BDS | 1,2,7,18,21,27 |
| Note: For this sub-test the satellite simulator shall generate all the GNSS signals supported by the UE for all the simulated satellites. |  |  |

-	Ionospheric model: see values in subclause 6.1.3

-	The levels of the simulated satellites shall all be at -125dBm +/- 6dB

# < Unchanged sections omitted >

Annex B (normative): GNSS data files

## B.1	GNSS data files for signalling tests

The GNSS orbital data files for use in GNSS signalling tests defined in TS 37.571-2 [7] subclauses 6.2.1 to 6.2.3 and subclauses 7 and 9 are contained in archives GNSS_Orbital_Data_Sig_V1.zip and GNSS_Data_Sig_V16.zip which accompanies the present document.

GNSS_Orbital_Data_Sig_V1.zip includes the assistance data for the scenarios defined in Tables 6.1.2-1 and 6.1.2-2, while GNSS_Data_Sig_V16.zip includes the assistance data for the scenarios defined in Tables 6.1.2-5 and 6.1.2-6.

## B.2	GNSS data files for Minimum Performance tests

The GNSS orbital data files for use in GNSS Minimum Performance tests defined in TS 37.571-1 [6] subclauses 6,  7 and 13 are contained in archive GNSS_ Orbital_Data_Perf_V1.zip which accompanies the present document. The different scenarios are designated with suffixes XX in the zip file, where XX is 01, 02, 03 etc. for scenarios #1, #2, #3 etc.

## B.3	GNSS data files for aerial tests

The GNSS data files for use in GNSS signalling tests using the scenario defined in TS 36.508 [20] subclause 4.12 are contained in archive GNSS_Data_Aerial_V1.zip which accompanies the present document.

The data files contained in the archive are recommended but not mandatory.

< End of changes >