3GPP TSG-RAN5 Meeting #93-e	R5-217128

# Electronic Meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-3 | CR | 0147 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to OTDOA pics parameters |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The presence of the for maxSupportedPrsBandwidth, maxSupportedPrsConfigs and numberOfRXantennas field cannot be evaluated with the current pics pc_OTDOA_maxSupportedPrsBandwidth, pc_OTDOA_numberOfRXantennas, pc_OTDOA_maxSupportedPrsConfigs. These 3 pics are of type enum and meant to evaluate the value of the aforementioned fields. Therefore, new pics of type boolean are needed to check the presence of the aforementioned fields. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Include 3 new boolean pics to evaluate the support for maxSupportedPrsBandwidth, maxSupportedPrsConfigs and numberOfRXantennas and rename the existing OTDOA enum pics as _r14 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Implementation for 37.571-4 could lead into wrong UE fail verdicts |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

Table A.4.3-3A: OTDOA Measurements

| Item | OTDOA Measurements | Ref. | Release | Mnemonic | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | Support of interFreqRSTDmeasurement | 36.355, 6.5.1.7 | Rel-10 | pc_OTDOA_interFreqRSTDmeasurement |  |
| 2 | Support of additionalNeighbourCellInfoList | 36.355, 6.5.1.7 | Rel-10 | pc_OTDOA_additionalNeighbourCellInfoList |  |
| 3 | Support of prs-id | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_prs_id |  |
| 4 | Support of tp-separation-via-muting | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_tp_separation_via_muting |  |
| 5 | Support of additional-prs-config | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_additional_prs_config |  |
| 6 | Support of prs-based-tbs | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_prs_based_tbs |  |
| 7 | Support of additionalPathsReport | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_additionalPathsReport |  |
| 8 | Support of densePrsConfig | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_densePrsConfig |  |
| 9 | maxSupportedPrsBandwidth | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_maxSupportedPrsBandwidth _r14 |  |
| 10 | Support of prsOccGroup | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_prsOccGroup |  |
| 11 | Support of prsFrequencyHopping | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_prsFrequencyHopping |  |
| 12 | maxSupportedPrsConfigs | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_maxSupportedPrsConfigs _r14 |  |
| 13 | Support of periodicalReporting | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_periodicalReporting |  |
| 14 | Support of multiPrbNprs | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_multiPrbNprs |  |
| 15 | Support of idleStateForMeasurements | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_idleStateForMeasurements |  |
| 16 | numberOfRXantennas | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_numberOfRXantennas _r14 |  |
| 17 | Support of motionMeasurements | 37.355, 6.5.1.7 | Rel-15 | pc_OTDOA_motionMeasurements |  |
| 18 | Support of interRAT-RSTDmeasurement | 37.355, 6.5.1.7 | Rel-15 | pc_OTDOA_interRAT_RSTDmeasurement | Inter-RAT RSTD for EUTRA measurements as described in 38.215 [18], 5.1.13 |
| 19 | Support of maxSupportedPrsBandwidth | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_maxSupportedPrsBandwidth |  |
| 20 | Support of maxSupportedPrsConfigs | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_maxSupportedPrsConfigs |  |
| 21 | Support of numberOfRXantennas | 36.355, 6.5.1.7 | Rel-14 | pc_OTDOA_numberOfRXantennas |  |

< End of changes >