3GPP TSG-RAN5 Meeting #93-e	R5-217140

# Electronic Meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.571-1 | CR | 0352 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add OTDOA feMTC test case 9.4.17 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Rohde & Schwarz, Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_eMTC4-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The OTDOA feMTC test case for measurement period with longer PRS occasions is not included in TS 37.571-1 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added a new test case for RSTD measurement period with longer PRS occasions, including test purpose, test applicability, minimum requirements and test procedure |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The OTDOA eMTC4 work plan cannot be completed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# < Unchanged sections omitted >

#### 9.4.17	E-UTRAN HD-FDD inter-frequency RSTD measurement period test case in CE Mode B with longer PRS occasions

###### 9.4.17.1	Test purpose

To verify that the RSTD measurement reporting delay with longer PRS occasions for UE Category M1 and M2 meets the performance requirements in normal coverage mode in an environment with fading propagation conditions.

###### 9.4.17.2	Test applicability

This test applies to E-UTRA HD-FDD UE Category M1 and M2 release 15 and forward that supports UE-assisted OTDOA, CE Mode B, inter-frequency RSTD measurements and dense PRS configuration or additional PRS configuration. Test 2 is applicable only to UE Category M2.

###### 9.4.17.3	Minimum conformance requirements

Same as in clause 9.4.4.1.3

The normative reference for this requirement is TS 36.133 [23] clause 8.13.3.7.3, 8.16.3.2.3 and A.8.13.13.

###### 9.4.17.4	Test description

##### 9.4.17.4.1	Initial conditions

Same as in clause 9.4.13.4.1 but replacing Table 9.4.13.4.1-1 with Table 9.4.17.4.1-1

Table 9.4.17.4.1-1: General test parameters

| Parameter | Unit | Value |  | Comment |
| --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 |  |
| Reference cell |  | Cell 1 |  | Reference cell is the cell in the OTDOA assistance data with respect to which the RSTD measurement is defined, as specified in 3GPP TS 36.214 [6] and 3GPP TS 36.355 [4]. The reference cell is the serving cell in this test case. |
| Neighbour cells |  | Cell 2 and Cell 3 |  | Cell 2 and Cell 3 appear at random places in the neighbour cell list in the OTDOA assistance data, but Cell 2 always appears in the first half of the list, whilst Cell 3 appears in the second half of the list. |
| MPDCCH |  | DL Reference Measurement Channel R.8 HD-FDD |  | As specified in TS 36.521-3 [25] clause A.7.1 |
| mPDCCH-startSF-UESS |  | 10 |  | Parameter G in which determines subframe k0 in which MPDCCH starts |
| Channel Bandwidth (BWchannel) | MHz | 10 |  |  |
| Gap pattern Id |  | rstd12 | rstd2 | As specified in TS 36.133 [23] Table 8.1.2.1-3. Applies for measurements on Cell 1, Cell 2, and Cell 3 |
| Gap offset |  | 151 |  | As specified in TS 36.331 [22], Clause 6.3.5 |
| PRS Transmission Bandwidth Note 2 | RB | 50 |  | PRS are transmitted over the system bandwidth |
| PRS configuration index Note 2 |  | Cell 1: 232, Cell 2, Cell 3: 312 |  | This corresponds to periodicity of 320 ms and PRS subframe offset of DL subframes, as defined in 3GPP TS 36.211 [26], Table 6.10.4.3-1 |
| Number of consecutive downlink positioning subframes Note 2 |  | 30 | 8 | As defined in 3GPP TS 36.211 [26]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [4], for UE with additional-prs-config capability |
|  |  | 40 | 10 | As defined in 3GPP TS 36.211 [26]. The number of subframes in a positioning occasion. Corresponds to parameter add-numDL-Frames in TS 36.355 [4], for UE with densePrsConfig capability |
| Physical cell ID PCI Note 2 |  | (PCI of Cell 1 – PCI of Cell 2)mod6=0 and (PCI of Cell 1 – PCI of Cell 3)mod6=0 |  | The cell PCIs are selected such that the relative shifts of PRS patterns among cells are as given by the test parameters |
| CP length Note 2 |  | Normal |  |  |
| DRX |  | ON |  | DRX parameters are further specified in Table 9.3.16.4.1-2 |
| prs-SubframeOffset |  | 80 |  | Number of subframes rounded to the closest integer. The corresponding parameter in the OTDOA assistance data is prs-SubframeOffset specified in TS 36.355 [4] |
| slotNumberOffset |  | 0 |  | The slot number offset at the transmitter between a neighbour cell and the assistance data reference cell specified in TS 36.355 [4] |
| Radio frame receive time offset between the cells at the UE antenna connector Note 3 | s | Cell 2 to Cell 1: 1 Cell 3 to Cell 1: -1 |  | PRS are transmitted from synchronous cells |
| Expected RSTD Note 1 | s | Cell 2: -2 Cell 3: 2 Other neighbour cells: randomly between -3 and 3 |  | The expected RSTD is what is expected at the receiver. The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [4] is the expectedRSTD indicator |
| Expected RSTD uncertainty for all neighbour cells Note 1 | s | 5 |  | The corresponding parameter in the OTDOA assistance data specified in TS 36.355 [4] is the expectedRSTD-Uncertainty index |
| Number of cells provided in OTDOA assistance data |  | 16 |  | Including the reference cell |
| PRS muting info Note 2 |  | Cell 1: ‘1111111100000000’ Cell 2: ‘0000000011111111’ Cell 3: ‘1111111100000000’ |  | Corresponds to prs-MutingInfo defined in TS 36.355 [4] |
| T1 | s | 3 |  | The length of the time interval from the beginning of each test |
| T2 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T1 |
| T3 | s | 2.56 |  | The length of the time interval that follows immediately after time interval T2 |
| Note 1: Parameters “Expected RSTD” and “Expected RSTD uncertainty for all neighbour cells” are not settable parameters. These are parameters signalled in LPP only. For the values to be used in LPP see Table 9.3.16.4.3-5 and TS 37.571-5 [20], clause 7.2.5. Note 2: Parameters “PRS Transmission Bandwidth”, “PRS configuration index”, “Number of consecutive downlink positioning subframes”, “Physical cell ID PCI”, “CP length”, and “PRS muting info” are settable parameters and also parameters signalled in LPP. The values to be used for “Physical cell ID PCI” are as follows: Cell 1: 0, Cell 2: 6, Cell 3: 12. For all the values to be used in LPP see Table 9.3.16.4.3-5 and TS 37.571-5 [20], clause 7.2.5. Note 3: The parameter “Radio frame receive time offset between the cells at the UE antenna connector” is not a settable parameter but is used to set the “true RSTD” values in step 6 of clause 9.3.16.4.1. Note 4: If the PRS transmission bandwidth is larger than the UE RF bandwidth, the UE is measuring RSTD within its RF bandwidth. |  |  |  |  |

##### 9.4.17.4.2	Test procedure

Same as in clause 9.4.14.4.2 but with the following additional step:

14.	Repeat step 1-13 for each sub-test in Table 9.4.17.4.1-1 as appropriate

##### 9.4.17.4.3	Message contents

Same as in clause 9.4.13.4.3.

#### 9.4.17.5	Test requirement

Same as in clause 9.4.13.5.

< End of changes >