3GPP TSG-RAN WG5 Meeting #93-e                                                                R5-217244

# Electronic Meeting, 8th – 19th November 2021

Agenda Item:	5.3.1.17

Source: 	CAICT

Title: 	Discussion on Alignment of the description for initial set up of downlink and uplink signals in TS 38.521-2

Document for:	Discussion and Endorsement

Introduction

The descriptions of initial set up for downlink and uplink signals are inconsistent in the spec and with errors for some test cases. Below are the examples of typical different kinds of descriptions in the spec:

The contents of Annex C and Annex G in TS 38.521-2 are as below:

- Annex C (normative): Downlink Physical Channels

- C.0	Downlink signal levels

Editor’s Note : Consideration to minimize the required number of additional FR2 link is under discussion

The downlink power settings in Table C.0-1 is used unless otherwise specified in a test case.

Table C.0-1: Default Downlink power levels for NR

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | -70 | -67 | -64 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | -70 | -67 | -64 | -61 |
|  | SS/PBCH SSS EPRE | dBm/SCS | -99 for DL SCS= 60 kHz -96 for DL SCS = 120 kHz | -99 for DL SCS = 60 kHz -96 for DL SCS = 120 kHz | -99 for DL SCS = 60 kHz -96 for DL SCS = 120 kHz | -99 for DL SCS = 60 kHz -96 for DL SCS = 120 kHz |
| Note 1: The channel bandwidth powers are informative, based on [-99]dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the centre of quiet zone. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of [–99]dBm/60kHz. |  |  |  |  |  |  |

The default downlink signal level uncertainty is +/- TBD dB, for any level specified. If the uncertainty value is critical for the test purpose, a tighter uncertainty is specified for the related test case in Annex F.

For TRP measurement, DL signal may be supplied from RSRP based pathloss compensation link. Downlink signal level using RSRP based pathloss compensation link is specified in Table C.0-2 or Table C.0-3.

Table C.0-2: Downlink power levels for RSRP based pathloss compensation link for TRP measurement for n257, n258 and n260

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | ≥ -87 | ≥ -84 | ≥ -80 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | ≥ -87 | ≥ -84 | ≥ -80 | ≥ -77 |
|  | SS/PBCH SSS EPRE | dBm/SCS | ≥ -115.5 for DL SCS = 60 kHz ≥ -112.5 for DL SCS = 120 kHz | ≥ -115.5 for DL SCS = 60 kHz ≥ -112.5 for DL SCS = 120 kHz | ≥ -115.5 for DL SCS = 60 kHz ≥ -112.5 for DL SCS = 120 kHz | ≥ -115.5 for DL SCS = 60 kHz ≥ -112.5 for DL SCS = 120 kHz |
| Note 1: The channel bandwidth powers are informative, based on -115.5dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the RSRP reference point as defined in TS 38.215 [24]. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of ≥ –115.5 dBm/60kHz. |  |  |  |  |  |  |

Table C.0-3: Downlink power levels for RSRP based pathloss compensation link for TRP measurement for n261

| SCS (kHz) |  | Unit | Channel Bandwidth |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 50 MHz | 100 MHz | 200 MHz | 400 MHz |
| 60 | Number of RBs |  | 66 | 132 | 264 | N/A |
|  | Channel BW power | dBm | ≥ -84 | ≥ -81 | ≥ -78 | N/A |
| 120 | Number of RBs |  | 32 | 66 | 132 | 264 |
|  | Channel BW power | dBm | ≥ -84 | ≥ -81 | ≥ -78 | ≥ -75 |
|  | SS/PBCH SSS EPRE | dBm/SCS | ≥ -113 for DL SCS = 60 kHz ≥ -110 for DL SCS = 120 kHz | ≥ -113 for DL SCS = 60 kHz ≥ -110 for DL SCS = 120 kHz | ≥ -113 for DL SCS = 60 kHz ≥ -110 for DL SCS = 120 kHz | ≥ -113 for DL SCS = 60 kHz ≥ -110 for DL SCS = 120 kHz |
| Note 1: The channel bandwidth powers are informative, based on -113dBm/60kHz SS/PBCH SSS EPRE, then scaled according to the number of RBs and rounded to the nearest integer dBm value. Full RE allocation with no boost or deboost is assumed. Note 2: The power level is specified at the RSRP reference point as defined in TS 38.215 [24]. Note 3: DL level is applied for any of the Subcarrier Spacing configuration () with the same power spectrum density of ≥ –113 dBm/60kHz. |  |  |  |  |  |  |

- C.1	General

The following clauses describes the downlink Physical Channels that are transmitted during a connection i.e., when measurements are done.

- C.2	Setup

Table C.2-1 describes the downlink Physical Channels that are required for connection set up.

Table C.2-1: Downlink Physical Channels required for connection set-up

Physical Channel

PBCH

SSS

PSS

PDCCH

PDSCH

PBCH DMRS

PDCCH DMRS

PDSCH DMRS

CSI-RS

PTRS

As common PDSCH and PDCCH configuration parameters the parameters in Table A.3.1-1, C.2-2, C.2-3, and C.2-4 shall be used to bring up the connection setup for FR1 NR cell.

Table C.2-2: PDSCH and PDCCH configuration

| Parameter | Unit | Value |
| --- | --- | --- |
| Number of HARQ processes |  | 8 (TDD) |
| Aggregation level | CCE | 4 |

Table C.2-3: Additional test parameters for TDD for SCS 60 KHz

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DDSU |
| Special Slot Configuration (Note 2) |  |  | 11D+3G+0U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 60 |
|  | dl-UL-TransmissionPeriodicity | ms | 1 |
|  | nrofDownlinkSlots |  | 2 |
|  | nrofDownlinkSymbols |  | 11 |
|  | nrofUplinkSlot |  | 1 |
|  | nrofUplinkSymbols |  | 0 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | K1 = 3 if mod(i,4) = 0 K1 = 2 if mod(i,4) = 1 K1 = 5 if mod(i,4) = 2 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,39} |  |  |  |

Table C.2-4: Additional test parameters for TDD for SCS 120 KHz

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
| TDD Slot Configuration pattern (Note 1) |  |  | DDDSU |
| Special Slot Configuration (Note 2) |  |  | 10D+2G+2U |
| UL-DL configuration (tdd-UL-DL-ConfigurationCommon) | referenceSubcarrierSpacing | kHz | 120 |
|  | dl-UL-TransmissionPeriodicity | ms | 0.625 |
|  | nrofDownlinkSlots |  | 3 |
|  | nrofDownlinkSymbols |  | 10 |
|  | nrofUplinkSlot |  | 1 |
|  | nrofUplinkSymbols |  | 2 |
| K1 value (PDSCH-to-HARQ-timing-indicator) |  |  | K1 = [4] if mod(i,5) = 0 K1 = [3] if mod(i,5) = 1 K1 = [2] if mod(i,5) = 2 K1 = [6] if mod(i,5) = 3 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index per frame; i = {0,…,79} |  |  |  |

- C.3	Connection

- C.3.0	Measurement of Transmitter Characteristics

Unless otherwise stated, Table C.3.0-1 is applicable for measurements on the Transmitter Characteristics (clause 6).

Table C.3.0-1: Downlink Physical Channels transmitted during a connection (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| SSS transmit power | W | Test specific |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH to SSS | dB | 0 |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |
| EPRE ratio of PDCCH to SSS | dB | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |
| EPRE ratio of PDSCH to SSS | dB | 0 |
| EPRE ratio of PDSCH to PDSCH DMRS (Note 1) | dB | -3 |
| EPRE ratio of CSI-RS to SSS | dB | 0 |
| EPRE ratio of PTRS to PDSCH | dB | Test specific |
| EPRE ratio of OCNG DMRS to SSS | dB | 0 |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) | dB | 0 |
| Note 1: No boosting is applied to any of the channels except PDSCH DMRS. For PDSCH DMRS, 3 dB power boosting is applied assuming DMRS Type 1 configuration when DMRS and PDSCH are TDM’ed and only half of the DMRS REs are occupied. Note 2: Number of DMRS CDM groups without data for PDSCH DMRS configuration for OCNG is set to 1. |  |  |

- C.3.1	Measurement of Receiver Characteristics

Unless otherwise stated, Table C.3.1-1 is applicable for measurements on the Receiver Characteristics (clause 7). For Adjacent channel selectivity testing, Table C.3.1-2 is applied.

Table C.3.1-1: Downlink Physical Channels transmitted during a connection (TDD)

| Parameter | Unit | Value |
| --- | --- | --- |
| SSS transmit power | W | Test specific |
| EPRE ratio of PSS to SSS | dB | 0 |
| EPRE ratio of PBCH to SSS | dB | 0 |
| EPRE ratio of PBCH to PBCH DMRS | dB | 0 |
| EPRE ratio of PDCCH to SSS | dB | 0 |
| EPRE ratio of PDCCH to PDCCH DMRS | dB | 0 |
| EPRE ratio of PDSCH to SSS | dB | 0 |
| EPRE ratio of PDSCH to PDSCH DMRS (Note 1) | dB | -3 |
| EPRE ratio of CSI-RS to SSS | dB | 0 |
| EPRE ratio of PTRS to PDSCH | dB | Test specific |
| EPRE ratio of OCNG DMRS to SSS | dB | 0 |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) | dB | 0 |
| Note 1: No boosting is applied to any of the channels except PDSCH DMRS. For PDSCH DMRS, 3 dB power boosting is applied assuming DMRS Type 1 configuration when DMRS and PDSCH are TDM’ed and only half of the DMRS REs are occupied. Note 2: Number of DMRS CDM groups without data for PDSCH DMRS configuration for OCNG is set to 1. |  |  |

Table C.3.1-2: PDCCH Aggregation Level for ACS testing

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| Aggregation level | CCE | 4 | CBW=50MHz when SCS=120kHz |
|  |  | 8 | CBW=50MHz when SCS=60kHz CBW=100MHz when SCS=120kHz |
|  |  | 16 | CBW>100 MHz when SCS=60kHz CBW>100 MHz when SCS=120kHz |

- Annex G (normative): Uplink Physical Channels

- G.0	Uplink Signal Levels

Please refer to Annex G.0 in TS 38.521-1 [13].

- G.1	General

Please refer to Annex G.1 in TS 38.521-1 [13].

- G.2	Set-up

Please refer to Annex G.2 in TS 38.521-1 [13].

- G.3	Connection

Please refer to Annex G.3 in TS 38.521-1 [13].

- G.3.0	Measurement of Transmitter Characteristics

Please refer to Annex G.3.0 in TS 38.521-1 [13].

- G.3.1	Measurement of Receiver Characteristics

Please refer to Annex G.3.1 in TS 38.521-1 [13].

Annex G in TS 38.521-2 refers to Annex G in TS 38.521-1, and Annex G in TS 38.521-1 is as below:

- Annex G (normative):
Uplink Physical Channels

- G.0	Uplink Signal Levels

Uplink signal power is a UE figure, which is configured by the Test System by means of:

RRC messages (IE-s), such as:

-	PUSCH-PowerControl

-	PUCCH-PowerControl

-	RACH-ConfigGeneric

-	SRS-Config

and L1/2 Power control commands (TPC).

The uplink power settings are specified in the test case.

Otherwise, the uplink power settings result from the default RRC messages described in TS 38.508 [5], and appropriate TPC-s, which are sent to the UE to transmit with an UL power level necessary for maintaining the call during the test.

- G.1	General

This annex specifies the uplink physical channels that are needed for setting a connection and channels that are needed during a connection. Table G.1-1 describes the mapping of uplink physical channels and signals to physical resources

Table G.1-1: Mapping of uplink physical channels and signals to physical resources

| Physical channel | Time Domain Location | Frequency Domain Location | Note |
| --- | --- | --- | --- |
| PRACH | Allowed by the parameter prach-ConfigurationIndex provided by higher layers | Allowed by the parameter msg1-FrequencyStart provided by higher layers | Mapping rule is specified in TS 38.211 [8] Section 6.3.3 |
| DMRS | For DMRS on PUCCH format 1: Every other symbols i.e., 0, 2, 4... For DMRS on PUCCH format 2: All the PUCCH symbols For DMRS on PUCCH format 3,4: PUCCH length dependent For One symbol DMRS on PUSCH: Symbol 2,7 and 11 of each slot | DMRS on CP-OFDM PUSCH: Specified by the parameters dmrs-Type provided by higher layers. DMRS on DFT-OFDM PUSCH: Allowed for DMRS configuration type1 DMRS on PUCCH: PUCCH bandwidth dependent. | Mapping rule of DMRS for PUCCH is specified in TS 38.211 [8] Section 6.4.1.3 Mapping rule of DMRS for PUSCH is specified in TS 38.211 [8] Sections 6.4.1.1, 6.4.1.2 |
| PUCCH | For PUCCH Format 0: 1 ~ 2 symbols each slot, specified by the parameters of nrofSymbols and startingSymbolIndex in PUCCH-format0 provided by the higher layer. For PUCCH Format 1: 4 ~ 14 symbols each slot, specified by the parameters of nrofSymbols and those of startingSymbolIndex of PUCCH-format1 provided by the higher layer. For PUCCH Format 2, 1 ~ 2 symbols each slot, specified by the parameters of nrofSymbols and startingSymbolIndex in PUCCH-format2 provided by the higher layer. For PUCCH Format 3: 4 ~ 14 symbols each slot, allowed by the parameters of nrofSymbols and startingSymbolIndex in PUCCH-format3, provided by the higher layer. For PUCCH Format 4: 4 ~ 14 symbols each slot, specified by the parameters of nrofSymbols and startingSymbolIndex in PUCCH-format4, provided by higher layer. | For PUCCH Format 0, 1 1 RB, the position specified by the parameters of startingPRB and intraSlotFrequencyHopping in the corresponding PUCCH-Resource provided by the higher layer. For PUCCH Format 2, 3: 1~16 RBs, specified by the parameter of nrofPRBs in PUCCH-format2 and PUCCH-format3 respectively; additionally the position specified by the parameters of startingPRB and intraSlotFrequencyHopping in the corresponding PUCCH-Resource provided by the higher layer. For PUCCH Format 4 1 RB, the position specified by the parameters of startingPRB and intraSlotFrequencyHopping in the corresponding PUCCH-Resource provided by the higher layer | Mapping rule is specified in TS 38.211 [8] Section 6.3.2 and 38.213 [9] Section 9.2 |
| PUSCH | All remaining uplink symbols of each slot not allocated to DMRS | RBs allocated according to Reference Measurement channel in Annex A.2 | Mapping rule is specified in TS 38.211 [8] Section 6.3 and 38.214 [12] Section 6.1 |
| SRS | 1, 2, or 4 symbols among the last 6 symbols in each SRS transmission slot specified by the parameters of resourceMapping, and resourceType in SRS-Config provided by the higher layer. | RBs specified by the ue-specific parameters of freqDomainPosition, freqDomainShift and freqHopping in SRS-Config provided by the higher layer. | Mapping rule is specified in TS 38.211 [8] Section 6.4.1.4.3 |

- G.2	Set-up

Table G.2-1 describes the uplink physical channels that are required for connection set up.

Table G.2-1: Uplink Physical Channels required for connection set-up

Physical Channel

PRACH

PUCCH

PUSCH

PUCCH DMRS

PUSCH DMRS

SRS

In case of supplementary test, Table G.2-2 describes the supplementary uplink physical channels that are required for connection set-up, and unless stated otherwise, there is no other uplink physical channels configured on the NON-SUL carrier except PRACH.

Table G.2-2: Supplementary Uplink Physical Channels required for connection set-up

Physical Channel

PRACH

DMRS

PUCCH

PUSCH

- G.3	Connection

The following clauses describes the uplink physical channels that are transmitted during a connection i.e., when measurements are done.

- G.3.0	Measurement of Transmitter Characteristics

As specified in the test case. Otherwise:

-	PUSCH + DMRS for PUSCH (and DMRS) measurements.

-	PUCCH + DMRS for PUCCH (and DMRS) measurements.

-	PRACH for PRACH measurements.

SRS for SRS measurements.

- G.3.1	Measurement of Receiver Characteristics

As specified in the test case. Otherwise:

-	PUSCH + DMRS for measurements with uplink interference configured.

-	PUCCH + DMRS for measurements without uplink interference configured.

- G.3.2	Measurement of Performance Requirements

As specified in the test case. Otherwise:

PUCCH + DMRS for measurements without CSI feedback, or with CSI feedback in PUCCH mode.

PUSCH + DMRS for measurements with CSI feedback in PUSCH mode.

Observation 1: Annexes C.0 and G.0 define the default downlink and uplink power settings.

Observation 2: Annexes C.2 and G.2 describe the default downlink and uplink Physical Channels that are required for connection set up.

Observation 3: Annexes C.3.0 and G.3.0 describes the default downlink and uplink physical channels that are transmitted during a connection when measurements of Transmitter Characteristics are done.

Observation 4: Annexes C.3.1 and G.3.1 describes the default downlink and uplink physical channels that are transmitted during a connection when measurements of Receiver Characteristics are done.

Section 5.2.1.1 of TS 38.508-1 is as below；

- 5.2.1	Signal levels

- 5.2.1.1	Signal Levels for conducted testing

For NR FR1 cell, the downlink power settings are specified in TS 38.521-1[14] and TS 38.521-3[16].

The uncertainty value is specified in TS 38.521-1 [14] Annex F or in TS 38.521-2 [15] Annex F.

- 5.2.1.2	Signal Levels for OTA testing

- 5.2.1.2.1	Downlink Signal Levels

For E-UTRA cell in EN-DC with FR2 NR, the downlink power settings are specified in clause 4.7 of TS 38.521-3[16].

For FR2 NR cell, the downlink power settings are specified in Annex C.0 of TS 38.521-2[15] and Annex C.0 of TS 38.521-3[16].

Observation 5: There is no section 5.2.1.1.1 in TS 38.508-1 and the Signal Levels of FR2 NR cell for OTA testing refer to Annex C.0 of TS 38.521-2.

Option 1: Update the description of initial set up for downlink and uplink signals as below:

For TX test cases:

3.	Downlink signals are initially set up according to Annex C.0, C.2 and C.3.0, and uplink signals according to Annex G.0, G.2 and G.3.0.

For RX test cases:

3.	Downlink signals are initially set up according to Annex C.0, C.2 and C.3.1, and uplink signals according to Annex G.0, G.2 and G.3.1.

Option 2: Update the description of initial set up for downlink and uplink signals as below for both TX and RX test cases:

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

Option 1 is more accurate but complex and easy to make mistakes as the current spec. Option 2 is simple and with no ambiguous as Annex C and Annex G are clearly enough.

Proposal: Update the description of initial set up for downlink and uplink signals in TS 38.521-2 with Option 2.

Conclusions

Observation 1: Annexes C.0 and G.0 define the default downlink and uplink power settings.

Observation 2: Annexes C.2 and G.2 describe the default downlink and uplink Physical Channels that are required for connection set up.

Observation 3: Annexes C.3.0 and G.3.0 describes the default downlink and uplink physical channels that are transmitted during a connection when measurements of Transmitter Characteristics are done.

Observation 4: Annexes C.3.1 and G.3.2 describes the default downlink and uplink physical channels that are transmitted during a connection when measurements of Receiver Characteristics are done.

Observation 5: There is no section 5.2.1.1.1 in TS 38.508-1 and the Signal Levels of FR2 NR cell for OTA testing in TS 38.508-1  refer to Annex C.0 of TS 38.521-2.

Option 1: Update the description of initial set up for downlink and uplink signals as below:

For TX test cases:

3.	Downlink signals are initially set up according to Annex C.0, C.2 and C.3.0, and uplink signals according to Annex G.0, G.2 and G.3.0.

For RX test cases:

3.	Downlink signals are initially set up according to Annex C.0, C.2 and C.3.1, and uplink signals according to Annex G.0, G.2 and G.3.1.

Option 2: Update the description of initial set up for downlink and uplink signals as below for both TX and RX test cases:

3.	Downlink signals are initially set up according to Annex C, and uplink signals according to Annex G.

Option 1 is more accurate but complex and easy to make mistakes as the current spec. Option 2 is simple and with no ambiguous as Annex C and Annex G are clearly enough.

Proposal: Update the description of initial set up for downlink and uplink signals in TS 38.521-2 with Option 2.