3GPP TSG-RAN5 Meeting #93-e	R5-217250

# Electronic Meeting, 8th - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0647 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of Table 6.2.2.4.1-9 for Test Frequency |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CAICT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Test Frequency in Table 6.2.2.4.1-9 is not correct with Mid range missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Mid range was added in Table 6.2.2.4.1-9. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The spec will remain incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< Unchanged sections omitted >>

Table 6.2.2.4.1-9: Test Configuration Table (Power Class 2, 3 and 4, MPRWT, BWchannel = 400 MHz)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | 400 MHz |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | 120kHz |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Maximum Power Reduction (MPR) test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_1RB_Right |
| 3 | Mid |  |  |  | DFT-s-OFDM PI/2 BPSK | Outer_Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK | Outer_1RB_Right |
| 6 | Mid |  |  |  | DFT-s-OFDM QPSK | Outer_Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM | Outer_1RB_Right |
| 9 | Mid |  |  |  | DFT-s-OFDM 16 QAM | Outer_Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM | Outer_1RB_Right |
| 12 | Mid |  |  |  | DFT-s-OFDM 64 QAM | Outer_Full |
| 13 | Low |  |  |  | CP-OFDM QPSK | Outer_1RB_Left |
| 14 | High |  |  |  | CP-OFDM QPSK | Outer_1RB_Right |
| 15 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 16 | Low |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Left |
| 17 | High |  |  |  | CP-OFDM 16 QAM | Outer_1RB_Right |
| 18 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 19 | Low |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 64 QAM | Outer_1RB_Right |
| 21 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1. |  |  |  |  |  |  |

<< End of changes >>