3GPP TSG-RAN5 Meeting #93-e	R5-217270

# Electronic Meeting, 8th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2602 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR V2X test case 12.1.6.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of new NR V2X test case 12.1.6.4 in accordance with work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new NR V2X test case 12.1.6.4: PC5-only operation / Sidelink failure / Sidelink radio link failure / Reception side |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 12.1.6.4 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0184 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

12.1.6.4	PC5-only operation / Sidelink failure / Sidelink radio link failure / Reception side

12.1.6.4.1	Test Purpose (TP)

(1)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE fails the integrity check on SL SRB2 }

then { UE releases PC5-RRC connection and indicates the release to upper layer }

}

(2)

with { UE has established PC5 RRC connection with peer UE on unicast sidelink }

ensure that {

when { UE fails the integrity check on SL SRB3 }

then { UE releases PC5-RRC connection and indicates the release to upper layer }

}

12.1.6.4.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 38.331 [22], subclause 5.8.9.3. Unless otherwise stated these are Rel-16 requirements.

[TS 38.331, clause 5.8.9.3]

The UE shall:

…

1>	upon integrity check failure indication from sidelink PDCP entity concerning SL-SRB2 or SL-SRB3 for a specific destination:

2>	consider sidelink radio link failure to be detected for this destination;

2>	release the DRBs of this destination, in according to sub-clause 5.8.9.1a.1;

2>	release the SRBs of this destination, in according to sub-clause 5.8.9.1a.3;

2>	discard the NR sidelink communication related configuration of this destination;

2>	reset the sidelink specific MAC of this destination;

2>	consider the PC5-RRC connection is released for the destination;

2>	indicate the release of the PC5-RRC connection to the upper layers for this destination (i.e. PC5 is unavailable);

2>	if UE is in RRC_CONNECTED:

3>	perform the sidelink UE information for NR sidelink communication procedure, as specified in 5.8.3.3;

NOTE:	It is up to UE implementation on whether and how to indicate to upper layers to maintain the keep-alive procedure [55].

12.1.6.4.3	Test description

12.1.6.4.3.1	Pre-test conditions

System Simulator:

-	SS-UE

-	Operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	GNSS simulator

UE:

- UE is authorised to perform NR sidelink communication.

Preamble:

-	The UE is in state 4-A as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On).

12.1.6.4.3.2	Test procedure sequence

Table 12.1.6.4.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS-UE transmits a DIRECT LINK IDENTIFIER UPDATE REQUEST message integrity protected on SL-SRB2. The MAC-I is corrupted so as to result in integrity failure at UE. | <-- | PC5-S: DIRECT LINK IDENTIFIER UPDATE REQUEST | - | - |
| 2 | Check: Does the UE transmit a DIRECT LINK RELEASE REQUEST message. | --> | PC5-S: DIRECT LINK RELEASE REQUEST | 1 | P |
| 3 | SS-UE transmits a DIRECT LINK RELEASE ACCEPT message. | <-- | PC5-S: DIRECT LINK RELEASE ACCEPT | - | - |
| 4-10 | Steps 1 to 7 of the Procedure for establishing unicast mode sidelink communication in TS 38.508-1 [4] Table 4.9.22.2.2-1 are executed to successfully complete. | - | - | - | - |
| 11 | The SS-UE transmits an RRCReconfigurationSidelink message integrity protected on SL-SRB3. The MAC-I is corrupted so as to result in integrity failure at UE. | <-- | PC5 RRC: RRCReconfigurationSidelink | - | - |
| 12 | Check: Does the UE transmit a DIRECT LINK RELEASE REQUEST message. | --> | PC5-S: DIRECT LINK RELEASE REQUEST | 2 | P |
| 13 | The SS-UE transmits a DIRECT LINK RELEASE ACCEPT message. | <-- | PC5-S: DIRECT LINK RELEASE ACCEPT | - | - |

12.1.6.4.3.3	Specific message contents

None

<End of Changed Section>