3GPP TSG-RAN5 Meeting #93-e	R5-217287

# Electronic Meeting, 8th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2615 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of NR V2X test case 13.2.5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | TDIA, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of new NR V2X test case 13.2.5 in accordance with work plan |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new NR V2X test case 13.2.5: PC5 unicast / link identifier update |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan will not be complete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 13.2.5(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS/TR 38.523-2 CR 0184 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Changed Section>

13.2.5	PC5 unicast / link identifier update

13.2.5.1	Test Purpose (TP)

(1)

with { UE has established a DIRECT LINK with Layer 2 ID-1 to a SS-UE }

ensure that {

when { UE receives a DIRECT LINK IDENTIFIER UPDATE REQUEST message with same Layer 2 ID-1 }

then { UE transmits a DIRECT LINK IDENTIFIER UPDATE REJECT message }

}

(2)

with { UE has established a DIRECT LINK with old Layer 2 ID-1 to a SS-UE. UE receives a DIRECT LINK IDENTIFIER UPDATE REQUEST message and respond with a DIRECT LINK IDENTIFIER UPDATE ACCEPT message with new Layer 2 ID-2 }

ensure that {

when { UE received a V2X packet from the SS-UE }

then { the Layer 2 ID associated with the V2X packet should be old Layer 2 ID-1 }

}

(3)

with { UE has established a DIRECT LINK with old Layer 2 ID-1 to a SS-UE. UE receives a DIRECT LINK IDENTIFIER UPDATE REQUEST message and respond with a DIRECT LINK IDENTIFIER UPDATE ACCEPT message with new Layer 2 ID-2 }

ensure that {

when { UE received a DIRECT LINK IDENTIFIER UPDATE ACK message from the SS-UE }

then { UE transmit a V2X packet and the Layer 2 ID associated with the V2X packet should be new Layer 2 ID-2 }

}

13.2.5.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.587 [FFS], subclause 6.1.2.5.3, 6.1.2.5.4 and 6.1.2.5.6. Unless otherwise stated these are Rel-16 requirements.

[TS 24.587, subclause 6.1.2.5.3]

Upon receipt of a DIRECT LINK IDENTIFIER UPDATE REQUEST message, if the target UE determines:

a)	the PC5 unicast link associated with this request message is still valid; and

b)	the timer T5010 for the PC5 unicast link identified by this request message is not running,

then the target UE accepts this request, and responds with a DIRECT LINK IDENTIFIER UPDATE ACCEPT message.

The target UE shall create the DIRECT LINK IDENTIFIER UPDATE ACCEPT message. In this message, the target UE:

a)	shall include the target UE's new layer-2 ID assigned by itself;

b)	shall include the new LSB of KNRP-sess ID;

c)  shall include the initiating UE's new MSB of KNRP-sess ID;

d)  shall include the initiating UE's new layer-2 ID;

e)	shall include the target UE's new application layer ID if received from upper layer;

f)	shall include the initiating UE's new IP address/prefix if received from the initiating UE and IP communication is used;

g)	shall include the initiating UE's new application layer ID if received from the initiating UE; and

h)	shall include the target UE's new IP address/prefix if IP communication is used and changed.

After the DIRECT LINK IDENTIFIER UPDATE ACCEPT message is generated, the target UE shall pass this message to the lower layers for transmission along with the initiating UE's old layer-2 ID for unicast communication and the target UE's old layer-2 ID for unicast communication, and start timer T5010. The UE shall not send a new DIRECT LINK IDENTIFIER UPDATE ACCEPT message to the same initiating UE while timer T5010 is running.

Before target UE receives the traffic using the new layer-2 IDs, the target UE shall continue to receive the traffic with the old layer-2 IDs (i.e. initiating UE's old layer-2 ID and target UE's old layer-2 ID) from initiating UE.

Before target UE receives the DIRECT LINK IDENTIFIER UPDATE ACK message from initiating UE, the target UE shall keep sending traffic to the initiating UE using the old layer-2 IDs (i.e. initiating UE's old layer-2 ID for unicast communication and target UE's old layer-2 ID for unicast communication).

[24.587, subclause 6.1.2.5.4]

Upon receipt of the DIRECT LINK IDENTIFIER UPDATE ACCEPT message, the initiating UE shall stop timer T5009 and respond with a DIRECT LINK IDENTIFIER UPDATE ACK message. In this message, the initiating UE:

a)	shall include the target UE's new layer-2 ID;

b)	shall include the target UE's new LSB of KNRP-sess ID;

c)	shall include the target UE's new application layer ID, if received; and

d)	shall include the target UE's new IP address/prefix, if received.

After the DIRECT LINK IDENTIFIER UPDATE ACK message is generated, the initiating UE shall pass this message to the lower layers for transmission along with the initiating UE's old layer-2 ID for unicast communication and the target UE's old layer-2 ID for unicast communication and shall stop timer T5011 if running and start a timer T5011 as configured if at least one of V2X service identifiers for the PC5 unicast link satisfying the privacy requirements as specified in clause 5.2.3.

Upon sending the DIRECT LINK IDENTIFIER UPDATE ACK message, the initiating UE shall update the associated PC5 unicast link context with the new identifiers and pass the new layer-2 IDs (i.e. initiating UE's new layer-2 ID for unicast communication and target UE's new layer-2 ID for unicast communication if changed) along with the PC5 link identifier down to the lower layer. Then the initiating UE shall use the new layer-2 IDs (i.e. initiating UE's new layer-2 ID for unicast communication and target UE's new layer-2 ID for unicast communication if changed) to transmit the PC5 signalling message and PC5 user plane data.

The initiating UE shall continue to receive traffic with the old layer-2 IDs (i.e. initiating UE's old layer-2 ID for unicast communication and target UE's old layer-2 ID for unicast communication) from the target UE until it receives traffic with the new layer-2 IDs (i.e. initiating UE's new layer-2 ID and target UE's new layer-2 ID if changed) from the target UE.

[24.587, subclause 6.1.2.5.6]

If the DIRECT LINK IDENTIFIER UPDATE REQUEST message cannot be accepted, the target UE shall send a DIRECT LINK IDENTIFIER UPDATE REJECT message. The DIRECT LINK IDENTIFIER UPDATE REJECT message contains a PC5 signalling protocol cause IE set to one of the following cause values:

#3	conflict of layer-2 ID for unicast communication is detected; or

#111	protocol error, unspecified.

For a received DIRECT LINK IDENTIFIER UPDATE REQUEST message from a layer-2 ID (for unicast communication), if the target UE already has an existing link using this layer-2 ID or is currently processing a DIRECT LINK IDENTIFIER UPDATE REQUEST message from the same layer-2 ID, but with user info different from the user info IE included in this new incoming message, the target UE shall send a DIRECT LINK IDENTIFIER UPDATE REJECT message with PC5 signalling protocol cause value #3 "conflict of layer-2 ID for unicast communication is detected".

NOTE:	After receiving the DIRECT LINK IDENTIFIER UPDATE REJECT message, whether the initiating UE initiates the PC5 unicast link release procedure or initiates another PC5 unicast link identifier update procedure with a new layer-2 ID depends on UE implementation.

For other reasons causing the failure of link identifier update, the target UE shall send a DIRECT LINK IDENTIFIER UPDATE REJECT message with PC5 signalling protocol cause value #111 "protocol error, unspecified".

Upon receipt of the DIRECT LINK IDENTIFIER UPDATE REJECT message, the initiating UE shall stop timer T5009 and abort this PC5 unicast link identifier update procedure.

13.2.5.3	Test description

13.2.5.3.1	Pre-test conditions

System Simulator:

-	SS-UE

-	Operating as NR sidelink communication device on the resources (i.e. the frequency included in pre-configuration) that UE is expected to use for transmission and reception via PC5 interface.

-	GNSS simulator

UE:

- UE is authorised to perform NR sidelink communication.

Preamble:

-	The UE is in state 4-A and Test Loop Function (On) with UE test loop mode E as defined in TS 38.508-1 [4], subclause 4.4A using generic procedure parameter Sidelink (On), Cast Type (Unicast), GNSS Sync (On).

13.2.5.3.2	Test procedure sequence

Table 13.2.5.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS-UE transmits a DIRECT LINK IDENTIFIER UPDATE REQUEST message including new Layer 2 ID. | <-- | PC5-S: DIRECT LINK IDENTIFIER UPDATE REQUEST | - | - |
| 2 | The UE transmits a DIRECT LINK IDENTIFIER UPDATE ACCEPT message. | --> | PC5-S: DIRECT LINK IDENTIFIER UPDATE ACCEPT | - | - |
| 3 | The SS-UE transmits one V2X packet on old Layer 2 ID to the UE. | <-- | V2X packet | - | - |
| 4 | Check: Does the UE loop back the V2X packet on old Layer 2 ID? | - | - | 2 | P |
| 5 | The UE transmits a DIRECT LINK IDENTIFIER UPDATE ACCEPT message. | --> | PC5-S: DIRECT LINK IDENTIFIER UPDATE ACCEPT | - | - |
| 6 | The SS-UE transmits a DIRECT LINK IDENTIFIER UPDATE ACK message. | <-- | PC5-S: DIRECT LINK IDENTIFIER UPDATE ACK | - | - |
| 7 | The SS-UE transmits one V2X packet on new Layer 2 ID to the UE. | <-- | V2X packet | - | - |
| 8 | Check: Does the UE loop back the V2X packet on new Layer 2 ID? | - | - | 3 | P |
| 9 | The SS-UE transmits a DIRECT LINK IDENTIFIER UPDATE REQUEST message including same Layer 2 ID. | <-- | PC5-S: DIRECT LINK IDENTIFIER UPDATE REQUEST | - | - |
| 10 | Check: Does the UE transmit a DIRECT LINK IDENTIFIER UPDATE REJECT message? | --> | PC5-S: DIRECT LINK IDENTIFIER UPDATE REJECT | 1 | P |

13.2.5.3.3	Specific message contents

FFS

<End of Changed Section>