3GPP TSG-RAN WG5 Meeting #93-e	R5-217349

# Electronic meeting, 8th - 19th Nov, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.522 | CR | 0118 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 3.1 for definitions of CA and DC configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The definitions for DL_NR_nCC(table_index) and UL_NR_nCC(table_index) are missing in clause 3.1. In addition, some editorial corrections should be made in clause 3.1 and Table 4.0-3. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add the definitions for DL_NR_nCC(table_index) and UL_NR_nCC(table_index) in clause 3.1. - Editorial corrections in clause 3.1 and Table 4.0-3. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The definitions for DL_NR_nCC and UL_NR_nCC will be missing and some typos will remain. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 4.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<< start of changes  >>

<< Unchanged sections omitted >>

3.1	Definitions

For the purposes of the present document, the terms and definitions given in TR 21.905 [9] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in TR 21.905 [9].

DL_nCC: DL_nCC(table_index) includes all supported n-carrier CA/DC configurations in Table table_index in TS 38.508-2 [8].

DL_NR_nCC: DL_NR_nCC(table_index) includes all supported DC configurations with n-carrier NR DL CA configuration in Table table_index in TS 38.508-2 [8].

EIRP(Link=Link angle, Meas=Link angle): measurement of the UE such that the link angle is aligned with the measurement angle. EIRP (indicator to be measured) can be replaced by EIS, Frequency, EVM, carrier Leakage, In-band emission and OBW. Beam peak search grids, TX beam peak direction, and RX beam peak direction can be selected to describe Link.

EIRP(Link=Link angle, Meas=beam peak direction): measurement of the EIRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement error uncertainty.

Implementation Conformance Statement (ICS): statement made by the supplier of an implementation or system claimed to conform to a given specification, stating which capabilities have been implemented

ICS proforma: document, in the form of a questionnaire, which when completed for an implementation or system becomes an ICS

Implementation extra Information for Testing (IXIT): A statement made by a supplier or implementer of an UEUT which contains or references all of the information (in addition to that given in the ICS) related to the UEUT and its testing environment, which will enable the test laboratory to run an appropriate test suite against the UEUT

Inter-band carrier aggregation: Carrier aggregation of component carriers in different operating bands.

NOTE:	Carriers aggregated in each band can be contiguous or non-contiguous.

Intra-band contiguous carrier aggregation: Contiguous carriers aggregated in the same operating band.

Intra-band non-contiguous carrier aggregation: Non-contiguous carriers aggregated in the same operating band.

IXIT proforma: A document, in the form of a questionnaire, which when completed for an UEUT becomes an IXIT

Protocol Implementation Conformance Statement (PICS): An ICS for an implementation or system claimed to conform to a given protocol specification

Protocol Implementation eXtra Information for Testing (PIXIT): An IXIT related to testing for conformance to a given protocol specification

Static conformance review: A review of the extent to which the static conformance requirements are claimed to be supported by the UEUT, by comparing the answers in the ICS(s) with the static conformance requirements expressed in the relevant specification(s)

TRP(Link=Link angle): measurement of the TRP of the UE such that the measurement angle is aligned with the beam peak direction within an acceptable measurement uncertainty. TX beam peak direction and RX beam peak direction can be selected to describe Link.

NOTE:	For requirements based on EIRP/EIS, the radiated interface boundary is associated to the far-field region

UL: UL(table_index) includes all supported CA Configurations where at least one UL CA configuration was declared in column "Supported CA Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8].

UL_nCC: UL_nCC(table_index) includes all supported CA Configurations where at least one n-carrier UL CA configuration was declared in column "Supported CA Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8].

UL_NR_nCC: UL_NR_nCC(table_index) includes all supported DC Configurations where at least one DC configuration with n-carrier NR UL CA configuration was declared in column "Supported EN-DC Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8].

ULTxSwitching: ULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported ULTxSwitching Band Pair" in Table table_index in TS 38.508-2 [8].

<< Unchanged sections omitted >>

4.0	Test case conditions and selection criteria

For the purposes of the present document, the applicability of conformance test cases conditions given in Table 4.0-1 apply. The tested bands selection criteria given in Table 4.0-2 apply. The tested CA/DC configuration selection criteria given in Table 4.0-3 apply. The ICS proformas used in Table 4.0-1, Table 4.0-2 and Table 4.0-3 are defined in TS 38.508-2 [8] unless otherwise stated.

Table 4.0-1: Applicability of conformance test cases conditions

C001 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C001a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.1-7/3 THEN R ELSE N/A

C001b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C002 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C003 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C003b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) AND (A.4.3.2-1/58 OR A.4.3.2-1/59 OR A.4.3.2-1/60) THEN R ELSE N/A

C003a IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C004 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-2/1 THEN R ELSE N/A

C005 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.1-2/4 AND A.4.3.2A.1-1/1 AND A.4.1-3/1 THEN R ELSE N/A

C006 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 THEN R ELSE N/A

C006a IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C006b IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/31a THEN R ELSE N/A

C007 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.2-1/22 THEN R ELSE N/A

C008 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND NOT(A.4.3.2-1/22) THEN R ELSE N/A

C009 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C009a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C009z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/1 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C010 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C010a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C010z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/2 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C011 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C011a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-1/1 THEN R ELSE N/A

C011b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2A/1 THEN R ELSE N/A

C011c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-1A/1 THEN R ELSE N/A

C011z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/3 AND A.4.3.2B.2.0-2A/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C012 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/1 THEN R ELSE N/A

C012a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1A/1 THEN R ELSE N/A

C012b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/2 THEN R ELSE N/A

C012c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/3 THEN R ELSE N/A

C012d IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/4 THEN R ELSE N/A

C012e IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A

C012f IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2/1 AND A.4.3.2-1/31a THEN R ELSE N/A

C012z IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.1-4/4 AND A.4.3.2B.2.0-2A/1 AND A.4.3.2-1/25 THEN R ELSE N/A

C013 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (A.4.1-4/3 OR A.4.1-4/4) THEN R ELSE N/A

C014 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/4) THEN R ELSE N/A

C015 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/6 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015x IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C015y IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/33 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C016 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016b IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/6 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016x IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C016y IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/20 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C017 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C017b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/6 THEN R ELSE N/A

C017x IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.9-1/1 THEN R ELSE N/A

C017y IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.2-1/33 THEN R ELSE N/A

C018 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A

C019 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C019b IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.2-1/6 THEN R ELSE N/A

C019x IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.9-1/1 THEN R ELSE N/A

C020 IF (A.4.1-1/1 OR A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) THEN R ELSE N/A

C021 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C021a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C021b IF A.4.1-1/1 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C022 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 THEN R ELSE N/A

C022a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C023 IF A.4.1-4/5 AND A.4.1-3/2 THEN R ELSE N/A

C023a IF A.4.1-4/5 AND A.4.1-3/2 AND A.4.3.5-1/1 THEN R ELSE N/A

C024 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 THEN R ELSE N/A

C025 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 THEN R ELSE N/A

C025a IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-3/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C026 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND 4.3.6-1/11 THEN R ELSE N/A

C027 Void

C028 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.6-1/11 THEN R ELSE N/A

C029 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/1 AND 4.3.2-1/9 THEN R ELSE N/A

C030 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/9 THEN R ELSE N/A

C030a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/9 THEN R ELSE N/A

C031 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C032 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C033 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/2 THEN R ELSE N/A

C034 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/6 THEN R ELSE N/A

C035 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/6 THEN R ELSE N/A

C036 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.3.2A.1-1/3 THEN R ELSE N/A

C037 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5 OR A.4.1-4A/7) AND A.4.1-5/1 AND A.4.3.6-1/41 THEN R ELSE N/A

C037a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5 OR A.4.1-4A/7) AND A.4.1-5/1 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C037b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5 OR A.4.1-4A/7) AND A.4.1-5/1 AND A.4.3.5-1/1 THEN R ELSE N/A

C037c IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5 OR A.4.1-4A/7) AND A.4.1-5/1 THEN R ELSE N/A

C038 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 THEN R ELSE N/A

C038a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C039 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4/5 OR A.4.1-4/7) AND A.4.1-5/1 AND A.4.3.6-1/41 THEN R ELSE N/A

C040 IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.6-1/41 THEN R ELSE N/A

C041 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 THEN R ELSE N/A

C041a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C041b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-5/1 AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C042 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 THEN R ELSE N/A

C042a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C042b IF A.4.1-1/1 AND A.4.1-2/7 AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C043 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND (4.3.6-1/43 OR 4.3.6-1/44) THEN R ELSE N/A

C043a IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.3.2-1/34 AND A.4.3.6-1/41 AND A.4.3.5-1/1 THEN R ELSE N/A

C044 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND 4.3.6-1/42 THEN R ELSE N/A

C045 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/2 AND (A.4.1-4/1 OR (A.4.1-4/3 AND A.4.3.2B.2.0-1A/2)) THEN R ELSE N/A

C046 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/3 AND (A.4.1-4/1 OR (A.4.1-4/3 AND A.4.3.2B.2.0-1A/3)) THEN R ELSE N/A

C047 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/4 AND (A.4.1-4/3 AND A.4.3.2B.2.0-1A/4) THEN R ELSE N/A

C048 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/2 AND A.4.1-4/1 THEN R ELSE N/A

C049 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND A.4.3.2B.2.0-1/3 AND A.4.1-4/1 THEN R ELSE N/A

C050 IF A.4.1-1/2 AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/36 THEN R ELSE N/A

C051 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.1-4A/5 AND A.4.3.2A.1-2/1 AND A.4.3.2-1/37 THEN R ELSE N/A

C052 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.11-1/1 THEN R ELSE N/A

C053 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/1 THEN R ELSE N/A

C054 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/2 THEN R ELSE N/A

C055 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/3 THEN R ELSE N/A

C056 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/4 THEN R ELSE N/A

C057 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/5 THEN R ELSE N/A

C058 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/6 THEN R ELSE N/A

C059 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.1-4A/3 OR A.4.1-4A/4) AND A.4.3.2A.1-2/7 THEN R ELSE N/A

C060 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND (A.4.3.2-1/14 OR A.4.3.2-1/15) THEN R ELSE N/A

C061 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) THEN R ELSE N/A

C062 IF A.4.1-1/2 AND A.4.1.2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.9-1/1 THEN R ELSE N/A

C063 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/2 THEN R ELSE N/A

C064 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/3 THEN R ELSE N/A

C064a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/4 THEN R ELSE N/A

C064b IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-3/2 AND (A.4.1-4/3 OR A.4.1-4/2) AND A.4.3.2B.2.0-1A/5 THEN R ELSE N/A

C065 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) AND A.4.1-3/2 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) THEN R ELSE N/A

C065a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3) AND A.4.1-3/2 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C066 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) THEN R ELSE N/A

C066a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.2-1/42 OR A.4.3.2-1/43 OR A.4.3.2-1/44) AND (A.4.3.2-1/24 OR A.4.3.2-1/24A) AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C067 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND (A.4.1-4A/1 OR A.4.1-4A/2 OR A.4.1-4A/5) AND A.4.1-3/2 THEN R ELSE N/A

C068 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND [10] A.4.6-1/1 AND A.4.1-3/2 THEN R ELSE N/A

C069 IF (A.4.1-4/4 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/6 THEN R ELSE N/A

C070 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C071 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C072 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C073 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/41 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C074 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C075 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C076 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C077 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/39 AND A.4.3.2-1/40 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C078 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.1-2/3 OR A.4.1-2/5) AND A.4.1-4A/1 AND A.4.3.2A.1-1/1 THEN R ELSE N/A

C079 IF A.4.1-1/3 AND A.4.1-2/7 THEN R ELSE N/A

C080 IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND A.4.1-1/2 AND A.4.1-2/8 THEN R ELSE N/A

C080a IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND A.4.1-1/2 AND A.4.1-2/8 AND A.4.3.5-1/1 THEN R ELSE N/A

C081 IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/5) AND (A.4.3.6-1/46 OR A.4.3.6-1/47) THEN R ELSE N/A

C081a IF ([10]A.4.1-1/1 OR [10]A.4.1-1/2) AND (A.4.1-3/2 OR A.4.1-3/5) AND [10]A.4.4-1a/5 AND (A.4.3.6-1/46 OR A.4.3.6-1/47) THEN R ELSE N/A

C082 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/63 AND A.4.3.2-1/65 THEN R ELSE N/A

C082a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.2-1/63 AND A.4.3.2-1/65 AND A.4.3.5-1/1 THEN R ELSE N/A

C083 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/41 AND A.4.3.2-1/64 AND A.4.3.2-1/65 THEN R ELSE N/A

C083a IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.6-1/41 AND A.4.3.2-1/64 AND A.4.3.2-1/65 AND A.4.3.5-1/1 THEN R ELSE N/A

C084 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/63 AND A.4.3.2-1/65 THEN R ELSE N/A

C084a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/63 AND A.4.3.2-1/65 AND A.4.3.5-1/1THEN R ELSE N/A

C085 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/41 AND A.4.3.2-1/64 AND A.4.3.2-1/65 THEN R ELSE N/A

C085a IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/41 AND A.4.3.2-1/64 AND A.4.3.2-1/65 AND A.4.3.5-1/1 THEN R ELSE N/A

C086 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 THEN R ELSE N/A

C086a IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 AND A.4.3.5-1/1 THEN R ELSE N/A

C087 IF A.4.1-1/2 AND A.4.1.2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/3 THEN R ELSE N/A

C088 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/2 AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C089 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C090 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND NOT A.4.3.1-7a/3 AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C091 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND (NOT A.4.3.9-1/2 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C092 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.5-1/1 AND A.4.3.5-1/5 THEN R ELSE N/A

C093 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.6-1/45 THEN R ELSE N/A

C094 IF ((A.4.1-1/1 AND A.4.1-1/1) OR (A.4.1-1/1 AND A.4.1-1/2) OR (A.4.1-1/2 AND A.4.1-1/1) OR (A.4.1-1/2 AND A.4.1-1/2)) AND A.4.1-3/1 AND A.4.3.6-1/45 THEN R ELSE N/A

C095 IF A.4.1-1/2 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.6-1/45 THEN R ELSE N/A

C096 IF ((A.4.1-1/1 OR A.4.1-1/2) AND [10] A.4.1-1/3) THEN R ELSE N/A

C097 IF ((A.4.1-1/1 AND [10]A.4.1-1/1) OR (A.4.1-1/1 AND [10]A.4.1-1/2) OR (A.4.1-1/2 AND [10]A.4.1-1/1) OR (A.4.1-1/2 AND [10]A.4.1-1/2)) AND A.4.1-2/7 AND A.4.1-3/1 AND (A.4.3.11-1/1 OR A.4.3.11-1/4) THEN R ELSE N/A

C098 IF (A.4.1-4/1 OR A.4.1-4/2 OR A.4.1-4/3 OR A.4.1-4/5) AND A.4.1-3/2 AND A.4.3.5-1/1 AND (A.4.3.11-1/1 OR A.4.3.11-1/3) THEN R ELSE N/A THEN R ELSE N/A

C099 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.11-1/2 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C100 IF A.4.1-1/3 AND A.4.1-2/7 AND A.4.3.10-1/3 THEN R ELSE N/A

C101 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/9 THEN R ELSE N/A

C102 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/16 THEN R ELSE N/A

C103 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.8-1/15 AND A.4.3.8-1/18 THEN R ELSE N/A

C104 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.8-1/17 AND A.4.3.8-1/18 THEN R ELSE N/A

C105 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/11 THEN R ELSE N/A

C106 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/8 AND A.4.1-3/1 AND A.4.3.8-1/11 THEN R ELSE N/A

C107 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/15 THEN R ELSE N/A

C108 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/17 THEN R ELSE N/A

C109 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/15 AND A.4.3.8-1/18 THEN R ELSE N/A

C110 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.8-1/17 AND A.4.3.8-1/18 THEN R ELSE N/A

C111 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/31 AND A.4.3.2-1/57 THEN R ELSE N/A

C112 IF (A.4.1-1/1 OR A.4.1-1/2) AND A.4.1-2/7 AND A.4.1-3/1 AND A.4.3.2-1/31 AND A.4.3.2-1/57 AND (A.4.1-2/3 OR A.4.1-2/5) THEN R ELSE N/A

C113 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C113a IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C113b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C113c IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/53 AND A.4.3.2-1/56 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C114 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/54 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C114a IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/54 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C114b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/54 AND A.4.3.2-1/56 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C114c IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/54 AND A.4.3.2-1/56 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C115 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/55 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C115a IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/55 AND A.4.3.2-1/56 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C115b IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/55 AND A.4.3.2-1/56 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C115c IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/55 AND A.4.3.2-1/56 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C116 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/61 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C117 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/61 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C118 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/61 AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C119 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/61 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C120 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/12 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C121 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/62 AND NOT A.4.3.1-7a/2 THEN R ELSE N/A

C122 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/12 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C123 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/62 AND NOT A.4.3.1-7a/3 THEN R ELSE N/A

C124 IF A.4.1-1/1 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/12 AND AND (NOT A.4.3.9-1/2 AND A.4.3.9-4a/7 OR (A.4.3.9-4a/1 OR A.4.3.9-4a/2 OR A.4.3.9-4a/3 OR A.4.3.9-4a/66 OR A.4.3.9-4a/70)) THEN R ELSE N/A

C125 IF A.4.1-1/2 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) AND A.4.3.2-1/12 AND (A.4.3.9-4b/38 OR A.4.3.9-4b/41 OR A.4.3.9-4b/77 OR A.4.3.9-4b/78 OR A.4.3.9-4b/79) OR (A.4.3.9-4b/34 OR A.4.3.9-4b/39 OR A.4.3.9-4b/40 OR A.4.3.9-4b/48)) THEN R ELSE N/A

C126 IF A.4.1-1/2 AND A.4.1-2/8 AND (A.4.1-3/1 OR A.4.1-3/2 OR A.4.1-3/3 OR A.4.1-3/5) and A.4.3.2-1/3 THEN R ELSE N/A

NOTE 1: Cxxxx applicability is defined for enhanced type 1 receiver for NR related tests (A.4.3.9-1/1). NOTE 2: Cxxxy applicability is defined for alternative additional DMRS position for co-existence with LTE CRS related tests (A.4.3.2-1/20). NOTE 3: Cxxxz applicability is defined for modified MPR behaviour related test (A.4.3.2-1/25).

Table 4.0-2: Tested Bands Selection Criteria

| Code | Tested Bands Selection Criteria | Comment |
| --- | --- | --- |
| D001 | (A.4.3.1-1 OR A.4.3.1-2) AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FR1 Bands without SUL/SDL bands |
| D002 | A.4.3.1-4 | All supported FR1 PC2 Bands |
| D003 | A.4.3.1-5 | All supported FR1 SUL Bands |
| D004 | {1,2,3,5,7,8,12,14,20,25,28,30,34,38,39,40,41,50,51,65,66,70,71,74,75,76} | UE supported bands among n1,n2,n3,n5,n7,n8,n12,n14,n20,n25,n28,n30,n34,n38,n39,n40,n41,n50,n51,n65,n66,n70,n71,n74,n75,n76 |
| D005 | A.4.3.1-3 | All supported FR2 Bands |
| D006 | A.4.3.1-1 OR A.4.3.1-2 | All supported FR1 Bands |
| D007 | A.4.3.1-1 OR A.4.3.1-2 OR A.4.3.1-3 | All supported NR Bands |
| D008 | ANY((A.4.3.1-1) AND 10MHz) | Any FDD FR1 band within the set supporting 10 MHz UE Channel BW |
| D009 | ANY((A.4.3.1-2) AND 20MHz) | Any TDD FR1 band within the set supporting 20 MHz UE Channel BW |
| D010 | ANY((A.4.3.1-2) AND 40MHz) | Any TDD FR1 band within the set supporting 40 MHz UE Channel BW |
| D011 | A.4.3.9-4a OR A.4.3.9-4b | All supported 4 Rx antenna ports Bands |
| D012 | A.4.3.1-1 AND NOT (A.4.3.1-5 OR A.4.3.1-6) | All supported FDD bands without SUL/SDL bands |
| D013 | ANY((A.4.3.1-3) AND 50MHz) | Any TDD FR2 band within the set supporting 50 MHz UE Channel BW |
| D014 | ANY((A.4.3.1-3) AND 100MHz) | Any TDD FR2 band within the set supporting 100 MHz UE Channel BW |
| D015 | ANY((A.4.3.1-3) AND 200MHz) | Any TDD FR2 band within the set supporting 200 MHz UE Channel BW |
| D016 | A.4.3.1-9 | All supported FR1 sidelink bands |
| D017 | {40, 41, 77, 78, 79} | UE supported TDD bands among n40, n41, n77, n78, n79 |
| NOTE 1: Band Selection is based on set theory. For each feature, item number shall correspond to the Band number. The result is the set of bands for which the test shall be conducted. The following operators are used: AND: Set intersection ( ). {1,2} AND {2,3} = {2} OR: Set union ( ). {1,2} OR {2,3} = {1,2,3} NOT: Set complement (\), full set being all bands. NOT{1} = {2 ...256} Also note that this is set without repetitions so {1} AND {1} = {1} The following basic sets are used: {1,2}: Explicitly given band set 10MHz: All bands supporting 10 MHz |  |  |

Table 4.0-3: Tested CA/DC Configuration Selection Criteria

| Code | Tested CA/DC Configuration Selection Criteria | Comment |
| --- | --- | --- |
| E001 | DL_2CC(A.4.3.2A.2.1-3) AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.2.1-2) | All supported intra-band contiguous CA Configurations with 2 carriers in DL but no CA in UL |
| E002 | DL_2CC(A.4.3.2A.4.1-3) AND A.4.3.2B.2.0-1/1 AND NOT UL(A.4.3.2A.4.1-2) | All supported inter-band CA Configurations with 2 carriers in DL but no CA in UL |
| E003 | UL_2CC(A.4.3.2B.2.1-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band contiguous EN-DC configurations in FR1 (2UL CCs) |
| E003a | DL_2CC(A.4.3.2B.2.1-2) AND A.4.3.2B.2.0-1/1 | All supported Intra-band contiguous EN-DC configurations in FR1 (2DL CCs) |
| E004 | UL_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-2/1 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2UL CCs) |
| E004a | DL_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1/1 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2DL CCs) |
| E005 | UL_2CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-2/1 | All supported Inter-band EN-DC configurations within FR1 (2UL CCs) |
| E005a | DL_2CC(A.4.3.2B.2.3.1-2) AND A.4.3.2B.2.0-1/1 | All supported Inter-band EN-DC configurations within FR1 (2DL CCs) |
| E005b | UL_NR_1CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-2A/1 | All supported Inter-band EN-DC configurations within FR1 with 1 NR UL CC and one or more LTE UL CC(s) |
| E005c | DL_NR_1CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/1 | All supported Inter-band EN-DC configurations within FR1 with 1 NR DL CC and one or more LTE DL CC(s) |
| E005d | A.4.3.2B.2.3.1-3 | All supported PC2 Inter-band EN-DC configurations within FR1 |
| E006 | DL_3CC(A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/2 | All supported EN-DC configurations within FR1 (3DL CCs) |
| E007 | DL_4CC(A.4.3.2B.2.1-2 OR A.4.3.2B.2.2-2 OR A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/3 | All supported EN-DC configurations within FR1 (4DL CCs) |
| E008 | DL_5CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/4 | All supported EN-DC configurations within FR1 (5DL CCs) |
| E009 | DL_6CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1/5 | All supported EN-DC configurations within FR1 (6DL CCs) |
| E010 | UL_NR_1CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-2 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/1 | All supported Inter-band EN-DC configurations including FR2 (1NR UL CC) |
| E010a | DL_NR_1CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-2 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/1 | All supported Inter-band EN-DC configurations including FR2 (1NR DL CC) |
| E011 | UL_NR_2CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/2 AND A.4.3.2B.2.0-2A/2 | All supported Inter-band EN-DC configurations including FR2 (2UL NR CCs) |
| E011a | DL_NR_2CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/2 AND A.4.3.2B.2.0-1A/2 | All supported Inter-band EN-DC configurations including FR2 (2DL NR CCs) |
| E012 | UL_NR_3CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/3 AND NR_A.4.3.2B.2.0-2A/3 | All supported Inter-band EN-DC configurations including FR2 (3UL NR CCs) |
| E012a | DL_NR_3CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/3 AND A.4.3.2B.2.0-1A/3 | All supported Inter-band EN-DC configurations including FR2 (3DL NR CCs) |
| E013 | UL_NR_4CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-2/4 AND A.4.3.2B.2.0-2A/4 | All supported Inter-band EN-DC configurations including FR2 (4UL NR CCs) |
| E013a | DL_NR_4CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/4 AND A.4.3.2B.2.0-1A/4 | All supported Inter-band EN-DC configurations including FR2 (4DL NR CCs) |
| E014 | DL_NR_5CC(A.4.3.2B.2.3.6-2 OR A.4.3.2B.2.3.7-3 OR A.4.3.2B.2.3.8-2 OR A.4.3.2B.2.3.9-2) AND A.4.3.2B.2.0-1/5 AND A.4.3.2B.2.0-1A/5 | All supported Inter-band EN-DC configurations including FR2 (5DL NR CCs) |
| E015 | UL_2CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-2/1 | All supported FR1 2UL CA configurations |
| E016 | DL_2CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3) AND A.4.3.2A.1-1/1 | All supported FR1 2DL CA configurations |
| E017 | DL_3CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-1/2 | All supported FR1 3DL CA configurations |
| E018 | DL_4CC(A.4.3.2A.2.1-3 OR A.4.3.2A.3.1-3 OR A.4.3.2A.4.1-3 OR A.4.3.2A.4.2-3) AND A.4.3.2A.1-1/3 | All supported FR1 4DL CA configurations |
| E019 | ULTxSwitching(A.4.3.2A.4.1-3) | All supported FR1 2UL CA configurations with ULTxSwitching capability |
| E020 | UL_2CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/1 | All supported FR2 2UL CA configurations |
| E021 | UL_3CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/2 | All supported FR2 3UL CA configurations |
| E022 | UL_4CC(A.4.3.2A.2.2-3 OR A.4.3.2A.3.2-3 OR A.4.3.2A.3.2-3a) AND A.4.3.2A.1-2/3 | All supported FR2 4UL CA configurations |
| E023 | UL_5CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/4 | All supported FR2 5UL CA configurations |
| E024 | UL_6CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/5 | All supported FR2 6UL CA configurations |
| E025 | UL_7CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/6 | All supported FR2 7UL CA configurations |
| E026 | UL_8CC(A.4.3.2A.2.2-3) AND A.4.3.2A.1-2/7 | All supported FR2 8UL CA configurations |
| E027 | DL_NR_2CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/2 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (2DL NR CCs) |
| E028 | DL_NR_3CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/3 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (3DL NR CCs) |
| E028a | DL_NR_4CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/4 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (4DL NR CCs) |
| E028b | DL_NR_5CC(A.4.3.2B.2.2-2) AND A.4.3.2B.2.0-1A/5 | All supported Intra-band non-contiguous EN-DC configurations in FR1 (5DL NR CCs) |
| E029 | DL_NR_2CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/2 | All supported Inter-band EN-DC configurations within FR1 (2DL NR CCs) |
| E030 | DL_NR_3CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/3 | All supported Inter-band EN-DC configurations within FR1 (3DL NR CCs) |
| E030a | DL_NR_4CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/4 | All supported Inter-band EN-DC configurations within FR1 (4DL NR CCs) |
| E030b | DL_NR_5CC(A.4.3.2B.2.3.1-2 OR A.4.3.2B.2.3.2-2 OR A.4.3.2B.2.3.3-2 OR A.4.3.2B.2.3.4-2 OR A.4.3.2B.2.3.5-2) AND A.4.3.2B.2.0-1A/5 | All supported Inter-band EN-DC configurations within FR1 (5DL NR CCs) |
| E031 | A.4.3.2C.3-2 | All supported FR1 intra-band contiguous 2DL CA with SUL in uplink Configurations |
| NOTE 1: UL(table_index) includes all supported CA Configurations where at least one UL CA configuration was declared in column "Supported CA Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8]. NOTE 2: UL_nCC(table_index) includes all supported CA or DC Configurations where at least one n-carrier UL CA or DC configuration was declared in column "Supported CA Bandwidth Class(es) in UL" or “Supported EN-DC Bandwidth Class(es) in UL” in Table table_index in TS 38.508-2 [8]. NOTE 3: UL_NR_nCC(table_index) includes all supported DC Configurations where at least one DC configuration with n-carrier NR UL CA configuration was declared in column "Supported EN-DC Bandwidth Class(es) in UL" in Table table_index in TS 38.508-2 [8]. NOTE 4: DL_nCC(table_index) includes all supported n-carrier CA/DC Configurations in Table table_index in TS 38.508-2 [8]. NOTE 5: DL_NR_nCC(table_index) includes all supported DC Configurations with n-carrier NR DL CA configuration in Table table_index in TS 38.508-2 [8]. NOTE 6: ULTxSwitching(table_index) includes all supported CA/DC/SUL Configurations where at least one uplink band pair was declared in column “Supported ULTxSwitching Band Pair" in Table table_index in TS 38.508-2 [8]. |  |  |

==============================================================

<< End of changes >>