3GPP TSG-RAN5 Meeting #93-e	R5-217437

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0443 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to frequencyDomainResources in PDCCHConfigCommon message exception |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | According to TS 38.331, the bandwidth of commonControlResourceSet in SIB1 should not exceed the bandwidth of CORESET#0, as quoted below: “commonControlResourceSet An additional common control resource set which may be configured and used for any common or UE-specific search space. If the network configures this field, it uses a ControlResourceSetId other than 0 for this ControlResourceSet. The network configures the commonControlResourceSet in SIB1 so that it is contained in the bandwidth of CORESET#0.” For the current test configurations, the CORESET#0 index values that will be derived correspond to a bandwidth of 24RBs as per TS 38.213 cl. 13 so “frequencyDomainResources” under commonControlResourceSet should only have 4 bits set to 1. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated first (leftmost) 8 bits of frequencyDomainResources under commonControlResourceSet from “11111111” to “11110000” |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | A conformant may fail the test. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.2.2.1.4_1, 5.2.2.2.4_1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

###### 5.2.2.1.4_1	2Rx FDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA

5.2.2.1.4_1.1	Test purpose

To verify the Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions with CRS rate matching configured.

5.2.2.1.4_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 15 and forward supporting capability IE rateMatchingLTE-CRS but not supporting capability IE additionalDMRS-DL-Alt.

Test1-1 also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and capability IE rateMatchingLTE-CRS but not supporting capability IE additionalDMRS-DL-Alt.

Test 1-2 applies to all types of NR UE release 15 and forward supporting capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

Test 1-2 also applies to all types of EUTRA UE release 15 and forward supporting EN-DC and capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

5.2.2.1.4_1.3	Test description

5.2.2.1.4_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.6 for TE diagram and section A.3.2.3 for UE diagram.

2.	The parameter settings for the cell are set up according to Table 5.2-1, Table 5.2.2.1.4.0-2 and Table 5.2.2.1.4.0-3 as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA with Connected without release On, Test Mode On or EN-DC, DC bearer MCG and SCG, Connected without release On, Test Mode On, for NSA according to TS 38.508-1 [6] clause 4.5. Message contents are defined in clause 5.2.2.1.4_1.3.3.

5.2.2.1.4_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.2.1.4.0-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Table 5.2.2.1.4_1.3.4-1.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

5.2.2.1.4_1.3.3	Message contents

5.2.2.1.4_1.3.3_1	Message exceptions for SA

As defined in clause 5.4.2 of TS 38.508-1 [6] with the following exceptions:

Table 5.2.2.1.4_1.3.3_1-1: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-19 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 2 entry |  | FR1 |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| k0 | 0 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 94 | Start symbol(S)=3, Length(L)=9 for Test 1-1 |  |
|  | 66 | Start symbol(S)=3, Length(L)=11 for Test 1-2 |  |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| k0 | 0 |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 66 | Start symbol(S)=3, Length(L)=11 for Test 1-2 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-2: SearchSpace

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and5.4.2.0-7 using condition USS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| monitoringSymbolsWithinSlot | 00100000000000 |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-3: ServingCellConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| dmrs-TypeA-Position | pos3 |  |  |
| lte-CRS-ToMatchAround | RateMatchPatternLTE-CRS |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-4: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2-20 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| carrierFreqDL | Same as NR carrier centre subcarrier location |  |  |
| carrierBandwidthDL | n50 | 10MHz |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| nrofCRS-Ports | n4 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-5: Void

Table 5.2.2.1.4_1.3.3_1-6: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| frequencyShift7p5khz | true |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-7: PDCCH-ControlResourceSet

| Derivation Path: Table TS 38.508-1 [6], 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 1 | SearchSpace duration of 1 symbol from third symbol |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-8: PDCCH-ConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| commonControlResourceSet ::= SEQUENCE { |  |  | SA |
| controlResourceSetId | 1 |  |  |
| frequencyDomainResources | 11110000 00000000 00000000 00000000 00000000 00000 |  |  |
| Duration | 1 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | Null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_1-9: SearchSpace for CSS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and 5.4.2-4 using condition CSS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition CSS |  | CSS |
| controlResourceSetId | 1 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| duration | Not present | 1 slot per default |  |
| monitoringSymbolsWithinSlot | 00100000000000 |  |  |
| } |  |  |  |

5.2.2.1.4_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.1.4_1.3.3_1 with the following exceptions:

Table 5.2.2.1.4_1.3.3_2-1: SearchSpace

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and 5.4.2.0-7 using condition USS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| monitoringSymbolsWithinSlot | 00100000000000 |  |  |
| } |  |  |  |

Table 5.2.2.1.4_1.3.3_2-2: PDCCH-ControlResourceSet

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| duration | 1 | SearchSpace duration of 1 symbol from third symbol |  |
| } |  |  |  |

5.2.2.1.4_1.3.4	Test requirement

Table 5.2.2.1.4.0-3 defines the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A for each throughput test shall meet or exceed the specified value in Table 5.2.2.1.4_1.3.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.1.4_1.3.4-1: Test requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-7.1 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | 0.0 |
| 1-2 | R.PDSCH.1-7.2 FDD | 10 / 15 | QPSK, 0.30 | TDLA30-10 | 4x2, ULA Low | 70 | 0.0 |

<<Unchaged sections skipped>>

5.2.2.2.4_1	2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA

5.2.2.2.4_1.1	Test purpose

To verify the PDSCH mapping Type A coexistence performance under 2 receive antenna conditions for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput with baseline receiver configuration.

5.2.2.2.4_1.2	Test applicability

This test applies to all types of NR UE release 16 and forward supporting capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

This test also applies to all types of E-UTRA UE release 16 and forward supporting EN-DC and capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

5.2.2.2.4_1.3	Test description

5.2.2.2.4_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.1 for TE diagram and clause A.3.2.3 for UE diagram.

2.	The parameter settings for the cell are set up according to Tables 5.2-1 and 5.2.2.2.4.0-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2, C.3.1 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1.

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA or (EN-DC, DC bearer MCG and SCG, Connected without release On) for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 5.2.2.2.4_1.3.3.

5.2.2.2.4_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.2.2.3.0-2. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.2.4_1.4-1 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

5.2.2.2.4_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.2.2.2.4_1.3.3_1	Message exceptions for SA

Table 5.2.2.2.4_1.3.3_1-1: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n8 |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-2: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 2 entry |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 94 | Start symbol(S)=3, Length(L)=9 | Test 1-1 |
|  | 66 | Start symbol(S)=3, Length(L)=11 | Test 1-2 |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 66 | Start symbol(S)=3, Length(L)=11 |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-3: SearchSpace

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and5.4.2.0-7 using condition USS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-4: ServingCellConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| dmrs-TypeA-Position | pos3 |  |  |
| lte-CRS-ToMatchAround | RateMatchPatternLTE-CRS |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-5: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| carrierFreqDL | Same as NR carrier centre subcarrier location |  |  |
| carrierBandwidthDL | n50 | 10MHz |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| nrofCRS-Ports | n4 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-6: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| frequencyShift7p5khz | true |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-7: PDCCH-ControlResourceSet

| Derivation Path: Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 1 | SearchSpace duration of 1 symbol from third symbol |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-8: PDCCH-ConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| commonControlResourceSet ::= SEQUENCE { |  |  | SA |
| controlResourceSetId | 1 |  |  |
| frequencyDomainResources | 11110000 00000000 00000000 00000000 00000000 00000 |  |  |
| Duration | 1 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | Null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-9: SearchSpace for CSS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and 5.4.2.0-7 using condition CSS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition CSS |  | CSS |
| controlResourceSetId | 1 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| duration | Not present | 1 slot per default |  |
| } |  |  |  |

5.2.2.2.4_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.2.4_1.3.3_1 with the following exceptions:

Table 5.2.2.2.4_1.3.3_2-1: SearchSpace

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and 5.4.2.0-7 using condition USS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_2-2: PDCCH-ControlResourceSet

| Derivation Path: Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| duration | 1 | SearchSpace duration of 1 symbol from third symbol |  |
| } |  |  |  |

5.2.2.2.4_1.4	Test requirement

Table 5.2.2.2.3.0-3 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A.3.2.2 for each throughput test shall meet or exceed the specified value in Table 5.2.2.2.4_1.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.2.4_1.4-1: Test Requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 TDD | 10 / 15 | QPSK, 0.30 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 0.1 |
| 1-2 | R.PDSCH.1-1.2 TDD | 10 / 15 | QPSK, 0.30 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 0.1 |

<<End of change>>