3GPP TSG-RAN5 Meeting #93-e	R5-217438

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1551 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to SIB4 exceptions in Annex H |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | SIB4 message exception for “dl-CarrierFreq” and “cellReselectionPriority” only consider when Cell 1 is sending SIB4. Hence, SIB4’s “dl-CarrierFreq” and “cellReselectionPriority” (i.e. no difference between higher and lower priority) are lacking when Cell 2 is sending it. SIB4’s SMTC.2 condition is wrongly placed in value “sf1” of “duration”. It must be in “sf5”. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added row in SIB4 for “dl-CarrierFreq” with value “Downlink NR SSB ARFCN of NR Cell 1”. Added row in SIB4 for “cellReselectionPriority” with value “4”. Added condition of NR Cell 1 and NR Cell 2 (similar to SIB2 table). Removed SMTC.2 condition in value “sf1” of “duration”. Added SMTC.2 condition in value “sf5” of “duration”. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect settings due to misleading description may occur. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex H.2.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchaged sections skipped>>

H.2.2	System information blocks message contents exceptions for NR inter frequency cell re-selection

SystemInformationBlockType2: for NR inter-frequency cell re-selection

Table H.2.2-1: SIB2: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-1 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB2 ::= SEQUENCE { |  |  |  |
| cellReselectionInfoCommon SEQUENCE { |  |  |  |
| rangeToBestCell | Not present |  |  |
| } |  |  |  |
| cellReselectionServingFreqInfo SEQUENCE { |  |  |  |
| s-NonIntraSearchP | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| threshServingLowP | 22 | 44 is actual value in dB (22 * 2 dB) |  |
| cellReselectionPriority | 4 |  | NR Cell 1 |
|  | 5 |  | NR Cell 2 |
| } |  |  |  |
| } |  |  |  |

SystemInformationBlockType4: for NR inter-frequency cell re-selection

Table H.2.2-2: SIB4: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.2-3 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SIB4 ::= SEQUENCE { |  |  |  |
| interFreqCarrierFreqList SEQUENCE (SIZE (1..maxFreq)) OF SEQUENCE { | 1 Entry |  |  |
| dl-CarrierFreq[1] | Downlink NR SSB ARFCN of NR Cell 2 |  | NR Cell 1 |
|  | Downlink NR SSB ARFCN of NR Cell 1 |  | NR Cell 2 |
| smtc SEQUENCE { |  |  |  |
| periodicityAndOffset | sf20 : 0 |  | SMTC.1, SMTC.2 |
|  | sf20 : 17 |  | SMTC.6 |
| duration | sf1 |  | SMTC.1, SMTC.2 |
|  | sf5 |  | SMTC.6 |
| deriveSSB-IndexFromCell[1] | false |  | Asynchronous cells |
|  | true |  | Synchronous cells |
| q-RxLevMin[1] | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| threshX-HighP[1] | 24 | 48 is actual value in dB (24 * 2 dB) |  |
| threshX-LowP[1] | 25 | 50 is actual value in dB (25 * 2 dB) |  |
| cellReselectionPriority[1] | 5 |  | NR Cell 1 |
|  | 4 |  | NR Cell 2 |
| q-OffsetFreq[1] | dB0 | 0 is actual value in dB (0 * 2 dB). |  |
| } |  |  |  |
| } |  |  |  |

| Condition | Explanation |
| --- | --- |
| SSB.n FRm | SSB pattern n in FRm according to TS 38.133 [6] A.3.10 |
| Synchronous cells | SSB indices of neighbour cells can be derived from timing of serving cell |
| Asynchronous cells | SSB indices of neighbour cells cannot be derived from timing of serving cell |

SystemInformationBlockType1: for NR inter frequency cell re-selection

Table H.2.2-3: SIB1: NR inter frequency cell re-selection

| Derivation Path: TS 38.508-1 [14], Table 4.6.1-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| SIB1 ::= SEQUENCE { |  |  |  |  |
| cellSelectionInfo SEQUENCE { |  |  |  |  |
| q-RxLevMin |  | -70 | -140 is actual value in dBm (-70 * 2 dBm) | dBm/15kHz |
|  |  | -69 | -137 is actual value in dBm (-69 * 2 +1 dBm) | dBm/30kHz |
| } |  |  |  |  |
| } |  |  |  |  |

<<End of change>>