3GPP TSG-RAN5 Meeting #93-e	R5-217440

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-4 | CR | 0444 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to TC 5.2.2.2.4_1 and editorial corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Anritsu |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_perf_enh-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - There are typos. - Applicability for TC 5.2.2.2.4_1 is missing. - RMC for TC 5.2.2.2.4_1 is missing. - Minimum test time for TC 5.2.2.2.4_1 is missing. - Calculation method for Scaling Factor is clarified by R5-214088 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Modified typos. - Applicability for TC 5.2.2.2.4_1 is introduced from core requirement. - RMC for TC 5.2.2.2.4_1 is introduced from core requirement. - Minimum test time for TC 5.2.2.2.4_1 is added. - Table G.1.5-1 is corrected. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test will not be performed correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.1.1.3, 5.1.1.4, 5.2.2.2.4, A.3.2.2, G.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<Unchanged sections skipped>>

5.1.1.3	Applicability of requirements for optional UE features

The performance requirements in Table 5.1.1.3-1 shall apply for UEs which support optional UE features only.

Table 5.1.1.3-1: Requirements applicability for optional UE features

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| SU-MIMO Interference Mitigation advanced receiver | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 3-1) Clause 5.2.3.1.1 (Test 5-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 3-1) Clause 5.2.3.2.1 (Test 5-1) |  |
| Alternative additional DMRS position for co-existence with LTE CRS (additionalDMRS-DL-Alt) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Test 1-2) Clause 5.2.3.1.4 (Test 1-2) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 (Test 1-2) Clause 5.2.3.2.4 (Test 1-2) |  |
| Basic DL NR-NR CA operation (supportedBandCombinationList) | NR CA | SDR | Clause 5.5A.1 | 1) Up to 16 DL carriers 2) Same numerology across carrier for data/control channel at a given time |
| Enhanced demodulation processing for HST-SFN joint transmission scheme with velocity up to 500km/h | FR1 FDD | PDSCH | Clause 5.2.2.1.9 (Test 1-1) Clause 5.2.3.1.9 (Test 1-1) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.9 (Test 1-1) Clause 5.2.3.2.9 (Test 1-1) |  |

5.1.1.4	Applicability of requirements for mandatory UE features with capability signalling

The performance requirements in Table 5.1.1.4-1 shall apply for UEs which support mandatory UE features with capability signalling only.

Table 5.1.1.4-1: Requirements applicability for mandatory features with UE capability signalling

| UE feature/capability [14] | Test type |  | Test list | Applicability notes |
| --- | --- | --- | --- | --- |
| 256QAM modulation scheme for PDSCH for FR1 (pdsch-256QAM-FR1) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Test 1-3) Clause 5.2.3.1.1 (Test 1-3) |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Test 1-3) Clause 5.2.3.2.1 (Test 1-3) |  |
| PDSCH mapping type B (pdsch-MappingTypeB) | FR1 FDD | PDSCH | Clause 5.2.2.1.3 Clause 5.2.3.1.3 |  |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.3 Clause 5.2.3.2.3 |  |
| Rate-matching around LTE CRS (rateMatchingLTE-CRS) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 Clause 5.2.3.1.4 | For UEs supporting “Alternative additional DMRS position for co-existence with LTE CRS”, if Test 1-2 is tested, the test coverage can be considered fulfilled without executing Test 1-1. Otherwise, only Test 1-1 is tested. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.4 Clause 5.2.3.2.4 |  |
| Supported maximum number of ports across all configured NZP-CSI-RS resources per CC (maxConfigNumberPortsAcrossNZP-CSI-RS-PerCC) | FR1 FDD | PDSCH | Clause 5.2.2.1.4 (Tests 1-1, 1-2) Clause 5.2.3.1.1 (Tests 3-1, 4-1, 5-1) Clause 5.2.3.1.4 (Tests 1-1, 1-2) Clause 5.2.3.2.1 (Tests 3-1, 4-1, 5-1) | The requirements apply only in case the number of NZP-CSI-RS ports in the test case satisfies UE capability on maximum number of NZP-CSI-RS ports |
|  | FR1 TDD | PDSCH | Clause 5.2.3.2.1 (Tests 3-1, 4-1, 5-1) |  |
| Supported maximum number of PDSCH MIMO layers (maxNumberMIMO-LayersPDSCH) | FR1 FDD | PDSCH | Clause 5.2.2.1.1 (Tests 2-1, 2-2, 3-1) Clause 5.2.2.1.2 Clause 5.2.3.1.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1) Clause 5.2.3.1.2 | The requirements apply only in case the PDSCH MIMO rank in the test case does not exceed UE PDSCH MIMO layers capability |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.1 (Tests 2-1, 2-2, 3-1) Clause 5.2.2.2.2 Clause 5.2.3.2.1 (Tests 2-1, 2-2, 3-1, 4-1, 5-1) Clause 5.2.3.2.2 |  |
| Support number of active TCI states per BWP per CC, including control and data (maxNumberActiveTCI-PerBWP) | FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-2) Clause 5.2.3.1.10 (Test 1-2) | The requirements apply only when maxNumberActiveTCI-PerBWP is other than n1. |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-2) Clause 5.2.3.2.10 (Test 1-2) |  |
| Support for maximum number of TRS resource sets per CC which the UE can track simultaneously (maxSimultaneousResourceSetsPerCC) | FR1 FDD | PDSCH | Clause 5.2.2.1.10 (Test 1-2) Clause 5.2.3.1.10 (Test 1-2) | The requirements apply only when maxSimultaneousResourceSetsPerCC ≥ 2 |
|  | FR1 TDD | PDSCH | Clause 5.2.2.2.10 (Test 1-2) Clause 5.2.3.2.10 (Test 1-2) |  |

<<Unchanged sections skipped>>

5.2.2.2.4	2Rx TDD FR1 PDSCH mapping Type A and LTE-NR coexistence performance

5.2.2.2.4.0	Minimum conformance requirements

The performance requirements are specified in Table 5.2.2.2.4.0-3, with the addition of test parameters in Table 5.2.2.2.4.0-2 and the downlink physical channel setup according to Annex C.2.1.

The test purposes are specified in Table 5.2.2.2.4.0-1.

Table 5.2.2.2.4.0-1: Tests purpose

| Purpose | Test index |
| --- | --- |
| Verify the PDSCH mapping Type A normal performance under 2 receive antenna conditions with CRS rate matching configured | 1-1, 1-2 |

Table 5.2.2.2.4.0-2: Test parameters

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | TDD |
| Active DL BWP index |  |  | 1 |
| NR UL transmission with a 7.5 kHz shift to the LTE raster |  |  | true |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 3 |
|  | Length (L) |  | 9 for Test 1-1 11 for Test 1-2 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Position of the first DM-RS for downlink |  | 3 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| CRS for rate matching (Note 1) | LTE carrier centre subcarrier location |  | Same as NR carrier centre subcarrier location |
|  | LTE carrier BW | MHz | 10 |
|  | Number of antenna ports |  | 4 |
|  | v-shift |  | 0 |
| Number of HARQ Processes |  |  | 8 |
| The number of slots between PDSCH and corresponding HARQ-ACK information |  |  | Specific to each TDD UL-DL pattern and as defined in Annex A.1.2 |
| Note 1: No MBSFN is configured on LTE carrier |  |  |  |

Table 5.2.2.2.4.0-3: Minimum performance for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 TDD | 10 / 15 | QPSK, 0.30 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | -0.8 |
| 1-2 | R.PDSCH.1-1.2 TDD | 10 / 15 | QPSK, 0.30 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | -0.8 |

The normative reference for this requirement is TS 38.101-4 [5] clause 5.2.2.2.4.

5.2.2.2.4_1	2Rx TDD FR1 PDSCH Mapping Type A and LTE-NR coexistence performance - 4x2 MIMO with baseline receiver for both SA and NSA

5.2.2.2.4_1.1	Test purpose

To verify the PDSCH mapping Type A coexistence performance under 2 receive antenna conditions for a specified downlink Reference Measurement Channel (RMC) to achieve a certain throughput with baseline receiver configuration.

5.2.2.2.4_1.2	Test applicability

Test 1-1 applies to all types of NR UE release 16 and forward supporting capability IE rateMatchingLTE-CRS but not supporting capability IE additionalDMRS-DL-Alt.

Test 1-1 also applies to all types of E-UTRA UE release 16 and forward supporting EN-DC and capability IE rateMatchingLTE-CRS but not supporting capability IE additionalDMRS-DL-Alt.

Test 1-2 applies to all types of NR UE release 16 and forward supporting capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

Test 1-2 also applies to all types of E-UTRA UE release 16 and forward supporting EN-DC and capability IE additionalDMRS-DL-Alt and rateMatchingLTE-CRS.

5.2.2.2.4_1.3	Test description

5.2.2.2.4_1.3.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1 and Table 5.3.6-1 of 38.521-1 [7].

Configurations of PDSCH and PDCCH before measurement are specified in Annex C.

Test Environment: Normal, as defined in TS 38.508-1 [6] clause 4.1.

Frequencies to be tested: Mid Range, as defined in TS 38.508-1 [6] clause 4.3.1.1.

For EN-DC within FR1 operation, setup the LTE link according to Annex D.

1.	Connect the SS, the faders and AWGN noise source to the UE antenna connectors as shown in TS 38.508-1 [6] Annex A, in Figure A.3.1.7.6 for TE diagram and clause A.3.2.3 for UE diagram.

2.	The parameter settings for the cell are set up according to Tables 5.2-1 and 5.2.2.2.4.0-2 and as appropriate.

3.	Downlink signals for NR cell are initially set up according to Annexes C.0, C.1, C.2 and uplink signals according to Annexes G.0, G.1, G.2, G.3.1 of TS 38.521-1 [7].

4.	Propagation conditions are set according to Annex B.0.

5.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR for SA or (EN-DC, DC bearer MCG and SCG, Connected without release On) for NSA according to TS 38.508-1 [6] clause 4.5. Message content are defined in clause 5.2.2.2.4_1.3.3.

5.2.2.2.4_1.3.2	Test procedure

1.	SS transmits PDSCH via PDCCH DCI format 1_1 for C_RNTI to transmit the DL RMC according to Table 5.2.2.2.4.0-3. The SS sends downlink MAC padding bits on the DL RMC.

2.	Set the parameters of the bandwidth, MCS, reference channel, the propagation condition, the correlation matrix and the SNR according to Tables 5.2.2.2.4_1.4-1 as appropriate.

3.	Measure the average throughput for a duration sufficient to achieve statistical significance according to Annex G clause G.1.5. Count the number of NACKs, ACKs and statDTXs on the UL during each subtest and decide pass or fail according to Table G.1.5-1 in Annex G clause G.1.5.

5.2.2.2.4_1.3.3	Message contents

Message contents are according to TS 38.508-1 [6] clause 4.6.1 and 5.4.2.

5.2.2.2.4_1.3.3_1	Message exceptions for SA

Table 5.2.2.2.4_1.3.3_1-1: PDSCH-ServingCellConfig

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-102 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-ServingCellConfig ::= SEQUENCE { |  |  |  |
| nrofHARQ-ProcessesForPDSCH | n8 |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-2: PDSCH-TimeDomainResourceAllocationList

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-103 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDSCH-TimeDomainResourceAllocationList::= SEQUENCE(SIZE(1..maxNrofDL-Allocations)) OF { | 2 entry |  |  |
| PDSCH-TimeDomainResourceAllocation[1] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 94 | Start symbol(S)=3, Length(L)=9 | Test 1-1 |
|  | 66 | Start symbol(S)=3, Length(L)=11 | Test 1-2 |
| } |  |  |  |
| PDSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  |  |  |
| K0 | Not present |  |  |
| mappingType | typeA |  |  |
| startSymbolAndLength | 66 | Start symbol(S)=3, Length(L)=11 | Test 1-2 |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-3: SearchSpace

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and5.4.2.0-7 using condition USS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-4: ServingCellConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommon ::= SEQUENCE { |  |  |  |
| dmrs-TypeA-Position | pos3 |  |  |
| lte-CRS-ToMatchAround | RateMatchPatternLTE-CRS |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-5: RateMatchPatternLTE-CRS

| Derivation Path: TS 38.508-1 [6], Table 5.4.2.0-28 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RateMatchPatternLTE-CRS ::= SEQUENCE { |  |  |  |
| carrierFreqDL | Same as NR carrier centre subcarrier location |  |  |
| carrierBandwidthDL | n50 | 10MHz |  |
| mbsfn-SubframeConfigList | Not present |  |  |
| nrofCRS-Ports | n4 |  |  |
| v-Shift | n0 |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-6: FrequencyInfoUL-SIB

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-62 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| FrequencyInfoUL-SIB SEQUENCE { |  |  |  |
| frequencyShift7p5khz | true |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-7: PDCCH-ControlResourceSet

| Derivation Path: Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 2 |  |  |
| duration | 1 | SearchSpace duration of 1 symbol from third symbol |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-8: PDCCH-ConfigCommon

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-96 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PDCCH-ConfigCommon ::= SEQUENCE { |  |  |  |
| commonControlResourceSet ::= SEQUENCE { |  |  | SA |
| controlResourceSetId | 1 |  |  |
| frequencyDomainResources | 11111111 00000000 00000000 00000000 00000000 00000 |  |  |
| Duration | 1 |  |  |
| cce-REG-MappingType CHOICE { |  |  |  |
| nonInterleaved | Null |  |  |
| } |  |  |  |
| precoderGranularity | sameAsREG-bundle |  |  |
| } |  |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_1-9: SearchSpace for CSS

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and 5.4.2.0-7 using condition CSS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| searchSpaceId | SearchSpaceId with condition CSS |  | CSS |
| controlResourceSetId | 1 |  |  |
| monitoringSlotPeriodicityAndOffset CHOICE { |  |  |  |
| sl1 | NULL |  |  |
| } |  |  |  |
| duration | Not present | 1 slot per default |  |
| } |  |  |  |

5.2.2.2.4_1.3.3_2	Message exceptions for NSA

Same as 5.2.2.2.4_1.3.3_1 with the following exceptions:

Table 5.2.2.2.4_1.3.3_2-1: SearchSpace

| Derivation Path: TS 38.508-1 [6], Table 4.6.3-162 and 5.4.2.0-7 using condition USS, FR1_10MHz, Long_DCI |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| SearchSpace ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| } |  |  |  |

Table 5.2.2.2.4_1.3.3_2-2: PDCCH-ControlResourceSet

| Derivation Path: Table 5.4.2.0-6 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ControlResourceSet ::= SEQUENCE { |  |  |  |
| controlResourceSetId | 1 |  |  |
| duration | 1 | SearchSpace duration of 1 symbol from third symbol |  |
| } |  |  |  |

5.2.2.2.4_1.4	Test requirement

Table 5.2.2.2.4.0-3 define the primary level settings.

The fraction of maximum throughput percentage for the downlink reference measurement channels specified in Annex A.3.2.2 for each throughput test shall meet or exceed the specified value in Table 5.2.2.2.4_1.4-1 for the specified SNR including test tolerances for all throughput tests.

Table 5.2.2.2.4_1.4-1: Test Requirement for Rank 1

| Test num. | Reference channel | Bandwidth (MHz) / Subcarrier spacing (kHz) | Modulation format and code rate | TDD UL-DL pattern | Propagation condition | Correlation matrix and antenna configuration | Reference value |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Fraction of maximum throughput (%) | SNR (dB) |
| 1-1 | R.PDSCH.1-1.1 TDD | 10 / 15 | QPSK, 0.30 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 0.1 |
| 1-2 | R.PDSCH.1-1.2 TDD | 10 / 15 | QPSK, 0.30 | FR1.15-1 | TDLA30-10 | 4x2, ULA Low | 70 | 0.1 |

<<Unchanged sections skipped>>

A.3.2.2	TDD

A.3.2.2.1	Reference measurement channels for SCS 15 kHz FR1

Table A.3.2.2.1-1: PDSCH Reference Channel for TDD UL-DL pattern FR1.15-1 and LTE-NR coexistence scenario

| Parameter | Unit | Value |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Reference channel |  | R.PDSCH.1-1.1 TDD | R.PDSCH.1-1.2 TDD |  |  |  |
| Channel bandwidth | MHz | 10 | 10 |  |  |  |
| Subcarrier spacing | kHz | 15 | 15 |  |  |  |
| Allocated resource blocks | PRBs | 52 | 52 |  |  |  |
| Number of consecutive PDSCH symbols |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 9 | 11 |  |  |  |
| Allocated slots per 2 frames |  | 7 | 7 |  |  |  |
| MCS table |  | 64QAM | 64QAM |  |  |  |
| MCS index |  | 4 | 4 |  |  |  |
| Modulation |  | QPSK | QPSK |  |  |  |
| Target Coding Rate |  | 0.30 | 0.30 |  |  |  |
| Number of MIMO layers |  | 1 | 1 |  |  |  |
| Number of DMRS REs |  |  |  |  |  |  |
| For Slot i, if mod(i, 5) = 3 for i from {0,…,19} |  | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = {0,1,2} for i from {1,…,19} |  | 12 | 12 |  |  |  |
| Overhead for TBS determination |  | 18 | 18 |  |  |  |
| Information Bit Payload per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 2472 | 3240 |  |  |  |
| Transport block CRC per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | Bits | 16 | 16 |  |  |  |
| Number of Code Blocks per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | CBs | N/A | N/A |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,19} | CBs | 1 | 1 |  |  |  |
| Binary Channel Bits Per Slot |  |  |  |  |  |  |
| For Slot 0 and Slot i, if mod(i, 5) = {2,3,4} for i from {0,…,19} | Bits | N/A | N/A |  |  |  |
| For Slots i = 10, 11 | Bits | 7760 | 10256 |  |  |  |
| For Slot i, if mod(i, 5) = {0,1} for i from {1,…,9,12,…,19} | Bits | 8384 | 10880 |  |  |  |
| Max. Throughput averaged over 2 frames | Mbps | 0.865 | 1.134 |  |  |  |
| Note 1: SS/PBCH block is transmitted in slot #0 with periodicity 20 ms Note 2: Slot i is slot index per 2 frames Note 3: No user data is scheduled on slots with LTE PBCH/PSS/SSS |  |  |  |  |  |  |

<<Unchanged sections skipped>>

G.1.5	Minimum Test time

Editor's Note: Simulation method to derive minimum test time for FR2 needs to be evaluated.

If a pass fail decision in clause G.1.4 can be achieved earlier than the minimum test time, then the test shall not be decided, but continued until the minimum test time is elapsed.

The tables below contain the minimum number of slots for FDD and TDD.

By simulations the minimum number of active subframes (carrying DL payload) was derived (MNAS), then adding inactive subframes to the active ones. For TDD additional subframes contain no DL payload) then rounding up to full thousand.

Simulation method to derive minimum test time:

With a level, corresponding a throughput at the test limit (here 30 % or 70 % of the max. throughput) the preliminary throughput versus time converges towards the final throughput. The allowance of ± 0.2 dB around the above mentioned level is predefined by RAN5 to find the minimum test time. The allowance of ±0.2 dB maps through the function "final throughput versus level" into a throughput corridor. The minimum test time is achieved when the preliminary throughput escapes the corridor the last time. The two functions "final throughput versus level" and "preliminary throughput versus time" are simulation results, which are done individual for each demodulation scenario.

Figure G.1.5-1: Simulation method to derive minimum test time

Table G.1.5-1: Minimum Test time for PDSCH demodulation

| TDD UL-DL pattern | Reference Channel | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- | --- |
| NA | R.PDSCH.1-8.1 FDD | 750 Hz | FFS | 1.0526 | FFS |
| NA | R.PDSCH.1-1.1 FDD | 400 Hz | 10000 (Note 1) | 1.0526 | 11000 |
| NA | R.PDSCH.1-1.2 FDD, R.PDSCH.1-2.1 FDD, R.PDSCH.1-5.1 FDD, | 100 Hz | 20000 (Note 1) | 1.0526 | 22000 |
| NA | R.PDSCH.1-1.3 FDD R.PDSCH.1-2.2 FDD, R.PDSCH.1-2.3 FDD, R.PDSCH.1-2.4 FDD, R.PDSCH.1-3.1 FDD, R.PDSCH.1-4.1 FDD, R.PDSCH.1-7.1 FDD, R.PDSCH.1-7.2 FDD, R.PDSCH.2-1.1 FDD, | 10 Hz | 75000 (Note 1) | 1.0526 | 79000 |
| FR1.15-1 | R.PDSCH.1-1.1 TDD R.PDSCH.1-1.2 TDD | 10 Hz | 75000 (Note 1) | 2.8571 | 215000 |
| FR1.30-1A | R.PDSCH.2-1.1 TDD | 400 Hz | 10000 (Note 1) | 1.2903 | 13000 |
| FR1.30-5 | R.PDSCH.2-11.1 TDD | 400Hz | 10000 (Note 1) | 1.2903 | 13000 |
| FR1.30-6 | R.PDSCH.2-12.1 TDD | 400Hz | 10000 (Note 1) | 1.2903 | 13000 |
| FR1.30-1 | R.PDSCH.2-1.2 TDD, R.PDSCH.2-2.1 TDD, R.PDSCH.2-7.1 TDD | 100 Hz | 20000 (Note 1) | 1.2903 | 26000 |
| FR1.30-1 | R.PDSCH.2-4.1 TDD, R.PDSCH.2-3.1 TDD, R.PDSCH.2-2.2 TDD R.PDSCH.2-1.3 TDD R.PDSCH.2-2.3 TDD R.PDSCH.2-2.4 TDD | 10 Hz | 75000 (Note 1) | 1.2903 | 97000 |
| FR1.30-2 | R.PDSCH.2-5.1 TDD | 10 Hz | 75000 (Note 1) | 1.2903 | 97000 |
| FR1.30-3 | R.PDSCH.2-6.1 TDD | 10 Hz | 75000 (Note 1) | 1.4815 | 112000 |
| FR1.30-4 | R.PDSCH.2-9.1 TDD | 10 Hz | 75000 (Note 1) | 1.2903 | 97000 |
| FR2.60-1 | R.PDSCH.4-1.1 TDD | 75 Hz | 20000 (Note 2) | 1.33 | 27000 |
| FR2.120-1 | R.PDSCH.5-1.1 TDD R.PDSCH.5-2.1 TDD R.PDSCH.5-3.1 TDD R.PDSCH.5-2.2 TDD R.PDSCH.5-2.3 TDD | 300 Hz | 10000 (Note 2) | 1.25 | 13000 |
| FR2.120-2 | R.PDSCH.5-4.1 TDD R.PDSCH.5-5.1 TDD R.PDSCH.5-5.2 TDD R.PDSCH.5-6.1 TDD | 75 Hz | 20000 (Note 2) | 1.33 | 27000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots |  |  |  |  |  |

Table G.1.5-2: Minimum Test time for PDCCH demodulation

| Reference Channel | Demodulation scenario (doppler speed) | Minimum number of active subframes (MNAS) | MNAS to MNS Scaling factor (Note 3) | Minimum Number of Subframes (MNS) after rounding up to nearest thousand MNS= |
| --- | --- | --- | --- | --- |
| R.PDCCH 1-1.1 FDD, R.PDCCH.1-1.3 FDD, R.PDCCH.1-2.1 FDD, R.PDCCH.1-2.2 FDD, R.PDCCH.1-2.3 FDD, R.PDCCH.1-2.4 FDD, R.PDCCH.1-2.5 FDD, R.PDCCH.1-2.6 FDD | 10, 100, 400 Hz | 100000 (Note 1) | 1.0526 | 106000 |
| R.PDCCH.2-1.1 TDD, R.PDCCH.2-1.2 TDD, R.PDCCH.2-2.1 TDD, R.PDCCH.2-1.3 TDD | 10, 100, 400 Hz | 100000 (Note 1) | 1.2903 | 130000 |
| R.PDCCH.5-1.1 TDD R.PDCCH.5-1.2 TDD R.PDCCH.5-1.3 TDD R.PDCCH.5-2.1 TDD | 75, 300 Hz | 100000 (Note 2) | 1.25 | 130000 |
| Note 1: MNAS determined by simulations. Note 2: For cases where MNS is not determined by simulations, use same MNAS as the similar case simulated (same doppler speed) Note 3: MNS/MNAS ratio decided by scheduling pattern and is ratio of all slots to DL slots |  |  |  |  |

<<End of change>>