3GPP TSG-RAN5 Meeting #93-e	R5-217527

# Electronic Meeting, 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2148 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | message content update for HST single tap 1Tx PDSCH test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Austria RFFE GmbH |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The NZP CSI-RS for CSI acquisition configuration is currently not specified for HST single tap 1Tx PDSCH demodulation test case |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated table 5.4.2.0-15 to include IE values for 1Tx test cases |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case implementation for HST single tap 1Tx test case will remain ambiguous |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.0 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.4.2.0	Parameters common to all Demod and CSI tests

<several sections skipped>

###### CSI-RS-ResourceMapping

Table 5.4.2.0-15: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| row2 | 000000000001 | K0 = 0, 1Tx test cases |  |
| other | 000001 | K0 = 0, row3, 2Tx test cases |  |
| row4 | 001 | K0 = 0, 4Tx test cases |  |
| } |  |  |  |
| nrofPorts | P2 | 2Tx test cases |  |
|  | P4 | 4Tx test cases |  |
|  | P1 | 1Tx test cases |  |
| firstOFDMSymbolInTimeDomain | 12 | I0 = 12 |  |
| cdm-Type | fd-CDM2 | 2Tx and 4Tx test cases |  |
|  | noCDM | 1Tx test cases |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

<end of changes>