Title: 	Discussion on test point selection for MOP for E-UTRA-NR V2X inter operation

Source: 	Huawei

Introduction

The purpose of this contribution is to provide the complete analysis for each parameter included in the Test Configuration Table and propose test points selection for MOP for E-UTRA/NR Uu and E-UTRA/NR SL inter operation in 38.521-3.

Discussion

Following subclauses introduce study for test environment, test frequencies, test bandwidth, test subcarrier spacing and uplink configuration aspects.

UE maximum output power for Intra-band contiguous V2X

For MOP of intra-band contiguous V2X, no exception requirements apply to NR sidelink operation or E-UTRA V2X operation. Given only single switched SL is supported for all intra-band contiguous configurations, the requirements can be well covered in clause 6.2.2G of TS 36.521-1 and clause 6.2E.1 of TS 38.521-1 and don’t need to be tested again

Proposal 1: No test point is needed for intra-band contiguous V2X.

UE maximum output power for Intra-band non-contiguous V2X

For MOP of intra-band non-contiguous V2X, no exception requirements apply to NR sidelink operation or E-UTRA V2X operation. Given only single switched SL is supported for all intra-band contiguous configurations, the requirements can be well covered in clause 6.2.2G of TS 36.521-1 and clause 6.2E.1 of TS 38.521-1 and don’t need to be tested again

Proposal 2: No test point is needed for intra-band non-contiguous V2X.

UE maximum output power for Inter-band concurrent V2X

For inter-band concurrent V2X configurations, concurrent operation is allowed. The analysis in this clause applies to both inter-band E-UTRA Uu + NR sidelink and NR Uu + E-UTRA V2X configurations.

    - Test Environment

MOP is one of the most critical requirements, and the test environment for Uu only or SL only test cases are selected as Normal, TL/VL, TL/VH, TH/VL and TH/VH. The same environment selection can be adopted for V2X test cases.

Proposal 3: Define test environment as Normal, TL/VL, TL/VH, TH/VL and TH/VH for inter-band concurrent V2X testing.

    - Test Subcarrier Spacing

Test subcarrier spacing for NR Uu or NR SL is selected as 15kHz so that the time slot could be aligned with E-UTRA subframe.

Proposal 4: Select 15kHz SCS for NR Uu or NR SL carrier for inter-band concurrent V2X testing.

    - Test Frequencies

Test frequencies are selected as Low, Mid and High range for Uu only or SL only test cases. For the sake of saving test time, it could be reduced to Low and High for inter-band concurrent V2X requirements.

Proposal 5: Test Low range for each carrier and High range for each carrier for inter-band concurrent V2X testing.

    - Test Channel Bandwidth

Test channel bandwidth is selected as Lowest, Mid and Highest for Uu only or SL only test cases. For the sake of saving test time, it could be reduced to Lowest and Highest for each carrier for inter-band concurrent V2X requirements.

Proposal 6: Test Lowest channel bandwidth for each carrier and Highest channel bandwidth for each carrier for inter-band concurrent V2X testing.

    - Modulations and RB allocation

MPR is always >0dB for E-UTRA/NR SL carrier. In order to reach the overall maximum output power, test configuration on Uu carrier shall be selected such that MPR=0dB, while the test configuration on Sl carrier shall be selected such that minimum MPR applies.

Proposal 7: For Uu carrier the combination of modulation and RB allocation is selected such that MPR=0dB. For SL carrier the combination of modulation and RB allocation is selected such that minimum MPR applies.

    - Number of Test Points

Based on above proposal, the total test points are tabulated below:

Table 2.3.6-1 Total test points of Inter-band concurrent V2X

| Environment conditions | Test SCS | Test Frequencies | Test ChBW | Test ID | Total Test Points |
| --- | --- | --- | --- | --- | --- |
| 5 | 1 | 2 | 2 | 1 | 20 |

Conclusion

It is proposed that RAN5 agrees below proposals on test points of MPR, SEM and ACLR for NR V2X:

Proposal 1: No test point is needed for intra-band contiguous V2X.

Proposal 2: No test point is needed for intra-band non-contiguous V2X.

Proposal 3: Define test environment as Normal, TL/VL, TL/VH, TH/VL and TH/VH for inter-band concurrent V2X testing.

Proposal 4: Select 15kHz SCS for NR Uu or NR SL carrier for inter-band concurrent V2X testing.

Proposal 5: Test Low range for each carrier and High range for each carrier for inter-band concurrent V2X testing.

Proposal 6: Test Lowest channel bandwidth for each carrier and Highest channel bandwidth for each carrier for inter-band concurrent V2X testing.

Proposal 7: For Uu carrier the combination of modulation and RB allocation is selected such that MPR=0dB. For SL carrier the combination of modulation and RB allocation is selected such that minimum MPR applies.

References

[1] R5-180065, “Discussion about Test parameters for NR TRx TCs“, NTT DOCOMO, RAN5 Ad hoc, Munich, Jan. 2018.

[2] 3GPP TS 38.101-3 v16.9.0 User Equipment (UE) radio transmission and reception; Part 3: Range 1 and Range 2 Interworking operation with other radios

[3] 3GPP TS 38.521-3 v17.2.0 User Equipment (UE) conformance specification; Radio transmission and reception; Part 1: Range 1 Standalone