3GPP TSG-RAN WG5 Meeting #93-e	R5-217671

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.523-1 | CR | 5070 | rev | - | Current version: | 16.10.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of UE capability test case 8.5.4.1 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | RACS-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | UE capability test case 8.5.4.1 needs to be updated to allow uplink segmentation for RACS. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Enabled uplink segmentation in the UE capability message for Rel-16 RACS. Added corresponding conformance requirements. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | RACS workplan will be incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | TTCN Impact |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of modified section>

#### 8.5.4.1	UE capability transfer / Success

8.5.4.1.1	Test Purpose (TP)

(1)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message before AS security is activated }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

(2)

with { UE in RRC_CONNECTED state }

ensure that {

when { UE receives an UECapabilityEnquiry message after AS security is activated }

then { UE transmits an UECapabilityInformation message including UE radio access capability information corresponding to the ue-CapabilityRequest variable }

}

8.5.4.1.2	Conformance requirements

References: The conformance requirements covered in the present TC are specified in: TS 36.331, clause 5.6.3.3, 5.6.22.2, 5.6.22.3 and TS 25.331, clause 8.1.16.3.

[TS 36.331, clause 5.6.3.3]

The UE shall:

1>	set the contents of UECapabilityInformation message as follows:

2>	if the ue-CapabilityRequest includes ‘eutra’:

3>	include the UE-EUTRA-Capability within a ue-CapabilityRAT-Container and with the rat-Type set to ‘eutra’;

2>	if the ue-CapabilityRequest includes ‘geran-cs’ and if the UE supports GERAN CS domain:

3>	include the UE radio access capabilities for GERAN CS within a ue-CapabilityRAT-Container and with the rat-Type set to ‘geran-cs’;

2>	if the ue-CapabilityRequest includes ‘geran-ps’ and if the UE supports GERAN PS domain:

3>	include the UE radio access capabilities for GERAN PS within a ue-CapabilityRAT-Container and with the rat-Type set to ‘geran-ps’;

2>	if the ue-CapabilityRequest includes ‘utra’ and if the UE supports UTRA:

3>	include the UE radio access capabilities for UTRA within a ueCapabilityRAT-Container and with the rat-Type set to ‘utra’;

2>	if the ue-CapabilityRequest includes ‘cdma2000-1XRTT’ and if the UE supports CDMA2000-1xRTT:

3>	include the UE radio access capabilities for CDMA 2000 within a ueCapabilityRAT-Container and with the rat-Type set to ‘cdma2000-1XRTT’;

…

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8]:

2>	initiate the UL message segment transfer procedure as specified in clause 5.6.22;

1>	else:

1> submit the UECapabilityInformation message to lower layers for transmission, upon which the procedure ends.

[TS 25.331, clause 8.1.16.3]

The UE shall:

1>	if the UE supports both UMTS TDD and FDD:

2>	determine whether capabilities are being requested for FDD or TDD mode; and

2>	use the capabilities associated with that mode for the remainder of this procedure.

1>	include the IE "UE security information", and the IE "UE security information2" if inter-RAT PS handover is supported by the UE; and

1>	not include the IE "UE Specific Behaviour Information 1 interRAT";

1>	in case support for the compressed version of the inter RAT handover info is indicated via the other radio access technology:

2>	if the other radio access technology is not E-UTRA:

3>	include of the following IEs the IE that after encoding has the smallest size: IE "Predefined configuration status information compressed" or the IE "Predefined configuration status information".

2>	else:

3>	exclude the IE "Predefined configuration status information" and "Predefined configuration status information compressed".

2>	include the IE "UE radio access capability compressed".

1>	else:

2>	if the other radio access technology is not E-UTRA:

3>	include the IE "Predefined configuration status information".

2>	else:

3>	exclude the IE "Predefined configuration status information".

2>	include the IE "UE capability container", containing the IE "UE radio access capability" and the IE "UE radio access capability extension", in accordance with the following:

3>	if the UE supports multiple UTRA FDD Frequency Bands; or

3>	if the UE supports a single UTRA FDD Frequency Band different from Band I [21]; or

3>	if the UE supports E-UTRA:

4>	include the IE "UE radio access capability", excluding IEs "RF capability FDD" and "Measurement capability" for FDD and including the IE "Measurement capability TDD" for TDD;

4>	include the IE "UE radio access capability extension", including the IEs "RF capability FDD extension", the "Measurement capability extension", the "Additional Secondary Cells" and the "Non-contiguous multi-cell" associated with each supported UTRA FDD frequency band indicated in the IE "Frequency band", but may omit all or part of these IEs for supported inter-RAT bands.

3>	else:

4>	include the IE "UE radio access capability", including the IEs "RF capability FDD" and "Measurement capability" associated with the Band I [21] for FDD and excluding the IE "Measurement capability TDD" for TDD;

4>	include the IE "UE radio access capability extension", including the IEs "RF capability FDD extension", the "Measurement capability extension", the "Additional Secondary Cells" and the "Non-contiguous multi-cell" associated with each supported UTRA FDD frequency band indicated in the IE "Frequency band".

1>	For FDD, include the IE "UE radio access capability comp 2";

1>	For 1.28 Mcps TDD, include the IE "UE radio access capability comp for 1.28 Mcps TDD";

1>	initiate the transfer of the INTER RAT HANDOVER INFO message via the other radio access technology, using radio access technology-specific procedures;

1>	store the following in the variable INTER_RAT_HANDOVER_INFO_TRANSFERRED if they were included in the INTER RAT HANDOVER INFO message:

2>	the IE "Predefined configuration status information";

2>	the IE "Predefined configuration status information compressed";

2>	the IE "UE security information";

2>	the IE "UE security information2";

2>	the IE "UE radio access capability";

2>	the IE "UE radio access capability extension"; and

2>	the IE "UE radio access capability compressed";

2>	if the IE "UE radio access capability compressed" were included in the INTER RAT HANDOVER INFO message:

3>	set the IE "Security Capability" to the mandatory R99 algorithms.

1>	and the procedure ends.

[TS 36.331, clause 5.6.22.2]

A UE capable of UL RRC message segmentation in RRC_CONNECTED will initiate the procedure when the following conditions are met:

1>	if the RRC message segmentation is enabled based on the field rrc-SegAllowed received, and

1>	if the encoded RRC message is larger than the maximum supported size of a PDCP SDU specified in TS 36.323 [8];

Upon initiating the procedure, the UE shall:

- initiate transmission of the ULDedicatedMessageSegment message as specified in 5.6.22.3;

[TS 36.331, clause 5.6.22.3]

The UE shall segment the encoded RRC PDU based on the maximum supported size of a PDCP SDU specified in TS 36.323 [8]. UE shall minimize the number of segments and set the contents of the ULDedicatedMessageSegment messages as follows:

1>	For each new UL DCCH message, set the segmentNumber to 0 for the first message segment and increment the segmentNumber for each subsequent RRC message segment;

1>	set rrc-MessageSegmentContainer to include the segment of the UL DCCH message corresponding to the segmentNumber;

1>	if the segment included in the rrc-MessageSegmentContainer is the last segment of the UL DCCH message:

2>	set the rrc-MessageSegmentType to lastSegment;

1>	else:

2>	set the rrc-MessageSegmentType to notLastSegment;

1>	submit all the ULDedicatedMessageSegment messages generated for the segmented RRC message to lower layers for transmission in ascending order based on the segmentNumber, upon which the procedure ends.

8.5.4.1.3	Test description

8.5.4.1.3.1	Pre-test conditions

System Simulator:

-	Cell 1

UE:

None.

Preamble:

-	The UE is in state Switched OFF (state 1) according to [18].

8.5.4.1.3.2	Test procedure sequence

Table 8.5.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The UE is switched on. | - | - | - | - |
| 2-8 | Steps 2-8 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 9 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E UTRA only with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 10a1 and 10a2 will be performed else step 10b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 10a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 10a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? | --> | ULDedicatedMessageSegment |  |  |
| 10a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 1 | P |
| 10b1 | Check: Does the UE transmit a UECapabilityInformation message? | --> | UECapabilityInformation | 1 | P |
| 11 | The SS transmits a SecurityModeCommand message to activate AS security. | <-- | SecurityModeCommand | - | - |
| 12 | The UE transmits a SecurityModeComplete message and establishes the initial security configuration. | --> | SecurityModeComplete | - | - |
| - | EXCEPTION: Steps 13a1 to 13a2 describe behaviour that depends on UE configuration; the "lower case letter" identifies a step sequence that take place if the UE has ESM information which needs to be transferred after SECURITY MODE COMPLETE message. | - | - | - | - |
| 13a1 | IF the UE sets the ESM information transfer flag in the last PDN CONNECTIVITY REQUEST message THEN the SS transmits a DLInformationTransfer message. This message includes an ESM INFORMATION REQUEST message. | <-- | DLInformationTransfer | - | - |
| 13a2 | The UE transmits a ULInformationTransfer message. This message includes an ESM INFORMATION RESPONSE message. | --> | ULInformationTransfer | - | - |
| 14-16c1 | Steps 14-16c1 of the generic procedure for UE registration specified in TS 36.508 subclause 4.5.2.3 are performed. | - | - | - | - |
| 17 | The SS transmits a UECapabilityEnquiry message to request UE radio access capability information for E UTRA only with rrc-segAllowed-r16 set to enabled. | <-- | UECapabilityEnquiry | - | - |
| - | EXCEPTION: The "lower case letter" identifies a step sequence that takes place depending on the size of UE capability message. IF pc_LTE_UL_Segmentation and UE’s encoded UECapabilityInformation message >= Max PDCP SDU size then steps 18a1 and 18a2 will be performed else step 18b1 will be performed | - | - | - | - |
| - | EXCEPTION: Step 18a1 is repeated a maximum of 16 times or till rrc-MessageSegmentType-16 IE within the UlDedicatedMessageSegment message indicate rrc-MessageSegmentType-r16 = lastSegment. | - | - | - | - |
| 18a1 | Check: Does the UE send segments of UECapabilityInformation message contained within the rrc-MessageSegmentContainer-r16 IE of the ULDedicatedMessageSegment message? | --> | ULDedicatedMessageSegment |  |  |
| 18a2 | Check: Is the received UE radio access capability information as per the ue-CapabilityRequest variable? | - | - | 2 | P |
| 18b1 | Check: Does the UE transmit a UECapabilityInformation message? | --> | UECapabilityInformation | 2 | P |
| - | EXCEPTION: Steps 19a1 to 19a2 describe behaviour that depends on the UE capability. | - | - | - | - |
| 19a1 | IF pc_FDD, pc_TDD_HCR, pc_TDD_LCR, pc_TDD_VHCR, pc_GERAN, pc_1xRTT or pc_HRPD THEN the SS transmits a UECapabilityEnquiry message to request UE radio access capability information for every other supported RATs. | <-- | UECapabilityEnquiry | - | - |
| 19a2 | Check: Does the UE transmit a UECapabilityInformation message? | --> | UECapabilityInformation | 2 | P |
| 20-22 | IF MULTI_PDN = TRUE (NOTE) THEN steps 10-12 of the generic procedure for network initiated release of additional PDN connectivity specified in TS 36.508 subclause 4.5A.18.3 are performed for the non-IMS PDN. | - | - | - | - |
| NOTE: MULTI_PDN as defined in TS 36.508 subclause 4.5.2, |  |  |  |  |  |

8.5.4.1.3.3	Specific message contents

Table 8.5.4.1.3.3-1: UEcapabilityEnquiry (step 9 and 17, Table 8.5.4.1.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark |  | Comment |  | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |
| c1 CHOICE { |  |  |  |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { |  |  |  |  |  |  |
| ue-CapabilityRequest SEQUENCE (SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { |  | 1 entry |  | E-UTRA only |  |  |
| RAT-Type[1] |  | eutra |  |  |  |  |
| } |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| lateNonCriticalExtension |  | Not present |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| requestReducedFormat-r13 |  | Not present |  |  |  |  |  |  |
| requestSkipFallbackComb-r13 |  | Not present |  |  |  |  |  |  |
| requestedMaxCCsDL-r13 |  | Not present |  |  |  |  |  |  |
| requestedMaxCCsUL-r13 |  | Not present |  |  |  |  |  |  |
| requestReducedIntNonContComb-r13 |  | Not present |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| requestDiffFallbackCombList-r14 |  | Not present |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| requestedFreqBandsNR-MRDC-r15 |  | Not present |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  | Not present |  |  |  |  |  |  |
| requestSTTI-SPT-Capability-r15 |  | Not present |  |  |  |  |  |  |
| eutra-nr-only-r15 |  | Not present |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| requestedCapabilityNR-r15 |  | Not present |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| requestedCapabilityCommon-r15 |  | Not present |  |  |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |  |  |  |  |
| rrc-SegAllowed-r16 |  | enabled |  |  |  |  |  |  |
| nonCriticalExtension |  | Not present |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |
| } |  |  |  |  |  |  |

Table 8.5.4.1.3.3-2: UECapabilityInformation (step 10a2 and 18b1, Table 8.5.4.1.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/Remark | Comment |  | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| c1 CHOICE{ |  |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { | 1 entry only |  |  |  |
| rat-Type[1] | eutra | E-UTRA only |  |  |
| ueCapabilitiesRAT-Container[1] OCTET STRING { |  |  |  |  |
| UE-EUTRA-Capability SEQUENCE { |  |  |  |  |
| accessStratumRelease | Not checked | Value should be based on Rel of Access stratum supported |  |  |
| ue-Category | Checked against UE Category indications in the PICS |  |  | Rel-8 or Rel-9 |
| pdcp-Parameters | Not checked |  |  | Rel-8 |
| pdcp-Parameters SEQUENCE { |  |  |  | > Rel-8 |
| supportedROHC-Profiles SEQUENCE { |  |  |  |  |
| profile0x0001 | Checked |  |  | pc_ROHC_profile0x0001 |
| profile0x0002 | Checked |  |  | pc_ROHC_profile0x0002 |
| profile0x0003 | Not checked |  |  |  |
| profile0x0004 | Not checked |  |  |  |
| profile0x0006 | Not checked |  |  |  |
| profile0x0101 | Not checked |  |  |  |
| profile0x0102 | Not checked |  |  |  |
| profile0x0103 | Not checked |  |  |  |
| profile0x0104 | Not checked |  |  |  |
| } |  |  |  |  |
| maxNumberROHC-ContextSessions | Not checked |  |  |  |
| } |  |  |  |  |
| phyLayerParameters SEQUENCE { |  |  |  |  |
| ul-AntennaSelectionSupported | Not checked |  |  |  |
| ue-SpecificRefSigsSupported | Not checked |  |  |  |
| } |  |  |  |  |
| rf-Parameters SEQUENCE { |  |  |  |  |
| supportedBandListEUTRA (SIZE (1..maxBands)) OF SEQUENCE { | n1 entries where n2 is the sum of pc_eBandα_Supp for α= 1 to 64, n3 is the sum of reported bands without pc_eBandα_Supp for α= 1 to 64, and n4 is the number of extended bands indicated as maxFBI (=64) corresponding to the extended bands in supportedBandListEUTRA_v9e0 (Note 14). | n1= n2+n3+n4 |  | not pc_FDD_TypeB_HalfDuplex |
| BandListEUTRA SEQUENCE { |  |  |  |  |
| bandEUTRA [α = 1..n2] | Any value β such that pc_eBandβ_Supp is TRUE and different from all eutra-Band[k] where k = 1 to α – 1. Except bands indicated as extended bands. |  |  |  |
| halfDuplex[α = 1..n2] | False |  |  | FDD |
| } |  |  |  |  |
| } |  |  |  |  |
| supportedBandListEUTRA (SIZE (1..maxBands)) OF SEQUENCE { | n1 entries where n2 is the sum of pc_eBandα_Supp for α= 1 to 64, n3 is the sum of reported bands without pc_eBandα_Supp for α= 1 to 64, and n4 is the number of extended bands indicated as maxFBI (=64) corresponding to the extended bands in supportedBandListEUTRA_v9e0 (Note 14). | n1=n2+n3+n4 |  | pc_FDD_TypeB_HalfDuplex |
| BandListEUTRA SEQUENCE { |  |  |  |  |
| bandEUTRA [α = 1..n2] | Any value β such that pc_eBandβ_Supp is TRUE and different from all eutra-Band[k] where k = 1 to α – 1. . Except bands indicated as extended bands. |  |  |  |
| halfDuplex[α = 1..n2] | true |  |  | FDD |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measurementParameters SEQUENCE { |  |  |  |  |
| BandListEUTRA(SIZE (1..maxBands)) OF SEQUENCE { | same number of entries like in SupportedBandListEUTRA |  |  |  |
| BandInfoEUTRA SEQUENCE { |  |  |  |  |
| InterFreqBandList (SIZE (1..maxBands)) OF SEQUENCE { | m entries (contents not checked, but m shall be equal to the number of bands listed in each IE present in interRAT-Parameters) |  |  |  |
| InterFreqBandInfo |  |  |  |  |
| } |  |  |  |  |
| InterRAT-BandList (SIZE (1..maxBands)) OF SEQUENCE { |  |  |  |  |
| InterRAT-BandInfo | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureGroupIndicators | shall be set according to the corresponding PICS items |  |  |  |
| -- FGI 1 | Checked |  |  |  |
| -- FGI 2 | Checked |  |  |  |
| -- FGI 3 | Checked |  |  |  |
| -- FGI 4 | Checked |  |  |  |
| -- FGI 5 | Checked |  |  |  |
| -- FGI 6 | Checked |  |  |  |
| -- FGI 7 | Checked |  |  |  |
| -- FGI 8 | Checked |  |  |  |
| -- FGI 9 | Checked |  |  |  |
| -- FGI 10 | Checked |  |  |  |
| -- FGI 11 | Checked |  |  |  |
| -- FGI 12 | Checked |  |  |  |
| -- FGI 13 | Checked |  |  |  |
| -- FGI 14 | Checked |  |  |  |
| -- FGI 15 | Checked |  |  |  |
| -- FGI 16 | Checked |  |  |  |
| -- FGI 17 | Checked |  |  |  |
| -- FGI 18 | Checked |  |  |  |
| -- FGI 19 | Checked |  |  |  |
| -- FGI 20 | Checked |  |  |  |
| -- FGI 21 | Checked |  |  |  |
| -- FGI 22 | Checked |  |  |  |
| -- FGI 23 | Checked |  |  |  |
| -- FGI 24 | Checked |  |  |  |
| -- FGI 25 | Checked |  |  |  |
| -- FGI 26 | Checked |  |  |  |
| -- FGI 27 | Checked |  |  |  |
| -- FGI 28 | Checked |  |  |  |
| -- FGI 29 | Checked |  |  |  |
| -- FGI 30 | Checked |  |  |  |
| -- FGI 31 | Checked |  |  |  |
| -- FGI 32 | ‘0’B (Undefined) |  |  |  |
| interRAT-Parameters SEQUENCE { | m elements are present |  |  |  |
| utraFDD | Present but value not checked | See Note 11 |  | pc_FDD AND (px_ReportUTRA_RAT=fdd OR px_ReportUTRA_RAT=both) |
| utraTDD128 | Present but value not checked | See Note 11 |  | pc_TDD_LCR AND Not px_NoTDD_LCR_onLTE AND (px_ReportUTRA_RAT=tdd OR px_ReportUTRA_RAT=both) Note 4 |
| utraTDD384 | Present but value not checked | See Note 11 |  | pc_TDD_HCR AND (px_ReportUTRA_RAT=tdd OR px_ReportUTRA_RAT=both) |
| utraTDD768 | Present but value not checked | See Note 11 |  | pc_TDD_VHCR AND (px_ReportUTRA_RAT=tdd OR pc_ReportUTRA_RAT=both) |
| geran | Present but value not checked |  |  | pc_GERAN |
| cdma2000-HRPD | Present but value not checked |  |  | pc_HRPD |
| cdma2000-1xRTT | Present but value not checked |  |  | pc_1xRTT |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { | See Note 2 |  |  |  |
| phyLayerParameters-v920 | Not checked |  |  |  |
| interRAT-ParametersGERAN-v920 | Not checked |  |  |  |
| interRAT-ParametersUTRA-v920 | Not checked |  |  |  |
| interRAT-ParametersCDMA2000-v920 | Not checked |  |  |  |
| deviceType-r9 | Not checked |  |  |  |
| csg-ProximityIndicationParameters-r9 | Not checked |  |  |  |
| neighCellSI-AcquisitionParameters-r9 | Not checked |  |  |  |
| son-Parameters-r9 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lateNonCriticalExtension SEQUENCE { |  |  |  |  |
| featureGroupIndRel9Add-r9 | If present, shall be set according to the corresponding PICS items |  |  |  |
| -- FGI 33 | Checked |  |  |  |
| -- FGI 34 | Checked |  |  |  |
| -- FGI 35 | Checked |  |  |  |
| -- FGI 36 | Checked |  |  |  |
| -- FGI 37 | Checked |  |  |  |
| -- FGI 38 | Checked |  |  |  |
| -- FGI 39 | Checked |  |  |  |
| -- FGI 40 | Checked |  |  |  |
| -- FGI 41 | Checked |  |  |  |
| -- FGI 42-64 | ‘0’B (Undefined) |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-r9 SEQUENCE { |  |  |  |  |
| phyLayerParameters-r9 | Not checked |  |  |  |
| featureGroupIndicators-r9 | Shall be set according to the corresponding PICS items. Checked. See Note 1. | BITSTRING 32 |  |  |
| -- FGI 1_F | Checked |  |  |  |
| -- FGI 2_F | Checked |  |  |  |
| -- FGI 3_F | Checked |  |  | FDD = TDD |
| -- FGI 4_F | Checked |  |  |  |
| -- FGI 5_F | Checked |  |  | FDD = TDD |
| -- FGI 6_F | Checked |  |  | FDD = TDD |
| -- FGI 7_F | Checked |  |  | FDD = TDD |
| -- FGI 8_F | Checked |  |  |  |
| -- FGI 9_F | Checked |  |  |  |
| -- FGI 10_F | Checked |  |  |  |
| -- FGI 11_F | Checked |  |  |  |
| -- FGI 12_F | Checked |  |  |  |
| -- FGI 13_F | Checked |  |  | FDD = TDD |
| -- FGI 14_F | Checked |  |  | FDD = TDD |
| -- FGI 15_F | Checked |  |  |  |
| -- FGI 16_F | Checked |  |  |  |
| -- FGI 17_F | Checked |  |  | FDD = TDD |
| -- FGI 18_F | Checked |  |  | FDD = TDD |
| -- FGI 19_F | Checked |  |  |  |
| -- FGI 20_F | Checked |  |  | FDD = TDD |
| -- FGI 21_F | Checked |  |  | FDD = TDD |
| -- FGI 22_F | Checked |  |  |  |
| -- FGI 23_F | Checked |  |  |  |
| -- FGI 24_F | Checked |  |  |  |
| -- FGI 25_F | Checked |  |  | FDD = TDD |
| -- FGI 26_F | Checked |  |  |  |
| -- FGI 27_F | Checked |  |  |  |
| -- FGI 28_F | Checked |  |  |  |
| -- FGI 29_F | Checked |  |  |  |
| -- FGI 30_F | Checked |  |  | FDD = TDD |
| -- FGI 31_F | Checked |  |  |  |
| -- FGI 32_F | ‘0’B (Undefined) |  |  |  |
| featureGroupIndRel9Add-r9 | Shall be set according to the corresponding PICS items. Checked. See Note 1. | BITSTRING 32 |  |  |
| -- FGI 33_F | Checked |  |  |  |
| -- FGI 34_F | Checked |  |  |  |
| -- FGI 35_F | Checked |  |  |  |
| -- FGI 36_F | Checked |  |  |  |
| -- FGI 37_F | Checked |  |  |  |
| -- FGI 38_F | Checked |  |  |  |
| -- FGI 39_F | Checked |  |  |  |
| -- FGI 40_F | Checked |  |  |  |
| -- FGI 41_F | Checked |  |  |  |
| -- FGI 42_F | Checked |  |  |  |
| -- FGI 43-64_F | ‘0’B (Undefined) |  |  |  |
| interRAT-ParametersGERAN-r9 | Not checked |  |  |  |
| interRAT-ParametersUTRA-r9 | Not checked |  |  |  |
| interRAT-ParametersGERAN-r9 | Not checked |  |  |  |
| interRAT-ParametersCDMA2000-r9 | Not checked |  |  |  |
| neighCellSI-AcquisitionParameters-r9 | Not checked |  |  |  |
| } |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-r9 SEQUENCE { |  |  |  |  |
| phyLayerParameters-r9 | Not checked |  |  |  |
| featureGroupIndicators-r9 | Shall be set according to the corresponding PICS items. Checked. See Note 1. | BITSTRING 32 |  |  |
| -- FGI 1_T | Checked |  |  |  |
| -- FGI 2_T | Checked |  |  |  |
| -- FGI 3_T | Checked |  |  | FDD = TDD |
| -- FGI 4_T | Checked |  |  |  |
| -- FGI 5_T | Checked |  |  | FDD = TDD |
| -- FGI 6_T | Checked |  |  | FDD = TDD |
| -- FGI 7_T | Checked |  |  | FDD = TDD |
| -- FGI 8_T | Checked |  |  |  |
| -- FGI 9_T | Checked |  |  |  |
| -- FGI 10_T | Checked |  |  |  |
| -- FGI 11_T | Checked |  |  |  |
| -- FGI 12_T | Checked |  |  |  |
| -- FGI 13_T | Checked |  |  | FDD = TDD |
| -- FGI 14_T | Checked |  |  | FDD = TDD |
| -- FGI 15_T | Checked |  |  |  |
| -- FGI 16_T | Checked |  |  |  |
| -- FGI 17_T | Checked |  |  | FDD = TDD |
| -- FGI 18_T | Checked |  |  | FDD = TDD |
| -- FGI 19_T | Checked |  |  |  |
| -- FGI 20_T | Checked |  |  | FDD = TDD |
| -- FGI 21_T | Checked |  |  | FDD = TDD |
| -- FGI 22_T | Checked |  |  |  |
| -- FGI 23_T | Checked |  |  |  |
| -- FGI 24_T | Checked |  |  |  |
| -- FGI 25_T | Checked |  |  | FDD = TDD |
| -- FGI 26_T | Checked |  |  |  |
| -- FGI 27_T | Checked |  |  |  |
| -- FGI 28_T | Checked |  |  |  |
| -- FGI 29_T | Checked |  |  |  |
| -- FGI 30_T | Checked |  |  | FDD = TDD |
| -- FGI 31_T | Checked |  |  |  |
| -- FGI 32_T | ‘0’B (Undefined) |  |  |  |
| featureGroupIndRel9Add-r9 | Shall be set according to the corresponding PICS items. Checked. See Note 1 | BITSTRING 32 |  |  |
| -- FGI 33_T | Checked |  |  |  |
| -- FGI 34_T | Checked |  |  |  |
| -- FGI 35_T | Checked |  |  |  |
| -- FGI 36_T | Checked |  |  |  |
| -- FGI 37_T | Checked |  |  |  |
| -- FGI 38_T | Checked |  |  |  |
| -- FGI 39_T | Checked |  |  |  |
| -- FGI 40_T | Checked |  |  |  |
| -- FGI 41_T | Checked |  |  |  |
| -- FGI 42_T | Checked |  |  |  |
| -- FGI 43-64_T | ‘0’B (Undefined) |  |  |  |
| interRAT-ParametersGERAN-r9 | Not checked |  |  |  |
| interRAT-ParametersUTRA-r9 | Not checked |  |  |  |
| interRAT-ParametersGERAN-r9 | Not checked |  |  |  |
| interRAT-ParametersCDMA2000-r9 | Not checked |  |  |  |
| neighCellSI-AcquisitionParameters-r9 | Not checked |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| interRAT-ParametersUTRA-v9c0 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |  |
| phyLayerParameters-v9d0 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |  |
| rf-Parameters-v9e0 SEQUENCE { |  |  |  |  |
| supportedBandListEUTRA-v9e0 (SIZE (1..maxBands)) OF SEQUENCE { | n1 entries where n2 is the sum of pc_eBandα_Supp for α= 65 to 256 and n3 is the sum of reported bands without pc_eBandα_Supp for α= 65 to 256 | n1= n2+n3 |  |  |
| BandListEUTRA-v9e0 SEQUENCE { |  |  |  |  |
| bandEUTRA-v9e0 [α = 1..n2] | Any value β such that pc_eBandβ_Supp is TRUE and different from all eutra-Band[k] where k = 1 to α - 1 | band 65 onwards, up to 64 bands |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { | See Note 2 |  |  |  |
| ue-Category-v1020 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| phyLayerParameters-v1020 | Not checked |  |  |  |
| rf-Parameters-v1020 { | Not checked |  |  |  |
| supportedBandCombination-r10 { | For as many band combinations as supported |  |  | Various conditions (Note 5) |
| BandCombinationParameters-r10 { |  |  |  |  |
| BandParameters-r10 { |  |  |  |  |
| bandEUTRA-r10 { | Not checked |  |  |  |
| } |  |  |  |  |
| BandParameterUL-r10 { |  |  |  |  |
| CA-MIMO-ParametersUL-r10 { | For as many bandwidth classes as supported |  |  |  |
| ca-BandwidthClassUL-r10 | Not checked |  |  |  |
| supportedMIMO-CapabilityUL-r10 | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| BandParameterDL-r10 { | For as many bandwidth classes as supported |  |  |  |
| CA-MIMO-ParametersDL-r10 { |  |  |  |  |
| ca-BandwidthClassDL-r10 | Not checked |  |  |  |
| supportedMIMO-CapabilityDL-r10 | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| measParameters-v1020 { | Not checked |  |  |  |
| bandCombinationListEUTRA-r10 { | For as many band combinations as supported |  |  |  |
| BandInfoEUTRA { |  |  |  |  |
| interFreqBandList { |  |  |  |  |
| InterFreqBandInfo { |  |  |  |  |
| interFreqNeedForGaps | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| interRAT-BandList { |  |  |  |  |
| InterRAT-BandInfo { |  |  |  |  |
| interRAT-NeedForGaps | Not checked |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| featureGroupIndRel10-r10 | Shall be set according to the corresponding PICS items. Checked. | BITSTRING 32 |  |  |
| -- FGI 101 | Checked |  |  |  |
| -- FGI 102 | Checked |  |  |  |
| -- FGI 103 | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 104 | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 105 | Checked |  |  |  |
| -- FGI 106 | Checked |  |  |  |
| -- FGI 107 | Checked |  |  |  |
| -- FGI 108 | Checked |  |  |  |
| -- FGI 109 | Checked |  |  |  |
| -- FGI 110 | Checked |  |  |  |
| -- FGI 111 | Checked |  |  |  |
| -- FGI 112 | Checked |  |  |  |
| -- FGI 113 | Checked | IF set to ‘1’ THEN pc_UL_CA_2Carriers shall be TRUE |  | pc_UL_CA_2Carriers |
| -- FGI 114 | Checked |  |  |  |
| -- FGI 115 | Checked |  |  |  |
| -- FGI 116 | Checked |  |  |  |
| -- FGI 117-132 | ‘0’B (Undefined) |  |  |  |
| interRAT-ParametersCDMA2000-v1020 | Not checked |  |  |  |
| ue-BasedNetwPerfMeasParameters-r10 | Not checked |  |  |  |
| interRAT-ParametersUTRA-TDD-v1020 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1060 | Shall be set according to the corresponding PICS items. Checked. See Note 1 | BITSTRING 32 |  |  |
| -- FGI 101_F | Checked |  |  |  |
| -- FGI 102_F | Checked |  |  |  |
| -- FGI 103_F | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 104_F | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 105_F | Checked |  |  |  |
| -- FGI 106_F | Checked |  |  |  |
| -- FGI 107_F | Checked |  |  |  |
| -- FGI 108_F | Checked |  |  |  |
| -- FGI 109_F | Checked |  |  |  |
| -- FGI 110_F | Checked |  |  |  |
| -- FGI 111_F | Checked |  |  |  |
| -- FGI 112_F | Checked |  |  |  |
| -- FGI 113_F | Checked | IF set to ‘1’ THEN pc_UL_CA_2Carriers shall be TRUE |  | pc_UL_CA_2Carriers |
| -- FGI 114_F | Checked |  |  |  |
| -- FGI 115_F | Checked |  |  |  |
| -- FGI 116_F | Checked |  |  |  |
| -- FGI 117-132_F | ‘0’B (Undefined) |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1060 | Shall be set according to the corresponding PICS items. Checked. See Note 1 | BITSTRING 32 |  |  |
| -- FGI 101_T | Checked |  |  |  |
| -- FGI 102_T | Checked |  |  |  |
| -- FGI 103_T | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 104_T | Checked |  |  | Rel-10 to Rel-14 OR (>=Rel 15 AND NOT(UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14)) (Note 12) |
|  | ‘1’B |  |  | >= Rel-15 AND (UE category 8 OR UE category 11 OR UE category 12 OR UE DL Category 11 OR UE DL Category 12 OR UE DL Category 14) (Note 12) |
| -- FGI 105_T | Checked |  |  |  |
| -- FGI 106_T | Checked |  |  |  |
| -- FGI 107_T | Checked |  |  |  |
| -- FGI 108_T | Checked |  |  |  |
| -- FGI 109_T | Checked |  |  |  |
| -- FGI 110_T | Checked |  |  |  |
| -- FGI 111_T | Checked |  |  |  |
| -- FGI 112_T | Checked |  |  |  |
| -- FGI 113_T | Checked | IF set to ‘1’ THEN pc_UL_CA_2Carriers shall be TRUE |  | pc_UL_CA_2Carriers |
| -- FGI 114_T | Checked |  |  |  |
| -- FGI 115_T | Checked |  |  |  |
| -- FGI 116_T | Checked |  |  |  |
| -- FGI 117-132_T | ‘0’B (Undefined) |  |  |  |
| rf-Parameters-v1060 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rf-Parameters-v1090 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { | See Note 2 |  |  |  |
| pdcp-Parameters-v1130 | Not checked |  |  |  |
| phyLayerParameters-v1130 | Not checked |  |  | Rel-8 to Rel-10 |
| phyLayerParameters-v1130 SEQUENCE { |  |  |  | > Rel-10 |
| crs-InterfHandl-r11 | Checked |  |  | pc_CRS_Interference_Handling (Note 10) |
| ePDCCH-r11 | Not checked |  |  |  |
| multiACK-CSI-Reporting-r11 | Not checked |  |  |  |
| ss-CCH-InterfHandl-r11 | Checked |  |  | pc_ss_CCH_Interference_Handling (Note 6) |
| tdd-SpecialSubframe-r11 | Not checked |  |  |  |
| txDiv-PUCCH1b-ChSelect-r11 | Not checked |  |  |  |
| ul-CoMP-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| rf-Parameters-v1130 | Not checked |  |  |  |
| measParameters-v1130 | Not checked |  |  |  |
| interRAT-ParametersCDMA2000-v1130 | Not checked |  |  |  |
| otherParameters-r11 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1130 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1130 |  |  |  | > Rel-10 |
| phyLayerParameters-v1130 SEQUENCE { |  |  |  |  |
| crs-InterfHandl-r11 | Checked |  |  | pc_CRS_Interference_Handling (Note 10) |
| ePDCCH-r11 | Not checked |  |  |  |
| multiACK-CSI-Reporting-r11 | Not checked |  |  |  |
| ss-CCH-InterfHandl-r11 | Checked |  |  | pc_ss_CCH_Interference_Handling (Note 6) |
| tdd-SpecialSubframe-r11 | Not checked |  |  |  |
| txDiv-PUCCH1b-ChSelect-r11 | Not checked |  |  |  |
| ul-CoMP-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| measParameters-v1130 | Not checked |  |  |  |
| otherParameters-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1130 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1130 |  |  |  | > Rel-10 |
| phyLayerParameters-v1130 SEQUENCE { |  |  |  |  |
| crs-InterfHandl-r11 | Checked |  |  | pc_CRS_Interference_Handling (Note 10) |
| ePDCCH-r11 | Not checked |  |  |  |
| multiACK-CSI-Reporting-r11 | Not checked |  |  |  |
| ss-CCH-InterfHandl-r11 | Checked |  |  | pc_ss_CCH_Interference_Handling (Note 6) |
| tdd-SpecialSubframe-r11 | Not checked |  |  |  |
| txDiv-PUCCH1b-ChSelect-r11 | Not checked |  |  |  |
| ul-CoMP-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| measParameters-v1130 | Not checked |  |  |  |
| otherParameters-r11 | Not checked |  |  |  |
| } |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| phyLayerParameters-v1170 | Not checked |  |  |  |
| ue-Category-v1170 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rf-Parameters-v1180 | Not checked |  |  |  |
| mbms-Parameters-r11 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1180 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1180 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-Category-v11a0 | Not checked |  |  |  |
| measParameters-v11a0 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| phyLayerParameters-v1250 | Not checked |  |  |  |
| rf-Parameters-v1250 SEQUENCE { |  |  |  |  |
| supportedBandListEUTRA-v1250 | Not checked |  |  |  |
| supportedBandCombination-v1250 | Not checked |  |  |  |
| supportedBandCombinationAdd-v1250 | Not checked |  |  |  |
| freqBandPriorityAdjustment-r12 | Checked |  | pc_freqBandPriorityAdjustment |  |
| } |  |  |  |  |
| rlc-Parameters-r12 | Not checked |  |  |  |
| ue-BasedNetwPerfMeasParameters-v1250 | Not checked |  |  |  |
| ue-CategoryDL-r12 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| ue-CategoryUL-r12 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| wlan-IW-Parameters-r12 | Not checked |  |  |  |
| measParameters-v1250 SEQUENCE { |  |  |  |  |
| timerT312-r12 | Not checked |  |  |  |
| alternativeTimeToTrigger-r12 | Not checked |  |  |  |
| incMonEUTRA-r12 | Not checked |  |  |  |
| incMonUTRA-r12 | Not checked |  |  |  |
| extendedMaxMeasId-r12 | Not checked |  |  |  |
| extendedRSRQ-LowerRange-r12 | Not checked |  |  |  |
| rsrq-OnAllSymbols-r12 | Not checked |  |  |  |
| crs-DiscoverySignalsMeas-r12 | Not checked |  |  |  |
| csi-RS-DiscoverySignalsMeas-r12 | Checked |  | pc_CSI-RS_DS_Meas |  |
| } |  |  |  |  |
| dc-Parameters-r12 | Not checked |  |  |  |
| mbms-Parameters-v1250 | Not checked |  |  |  |
| mac-Parameters-r12 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1250 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1250 | Not checked |  |  |  |
| sl-Parameters-r12 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-CategoryDL-v1260 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| rf-Parameters-v1270 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { | Not checked |  |  |  |
| phyLayerParameters-v1280 |  |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-CategoryDL-v1310 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 | See Note 3 | pc_ue_CategoryDL_M1 |  |
| ue-CategoryUL-v1310 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 | See Note 3 | pc_ue_CategoryUL_M1 |  |
| pdcp-Parameters-v1310 | Not checked |  |  |  |
| rlc-Parameters-v1310 | Not checked |  |  |  |
| mac-Parameters-v1310 SEQUENCE { |  |  |  |  |
| extendedMAC-LengthField-r13 | Not checked |  |  |  |
| extendedLongDRX-r13 | Checked |  | pc_extendedlongDRX |  |
| } |  |  |  |  |
| phyLayerParameters-v1310 | Not checked |  |  |  |
| rf-Parameters-v1310 | Not checked |  |  |  |
| measParameters-v1310 SEQUENCE { |  |  |  |  |
| rs-SINR-Meas-r13 | Not checked |  |  |  |
| whiteCellList-r13 | Not checked |  |  |  |
| extendedMaxObjectId-r13 | Not checked |  |  |  |
| ul-PDCP-Delay-r13 | Not checked |  |  |  |
| extendedFreqPriorities-r13 | Not checked |  |  |  |
| multiBandInfoReport-r13 | Not checked |  |  |  |
| rssi-AndChannelOccupancyReporting-r13 | Checked |  | pc_rssiAndChannelOccupancyReporting |  |
| } |  |  |  |  |
| dc-Parameters-v1310 | Not checked |  |  |  |
| sl-Parameters-v1310 | Not checked |  |  |  |
| scptm-Parameters-r13 | Not checked |  |  |  |
| ce-Parameters-r13 SEQUENCE { |  |  |  |  |
| ce-ModeA-r13 | Checked |  | pc_CEmodeA |  |
| ce-ModeB-r13 | Checked |  | pc_CEmodeB |  |
| } |  |  |  |  |
| interRAT-ParametersWLAN-r13 | Not checked |  |  |  |
| laa-Parameters-r13 SEQUENCE { |  |  |  |  |
| crossCarrierSchedulingLAA-DL-r13 | Not checked |  |  |  |
| csi-RS-DRS-RRM-MeasurementsLAA-r13 | Not checked |  |  |  |
| downlinkLAA-r13 | Checked |  | pc_downlink_LAA |  |
| endingDwPTS-r1310 | Not checked |  |  |  |
| secondSlotStartingPosition- r13 | Not checked |  |  |  |
| tm9-LAA-r13 | Not checked |  |  |  |
| tm10-LAA-r13 | Not checked |  |  |  |
| } |  |  |  |  |
| lwa-Parameters-r13 | Not checked |  |  |  |
| wlan-IW-Parameters-v1310 | Not checked |  |  |  |
| lwip-Parameters-r13 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1310 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1310 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ | Not checked |  |  |  |
| ce-Parameters-v1320 SEQUENCE { |  |  |  |  |
| intraFreqA3-CE-ModeA-r13 | Checked |  | pc_IntraFreqA3_CE_ModeA |  |
| intraFreqA3-CE-ModeB-r13 | Not checked |  |  |  |
| intraFreqHO-CE-ModeA-r13 | Checked |  | pc_IntraFreqHO_CE_ModeA |  |
| intraFreqHO-CE-ModeB-r13 | Not checked |  |  |  |
| } |  |  |  |  |
| phyLayerParameters-v1320 | Not checked |  |  |  |
| rf-Parameters-v1320 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1320 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1320 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ | Not checked |  |  |  |
| ue-CategoryDL-v1330 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| phyLayerParameters-v1330 | Not checked |  |  |  |
| ue-CE-NeedULGaps-r13 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ | Not checked |  |  |  |
| ue-CategoryUL-v1340 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| nonCritialExtension SEQUENCE{ | Not checked |  |  |  |
| ue-CategoryDL-v1350 | Checked | See Note 7 | pc_ue_CategoryDL_1bis |  |
| ue-CategoryUL-v1350 | Checked | See Note 7 | pc_ue_CategoryUL_1bis |  |
| ce-Parameters-v1350 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |
| other-Parameters-v1360 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |
| phyLayerParameters-v1430 | Not checked |  |  |  |
| ue-CategoryDL-v1430 | Not checked | See Note 8 |  |  |
| ue-CategoryUL-v1430 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 | See Note 9 |  |  |
| ue-CategoryUL-v1430b | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  | pc_ue_CategoryUL_21 |  |
| mac-Parameters-v1430 | Not checked |  |  |  |
| measParameters-v1430 | Not checked |  |  |  |
| pdcp-Parameters-v1430 | Not checked |  |  |  |
| rlc-Parameters-v1430 | Not checked |  |  |  |
| rf-Parameters-v1430 | Not checked |  |  |  |
| laa-Parameters-v1430 | Not checked |  |  |  |
| lwa-Parameters-v1430 | Not checked |  |  |  |
| lwip-Parameters-v1430 | Not checked |  |  |  |
| otherParameters-v1430 | Not checked |  |  |  |
| mmtel-Parameters-r14 | Not checked |  |  |  |
| mobilityParameters-r14 | Not checked |  |  |  |
| ce-Parameters-v1430 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities-v1430 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities-v1430 | Not checked |  |  |  |
| mbms-Parameters-v1430 | Not checked |  |  |  |
| sl-Parameters-v1430 | Not checked |  |  |  |
| ue-BasedNetwPerfMeasParameters-v1430 | Not checked |  |  |  |
| highSpeedEnhParameters-r14 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| lwa-Parameters-v1440 | Not checked |  |  |  |
| mac-Parameters v1440 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE{ |  |  |  |  |
| phyLayerParameters-v1450 | Not checked |  |  |  |
| rf-Parameters-v1450 | Not checked |  |  |  |
| ue-CategoryDL-v1450 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| ue-CategoryDL-v1460 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  | pc_ue_CategoryDL_21 |  |
| otherParameters-v1460 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| irat-ParametersNR- r15 | Not checked |  |  |  |
| featureSetsEUTRA- r15 | Not checked |  |  |  |
| pdcp-ParametersNR- r15 | Not checked |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities- v1510 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities- v1510 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| measParameters- v1520 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| measParameters- v1530 SEQUENCE { |  |  |  |  |
| qoe-MeasReport-r15 | Not checked |  |  |  |
| qoe-MTSI-MeasReport-r15 | Not checked |  |  |  |
| ca-IdleModeMeasurements-r15 | Not checked |  |  |  |
| ca-IdleModeValidityArea-r15 | Not checked |  |  |  |
| heightMeas-r15 | Checked |  | pc_heightMeas |  |
| multipleCellsMeasExtension-r15 | Checked |  | pc_Multiple_Cells_Meas_Ext |  |
| } |  |  |  |  |
| otherParameters- v1530 SEQUENCE { |  |  |  |  |
| assistInfoBitForLC-r15 | Not checked |  |  |  |
| timeReferenceProvision-r15 | Not checked |  |  |  |
| flightPathPlan-r15 | Checked |  | pc_FlightPathPlan |  |
| neighCellSI-AcquisitionParameters- v1530 | Not checked |  |  |  |
| mac-Parameters- v1530 | Not checked |  |  |  |
| phyLayerParameters- v1530 | Not checked |  |  |  |
| rf-Parameters- v1530 | Not checked |  |  |  |
| pdcp-Parameters-v1530 | Not checked |  |  |  |
| ue-CategoryDL- v1530 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| ue-BasedNetwPerfMeasParameters- v1530 | Not checked |  |  |  |
| rlc-Parameters- v1530 | Not checked |  |  |  |
| sl-Parameters- v1530 | Not checked |  |  |  |
| extendedNumberOfDRBs- v1530 | Not checked |  |  |  |
| reducedCP-Latency- v1530 | Not checked |  |  |  |
| laa-Parameters- v1530 | Not checked |  |  |  |
| ue-CategoryUL- v1530 | Checked against UE Category indications in the PICS and requirements from 36.306 cl. 4.1 |  |  |  |
| fdd-Add-UE-EUTRA-Capabilities- v1530 | Not checked |  |  |  |
| tdd-Add-UE-EUTRA-Capabilities- v1530 | Not checked |  |  |  |
| nonCriticalExtension SEQUENCE {} |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| Rel-8 | Only for Rel-8 |
| >Rel-8 | For Rel-9 or later Releases |
| >Rel-10 | For Rel-11 or later Releases |
| FDD = TDD | UE is not allowed to signal different values for FDD and TDD |
| Note 1: For Rel-9 UEs: If FDD resp. TDD specific FGI values are reported then the FDD resp. TDD specific FGI PICS are checked against the reported FDD resp. TDD specific FGI values, else the FDD resp. TDD specifc FGI PICS are both checked against the reported common FGI values. For Rel-8 UEs the FDD resp. TDD specific FGI PICS are both checked against the reported common FGI values. For Rel-10 and higher release versions the same holds as stated above for Rel-9, except that the common values may be assumed if not reported. See 36.331 Annex B.1. Note 2: The present table shows the checking which is performed if a UE of the highest defined access stratum release indicator is tested. For UEs of lower release versions information related to higher release versions may be present if a UE has selective higher release features implemented. Such information is tolerated but not used for checking against any PICS items. Note 3: Only checked against pc_ue_CategoryDL_M1 and pc_ue_CategoryUL_M1. Note 4: Depending on UE implementation/configuration, the UE may not report certain RAT support based on information received from the network. Note 5: List of Conditions: pc_DL_intraBand_contCaBWclassB, pc_DL_intraBand_contCaBWclassC, pc_UL_intraBand_contCaBWclassB, pc_UL_intraBand_contCaBWclassC, pc_DL_intraBand_nonContCaBwClassComb_AA, pc_UL_intraBand_nonContCaBwClassComb_AA, pc_DL_interBand_CaBwClassComb_AA, pc_UL_SupportedInAllBandsInCAComb, pc_UL_interBand_CaBwClassComb_AA Checkpoints: pc_DL_intraBand_contCaBWclassB shall be set to true if at least one DL Intra-band contiguous CA BW Class B CA configuration is reported; pc_DL_intraBand_contCaBWclassC shall be set to true if at least one DL Intra-band contiguous CA BW ClassC CA configuration is reported; pc_UL_intraBand_contCaBWclassB shall be set to true if at least one UL Intra-band contiguous CA BW Class B CA configuration is reported; pc_UL_intraBand_contCaBWclassC shall be set to true if at least one UL Intra-band contiguous CA BW Class C CA configuration is reported; pc_DL_intraBand_nonContCaBwClassComb_AA shall be set to true if at least one DL_intraBand_nonContCaBwClassComb_AA configuration is reported; pc_UL_intraBand_nonContCaBwClassComb_AA shall be set to true if at least one UL_intraBand_nonContCaBwClassComb_AA configuration is reported; pc_DL_interBand_CaBwClassComb_AA shall be set to true if at least one DL_DL_interBand_CaBwClassComb_AA configuration is reported; pc_UL_SupportedInAllBandsInCAComb shall be set to true if UL_SupportedInAllBandsInCAComb configuration is reported for the CA band combination identified by px_EUTRA_CA_BandCombination; pc_UL_interBand_CaBwClassComb_AA shall be set to true if at least one UL_interBand_CaBwClassComb_AA configuration is reported. The UE shall include in supportedBandCombination-r10 as many as possible of the band combinations the UE supports. For each checkpoint listed above, in case none of the listed configuration is reported, the associated PICS item is assumed to be set correctly. Note 6: If the UE is Single Mode (FDD or TDD) support of ss-CCH-InterfHandl-r11 will be signaled in the common IE. If the UE is Dual Mode (FDD & TDD) and the support of ss-CCH-InterfHandl-r11 is different in both modes then it will be signaled under X-Add-UE-EUTRA-Capabilities-v1130 IE (X – fdd and/or tdd respectively), and it will not be present in the common IE. It will be signalled in the common IE if the value is the same on both modes (i.e. “supported” on both FDD and TDD). The IE may be signaled independently for either FDD or TDD or both. Note 7: Only check against pc_ue_CategoryDL_1bis and pc_ue_CategoryUL_1bis. Note 8: Not checked as no PICS are defined yet. Note 9: Checked as far as PICS are defined. Note 10: If the UE is Single Mode (FDD or TDD) support of crs-InterfHandl-r11 will be signaled in the common IE. If the UE is Dual Mode (FDD & TDD) and the support of crs-InterfHandl -r11 is different in both modes then it will be signaled under X-Add-UE-EUTRA-Capabilities-v1130 IE (X – fdd and/or tdd respectively), and it will not be present in the common IE. It will be signalled in the common IE if the value is the same on both modes (i.e. “supported” on both FDD and TDD). The IE may be signaled independently for either FDD or TDD or both. Note 11: Depending on UE implementation, the UE may not report both UTRA RATs supported. Note 12: The applicable categories for FGI 103/104 checking are (UE category = 8 OR UE category >=11 OR UE DL Category = 11 OR UE DL Category = 12 OR UE DL Category >= 14) according to TS 36.331. The UE DL Categories higher than 14 are not explicitly listed in the condition because in this version of core specification UE DL Category > 14 requires supporting of UE Category 11 or 12. Note 13: Void. Note 14: The extended bands represented by their placeholders need to be related to the bands reported in supportedBandListEUTRA-v9e0. |  |

Table 8.5.4.1.3.3-3: UEcapabilityEnquiry (step 19a1, Table 8.5.4.1.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-22 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityEnquiry ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE { |  |  |  |
| ueCapabilityEnquiry-r8 SEQUENCE { | Numbering of entries is according to conditions met | According to inter-RAT capabilities of UE |  |
| ue-RadioAccessCapRequest[i1] | utran | This entry is present if the UE is capable of any mode (FDD/TDD) in UMTS. | pc_FDD, pc_TDD_HCR, pc_TDD_LCR, pc_TDD_VHCR |
| ue-RadioAccessCapRequest[i2] | geran-cs |  | pc_GERAN and pc_CS |
| ue-RadioAccessCapRequest[i3] | geran-ps |  | pc_GERAN and pc_PS |
| ue-RadioAccessCapRequest[i4] | cdma2000-1XRTT |  | pc_1xRTT,pc_HRPD |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.1.3.3-4: UECapabilityInformation (step 19a2, Table 8.5.4.1.3.2-1)

| Derivation path: 36.508 clause 4.6.1 table 4.6.1-23 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| UECapabilityInformation ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| c1 CHOICE{ |  |  |  |
| ueCapabilityInformation-r8 SEQUENCE { SIZE (1..maxRAT-Capabilities)) OF SEQUENCE { |  | Stated capability shall be compatible with 3GPP TS 36.523-2 (ICS statements) and the user settings |  |
| rat-Type[i1] | utran |  | pc_FDD, pc_TDD_HCR, pc_TDD_LCR, pc_TDD_VHCR |
| ueCapabilitiesRAT-Container[i1] OCTET STRING {} | ueCapabilitiesRAT-Container-UTRAN | Encoded as an INTER RAT HANDOVER INFO messagewith UMTS TDD capabilities as defined in 3GPP TS 25.331 [17]. | pc_TDD_HCR OR pc_TDD_LCR OR pc_TDD_VHCR |
|  |  |  | (pc_TDD_LCR AND pc_FDD) OR (pc_TDD_HCR AND pc_FDD) OR (pc_TDD_VHCR AND pc_FDD)) (Note 1) |
|  |  | Encoded as an INTER RAT HANDOVER INFO message with UMTS FDD capabilities as defined in 3GPP TS 25.331 [17]. | pc_FDD |
|  |  |  | (pc_TDD_LCR AND pc_FDD) OR (pc_TDD_HCR AND pc_FDD) OR (pc_TDD_VHCR AND pc_FDD)) (Note 1) |
| rat-Type[i2] | geran-cs |  | pc_GERAN and pc_CS |
| ueCapabilitiesRAT-Container [i2] OCTET STRING {} | ueCapabilitiesRAT-Container-GERAN-CS | Encoded as the concatenation of IEs MS classmark 2 and MS classmark 3 as defined in 3GPP TS 24.008 [32]. | pc_GERAN and pc_CS |
| rat-Type[i3] | geran-ps |  | pc_GERAN and pc_PS |
| ueCapabilitiesRAT-Container [ i3] OCTET STRING {} | ueCapabilitiesRAT-Container-GERAN-PS | Encoded as MS radio access capability IE as defined in 3GPP TS 24.008 [32]. | pc_GERAN and pc_PS |
| rat-Type[i4] | cdma2000-1XRTT |  | pc_1xRTT, pc_HRPD |
| ueCapabilitiesRAT-Container [ i4] OCTET STRING {} | Not checked | Encoded as A21 Mobile Subscription Information as defined in 3GPP2 A.S0008-C v4.0 [33]. | pc_1xRTT, pc_HRPD |
| nonCriticalExtension SEQUENCE {} | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 8.5.4.1.3.3-5: ueCapabilitiesRAT-Container-UTRAN

| Derivation path: 25.331 clause 11.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element |  | Value/Remark | Comment | Condition |
| ueCapabilitiesRAT-Container-UTRAN ::= SEQUENCE { |  |  |  |  |
| predefinedConfigStatusList CHOICE { |  |  |  |  |
| absent |  | NULL |  |  |
| } |  |  |  |  |
| uE-SecurityInformation |  | Not checked | The value of start CS is not used for LTE to UMTS handover in Rel-8 |  |
| ue-CapabilityContainer CHOICE { |  |  |  |  |
| present |  | Not checked | Container including UE radio access capability |  |
| } |  |  |  |  |
| v390NonCriticalExtensions CHOICE { |  |  |  |  |
| present SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v390ext |  | Not checked | Positioning capability and dummy field |  |
| v3a0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v3a0ext |  | Not checked | Positioning capability |  |
| laterNonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v3d0ext |  | Not checked | Deprecated information |  |
| interRATHandoverInfo-r3-add-ext |  | Not checked if present | UE radio access capability for bands VIII to XIV, UE radio access capability extension, support of 2 DRX schemes in CELL_PCH, support of E-DPDCH power interpolation |  |
| v3g0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v3g0ext |  | Not checked | Positioning capability extension |  |
| v4b0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v4b0ext |  | Checked | Access Stratum Release indicator |  |
| v4d0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v4d0ext |  | Not checked if present | LCR TDD UE capability |  |
| v590NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v590ext |  | Not checked if present | Predefined configuration status information compressed, UE radio access capability compressed |  |
| v690NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v690ext SEQUENCE { |  |  |  |  |
| ue-SecurityInformation2 |  | Present but value not checked | START PS | pc_Featr Grp_8 |
| ue-SecurityInformation2 |  | Not present |  | NOT pc_FeatrGrp_8 |
| ue-RadioAccessCapabilityComp |  | Not checked | RF capability for bands VIII to XIV |  |
| ue-RadioAccessCapabilityComp2 |  | Present but value not checked | UE radio access capability comp 2 | pc_FDD AND UE reportsF DD capabilities (Note1) |
| ue-RadioAccessCapabilityComp2 |  | Not checked if present | UE radio access capability comp 2 | NOT (pc_FDD AND UE reportsF DD capabilities) (Note 1) |
| } |  |  |  |  |
| v6b0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v6b0ext |  | Not checked if present | Support for SIB11bis |  |
| v6e0NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v6e0ext |  | Not checked if present | Support of FDPCH |  |
| v770NonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v770ext |  | Not checked if present | TDD RF and physical channel capability extensions in Rel-7, support of GANSS, support of MAC-ehs, LCR TDD UE specific capability information |  |
| v790nonCriticalExtensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v790ext |  | Not checked if present | Support of E-DPCCH power boosting |  |
| v860NonCriticalExtensions0 SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v860ext |  | Not checked if present | UE radio access capability for additional bands, Rel-8 HS-DSCH physical layer category, support of MAC-iis |  |
| v880NonCriticalExtensions SEQUENCE { |  | Present but value not checked |  | pc_TDD_LCR AND NOT px_NoTDD_LCR_onLTE (Note 1) |
| v880NonCriticalExtensions SEQUENCE { |  | Not checked if present |  | NOT (pc_TDD_LCR AND NOT px_NoTDD_LCR_onLTE ) (Note 1) |
| v920NonCritical ExtensionsinterRATHandoverInfo-v880ext |  | Not checked | Support for priority reselection in UTRAN, Rel-8 radio access capability extensions for LCR TDD (e.g. multi-carrier operation) |  |
| SEQUENCE { |  | Not checked if present |  |  |
| interRATHandoverInfo-v920ext |  | Not checked if present |  |  |
| v8b0NonCritical Extensions SEQUENCE { |  | Present but value not checked |  | pc_TDD_LCR AND NOT px_NoTDD_LCR_onLTE (Note 1) |
| v8b0NonCritical Extensions SEQUENCE { |  | Not checked if present |  | NOT (pc_TDD_LCR AND NOT px_NoTDD_LCR_onLTE ) (Note 1) |
| interRATHandoverInfo-v8b0ext |  | Not checked if present |  |  |
| v950NonCritical Extensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-v950ext |  | Not checked if present |  |  |
| va40NonCritical Extensions SEQUENCE { |  |  |  |  |
| interRATHandoverInfo-va40ext |  | Not checked if present |  |  |
| va80NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-va80ext | Not checked if present |  |  |  |
| vb50NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-vb50ext | Not checked if present |  |  |  |
| vb70NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-vb70ext | Not checked if present |  |  |  |
| vbc0NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-vbc0ext | Not checked if present |  |  |  |
| vc50NonCriticalExtensions SEQUENCE { | Not checked if present |  |  |  |
| interRATHandoverInfo-vc50ext | Not checked if present |  |  |  |
| nonCritical Extensions SEQUENCE {} |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| Note 1: When the UE supports more than one mode (e.g. pc_FDD and pc_TDD_HCR) then the UE is expected to report in the UTRA Container the capabilities associated with one of the supported modes. |  |  |  |  |

Table 8.5.4.1.3.3-6: ueCapabilitiesRAT-Container-GERAN-CS

| Derivation path: 25.331 clause 11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| Mobile Station Classmark 2 | First byte is 33H Second byte is 3. Third, Fourth and Fifth bytes are ignored. |  |  |
| Mobile Station Classmark 3 | CSN.1 decoding shall be successful and the contents shall indicate that E-UTRA FDD or EUTRA TDD or both is supported. Other values are not checked. |  |  |

Table 8.5.4.1.3.3-7: ueCapabilitiesRAT-Container-GERAN-PS

| Derivation path: 25.331 clause 11.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/Remark | Comment | Condition |
| MS Radio Access Capability | CSN.1 decoding shall be succesful and the contents shall indicate that E-UTRA FDD or EUTRA TDD or both is supported. Other values are not checked. |  |  |

## <End of modifed section>