3GPP TSG-RAN5 Meeting #93-e	R5-217693

# Electronic Meeting, 8th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.905 | CR | 0531 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of test point analysis for relative power tolerance in FR2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ROHDE & SCHWARZ |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The power range for Pmin ≤ P ≤ Pint can be increased by selecting 50 MHz instead of 100 MHz CHBW due to lower relaxation. In order to avoid MPRnarrow for PC1 an allocation with at least 8 RBs has to be selected. |  |  |  |  |  |  |  |  |
| Summary of change: |  | CHBW has been updated from 100 MHz to 50 MHz. Allocation has been restricted to the range from 8 to 10 RBs. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test point analysis will remain inconsistent. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.2.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Related discussion paper in R5-217557. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

< Unchanged sections omitted >

#### 4.2.1.1	FR2 single carrier, NR CA and UL MIMO test cases

This clause contains information on test point analysis and test point selection for single carrier, NR CA and UL MIMO test cases in [3] clause 6 and 7 with information about transmitting test point selection for FR2 listed in table 4.2.1.1-1 and receiver test point selection in table 4.2.1.1-2.

Table 4.2.1.1-1: NR UE transmitter test point selection for FR2

| Subclause | Number of test points | Justification in attachment | Comments |
| --- | --- | --- | --- |
| 6.2.1 UE maximum output power | x | “38.521-2_TPanalysis_6.2.1_MOP_v2.zip” | RAN5#5-5G-NR Adhoc |
| 6.2.2 UE maximum output power reduction | power class 1: 90 power class 2&3&4: 84 | ”38.521-2_TPanalysis_6.2.2_MPR_6.5.2.1_SEM_6.5.2.3_NR_ACLR_v4.zip” | RAN5#89-e RAN5#90-e RAN5#91-e RAN5#92-e |
| 6.2A.1.1 UE maximum output power - EIRP and TRP for CA | TRP: 4 EIRP: 20 | “38.521-2_TPanalysis_6.2A.1.x_MOP_Spherical Coverage_CA_v1” | RAN5#84 |
| 6.2A.1.2 UE maximum output power - Spherical coverage for CA | 20 | “38.521-2_TPanalysis_6.2A.1.x_MOP_Spherical Coverage_CA_v1” | RAN5#84 |
| 6.2A.2 UE maximum output power reduction for CA | FFS | “38.521-2_TPanalysis_6.2A.2_MPR for CA” | RAN5#84 |
| 6.2D.2 UE maximum output power reduction for UL MIMO | x | ”38.521-2_TPanalysis_6.2.2_MPR_6.5.2.1_SEM_6.5.2.3_NR_ACLR_v4.zip” | RAN5#92-e |
| 6.3.1 Minimum output power | 9 | “38.521-2_TPanalysis_6.3.1_MinOP_v2.zip“ | RAN5#84 |
| 6.3.2 Transmit OFF power | 3 | “38.521-2_TPanalysis_6.3.2_Tx_OFF_power” | RAN5#83 |
| 6.3.3.2 General ON/OFF time mask | 9 | “38.521-2_TPanalysis_6.3.3.2_OnOff.zip” | RAN5#92-e |
| 6.3.4.2 Absolute power tolerance | 8 | “38.521-2_TPanalysis_6.3.4.2_AbsPtol.zip” | RAN5#91-e |
| 6.3.4.3 Relative power tolerance | Ramp up: 1 Ramp down: 1 Alternating: 1 | “38.521-2_TPanalysis_6.3.4.3_RelPtol_v3.zip” | RAN5#82 RAN5#91-e RAN5#93-e |
| 6.3.4.4 Aggregate power tolerance | PUCCH: 6 PUSCH: 6 | “38.521-2_TPanalysis_6.3.4.4_AggPtol_2.zip” | RAN5#82 RAN5#91-e |
| 6.3A.1.1 Minimum output power for CA (2UL CA) | 4 | “38.521-2_TPanalysis_6.3A.1.1_MinOP.zip” | RAN5#83 |
| 6.3A.2.1 Transmit OFF power for CA (2UL CA) | 3 | “38.521-2_TPanalysis_6.3A.2.1_Tx_OFF_Power_CA.zip” | RAN5#88-e |
| 6.3A.4.2.1 Absolute power tolerance for CA (2UL CA) | 6 | 38.521-2_TPanalysis_6.3A.4.2.1_AbsPCTol_CA.zip | RAN5#85 |
| 6.3A.4.2.2 Absolute power tolerance for CA (3UL CA) | 6 | 38.521-2_TPanalysis_6.3A.4.2.1_AbsPCTol_CA.zip | RAN5#85 |
| 6.3A.4.2.3 Absolute power tolerance for CA (4UL CA) | 6 | 38.521-2_TPanalysis_6.3A.4.2.1_AbsPCTol_CA.zip | RAN5#85 |
| 6.3A.4.2.4 Absolute power tolerance for CA (5UL CA) | 6 | 38.521-2_TPanalysis_6.3A.4.2.1_AbsPCTol_CA.zip | RAN5#85 |
| 6.3A.4.2.5 Absolute power tolerance for CA (6UL CA) | 6 | 38.521-2_TP analysis_6.3A.4.2.1_AbsPCTol_CA.zip | RAN5#85 |
| 6.3A.4.2.6 Absolute power tolerance for CA (7UL CA) | 6 | 38.521-2_TP analysis_6.3A.4.2.1_AbsPCTol_CA.zip | RAN5#85 |
| 6.3A.4.2.7 Absolute power tolerance for CA (8UL CA) | 6 | 38.521-2_TPanalysis_6.3A.4.2.1_AbsPCTol_CA.zip | RAN5#85 |
| 6.3A.4.4 Aggregate power tolerance for CA | 1 | 38.521-2_TPanalysis_6.3A.4.4_Aggregate power tolerance for CA.zip | RAN5#92-e |
| 6.3D.1 Minimum output power for UL MIMO | 9 | “38.521-2_TPanalysis_6.3.1_MinOP_v2.zip“ | RAN5#84 |
| 6.3D.3.4 SRS time mask for UL-MIMO | 18 | “38.521-2_TPanalysis_6.3.3.2_SRS_M_UL-MIMO.zip“ | RAN5#85 |
| 6.4.1 Frequency error | 1 | “38.521-2_TPanalysis_6.4.1_FreqErr.zip” | RAN5#80 |
| 6.4.2.1 Error Vector Magnitude | PUSCH: 168 PUCCH: 24 PRACH: 24 | “38.521-2_TPanalysis_6.4.2.1_EVM.zip” | RAN5#3-5G-NR Adhoc |
| 6.4.2.2 Carrier leakage | 3 | “38.521-2_TPanalysis_6.4.2.2_CarrLeak_v2.zip” | RAN5#89-e |
| 6.4.2.3 In-band emissions | PUSCH: 36 PUCCH: 18 | “38.521-1_TPanalysis_6.4.2.3_IE_v2.zip” | RAN5#89-e |
| 6.4.2.4 EVM equalizer spectrum flatness | 18 | “38.521-2_TP analysis_6.4.2.4_6.4.2.5_EVMequalizerSpectrumFlatness.zip” | RAN5#3-5G-NR Adhoc |
| 6.4.2.5 EVM spectral flatness for pi/2 BPSK modulation with spectrum shaping | 9 | “38.521-2_TP analysis_6.4.2.4_6.4.2.5_EVMequalizerSpectrumFlatness.zip” | RAN5#3-5G-NR Adhoc |
| 6.4A.1 Frequency error for CA | N (1 test point per UL carrier) | “38.521-2_TPanalysis_6.4A.1_FreqErr_CA_v2.zip” | RAN5#87-e RAN5#90-e |
| 6.4A.2.1 Error Vector Magnitude for CA | 224 | “38.521-2_TPanalysis_6.4A.2.1_EVM_CA.zip” | RAN5#91-e |
| 6.4A.2.2 Carrier leakage for CA | 2 | “38.521-2_TPanalysis_6.4A.2.2_CarrLeak_CA_v2.zip” | RAN5#89-e |
| 6.4D.3 Time alignment error for UL MIMO | 6 | “38.521-2_TPanalysis_6.4D.3_TAE_MIMO.zip” | RAN5#92-e |
| 6.5.1 Occupied Bandwidth | 12 | “38.521-2_TPanalysis_6.5.1_OccBW_v3.zip” | RAN5#89-e RAN5#91-e |
| 6.5.2.1 Spectrum Emission Mask | 90 | ”38.521-2_TPanalysis_6.2.2_MPR_6.5.2.1_SEM_6.5.2.3_NR_ACLR_v4.zip” | RAN5#2-5G-NR Adhoc RAN5#79 RAN5#80 RAN5#89-e RAN5#90-e RAN5#91-e RAN5#92-e |
| 6.5.2.3 Adjacent Channel Leakage Ratio | TBD | ”38.521-2_TPanalysis_6.2.2_MPR_6.5.2.1_SEM_6.5.2.3_NR_ACLR_v4.zip” | RAN5#2-5G-NR Adhoc RAN5#89-e RAN5#90-e RAN5#91-e RAN5#92-e |
| 6.5.3.1 Spurious emissions | 2 | “38.521-2_TPanalysis_6.5.3_TxSpurious_v2.zip” | RAN5#84 |
| 6.5.3.2 Spurious emissions UE band co-existence | 2 | “38.521-2_TPanalysis_6.5.3_TxSpurious_v2.zip” | RAN5#84 |
| 6.5.3.3 Additional spurious emission | NS202: 4 NS203: 4 | “38.521-2_TPanalysis_6.2.3_AMPR_NS_202.zip” “38.521-2_TPanalysis_6.2.3_AMPR_NS_203.zip” | RAN5#90-e |
| 6.5A.2.1 Spectrum Emission Mask for CA | 30 | “38.521-2_TPanalysis_6.5A.2.1_SEM_CA.zip” | RAN5#89-e |
| 6.5A.2.2 Adjacent channel leakage ratio for CA | 52 | “38.521-2_TPanalysis_6.5A.2.2_ACLR_CA.zip” | RAN5#89-e |
| 6.5A.3 Spurious emissions for CA | 4 | 38.521-2_TPanalysis_6.5A.3_Spur_CA.zip | RAN5#91-e |
| 6.5D.1 Occupied Bandwidth for UL MIMO | 12 | “38.521-2_TPanalysis_6.5.1_OccBW_v3.zip” | RAN5#91-e |
| 6.5D.2.1 Spectrum Emission Mask for UL MIMO | x | ”38.521-2_TPanalysis_6.2.2_MPR_6.5.2.1_SEM_6.5.2.3_NR_ACLR_v4.zip” | RAN5#92-e |
| 6.5D.2.2 Adjacent channel leakage ratio for UL MIMO | x | ”38.521-2_TPanalysis_6.2.2_MPR_6.5.2.1_SEM_6.5.2.3_NR_ACLR_v4.zip” | RAN5#92-e |
| 6.6 Beam Correspondence | 6 | “38.521-2_TPanalysis_6.6_Beam_Correspond_v1.zip” | RAN5#85 |

< End of changes >

Add the following attached .zip files to TR 38.905:

## See attachment “38.521-2_TPanalysis_6.3.4.3_RelPtol_v3.zip”

## Remove the following .zip files from TR 38.905:

## “38.521-2_TPanalysis_6.3.4.3_RelPtol_v2.zip”