3GPP TSG-RAN5 Meeting # 93-e	R5-217710

Electronic Meeting, 8th -19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-2 | CR | 0673 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | 38.521-2 FR2 Extreme Temperature Conditions applicability for UL-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Keysight Technologies UK Ltd |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Current FR2 UL MIMO test cases do not include test coverage under ETC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | As proposed in R5-217707, extend initial conditions to FR2 ETC for the following UL MIMO test cases: Test Case ID; Test case Title. 6.2D.1.1; UE maximum output power – EIRP and TRP for UL MIMO. 6.3D.3.1; General ON/OFF time mask for UL MIMO. 6.4D.1; Frequency error for UL MIMO. 6.5D.2.2; Adjacent channel leakage ratio for UL MIMO. |
| Consequences if not approved: |  | Specification will remain incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.1.1.4.1, 6.3D.3.1.4.1, 6.4D.1.4.1, 6.5D.2.2.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This document depends on the endorsement of R5-217707. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### << Start of changes 1>>

6.2D.1.1.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, and channel bandwidths based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each channel bandwidth and subcarrier spacing, are shown in table 6.2D.1.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2D.1.1.4.1-1: Test Configuration Table for 2-layer UL MIMO

NOTE:	No test points are defined since there is no configuration satisfying MPR=0dB requirements in RAN4.

Table 6.2D.1.1.4.1-2: Test Configuration Table for uplink full power transmission (ULFPTx)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | FFS |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | FFS |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | FFS |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  | Default | FFS | Modulation |  | RB allocation |
| 1 | FFS |  |  | FFS |  | FFS |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.2 and TS 38.508-1 [10] subclause 5.2.1.1.1, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement channels are set according to Table 6.2D.1.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.2D.1.1.4.3

<< End of changes 1>>

#### << Start of changes 2>>

6.3D.3.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.2-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.3D.3.1.4.1-1. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.3D.3.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |
| --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  | Lowest, Mid, Highest |  |
| Test SCS as specified in Table 5.3.5-1. |  | Highest |  |
| Test Parameters |  |  |  |
|  | Downlink Configuration | Uplink Configuration |  |
| Test ID | N/A for maximum output power test case | Modulation | RB allocation (NOTE 1) |
| 1 |  | CP-OFDM QPSK | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. |  |  |  |

1.	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A Figure A.3.3.1.1 for TE diagram and Figure A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The UL Reference Measurement Channels are set according to Table 6.3D.3.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.3D.3.1.4.3.

<< End of changes 2>>

#### << Start of changes 3>>

6.4D.1.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of channel bandwidth and sub-carrier spacing, are shown in table 6.4D.1.4.1-1. The details of the uplink and downlink reference measurement channels (RMCs) are specified in Annexes A.2 and A.3. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.4D.1.4.1-1: Test Configuration Table

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | FFS |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  | FFS |  |
| Test SCS as specified in Table 5.3.5-1. |  |  | FFS |  |
| Test Parameters |  |  |  |  |
|  | Downlink Configuration |  | Uplink Configuration |  |
| Test ID | Modulation | RB allocation | Modulation | RB allocation |
| 1 | FFS | FFS | FFS | FFS |

1. 	Connection between SS and UE is shown in TS 38.508-1 [10] Annex A, Figure A.3.3.1.1 for TE diagram and section A.3.4.1.1 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [10] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1 and C.3.0, and uplink signals according to Annex G.0, G.1 and G.3.0.

4.	The DL and UL Reference Measurement channels are set according to Table 6.4D.1.4.1-1.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [10] clause 4.5. Message contents are defined in clause 6.4D.1.4.3

<< End of changes 3>>

#### << Start of changes 4>>

6.5D.2.2.4.1	Initial condition

Same initial condition in clause 6.5.2.3.4.1 with following exceptions:

-	Instead of Table 6.5.2.3.4.1-1 use Table 6.5D.2.2.4.1-1.

-	Instead of Table 6.5.2.3.4.1-2  use Table 6.5D.2.2.4.1-2.

Table 6.5D.2.2.4.1-1: Test Configuration Table (Power Class 1)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, Mid range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | 8@0 |
| 2 | High |  |  |  | CP-OFDM QPSK | 8@NRB-8 |
| 3 | Mid |  |  |  | CP-OFDM QPSK | Outer_Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | 8@0 |
| 5 | High |  |  |  | CP-OFDM 16 QAM | 8@NRB-8 |
| 6 | Mid |  |  |  | CP-OFDM 16 QAM | Outer_Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | 8@0 |
| 8 | High |  |  |  | CP-OFDM 64 QAM | 8@NRB-8 |
| 9 | Mid |  |  |  | CP-OFDM 64 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Applicability of test IDs for for CHBWs and frequency ranges is FFS.NOTE 3: All test points in this table must also exist in table 6.2.2.4.1-1, table 6.2.2.4.1-2, table 6.2.2.4.1-3 (MPR). |  |  |  |  |  |  |

Table 6.5D.2.2.4.1-2: Test Configuration Table (Power Class 2, 3 and 4)

| Default Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [10] subclause 4.1 |  |  |  |  | Normal, TL, TH |  |
| Test Frequencies as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [10] subclause 4.3.1 |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  | Lowest, Highest |  |
| Test Parameters |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |
|  |  | Default | Default | N/A for Adjacent Channel Leakage Ratio test case | Modulation | RB allocation (NOTE 1) |
| 1 | Low |  |  |  | CP-OFDM QPSK | Edge_1RB_Left |
| 2 | High |  |  |  | CP-OFDM QPSK | Edge_1RB_Right |
| 3 | Default |  |  |  | CP-OFDM QPSK | Outer Full |
| 4 | Low |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 5 | High |  |  |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 6 | Default |  |  |  | CP-OFDM 16 QAM | Outer Full |
| 7 | Low |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 8 | High |  |  |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 9 | Default |  |  |  | CP-OFDM 64 QAM | Outer Full |
| NOTE 1: The specific configuration of each RF allocation is defined in Table 6.1-1 for PC2, PC3 and PC4 or Table 6.1-2 for PC1. NOTE 2: Following Test IDs shall be skipped for FR2b - All Test IDs for 400MHz Channel Bandwidth - All Test IDs for 200MHz Channel Bandwidth - Test ID 7-9 for 100MHz Channel Bandwidth NOTE 3: All test points in this table must also exist in table 6.2D.2.4.1-4, table 6.2D.2.4.1-5, table 6.2D.2.4.1-6 (MPR). |  |  |  |  |  |  |

<< End of changes 4>>