3GPP TSG-RAN5 Meeting #93-e	R5-217715

# Electronic Meeting, 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-2 | CR | 0283 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of PICS for NPN |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm CDMA Technologies |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NG_RAN_PRN_Vertical_LAN-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of PICS for Rel-16 nr-CGI-Reporting-NPN capability, Autonomous search function over NR for CAG and user initiated SNPN reselction in automatic mode for SNPN. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added PICS for Rel-16 nr-CGI-Reporting-NPN capability, Autonomous search function over NR for CAG and user initiated SNPN reselction in automatic mode for SNPN. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Applicability of the test cases cannot be correctly specified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.4.3.6, A.4.3.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Secretary to resolve xz,yz and zz and assign proper clause |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### A.4.3.6	Measurement Capabilities

Table A.4.3.6-1: UE Measurement Capabilities

| Item |  | UE Measurement Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated "Yes" the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support NR measurements and events A triggered reporting |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_eventA_MeasAndReport |  | Yes |  |  |  |  |  |
| 2 |  | Support two independent measurement gap configurations for FR1 and FR2 |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_independentGapConfig |  | No |  |  |  |  |  |
| 3 |  | Support NR intra-frequency and inter-frequency measurements and at least periodical reporting |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_intraAndInterF_MeasAndReport |  | Yes |  |  |  |  |  |
| 4 |  | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured with an associated SS/PBCH |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_csi_RSRP_AndRSRQ_MeasWithSSB |  | No |  |  |  |  |  |
| 5 |  | Support inter-RAT E-UTRA measurements and events B triggered reporting |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_eventB_MeasAndReport |  | Yes |  |  |  |  |  |
| 6 |  | Support SS-SINR measurents |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_ss_SINR_Meas |  | No |  |  |  |  |  |
| 7 |  | Support acquisition of relevant information from a neighbouring E-UTRA cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is not configured. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_eutra_CGI_Reporting |  | Yes |  |  |  |  |  |
| 8 |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when EN-DC is not configured. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_nr_CGI_Reporting |  | Yes |  |  |  |  |  |
| 9 |  | Support acquisition of relevant information from a neighbouring intra-frequency or inter-frequency NR cell by reading the SI of the neighbouring cell and reporting the acquired information to the network as specified in TS 38.331 [9] when the EN-DC is configured. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_nr_CGI_Reporting_ENDC |  | Yes |  |  |  |  |  |
| 10 |  | Support shorter measurement gap length (i.e. gp2 and gp3) for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC. |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp2_gp3_en_dc |  | No |  |  |  |  |  |
| 11 |  | Support NR supports gap pattern 4 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp4_en_dc |  | No |  |  |  |  |  |
| 12 |  | Support NR supports gap pattern 5 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp5_en_dc |  | No |  |  |  |  |  |
| 13 |  | Support NR supports gap pattern 6 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp6_en_dc |  | No |  |  |  |  |  |
| 14 |  | Support NR supports gap pattern 7 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp7_en_dc |  | No |  |  |  |  |  |
| 15 |  | Support NR supports gap pattern 8 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp8_en_dc |  | No |  |  |  |  |  |
| 16 |  | Support NR supports gap pattern 9 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp9_en_dc |  | No |  |  |  |  |  |
| 17 |  | Support NR supports gap pattern 10 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp10_en_dc |  | No |  |  |  |  |  |
| 18 |  | Support NR supports gap pattern 11 for independent measurement gap configuration on FR1 and per-UE gap in (NG)EN-DC |  | 36.331, 6.3.6 |  | Rel-15 |  | pc_gp11_en_dc |  | No |  |  |  |  |  |
| 19 |  | Support measurement gap pattern 2 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp2_nr |  | No |  |  |  |  |  |
| 20 |  | Support measurement gap pattern 3 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp3_nr |  | No |  |  |  |  |  |
| 21 |  | Support measurement gap pattern 4 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp4_nr |  | No |  |  |  |  |  |
| 22 |  | Support measurement gap pattern 5 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp5_nr |  | No |  |  |  |  |  |
| 23 |  | Support measurement gap pattern 6 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp6_nr |  | No |  |  |  |  |  |
| 24 |  | Support measurement gap pattern 7 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp7_nr |  | No |  |  |  |  |  |
| 25 |  | Support measurement gap pattern 8 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp8_nr |  | No |  |  |  |  |  |
| 26 |  | Support measurement gap pattern 9 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp9_nr |  | No |  |  |  |  |  |
| 27 |  | Support measurement gap pattern 10 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp10_nr |  | No |  |  |  |  |  |
| 28 |  | Support measurement gap pattern 11 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp11_nr |  | No |  |  |  |  |  |
| 29 |  | Support measurement gap pattern 12 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp12_nr |  | No |  |  |  |  |  |
| 30 |  | Support measurement gap pattern 15 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp15_nr |  | No |  |  |  |  |  |
| 31 |  | Support measurement gap pattern 16 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp16_nr |  | No |  |  |  |  |  |
| 32 |  | Support measurement gap pattern 17 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp17_nr |  | No |  |  |  |  |  |
| 34 |  | Support measurement gap pattern 18 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp18_nr |  | No |  |  |  |  |  |
| 35 |  | Support measurement gap pattern 19 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp19_nr |  | No |  |  |  |  |  |
| 36 |  | Support measurement gap pattern 20 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp20_nr |  | No |  |  |  |  |  |
| 37 |  | Support measurement gap pattern 21 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp21_nr |  | No |  |  |  |  |  |
| 38 |  | Support measurement gap pattern 22 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp22_nr |  | No |  |  |  |  |  |
| 39 |  | Support measurement gap pattern 23 configured by NR RRC. |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_gp23_nr |  | No |  |  |  |  |  |
| 40 |  | Support CSI-RSRP and CSI-RSRQ measurement as specified in TS38.215 [21], where CSI-RS resource is configured without an associated SS/PBCH |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_csi_RSRP_AndRSRQ_MeasWithoutSSB |  | No |  |  |  |  |  |
| 41 |  | Support CSI-RS based Radio Link Monitoring for FR1 |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_CSI_RS_RLM_FR1 |  | Yes |  |  |  | If the UE supports this feature, the UE needs to report maxNumberResource-CSI-RS-RLM in its capability report. If the UE doesn’t support CSI-RS based RLM, it will not include this IE in its capability report. |  |
| 42 |  | Support of E-UTRA RS-SINR measurements |  | 38.306, 4.2.10 |  | Rel-15 |  | pc_RS_SINR_MeasEUTRA |  | No |  |  |  |  |  |
| 43 |  | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in FDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_SFTD_MeasPSCell_MRDC_FDD |  | No |  |  |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |  |
| 44 |  | Support of SFTD measurements between a E-UTRA PCell and an NR PSCell in TDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_SFTD_MeasPSCell_MRDC_TDD |  | No |  |  |  | The SFTD measurement support should be indicated in MRDC capabilities for EN-DC. The support needs to be declared for FDD and TDD separately |  |
| 45 |  | Support of relaxed RRM measurements of neighbour cells in RRC_IDLE/RRC_INACTIVE |  | 38.306, 5.6 |  | Rel-16 |  | pc_Relaxed_Meas |  | No |  |  |  |  |  |
| 46 |  | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in FDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_SFTD_MeasNR_Cell_FDD |  | No |  |  |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |  |
| 47 |  | Support of SFTD measurements between a E-UTRA PCell and an NR neighbour cell in TDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_SFTD_MeasNR_Cell_TDD |  | No |  |  |  | The support needs to be declared for FDD and TDD separately The SFTD measurement support can only be indicated in MRDC capabilities for EN-DC |  |
| 48 |  | Support of SFTD measurements between a NR PCell and an NR neighbour cell in FDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_SFTD_MeasNR_Neigh_FDD |  | No |  |  |  | The support needs to be declared for FDD and TDD separately |  |
| 49 |  | Support of SFTD measurements between a NR PCell and an NR neighbour cell in TDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_SFTD_MeasNR_Neigh_TDD |  | No |  |  |  | The support needs to be declared for FDD and TDD separately |  |
| 50 |  | Support of SFTD measurements between a NR PCell and an NR PSCell in FDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_SFTD_MeasPSCell_NRDC_FDD |  | No |  |  |  | The SFTD measurement support should be indicated in UE-NR-Capability |  |
| 51 |  | Support of SFTD measurements between a NR PCell and an NR PSCell in TDD |  | 38.306, 4.2.9 |  | Rel-15 |  | pc_SFTD_MeasPSCell_NRDC_TDD |  | No |  |  |  | The SFTD measurement support should be indicated in UE-NR-Capability |  |
| zz |  | Support of acquisition of CGI related information from a neighbouring intra-frequency or inter-frequency NPN CAG cell |  | 38.306, 4.2.9 |  | Rel-16 |  | pc_nr_CGI_Reporting_NPN_r16 |  | No |  |  |  |  |  |

### A.4.3.7	General Capabilities

Table A.4.3.7-1: UE General Capabilities

| Item |  | UE General Capabilities |  | Ref. |  | Release |  | Mnemonic |  | M |  | If indicated “Yes” the feature shall be implemented and successfully tested for the corresponding release |  | Comments |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 |  | Support UL transmission via either MCG path or SCG path for the split SRB as specified in TS 37.340[20] |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_splitSRB_WithOneUL_Path |  | No |  |  |  |  |  |
| 2 |  | Support UL transmission via both MCG path and SCG path for the split DRB as specified in TS 37.340[20] |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_splitDRB_withUL_Both_MCG_SCG |  | Yes |  |  |  |  |  |
| 3 |  | Support direct SRB between the SN and the UE as specified in TS 37.340[20] |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_srb3 |  | Yes |  |  |  |  |  |
| 4 |  | Support of reflective QoS |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_as_ReflectiveQoS |  | No |  |  |  |  |  |
| 5 |  | Support of NAS reflective QoS |  | 24.501, 6.2.5.1.4.1, 9.11.4.1 |  | Rel-15 |  | pc_nas_ReflectiveQoS |  | No |  |  |  |  |  |
| 6 |  | Support of SMS over NAS |  | 24.501, 5.5.1.2 |  | Rel-15 |  | pc_sms_over_NAS |  | No |  |  |  |  |  |
| 7 |  | Support of CMAS message on NR |  | 38.331, 5.2.2.2.2 |  | Rel-15 |  | pc_CMAS_NR |  | No |  |  |  |  |  |
| 8 |  | Support of ETWS message on NR |  | 38.331, 5.2.2.2.2 |  | Rel-15 |  | pc_ETWS_NR |  | No |  |  |  |  |  |
| 9 |  | The UE supports additional UE-requested PDU establishment |  | 24.501, 6.4.1.5 |  | Rel-15 |  | pc_Additional_PDU_establishment |  | No |  |  |  | pc_ExpectedNoOfPDUSessionsAtRegistration +1 |  |
| 10 |  | The UE includes the SM PDU DN request container IE in the PDU SESSION ESTABLISHMENT REQUEST message |  | 24.501, 6.4.1.2 |  | Rel-15 |  | pc_SM_PDU_DN_RequestContainer |  | No |  |  |  |  |  |
| 11 |  | Support of emergency services fallback in NR connected to 5GCN |  | 24.501 |  | Rel-15 |  | pc_NR_5GC_EmergencyService_fallback |  | No |  |  |  |  |  |
| 12 |  | Support of EPS fallback |  | 24.501, |  | Rel-15 |  | pc_EPS_fallback |  | No |  |  |  |  |  |
| 13 |  | Support of UE requested PDU session modification |  | 24.501, 6.4.2.2 |  | Rel-15 |  | pc_MO_PDU_Session_Modification |  | Yes |  |  |  |  |  |
| 14 |  | Support of emergency services in NR connected to 5GCN |  | 24.501 |  | Rel-15 |  | pc_NR_5GC_EmergencyServices |  | No |  |  |  |  |  |
| 15 |  | Support of voiceFallbackIndication |  | 38.306, 4.2.13 |  | Rel-16 |  | pc_voiceFallbackIndication |  | No |  |  |  |  |  |
| 16 |  | Support provision of referenceTimeInfo |  | 38.306, 4.2.2 |  | Rel-16 |  | pc_referenceTimeProvision_r16 |  | No |  |  |  | specifically for TSC (time sensitive communication) services |  |
| 17 |  | Support of RACS |  | 24.501, 9.11.3.1 |  | Rel-16 |  | pc_5GC_RACS |  | No |  |  |  |  |  |
| 18 |  | Support of RRC message Segmentation in the UL |  | 38.306, 5.4 |  | Rel-16 |  | pc_NR_UL_Segmentation |  | No |  |  |  | UE supports segmenation of UECapabilityInformation message, IF size > maximum supported size of a PDCP SDU |  |
| 19 |  | Support of RRC_INACTIVE as specified in TS 38.331 [9]. |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_inactiveState |  | Yes |  |  |  |  |  |
| 20 |  | Support of UE local release when the security check is successful but SOR Transparent container indicates ACK has been NOT requested |  | 23.122 clause C.2 |  | Rel-15 |  | pc_SOR_ACKNotReqLocalRel |  | No |  |  |  |  |  |
| 21 |  | Support of RRC connection release with deprioritisation |  | 38.306, 5.3 |  | Rel-15 |  | pc_NR_RRC_Release_With_Deprioritisation |  | No |  |  |  |  |  |
| 22 |  | Support of RRC connection establishment failure with temporary offset |  | 38.306, 5.3 |  | Rel-15 |  | pc_NR_RRC_ConEstFail_With_TempOffset |  | No |  |  |  |  |  |
| 23 |  | Support of Closed Access Group |  | 24.501, 9.11.3.1 |  | Rel-16 |  | pc_CAG |  | No |  |  |  |  |  |
| 24 |  | Support of Stand-alone Non-Public Network |  | 23.501, 3.1 |  | Rel-16 |  | pc_SNPN |  | No |  |  |  |  |  |
| 25 |  | Support of test function SET UL MESSAGE for using a preconfigured UE capability container over NR |  | 38.509, 5.9 |  | Rel-16 |  | pc_Set_UE_Cap_Info_NR |  | No |  |  |  |  |  |
| 26 |  | Support of network slice-specific authentication and authorization |  | 24.501, 9.11.3.1 |  | Rel-16 |  | pc_5GC_NSSAA |  | No |  |  |  |  |  |
| 27 |  | Support of EAP-AKA’ as EAP method for network slice-specific authentication and authorization |  | 24.501, 5.4.7 |  | Rel-16 |  | pc_5GC_NSSAA_EAP_AKA_Prime |  | No |  |  |  |  |  |
| 28 |  | Support reduced control plane latency as defined in TS 38.331 [9] |  | 38.306, 4.2.2 |  | Rel-15 |  | pc_reducedCP_Latency |  | No |  |  |  |  |  |
| 29 |  | Support of release preference assistance information |  | 38.306, 4.2.2 |  | Rel-16 |  | pc_releasePreference_r16 |  | No |  |  |  |  |  |
| xz |  | Support of user initiated SNPN reselection in automatic mode |  | 23.122 |  | Rel-16 |  | pc_UserInitiated_SNPN_Reselection |  | No |  |  |  |  |  |
| yz |  | Support of autonomous search function to detect CAG cells on serving and non-serving frequencies |  | 38.304, 5.2.4.10 |  | Rel-16 |  | pc_Autonomous_search_function_nr_CAG |  | No |  |  |  |  |  |