3GPP TSG RAN Meeting #93-e	                   R5-217716

Electronic Meeting, 08th Nov – 16th Nov 2021

Agenda item:	5.3.1.17

Source:	Apple Portugal

Title:       Avoiding Scell Drop in FR2 RF UL-CA tests

Document for:	DISCUSSION AND ENDORSEMENT

# 1	Introduction

The purpose of this discussion paper is to summarize current status in RAN5 on the issue of FR2 EN-DC UEs dropping NR SCell during the TRx measurement and thereby ceasing to be in UL-CA mode. This is in lieu of the DUT following the prioritization rules in TS 38.101-2 [1] and TS 38.213 [2]. The issue has been discussed in length in RAN5 [5] [6] [7] [8].We further propose some solutions for agreement.

It should also be noted that there the agreement in RAN4 (Way Forward R4-2107762) discussed possible ways to limit the serving cell power to prevent excessive power scaling of secondary cells. While the actual method is still under discussion, the impact of this to conformance testing and possible options to consider in RAN5 spec are discussed.

# 2	Discussion

## 2.1     Test conditions

The stated UE behaviour can potentially occur when Rel-15 FR2 EN-DC UE is set to output a maximum power with multiple NR component carriers by continuous TPC UP command during tests such as Maximum Output Power (Peak EIRP).  Based on the description of TS 38.508-1 [3] and test procedures in TS 38.521-2 [4], the SCG of the FR2 UE (with dynamic power sharing) is defined to set P-Max as +26 dBm and the output power is increased by continuous TPC commands from a test equipment. In a power limited condition, when the PCell is prioritized then the Scell could reach a situation where it is dropped due to inadequate power allocation.

Observation 1: Scell Drop is possible when the EN-DC FR2 intra-band contiguous UL-CA test procedure configures the DUT to transmit at maximum power across CCs

## 2.2	Core and Test Specification background

It would be useful to understand the background from TS 38.213 [2] and TS 38.101-2 [1]. Associated clauses are extracted in the appendix at the bottom of this contribution. In summary:

- In TS 38.213 clause 7.5, UE is defined to prioritize the primary cell in case of same priority order of transmission and for operation with CA, which means the primary cell of the SCG (NR cell group) is prioritized than secondary cell in the EN-DC UE case. Moreover, this is also the default mode of operation in real deployments.

Observation 2: As per TS 38.213 prioritization rules, PCC is prioritized over SCC. This is also the mode of operation in the field

- In TS 38.101-2 clause 6.2A.4, configured transmitted power for CA is defined only with PCMAX, which is the total power of CCs as NR. Thus, it is possible to configure the output power of each component carrier with an imbalanced output power. This led to the question on whether “equal power spectral density for each RB in each component carrier” should be a mandatory condition or not.

Extract from TS 38.101-2 clause 6.2A.4
For uplink intra-band contiguous carrier aggregation, MPR is specified in clause 6.2A.2. PCMAX is calculated under the assumption that power spectral density for each RB in each component carrier is same.

RAN4 discussed this issue and conveyed their understanding to RAN5 via [ref]

RAN4 has reached the following understanding

- The UE Prioritizes Power on the Pcell and reduces the power on the Scell(s) for transmissions of a given priority when the UE is power limited (38.213)

- The RAN4 requirements on PCMAX for CA do not mandate the UE transmit with equal PSD across CCs at maximum output power

- Equal PSD was assumed for the development of MPR requirements, but the MPR thus specified for the total CA power is applicable to all transmit conditions (priorities)

In addition, RAN4 has confirmed the below

To this end, RAN4 has discussed the following two options for verification of the CA test cases:

- Option 1: Equal PSD between CCs.

- Option 2: Measure the UE as is even SCC output may be scaled down under CA mode.

RAN4 considers that equal PSD is a preferred test condition to verify the UL CA requirements. However, considering the actual UE behaviour in the field, which is subject to the prioritization rules in 38.213.RAN4 recognizes that testing details (configures/procedures) are ultimately up to RAN5.

Observation 3: RAN4 clarified via LS [2] to RAN5 that equal PSD is preferred test condition. However, actual UE behavior in the field has to be considered as well as prioritization rules in TS 38.213 to identify testing details.

Observation 4: RAN5 has agreed that the way forward recognizes prioritization rules in TS 38.213 [10]

Observation 5: RAN5 has to analyse and define test procedures which will ensure testability for intra-band UL-CA scenarios. Considering the ongoing RAN4 discussion, a potential solution from RAN4 cannot be precluded.

The most recent update from the core working groups has been an LS exchange which conveys the following (per LS response from RAN1 [11])

Question 1: Whether UE drop Scell power according to the priority rule defined in 38.213 is considered as an issue from RAN1 perspective.

Answer: RAN1 understands that SCell dropping due to power limitation can occur. But RAN1 doesn’t see changes to the priority rule defined in TS 38.213 as a way to address such issue since RAN1 has not identified any issue on the priority rules in TS 38.213.

Question 2: Whether UE drop Scell power according to the priority rule defined in 38.213 has been addressed from 16 or 17? If not, what expected solution(s) are?

Answer: No until this RAN1 meeting, RAN1 is not working to modify the power scaling priority rule in Rel-17 either, and understands that RAN4 is discussing potential RAN4 specification updates.

Question 3: If the problem above is solved in RAN4 specifications with solution by higher layer configuration, e.g. introduce additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping (see e.g. R4-2112826 or R4-2114551 for details), is there any expected RAN1 spec impact or possible conflict with UE behaviour defined in RAN1 specifications?

Answer: Regarding the solution examples, there is no RAN1 specification impact by introducing additional UE-specific configuration of power limits on Pcmax,f,c for each CC to prevent SCell dropping. For other aspects, including potential conflict with current RAN1 specification, a definite answer can only be provided when the detailed solution is provided by RAN4.

Observation 6: RAN1 confirms that no changes to prioritization rules is planned as there is no known issue as such. Further any UE-specific configuration of power limits to prevent SCell drop would not conflict with RAN1 specifications but a confirmation on this is dependent on the actual RAN4 solution.

## Discussion on possible ways forward for RAN5

### 2.3.1	Background: Previous RAN5 discussion on FR2 RF Tx tests impacted by this issue

As part of the discussions during earlier RAN5 meetings, a list of tests that are impacted was identified. This information is useful to determine the real impact on coverage solely because of the SCell drop issue and how much bandwidth to spend/how best to identify a temporary workaround for the issue before a RAN4/RAN2 solution is identified.

Observation 7: There are 3 (three) FR2 RF UL-CA tests that are blocked solely by SCell drop issue. Other tests either have multiple other blocking issues or are not yet introduced in the spec even in Release 15 work plan for the past many RAN5 meetings.

| TS38.521-2 clause number | Test case title | Coverage Blocked by SCell Drop Issue? | Comments |
| --- | --- | --- | --- |
| 6.2A.1 | UE maximum output power for CA | No | The MOP CA test case requires to be tested at MPR=0 and according to RAN 4 this condition is met only with 1CC. This means SCell drop cannot happen in this test case |
| 6.2A.2 | UE maximum output power reduction for CA | Yes, if other incomplete items are addressed | Multiple other incomplete items in test so SCell Drop is not only issue |
| 6.2A.3 | A-MPR for CA | No | No contents in the test case (FFS). |
| 6.2A.4 | Configured output power for CA | No | No contents in the test case (FFS). |
| 6.3A.4.2 | Absolute power tolerance | No | MU/TT is FFS |
| 6.3A.4.3 | Relative power tolerance | No | No contents in the test case (FFS). |
| 6.3A.4.4 | Aggregate power tolerance | No | No contents in the test case (FFS). |
| 6.5A.1 | Occupied bandwidth for CA | No | MU/TT is FFS, TP Analysis has been pending |
| 6.5A.2.1 | Spectrum emission mask for CA | Yes. |  |
| 6.5A.2.2 | Adjacent channel leakage ratio for CA | Yes. |  |
| 6.5A.3.0 | General spurious emissions for CA | No | Test is incomplete |
| 6.5A.3.1 | Spurious emission band UE co-existence for UL CA | No | Other incomplete items in test |
| 6.5A.3.2 | Additional spurious emissions | No | Other incomplete items in test |

### 2.3.2	Testability issues raised for approach using Pcell power limiting via TPC approach.

### The Aspect of beam peak search

It was identified by some contributors that the peak beam search will have to be performed uniquely for CA tests unless there is a UE vendor declaration of reusing the beam peak result from single carrier [9]. With the approach of limiting power on the PCell, one aspect that needs to be discussed is if this would apply during the peak beam search process as well. If so, then the limiting of PCell power would potentially prevent the SCell drop and peak beam search can be carried out in CA mode. However, the concern about time to obtain peak beam search while trying to limit the PCell power and attain equal PSD on both component carriers was raised at RAN5#92. For intra-band CA, it can be assumed that the peak beam would not differ significantly across component carriers. This has been confirmed by agreement on R5-215819 which can be extended to UL-CA.

Observation 8: RAN5 has agreed that PCC beam peak direction as the beam peak for intra-band DL CA for a frequency separation of up to 800 MHz. This can be extended to UL-CA.

### Dynamic Range Issues

Another testability issue that has been raised is dynamic range issues that are possible. An analysis of some test points around FR2a indicates a few points

- The impact of noise floor increases with channel bandwidth

- Impact is lower for lower MPR values

- Since measurement for UL-CA tests occurs on per CC basis, some of the dynamic range impact is mitigated.

Observation 9: Focusing on a reduced set of test points for MPR UL-CA tests is possible to avoid testability issues related to dynamic range while enabling testing for UL-CA configurations

2.3.3   Option 1: Use TPC to limit PCell Power

Most of the focus of RAN5 has been to explore test mode versus non test mode options to accomplish the desired objective of the FR2 RF CA test scenarios. There has been no agreement to adopt one or the other and in lieu of the ongoing RAN4 discussion.

As stated in [4] and [7], one way of preventing SCell dropping would be to limit the maximum power for the PCell to reserve power for SCell transmissions, at least for particular transmissions e.g. for PUSCH without UCI. Moreover, by limiting the SCell maximum power in addition, a behavior similar to that for LTE in which scaling applies uniformly for each serving cell as noted previously, from 36.213,

an “equal PSD” condition, could be achieved for PUSCH transmissions.

For FR1, the configured maximum output power is specified at the antenna connector and can be determined by . For FR2, on the other hand, both the configured power per cell  and the total configured power   are specified in implementation-specific plane of references internal to the UE. Absolute power limits configured by the network are therefore not viable for these parameters. Moreover, the UE power class for FR2 is specified in terms of EIRP that is impossible to control for UE operations in the field.

Again from [4], it can be gathered that a limit relative to the configured power can be specified to work around the issue of PCell and SCell power getting limited. This would also account for the actual power back-off (up to MPR) that is applied by the UE, which is unknown to the network but included in the PHR determination. The network would then configure the UE with UE-specific relative limits Xmax,f,c on the PCell and possibly also one or more SCells

relative to an absolute reference power Pref  that could be implementation specific. The relative limitation must not necessarily apply to all transmissions, only to specific transmissions like PUSCH without UCI or of priority 0. Other transmissions would not be limited.

From 38.521-2 clause 6.2.4.3, we know that

PPowerclass – MAX(MAX(MPRf,c, A- MPRf,c,) + ΔMBP,n, P-MPRf,c) – MAX{T(MAX(MPRf,c, A- MPRf,c,)), T(P-MPRf,c)} ≤ PUMAX,f,c ≤ EIRPmax

Also PCMAX,f,c for carrier f of a serving cell c is defined as that available to the reference point of a given transmitter branch that corresponds to the reference point of the higher-layer filtered RSRP measurement as specified in TS 38.215 [24].

For intra-band CA, the MPR for each serving cell is the same as that for the total power. Hence the same power back-off is normally assumed for both the  and the . Suppose we pick . If the network configures the UE with a value of  Xmax,f,PCell > 0 dB, then the  for the PCell should be reduced by this value and power for SCells would therefore be ensured as the total power  is unchanged.

An example of intra-band UL CA with allowed MPR for FR2 UE configured with a limit relative to    and four UL component carriers is shown in Figure 4.1. The ordinate shows the transmitted power density of the component carriers in relation to the , the   and the  as seen in the respective plane of reference (different at least for the power class). The power back-off up to the allowed MPR is measured relative to the power class. Configuration of the relative limit   reduces the configured power for the PCell, the remaining power up to  is available for the SCells. Setting  = 6 dB would make possible transmissions with equal power spectral density on all configured cells if the same limit is configured for the SCells.

Figure 4.1: UL intra-band contiguous CA – avoiding power limitation on SCell in FR2

Based on reference diagram in [4] with additional parameters added

Observation 10: To prevent SCell dropping due to a large power reduction, the discussed approach in RAN5 aligns with ongoing discussions in RAN4 that seek to attain the same objective i.e. limit the PCell power.

Option 1: To prevent SCell dropping due to a large power reduction, configure a power back-off on the PCC power via TPC so that remaining power up to Pcmax is available for Scells. Start with a limited set of test points on MPR CA tests to unblock UL-CA testing.

2.3.4	Option 2: Use Conformance Test Function to apply limit on PCell Power (no impact on PCC prioritization rules)

To help avoid some of the testability issues that are of concern when adjusting the power levels of PCC and SCCs during UL-CA RF conformance tests, it is proposed to have a test function that can apply the backoff or limit of power on the PCell. This will enable to UE to apply the backoff in response to the conformance test function and enable the SCells to stay active. This will enable testing of UL-CA configuration with prioritization rules enabled per TS 38.213. However, considering the impact on UEs that are already commercialized, this option should be limited to Release 16 since the RAN4/RAN2 solution is planned in Release 17. This option potentially serves as a compromise among the solutions available.

Option 2: Introduce a conformance test function, for Release 16 testing only, to apply the backoff Xmax,f,Pcell that the UE can apply during the UL-CA conformance tests that are configured to test at max transmit power.

2.3.5.    Await RAN4/RAN2 configuration updates

As per the way forward identified in RAN4 [11], which will be worked on in Release 17 timeframe, the discussion includes “a ‘RAN4 only solution’ but changes to RAN2 specifications also needed: configured power limits for UL serving cells and MAC-CE for enabling/disabling limits”.

Observation 11: RAN4 solution being worked on in Release 17 with possibility of RAN2 updates

Option 3: Based on the Observation 7 and Observation 11, the option is to wait for RAN4/RAN2 solution in Release 17 (configured power limits for PCell/SCell and MAC-CE for enabling/disabling limits)

Proposal 1: Prefer Option 1 to enable coverage on a limited set of test points for MPR CA tests prior to Release 17

We are open to options 2 and 3 as well based on feedback from other stakeholders on this topic.

# 3	Summary

In this contribution, the following observations were made

Observation 1: Scell Drop is possible when the EN-DC FR2 intra-band contiguous UL-CA test procedure configures the DUT to transmit at maximum power across CCs

Observation 2: As per TS 38.213 prioritization rules, PCC is prioritized over SCC. This is also the mode of operation in the field

Observation 3: RAN4 clarified via LS [2] to RAN5 that equal PSD is preferred test condition. However, actual UE behavior in the field has to be considered as well as prioritization rules in TS 38.213 to identify testing details.

Observation 4: RAN5 has agreed that the way forward recognizes prioritization rules in TS 38.213 [10]

Observation 5: RAN5 has to analyse and define test procedures which will ensure testability for intra-band UL-CA scenarios. Considering the ongoing RAN4 discussion, a potential solution from RAN4 cannot be precluded.

Observation 6: RAN1 confirms that no changes to prioritization rules is planned as there is no known issue as such. Further any UE-specific configuration of power limits to prevent SCell drop would not conflict with RAN1 specifications but a confirmation on this is dependent on the actual RAN4 solution.

Observation 7: There are 3 (three) FR2 RF UL-CA tests that are blocked solely by SCell drop issue. Other tests either have multiple other blocking issues or are not yet introduced in the spec even in Release 15 work plan for the past many RAN5 meetings

Observation 8: RAN5 has agreed that PCC beam peak direction as the beam peak for intra-band DL CA for a frequency separation of up to 800 MHz. This can be extended to UL-CA.

Observation 9: Focusing on a reduced set of test points for MPR UL-CA tests is possible to avoid testability issues related to dynamic range while enabling testing for UL-CA configurations

Observation 10: To prevent SCell dropping due to a large power reduction, the discussed approach in RAN5 aligns with ongoing discussions in RAN4 that seek to attain the same objective i.e. limit the PCell power.

Option 1: To prevent SCell dropping due to a large power reduction, configure a power back-off on the PCC power via TPC so that remaining power up to Pcmax is available for Scells. Start with a limited set of test points on MPR CA tests to unblock UL-CA testing.

Option 2: Introduce a conformance test function, for Release 16 testing only, to apply the backoff Xmax,f,Pcell (no impact on prioritization rules) that the UE can apply during the UL-CA conformance tests that are configured to test at max transmit power.

Observation 11: RAN4 solution being worked on in Release 17 with possibility of RAN2 updates

Option 3: Based on the Observation 7 and Observation 11, the option is to wait for RAN4/RAN2 solution in Release 17 (configured power limits for PCell/SCell and MAC-CE for enabling/disabling limits)

Proposal 1: Prefer Option 1 to enable coverage on a limited set of test points for MPR UL-CA tests prior to Release 17

We are open to options 2 and 3 as well based on feedback from other stakeholders on this topic.

# 4	References

Action_Points_RAN5#90-e_atRFsession

R5-2103124, LS from RAN4 to RAN5 on SCell Dropping due to PCC Prioritization, 3GPP TSG RAN WG5 Meeting #91-e, February 2021

R5-211227, Discussion on PCC prioritization, 3GPP TSG RAN WG5 Meeting #90-e, February 2021

R4-2101722 Scell Dropping behavior and verification thereof, Ericsson, 3GPP TSG RAN WG4 Meeting #98-e, February 2021

R5-212812 Discussion on Tx power allocation prioritization for FR2 UL CA testing, Qualcomm, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-213809 Discussion on SCell dropping and UL CA testing in NR FR2, Ericsson, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-213818 Discussion on FR2 test procedure updates to handle PCC Prioritization, Apple, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-212919, Discussion on FR1 and FR2 PCC priorization, Huawei, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-213809, Consideration on Scell drop issue for FR2 UL CA, Anritsu, 3GPP TSG RAN WG5 Meeting #91-e, May 2021

R5-211941, Discussion on PCC prioritization for FR1 and FR2 UL CA testing, Qualcomm Finland, 3GPP TSG RAN WG5 Meeting #90-e, February 2021

[11]   R4-2114949, WF on solution for Scell dropping issue, Huawei, RAN4#100