3GPP TSG-RAN5 Meeting #93-e	R5-217825

# Electronic Meeting, 8th November 2021 – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2658 | rev | 1 | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | New Test Case 11.6.1 Data Off / MO Voice Call |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | There is no test coverage for the Data Off feature. |  |  |  |  |  |  |  |  |
| Summary of change: |  | A new test case, 11.6.1 Data Off / MO Voice Call, was added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The Data Off feature will not be covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 11.6 and 11.6.1 (new clauses) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-217755 and the outcome of 1 revision to this document. |  |  |  |  |  |  |  |  |

Table 11.4.11.3.3-3: UL NAS TRANSPORT (step 1, Table 11.4.11.3.2-2)

| Derivation Path: TS 38.508-1 [4], Table 4.7.1-10, condition INITIAL_PDU_REQUEST. |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Request type | '100'B | existing emergency PDU session |  |
| S-NSSAI | Not Present |  |  |
| DNN | Not Present |  |  |

## 11.6	3GPP PS Data Off

### 11.6.1	Data Off / MO Voice Call

11.6.1.1	Test Purpose (TP)

(1)

with { UE in PDU SESSION ACTIVE state and in 5GMM-CONNECTED mode and PS Data Off configured to “inactive” }

ensure that {

when { UE PS Data Off status is changed to “active” }

then { UE performs a PDU session modification procedure providing the PS Data Off status set to “active” }

}

(2)

with { UE being registered to IMS and with PS Data Off configured to “inactive” }

ensure that {

when { UE PS Data Off status is changed to “active” }

then { UE sends REGISTER with PS Data Off configured to “active” }

}

(3)

with { UE being registered to IMS and with PS Data Off configured to “active” }

ensure that {

when { UE is being made to initiate a voice call }

then { UE sends INVITE for voice call }

}

11.6.1.2	Conformance Requirements

References: The conformance requirements covered in the present TC are specified in: TS 24.501, clause 6.2.10. TS 24.229 clause 4.17, Rel-15. Unless otherwise stated these are Rel-16 requirements.

[TS 24.501, clause 6.2.10]

In case of PLMN, a UE, which supports 3GPP PS data off (see 3GPP TS 23.501 [8]), can be configured with up to two lists of 3GPP PS data off exempt services as specified in 3GPP TS 24.368 [17] or in the EF3GPPPSDATAOFF USIM file as specified in 3GPP TS 31.102 [22]:

a)	a list of 3GPP PS data off exempt services to be used in the HPLMN or EHPLMN; and

b)	a list of 3GPP PS data off exempt services to be used in the VPLMN.

If only the list of 3GPP PS data off exempt services to be used in the HPLMN or EHPLMN is configured at the UE, this list shall be also used in the VPLMN.

In case of SNPN, a UE, which supports 3GPP PS data off (see 3GPP TS 23.501 [8]), can be configured with a list of 3GPP PS data off exempt services as specified in 3GPP TS 24.368 [17] for each SNPN whose entry exists in the "list of subscriber data":

a)	a list of 3GPP PS data off exempt services to be used in the SNPN.

If the UE supports 3GPP PS data off, the UE shall provide the 3GPP PS data off UE status in the Extended protocol configuration options IE during UE-requested PDU session establishment procedure except for the transfer of a PDU session from non-3GPP access to 3GPP access and except for the establishment of user plane resources on the other access for the MA PDU session(see subclause 6.4.1), and during UE-requested PDU session modification procedure (see subclause 6.4.2), regardless of associated access type of the PDU session. If the UE requests a PDU session establishment procedure in order to transfer a PDU session from non-3GPP access to 3GPP access, or in order to establish user plane resources on the other access for the MA PDU session over 3GPP access or non-3GPP access, and:

a)	if the 3GPP PS data off UE status has changed since the last providing to the network, the UE shall provide the 3GPP PS data off UE status in the Extended protocol configuration options IE; or

b)	if the 3GPP PS data off UE status has not changed since the last providing to the network, the UE need not provide the 3GPP PS data off UE status.

The network shall support of 3GPP PS data off.

The UE shall indicate change of the 3GPP PS data off UE status for the PDU session by using the UE-requested PDU session modification procedure as specified in subclause 6.4.2.

When the 3GPP PS data off UE status is "activated":

a)	the UE does not send uplink IP packets via 3GPP access except:

1)	for those services indicated in the list of 3GPP PS data off exempt services to be used in the HPLMN or EHPLMN as specified in 3GPP TS 24.368 [17] when the UE is in its HPLMN or EHPLMN or for those services indicated in the list of 3GPP PS data off exempt services to be used in the SNPN as specified in 3GPP TS 24.368 [17] when the UE is in an SNPN;

2)	for those services indicated in the list of 3GPP PS data off exempt services to be used in the HPLMN or EHPLMN when the UE is in the VPLMN, if only the list of 3GPP PS data off exempt services to be used in the HPLMN or EHPLMN is configured to the UE as specified in 3GPP TS 24.368 [17];

3)	for those services indicated in the list of 3GPP PS data off exempt services to be used in the VPLMN when the UE is in the VPLMN, if the list of 3GPP PS data off exempt services to be used in the VPLMN is configured to the UE as specified in 3GPP TS 24.368 [17];

4)	for those services indicated in the EF3GPPPSDATAOFF USIM file as specified in 3GPP TS 31.102 [22];

5)	any uplink traffic due to procedures specified in 3GPP TS 24.229 [14]; and

6)	any uplink traffic due to procedures specified in 3GPP TS 24.623 [20];

b)	the UE does not send uplink Ethernet user data packets via 3GPP access; and

c)	the UE does not send uplink Unstructured user data packets via 3GPP access.

Otherwise the UE sends uplink user data packets without restriction.

NOTE:	If the UE supports 3GPP PS data off, uplink IP packets are filtered as specified in 3GPP TS 24.229 [14] in U.3.1.5.

3GPP PS data off does not restrict sending of uplink user data packets via non-3GPP access of a single access PDU session or an MA PDU session.

[TS 24.229, Rel-15, clause 4.17]

The UE and the network can support the 3GPP PS data off.

When 3GPP PS data off is supported and active, IP packets that are associated with services that are not a 3GPP PS data off exempt service are prevented from transport over EPS IP-CAN, GPRS IP-CAN and 5GS IP-CAN as specified in 3GPP TS 23.228 [7]. The UE may be configured by the HPLMN or the EHPLMN with up to two indications whether a 3GPP IMS service is a 3GPP PS Data Off exempt service, one indication is valid for the UE is in the HPLMN or the EHPLMN and the other indication is valid for the UE is in the VPLMN. When the UE is only configured with the indication valid for the UE camping in the HPLMN or the EHPLMN, the UE shall use this indication also when the UE is in the VPLMN.

When 3GPP PS data off is supported and active and the UE is configured, either as specified in 3GPP TS 24.167 [8G] or in 3GPP TS 31.102 [15C], with services that are 3GPP PS data off exempt, then the UE will not send uplink IP packets related to any services that are not 3GPP PS data off exempt over EPS IP-CAN, GPRS IP-CAN and 5GS IP-CAN. The UE informs the network about its 3GPP PS data off status by including a g.3gpp.ps-data-off media feature tag specified in subclauce 7.9.8 in all REGISTER requests sent over GPRS IP-CAN, EPS IP-CAN or 5GS IP-CAN. The UE reregisters over EPS IP-CAN, GPRS IP-CAN and 5GS IP-CAN every time the 3GPP PS data off status is changed or the UE is provided by the network with a new list of 3GPP PS data off exempt services while the 3GPP PS data off status is "active".

An AS handling a service is configured with information whether the service is a 3GPP PS data off exempt service. If the 3GPP PS data off status is active and the service is not a 3GPP PS data off exempt service, the AS prevents downlink IP packets of the service from reaching the UE over EPS IP-CAN, GPRS IP-CAN and 5GS IP-CAN. The AS shall be configured with up to two indications whether a 3GPP IMS service is a 3GPP PS Data Off exempt service, one indication is valid for non-roaming users, and the other indication is valid for users roaming in the various VPLMNs with whom roaming agreements exist. When the AS is only configured with the indication valid for the UE camping in the HPLMN or the EHPLMN, the AS shall use this indication also when the UE is in the VPLMN.

[TS 24.229, Rel-15, clause U.3.1.0]

If the UE supports the 3GPP PS data off, then the UE shall in all REGISTER requests include the "+g.3gpp.ps-data-off" header field parameter defined in subclause 7.9.8 set to a value indicating the 3GPP PS data off status.

When the UE sends a REGISTER request, if the 3GPP PS data off status is "active", then the UE shall only include media feature tags associated with services that are 3GPP PS data off exempt services in the g.3gpp.icsi-ref media feature tag, as defined in subclause 7.9.2 and RFC 3840 [62], for the IMS communication services it intends to use.

If the UE is registered, and the 3GPP PS data off status is changed, then the UE shall perform a reregistration of the previously registered public user identity.

11.6.1.3	Test description

11.6.1.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	None.

Preamble:

-	UE is in state 3N-A (TS 38.508-1 [21]).

11.6.1.3.2	Test procedure sequence

Table 11.6.1.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to activate Data Off, see NOTE 1 | - | - | - | - |
| 2 | Check: Does the UE transmit a PDU SESSION MODIFICATION REQUEST message with Data Off status set to activated? | --> | 5GSM: PDU SESSION MODIFICATION REQUEST | 1 | P |
| 3 | The SS transmits an PDU SESSION MODIFICATION COMMAND message. | <-- | 5GSM: PDU SESSION MODIFICATION COMMAND | - | - |
| 4 | the UE transmit an PDU SESSION MODIFICATION COMPLETE message. | --> | 5GSM: PDU SESSION MODIFICATION COMPLETE | - | - |
| 5 | Check: does the UE re-register with "+g.3gpp.ps-data-off" header field parameter set to “active”? | --> | IMS: REGISTER | 2 | P |
| 6 | SS responds with 200 OK | <-- | IMS: 200 OK |  |  |
| 7 | UE is made to attempt an IMS voice call. | - | - |  |  |
| 8-25 | Check: is the MO Voice call procedure using preconditions performed, i.e., are Steps 1A-13 of Table 7.4.3.2-1 in TS 34.229-5 executed.? | - | - | 3 | P |
| 26-27 | SS releases the call. (Steps 1-2 of Annex A.8 in TS 34.229-5) | - | - | - | - |
| NOTE 1: This could be done by e.g. MMI or AT command. |  |  |  |  |  |

11.6.1.3.3	Specific message contents

Table 11.6.1.3.3-1: PDU SESSION MODIFICATION REQUEST (step 2, table 11.6.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol configuration options |  |  |  |  |
| Container ID n |  | ‘0017’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 1 octet |  |
| Container ID n contents |  | ‘02’H | 3GPP PS data off UE status activated |  |

Table 11.6.1.3.3-2: PDU SESSION MODIFICATION COMMAND (step 3, table 11.6.1.3.2-1)

| Derivation path: TS 38.508-1 [4], Table 4.7.2-7 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| Extended protocol configuration options |  |  |  |  |
| Container ID n |  | ‘0017’H | n assigned to next available number |  |
| Length of container ID n contents |  |  | 0 octets |  |
| Container ID n contents |  | empty | 3GPP PS data off support indication |  |

Table 11.6.1.3.3-3: REGISTER (step 5, table 11.6.1.3.2-1)

| Derivation Path: TS 34.229-1 [2], Annex A.1.1, Conditions A2, A4, A17, A32, A35 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| feature-param |  | +g.3gpp.ps-data-off=”active” |  | TS 24.229 [7] |