3GPP TSG-RAN5 Meeting #93-e	R5-217843

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 34.229-5 | CR | 0239 | rev | 1 | Current version: | 16.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new 5GS IMS test case 8.8 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-25 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | New 5GS IMS test case 8.8 need to be added according to WP. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added new test case 8.8 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test case will be missing |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-216571 |  |  |  |  |  |  |  |  |

8.8	Communication Forwarding Unconditional / Signalling / 5GS

8.8.1	Test Purpose (TP)

(1)

with { UE being registered to IMS and being made to initiate a voice call }

ensure that {

when { UE sends INVITE and receives a 181 Call Is Being Forwarded response }

then { UE continues and completes the voice call initiation }

}

8.8.2	Conformance Requirements

The conformance requirements covered in the present test case are, unless otherwise stated, Rel-15 requirements.

[TS 24.604, clause 4.2.1.2]:

The CFU service enables a served user to have the network redirect to another user communications which are addressed to the served user's address. The CFU service may operate on all communication, or just those associated with specified services. The served user's ability to originate communications is unaffected by the CFU supplementary service. After the CFU service has been activated, communications are forwarded independent of the status of the served user.

As a service provider option, a subscription option can be provided to enable the served user to receive an indication that the CFU service has been activated. This indication shall be provided when the served user originates a communication if the CFU service has been activated for the served user's address and for the service requested for the communication.

The maximum number of diversions permitted for each communication is a service provider option. The service provider shall define the upper limit of diversions. When counting the number of diversions, all types of diversion are included.

[TS 24.604, clause 4.5.2.1]:

When communication diversion has occurred on the served user side and the network option "Originating" user receives notification that his communication has been diverted (forwarded or deflected)" is set to true, the originating UA may receive a 181 (Call is being forwarded) response according to the procedures described in 3GPP TS 24.229.

The Information given by the History header could be displayed by the UA if it is a UE.

[TS 24.229, clause 9.2.3]:

Since the UE does not know that forking has occurred until a second provisional response arrives, the UE will request the radio/bearer resources as required by the first provisional response. For each subsequent provisional response that may be received, different alternative actions may be performed depending on the requirements in the SDP answer:

-	the UE has sufficient radio/bearer resources to handle the media specified in the SDP of the subsequent provisional response, or

-	the UE must request additional radio/bearer resources to accommodate the media specified in the SDP of the subsequent provisional response.

NOTE 1:	When several forked responses are received, the resources requested by the UE is the "logical OR" of the resources indicated in the multiple responses to avoid allocation of unnecessary resources. The UE does not request more resources than proposed in the original INVITE request.

NOTE 2:	When service-based local policy is applied, the UE receives the same authorization token for all forked requests/responses related to the same SIP session.

When an 199 (Early Dialog Terminated) response for the INVITE request is received for an early dialogue, the UE shall release reserved radio/bearer resources associated with that early dialogue.

When the first final 200 (OK) response for the INVITE request is received for one of the early dialogues, the UE proceeds to set up the SIP session using the radio/bearer resources required for this session. Upon the reception of the first final 200 (OK) response for the INVITE request, the UE shall release all unneeded radio/bearer resources.

GIBA:

NOTE 1:	GIBA does not allow SIP requests to be protected using an IPsec security association because it does not perform a key agreement procedure.

8.8.3	Test description

8.8.3.1	Pre-test conditions

System Simulator:

-	1 NR Cell connected to 5GC, default parameters.

UE:

-	The UE contains either ISIM and USIM applications or only USIM application on UICC.

-	The UE is configured to register for IMS after switch on.

-	The UE is configured to use preconditions.

Preamble:

-	UE is in state 1N-A (TS 38.508-1 [21]) and registered to IMS

8.8.3.2	Test procedure sequence

Table 8.8.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | UE is made to attempt an IMS voice call. | - | - | - | - |
| 2-7 | Steps 2-7 of generic procedure specified in Table 4.9.15.2.2-1 of TS 38.508-1 [21] are performed. | - | - | - | - |
| 8-9 | Steps 1-2 as defined in Annex A.4.1 are excuted. | - | - | - | - |
| 10 | SS sends 181 response to indicate that call forwarding has been started as the user is configured to Communication Forwarding Unconditional. |  | 181 Call is being forwarded | - | - |
| 11 | SS (simulating the phone to which the call was forwarded) responds with 183 Session Progress containing an SDP answer indicating support for AMR-WB codec and state of the local preconditions. |  | 183 Session Progress | - | - |
| 12 | Check: Does the UE send PRACK for 183 Session Progress |  | PRACK | 1 | P |
| 13 | SS responds to PRACK. |  | 200 OK | - | - |
| 14 | Check: Does the UE send a second SDP offer in an UPDATE request？ |  | UPDATE | 1 | P |
| 15 | SS responds to UPDATE. |  | 200 OK | - | - |
| 16 | The SS sends 180 Ringing response to the UE |  | 180 Ringing | - | - |
| 17 | UE acknowledges the receipt of 180 response by sending PRACK. |  | PRACK | - | - |
| 18 | The SS responds PRACK with 200 OK. |  | 200 OK | - | - |
| 19 | The SS responds INVITE with 200 OK to indicate that the virtual remote UE had answered the call |  | 200 OK | - | - |
| 20 | The UE acknowledges the receipt of 200 OK for INVITE |  | ACK | - | - |
| 21 | Make the UE release the call. | - | - | - | - |
| 22 | UE releases the voice call. (Steps 1-2 of Annex A.7.) | - | - | - | - |

8.8.3.3	Specific message content

Table 8.8.3.3-1: 181 Call is being forwarded for INVITE (Step 10, table 8.8.3.2-1)

Derivation Path: TS 34.229-1 [2], Table in subclause A.2.14.

Table 8.8.3.3-2: 183 Session Progress for INVITE (Step 11, table 8.8.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.3, condition A1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| To |  |  |  |  |
| tag |  | different tag must be used than the one used in steps 8-9 as this response is now from another UE and belongs to another dialog instance. Note that this new tag must be used within the rest of the steps (10-17) in this test case instead of the tag used within steps 8-9 |  |  |
| Contact |  |  |  |  |
| addr-spec |  | different URI must be used than the one used in step 8-9 as this is supposed now to represent another UE to which the call is being forwarded. Note that this new Contact must be used within the rest of the steps (13-14) in this test case. |  |  |
| Require |  |  |  |  |
| option-tag |  | precondition |  |  |
| Message-body |  | Same contents as specified in step 3 annex A.4.1 except for o-line: o=- 22222222 22222222 IN (addrtype) (unicast-address for new remote UE). |  |  |

Table 8.8.3.3-3: 180 Ringing for INVITE (Step 16, table 8.8.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.2.4 with conditions A1 and A7 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| addr-spec |  | Same value as in the 183 response of step 11 |  |  |
| History-Info |  |  |  |  |
| hi-targeted-to-uri |  | Same value as in the 181 response of step 10 |  |  |
| hi-index |  | Same value as in the 181 response of step 10 |  |  |

Table 8.8.3.3-4: 200 OK for INVITE (Step 19, table 8.8.3.2-1)

| Derivation Path: TS 34.229-1 [2], Table in subclause A.3.1, conditions A1, A10, and A19 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Header/param | Cond | Value/remark | Rel | Reference |
| Contact |  |  |  |  |
| addr-spec |  | Same value as in the 183 response of step 11 |  |  |
| History-Info |  |  |  |  |
| hi-targeted-to-uri |  | Same value as in the 181 response of step 10 |  |  |
| hi-index |  | Same value as in the 181 response of step 10 |  |  |