3GPP TSG-RAN WG5 Meeting #93-e	R5-217970

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-2 | CR | 0237 | rev | 1 | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of MCPTT Test Case 6.1.1.2 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCPTT-ConTest |  |  |  |  |  | Date: |  |  | 2021-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Floor Control Queueing (via parameter “mc_queueing”) is an optional parameter that the UE uses or does not use floor request queuing – see 27.380 clause 4.1.1.2 - Conditions ACK, EMERGENCY-CALL and IMMPERIL-CALL in Floor Control messages introduced - Generic procedue 5.3.10 and 5.3.12 for call release were renamed due to generalisation for all MC Services |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Pics pc_MCPTT_FloorRequestQueueing introduced and applied: - new test purpose for support of queueing - test sequence adapted - Conditions ACK, EMERGENCY-CALL and IMMPERIL-CALL in Floor Control messages applied - Rename of generic procedure 5.3.10 and 5.3.12 applied - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misleading procedures and message contents may result in incorrect MCPTT test case implementation, test case can not be validated |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.579-1 CR 0207, 0212, 0219 TS 36.579-4 CR 0018 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-216718 and the outcome of 1 revision to this document |  |  |  |  |  |  |  |  |

#### 6.1.1.2	On-network / On-demand Pre-arranged Group Call / Automatic Commencement Mode / Floor Control / Upgrade to Emergency Group Call / Cancel Emergency State / Upgrade to Imminent Peril Group Call / Cancel Imminent Peril State / Client Terminated (CT)

6.1.1.2.1	Test Purpose (TP)

(1)

with { UE (MCPTT Client) registered and authorised for MCPTT }

ensure that {
  when { the SS (MCPTT Server) initiates an On-demand Pre-arranged group call with Automatic Commencement Mode and implicit floor control }

then { UE (MCPTT Client) responds by sending a SIP 200 (OK) message and after indication from the MCPTT Server that the call was established notifies the user }

}

(2)

with { UE (MCPTT Client) having an ongoing MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the MCPTT User (MCPTT Client) engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server (Floor Request during a talk burst, Floor granting/release, Floor idle, Floor deny, Floor taken/revoked) }

}

(3)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call with Automatic  Commencement Mode }

ensure that {

when { the MCPTT User (MCPTT Client) wants to terminate the ongoing MCPTT Group Call }

then { UE (MCPTT Client) sends a SIP BYE request and leaves the MCPTT session }

}

(4)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call with Automatic  Commencement Mode }

ensure that {

when { the SS (MCPTT Server) upgrades the ongoing MCPTT Group Call to an MCPTT Emergency Group Call with floor control }

then { UE (MCPTT Client) responds to the SIP re-INVITE request with a SIP 200 (OK) response and considers the call as being upgraded to an Emergency Group Call (emergency group call state = "MEGC 3: emergency-call-granted") }

}

(5)

with { UE (MCPTT Client) in an upgraded Emergency Group Call }

ensure that {

when { the MCPTT User (MCPTT Client) continues communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server }

}

(6)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call with Automatic Commencement Mode which was upgraded to an Emergency Group Call }

ensure that {

when { the SS (MCPTT Server) cancels the ongoing MCPTT Emergency state }

then { UE (MCPTT Client) responds to the SIP re-INVITE request with a SIP 200 (OK) and considers the emergency condition cancelled }

}

(7)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the SS (MCPTT Server) upgrades the ongoing MCPTT Group Call to an MCPTT Imminent Peril Group Call with floor control }

then { UE (MCPTT Client) responds to the SIP re-INVITE request with a SIP 200 (OK) response and considers the call as being upgraded to an cImminent Peril Group Call (imminent peril group call state = "MIG 2: in-progress") }

}

(8)

with { UE (MCPTT Client) in an upgraded Imminent Peril Group Call }

ensure that {

when { the MCPTT User (MCPTT Client) continues communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control imposed by the MCPTT Server }

}

(9)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call with Automatic Commencement Mode which was upgraded to an Imminent Peril Group Call }

ensure that {

when { the SS (MCPTT Server) cancels the ongoing MCPTT Imminent Peril state }

then { UE (MCPTT Client) responds to the SIP re-INVITE request with a SIP 200 (OK) and considers the imminent peril condition cancelled }

}

(10)

with { UE (MCPTT Client) having an ongoing On-demand Pre-arranged Group Call with Automatic Commencement Mode }

ensure that {

when { the SS (MCPTT Server) needs to terminate the ongoing MCPTT Group Call }

then { the SS (MCPTT Server) sends a SIP BYE request and the UE (MCPTT Client) responds with a SIP 200 (OK) and leaves the MCPTT session }

}

(11)

with { UE (MCPTT Client) having established an MCPTT On-demand Pre-arranged Group Call with Automatic Commencement Mode and pc_MCPTT_FloorRequestQueueing = “true” }

ensure that {

when { the MCPTT User (MCPTT Client) engages in communication with the invited MCPTT User(s) }

then { UE (MCPTT Client) respects the floor control queue handling imposed by the MCPTT Server (Floor request queued, Floor queue position request and Floor queue position info) }

}

6.1.1.2.2	Conformance requirements

References: The conformance requirements covered in the current TC are specified in: TS 24.379 clauses 10.1.1.2.1.2, 6.2.2, 6.2.3.1.2, 6.2.6, 10.1.1.2.1.6, 6.2.5.1, 10.1.1.2.3.1, 10.1.1.2.3.3 and TS 24.380, clauses 6.2.4.5.3, 6.2.4.3.5, 6.2.4.4.2, 6.2.4.5.4, 6.2.4.4.4, 6.2.4.4.9, 6.2.4.9.9, 6.2.4.9.6, 6.2.4.9.4. The following represents a copy/paste extraction of the requirements relevant to the test purpose; any references within the copy/paste text should be understood within the scope of the core spec they have been copied from. Unless otherwise stated, these are Rel-13 requirements.

[TS 24.379, clause 10.1.1.2.1.2]

In the procedures in this subclause:

1)	emergency indication in an incoming SIP INVITE request refers to the <emergency-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

2)	imminent peril indication in an incoming SIP INVITE request refers to the <imminentperil-ind> element of the application/vnd.3gpp.mcptt-info+xml MIME body.

Upon receipt of an initial SIP INVITE request, the MCPTT client shall follow the procedures for termination of multimedia sessions in the IM CN subsystem as specified in 3GPP TS 24.229 [4] with the clarifications below.

The MCPTT client:

1)	may reject the SIP INVITE request if either of the following conditions are met:

a)	MCPTT client does not have enough resources to handle the call; or

b)	any other reason outside the scope of this specification;

and skip the rest of the steps;

2)	if the SIP INVITE request is rejected in step 1), shall respond toward participating MCPTT function either with appropriate reject code as specified in 3GPP TS 24.229 [4] and warning texts as specified in subclause 4.4.2 or with SIP 480 (Temporarily unavailable) response not including warning texts if the user is authorised to restrict the reason for failure and skip the rest of the steps of this subclause;

NOTE:	If the SIP INVITE request contains an emergency indication or imminent peril indication, the MCPTT client can by means beyond the scope of the present document choose to accept the request.

3)	shall check if a Resource-Priority header field is included in the incoming SIP INVITE request and may perform further actions outside the scope of the present document to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

4)	if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication that this is a SIP INVITE request for an MCPTT emergency group call and:

i)	should display the MCPTT ID of the originator of the MCPTT emergency group call contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body;

ii)	should display the MCPTT group identity of the group with the emergency condition contained in the <mcptt-calling-group-id> element; and

iii)	if the <alert-ind> element is set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information;

b)	shall set the MCPTT emergency group state to "MEG 2: in-progress";

c)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril"; and

d)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-gc-capable"; otherwise

5)	if the SIP INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCPTT user an indication that this is a SIP INVITE request for an MCPTT imminent peril group call and:

i)	should display the MCPTT ID of the originator of the MCPTT imminent peril group call contained in the <mcptt-calling-user-id> element of the application/vnd.3gpp.mcptt-info+xml MIME body; and

ii)	should display the MCPTT group identity of the group with the imminent peril condition contained in the <mcptt-calling-group-id> element; and

b)	shall set the MCPTT imminent peril group state to "MIG 2: in-progress";

6)	may display to the MCPTT user the MCPTT ID of the inviting MCPTT user;

7)	shall perform the automatic commencement procedures specified in subclause 6.2.3.1.2 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCPTT service setting at the invited MCPTT client for answering the call is set to automatic commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Auto" and the MCPTT service setting at the invited MCPTT client for answering the call is set to manual commencement mode, yet the invited MCPTT client allows the call to be answered with automatic commencement mode;

8)	shall perform the manual commencement procedures specified in subclause 6.2.3.2.2 if one of the following conditions are met:

a)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCPTT service setting at the invited MCPTT client for answering the call is to use manual commencement mode; or

b)	SIP INVITE request contains an Answer-Mode header field with the value "Manual" and the MCPTT service setting at the invited MCPTT client for answering the call is set to automatic commencement mode, yet the invited MCPTT client allows the call to be answered with manual commencement mode; and

9)	when the SIP 200 (OK) response to the SIP INVITE request is sent, may subscribe to the conference event package as specified in subclause 10.1.3.1.

[TS 24.379, clause 6.2.2]

When the MCPTT client receives an initial SDP offer for an MCPTT session, the MCPTT client shall process the SDP offer and shall compose an SDP answer according to 3GPP TS 24.229 [4].

When composing an SDP answer, the MCPTT client:

1)	shall accept the MCPTT speech media stream in the SDP offer;

2)	shall set the IP address of the MCPTT client for the accepted MCPTT speech media stream and, if included in the SDP offer, for the accepted media-floor control entity;

NOTE:	If the MCPTT client is behind a NAT the IP address and port included in the SDP answer can be a different IP address and port than the actual IP address and port of the MCPTT client depending on the NAT traversal method used by the SIP/IP Core.

3)	shall include an "m=audio" media-level section for the accepted MCPTT speech media stream consisting of:

a)	the port number for the media stream;

b)	media-level attributes as specified in 3GPP TS 24.229 [4]; and

c)	"i=" field set to "speech" according to 3GPP TS 24.229 [4]; and

4)	if included in the SDP offer, shall include the media-level section of the offered media-floor control entity consisting of:

a)	an "m=application" media-level section as specified in 3GPP TS 24.380 [5] clause 12; and

b)	'fmtp' attributes as specified in 3GPP TS 24.380 [5] clause 14.

[TS 24.379, clause 6.2.3.1.2]

When performing the automatic commencement mode procedures, the MCPTT client shall follow the procedures in subclause 6.2.3.1.1 with the following clarification:

-	The MCPTT client may include a P-Answer-State header field with the value "Confirmed" as specified in IETF RFC 4964 [34] in the SIP 200 (OK) response.

[TS 24.379, clause 6.2.6]

Upon receiving a SIP BYE request, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5]; and

2)	shall send SIP 200 (OK) response towards MCPTT server according to 3GPP TS 24.229 [4].

[TS 24.379, clause 10.1.1.2.1.6]

This subclause covers both on-demand session and pre-established sessions.

Upon receipt of a SIP re-INVITE request the MCPTT client:

1)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "true":

a)	should display to the MCPTT user the MCPTT ID of the originator of the MCPTT emergency group call and an indication that this is an MCPTT emergency group call;

b)	if the <mcpttinfo> element containing the <mcptt-Params> element contains an <alert-ind> element set to "true", should display to the MCPTT user an indication of the MCPTT emergency alert and associated information;

c)	shall set the MCPTT emergency group state to "MEG 2: in-progress";

d)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril"; and

e)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-gc-capable";

2)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "true":

a)	should display to the MCPTT user the MCPTT ID of the originator of the MCPTT imminent peril group call and an indication that this is an MCPTT imminent peril group call; and

b)	shall set the MCPTT imminent peril group state to "MIG 2: in-progress";

3)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <emergency-ind> element set to a value of "false":

a)	should display to the MCPTT user the MCPTT ID of the MCPTT user cancelling the MCPTT emergency group call;

b)	if the <mcpttinfo> element containing the <mcptt-Params> element contains an <alert-ind> element set to "false":

i)	should display to the MCPTT user an indication of the MCPTT emergency alert cancellation and the MCPTT ID of the MCPTT user cancelling the MCPTT emergency alert; and

ii)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body including an <originated-by> element:

A)	should display to the MCPTT user the MCPTT ID contained in the <originated-by> element of the MCPTT user that originated the MCPTT emergency alert; and

B)	if the MCPTT ID contained in the <originated-by> element is the MCPTT ID of the receiving MCPTT user shall set the MCPTT emergency alert state to "MEA 1: no-alert";

c)	shall set the MCPTT emergency group state to "MEG 1: no-emergency"; and

d)	if the MCPTT emergency group call state of the group is set to "MEGC 3: emergency-call-granted", shall set the MCPTT emergency group call state of the group to "MEGC 1: emergency-gc-capable";

4)	if the SIP re-INVITE request contains an application/vnd.3gpp.mcptt-info+xml MIME body with the <mcpttinfo> element containing the <mcptt-Params> element with the <imminentperil-ind> element set to a value of "false":

a)	should display to the MCPTT user the MCPTT ID of the MCPTT user cancelling the MCPTT imminent peril group call and an indication that this is an MCPTT imminent peril group call;

b)	shall set the MCPTT imminent peril group state to "MIG 1: no-imminent-peril"; and

c)	shall set the MCPTT imminent peril group call state to "MIGC 1: imminent-peril-gc-capable";

5)	shall check if a Resource-Priority header field is included in the incoming SIP re-INVITE request and may perform further actions outside the scope of the present document to act upon an included Resource-Priority header field as specified in 3GPP TS 24.229 [4];

6)	shall accept the SIP re-INVITE request and generate a SIP 200 (OK) response according to rules and procedures of 3GPP TS 24.229 [4];

7)	shall include the g.3gpp.mcptt media feature tag in the Contact header field of the SIP 200 (OK) response;

8)	shall include the g.3gpp.icsi-ref media feature tag containing the value of "urn:urn-7:3gpp-service.ims.icsi.mcptt" in the Contact header field of the SIP 200 (OK) response;

9)	if the SIP re-INVITE request was received within an on-demand session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4] with the clarifications given in subclause 6.2.2;

10)	if the SIP re-INVITE request was received within a pre-established session, shall include an SDP answer in the SIP 200 (OK) response to the SDP offer in the incoming SIP re-INVITE request according to 3GPP TS 24.229 [4], based upon the parameters already negotiated for the pre-established session;

NOTE:	The SIP re-INVITE request can be received within an on-demand session or a pre-established session. If the SIP re-INVITE request is received within a pre-established session, the media-level section for the MCPTT speech media stream and the media-level section of the media-floor control entity are expected to be the same as was negotiated in the existing pre-established session.

11)	shall send the SIP 200 (OK) response towards the MCPTT server according to rules and procedures of 3GPP TS 24.229 [4]; and

12)	shall interact with the media plane as specified in 3GPP TS 24.380 [5].

[TS 24.379, clause 6.2.5.1]

When the MCPTT client wants to release an MCPTT session established using on-demand session signalling, the MCPTT client:

1)	shall interact with the media plane as specified in 3GPP TS 24.380 [5];

2)	shall generate a SIP BYE request according to 3GPP TS 24.229 [4];

3)	shall set the Request-URI to the MCPTT session identity to release; and

4)	shall send a SIP BYE request towards MCPTT server according to 3GPP TS 24.229 [4].

Upon receiving a SIP 200 (OK) response to the SIP BYE request, the MCPTT client shall interact with the media plane as specified in 3GPP TS 24.380 [5].

[TS 24.379, clause 10.1.1.2.3.1]

When an MCPTT client wants to leave the MCPTT session that has been established using on-demand session, the MCPTT client shall follow the procedures as specified in subclause 6.2.4.1.

[TS 24.379, clause 10.1.1.2.3.3]

Upon receiving a SIP BYE request for releasing the prearranged MCPTT group call, the MCPTT client shall follow the procedures as specified in subclause 6.2.6.

[TS 24.380, clause 6.2.4.5.3]

Upon receiving an indication from the user to release the permission to send RTP media, the floor participant:

1.	shall send a Floor Release message towards the floor control server The Floor Release message:

a.	may include the first bit in the subtype of the Floor Release message set to '1' (acknowledgement is required) as specified in subclause 8.2.2;

NOTE:	It is an implementation option to handle the receipt of the Floor Ack message and what action to take if the Floor Ack message is not received.

b.	if the session is a broadcast call and if the session was established as a normal call, shall include the Floor Indicator with the A-bit set to '1' (Normal call); and

c.	if the Floor Granted message included the G-bit set to '1' (Dual floor), shall include the Floor Indicator with the G-bit set to '1' (Dual floor);

2.	shall remove the indication that the participant is overriding without revoke if this indication is stored;

3.	shall start timer T100 (Floor Release) and initialize counter C100 (Floor Release) to 1; and

4.	shall enter the 'U: pending Release' state.

[TS 24.380, clause 6.2.4.3.5]

Upon receiving an indication from the user to request permission to send media, the floor participant:

1.	shall send the Floor Request message toward the floor control server; The Floor Request message:

a.	if a different priority than the normal priority is required, shall include the Floor Priority field with the priority not higher than negotiated with the floor control server as specified in subclause 14.3.3; and

b.	if the floor request is a broadcast group call, system call, emergency call or an imminent peril call, shall include a Floor Indicator field indicating the relevant call types;

2.	shall start timer T101 (Floor Request) and initialise counter C101 (Floor Request) to 1; and

3.	shall enter the 'U: pending Request' state.

[TS 24.380, clause 6.2.4.4.2]

Upon receiving a Floor Granted message from the floor control server or a floor granted indication in an SIP 200 (OK) response in the application and signalling layer, the floor participant:

1.	if the first bit in the subtype of the Floor Granted message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '1' (Floor Granted); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	shall provide floor granted notification to the user, if not already done;

NOTE:	Providing the floor granted notification to the user prior to receiving the Floor Granted message is an implementation option.

3.	if the Floor Indicator field is included and the B-bit is set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	if the G-bit in the Floor Indicator is set to '1' (Dual floor) shall store an indication that the participant is overriding without revoke;

5.	shall stop the optional timer T103 (End of RTP media), if running;

6.	shall stop timer T101 (Floor Request); and

7.	shall enter the 'U: has permission' state.

[TS 24.380, clause 6.2.4.5.4]

Upon receiving a Floor Revoke message, the floor participant:

1.	shall inform the user that the permission to send RTP media is being revoked;

2.	may give information to the user about the reason for revoking the permission to send media;

3.	shall request the media in the MCPTT client discard any remaining buffered RTP media packets and to stop forwarding of encoded voice to the MCPTT server;

4	if the G-bit in the Floor Indicator is set to '1' (Dual floor):

a.	shall send a Floor Release message. In the Floor Release message:

i.	shall include the Floor Indicator with the G-bit set to '1' (Dual floor); and

ii.	may set the first bit in the subtype to '1' (Acknowledgment is required) as described in subclause 8.3.2;

5	if the G-bit in the Floor Indicator is set to '0' (not Dual floor):

a.	shall send a Floor Release message. In the Floor Release message:

i.	shall include the Floor Indicator with the G-bit set to '0' (not Dual floor); and

ii.	may set the first bit in the subtype to '1' (Acknowledgment is required) as described in subclause 8.3.2;

NOTE:	It is an implementation option to handle the receipt of the Floor Ack message and what action to take if the Floor Ack message is not received.

6.	shall start timer T100 (Floor Release) and initialize counter C100 (Floor Release) to 1; and

7.	shall enter the 'U: pending Release' state.

[TS 24.380, clause 6.2.4.4.4]

Upon receiving a Floor Deny message, the floor participant:

1.	if the first bit in the subtype of the Floor Deny message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '3' (Floor Deny); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	shall provide floor deny notification to the user;

3.	may display the floor deny reason to the user using information in the Reject Cause field;

4.	shall stop timer T101 (Floor Request); and

5.	shall enter the 'U: has no permission' state.

[TS 24.380, clause 6.2.4.4.9]

Upon receiving a Floor Queue Position Info message, the floor participant:

1.	if the first bit in the subtype of the Floor Queue Position Info message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '9' (Floor Queue Position Info); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	shall provide floor request queued response notification to the MCPTT user;

3.	may provide the queue position and priority to the MCPTT user; and

4.	shall enter the 'U: queued' state.

[TS 24.380, clause 6.2.4.9.9]

Upon receipt of an indication from the MCPTT client to request the queue position, the floor participant:

1.	shall send the Floor Queue Position Request message;

2.	shall start timer T104 (Floor Queue Position Request) and initialize counter C104 (Floor Queue Position Request) to 1; and

3.	remain in the 'U: queued' state.

[TS 24.380, clause 6.2.4.9.6]

Upon receiving an indication from the MCPTT user to release the queued floor request, the floor participant:

1.	shall send a Floor Release message: The Floor Release message:

a.	may include the Floor Indicator field changing a broadcast group call to a normal call;

2.	may set the first bit in the subtype of the Floor Release message to '1' (Acknowledgment is required) as described in subclause 8.3.2;

NOTE:	It is an implementation option to handle the receipt of the Floor Ack message and what action to take if the Floor Ack message is not received.

3.	shall start timer T100 (Floor Release) and initialise counter C10 (Floor Release) to 1;

4.	shall stop timer T104 (Floor Queue Position Request), if running; and

5.	shall enter the 'U: pending Release' state.

[TS 24.380, clause 6.2.4.9.4]

Upon receiving a Floor Granted message, the floor participant:

1.	if the first bit in the subtype of the Floor Granted message is set to '1' (Acknowledgment is required) as described in subclause 8.3.2, shall send a Floor Ack message. The Floor Ack message:

a.	shall include the Message Type field set to '1' (Floor Granted); and

b.	shall include the Source field set to '0' (the floor participant is the source);

2.	shall provide a floor granted notification to the MCPTT user;

3.	if the Floor Indicator field is included and the B-bit is set to '1' (Broadcast group call), shall provide a notification to the user indicating the type of call;

4.	shall stop timer T104 (Floor Queue Position Request), if running;

5.	shall start timer T132 (Queued granted user action);

6.	shall indicate the user that the floor is granted; and

7.	shall remain in the 'U: queued' state.

6.1.1.2.3	Test description

6.1.1.2.3.1	Pre-test conditions

System Simulator:

-	SS (MCPTT server)

-	For the underlying "transport bearer" over which the SS and the UE will communicate Parameters are set to the default parameters for the basic E-UTRA Single cell network scenarios, as defined in TS 36.508 [24] clause 4.4. The simulated Cell 1 shall belong to PLMN1 (the PLMN specified for MCPTT operation in the MCPTT configuration document).

IUT:

-	UE (MCPTT client)

-	The test USIM set as defined in TS 36.579-1 [2], subclause 5.5.10 is inserted.

Preamble:

-	The UE has performed the Generic Test Procedure for MCPTT UE registration as specified in TS 36.579-1 [2], subclause 5.4.2.

-	The MCPTT User performs the Generic Test Procedure for MCPTT Authorization/Configuration and Key Generation as specified in TS 36.579-1 [2], subclause 5.3.2.

-	UE States at the end of the preamble

-	The UE is in E-UTRA Registered, Idle Mode state.

-	The MCPTT Client Application has been activated and User has registered-in as the MCPTT User with the Server as active user at the Client.

6.1.1.2.3.2	Test procedure sequence

Table 6.1.1.2.3.2-1: Main Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Is the Generic Test Procedure for MCPTT CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to establish an on-demand pre-arranged group call with automatic commencement mode and implicit floor control correctly performed? | - | - | 1 | P |
| 2a1-9 | Void | - | - | - | - |
| 10 | Make the MCPTT User request to speak (e.g. pressing the PTT button) (NOTE 1). | - | - | - | - |
| 11 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Granted as described in TS 36.579-1 [2] Table 5.3.16.3-1? | - | - | 2 | P |
| 12-14 | Void | -- | - | - | - |
| 15 | The SS overrides the UE (MCPTT client) and grants the floor to a higher priority MCPTT Client. | - | - | - | - |
| 16 | The SS sends a Floor Revoke message with the Reject Cause set to #4 - Media Burst pre-empted. | <-- | Floor Revoke | - | - |
| - | EXCEPTION: In parallel to the events described in step 17, the step specified in Table 6.1.1.2.3.2-2 takes place. | - | - | - | - |
| 17 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Release – Floor Taken as described in TS 36.579-1 [2] Table 5.3.21.3-1? | - | - | 2 | P |
| 18 | Void | -- | - | - | - |
| 19 | Make the MCPTT User request to speak (e.g. pressing the PTT button) (NOTE 1). | - | - | - | - |
| 20 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Deny as described in TS 36.579-1 [2] Table 5.3.19.3-1? | - | - | 2 | P |
| 21-22 | Void | - | - | - | - |
| - | EXCEPTION: Steps 23a1 - 23b2 describe behaviour that depends on pc_MCPTT_FloorRequestQueueing parameter “true” or “false”. The “lower case letter” identifies a step sequence that takes place when one or the other is the case | - | - | - | - |
| 23a1 | IF pc_MCPTT_FloorRequestQueueing: Make the MCPTT User request to speak (e.g. pressing the PTT button) (NOTE 1). | - | - | - | - |
| 23a2 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Queue Position Info as described in TS 36.579-1 [2] Table 5.3.17.3-1? | - | - | 111 | P |
| 23a3 | Check: Does the UE (MCPTT client) provide floor request queued response notification to the MCPTT user (NOTE 1)? | - | - | 11 | P |
| 23a4 | Make the MCPTT User request the current position in the queue (NOTE 1). | - | - | - | - |
| 23a5 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Queuing Position Request as described in TS 36.579-1 [2] Table 5.3.18.3-1? | - | - | 11 | P |
| 23a6 | Check: Does the UE (MCPTT Client) provide floor granted notification to the MCPTT User? (NOTE 1) | - | - | 11 | P |
| 23a7 | Make the MCPTT User request to cancel the Floor Request and end being in the queue (e.g. releasing the PTT button) (NOTE 1). | - | - | - | - |
| 23a8 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Release – Floor Taken as described in TS 36.579-1 [2] Table 5.3.21.3-1 to cancel the queue position? | - | - | 11 | P |
| 23a9 | Make the MCPTT User request to speak (e.g. pressing the PTT button) (NOTE 1). | - | - | - | - |
| 23a10 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Queue Position Info as described in TS 36.579-1 [2] Table 5.3.17.3-1? | - | - | 11 | P |
| 23a11 | Check: Does the UE (MCPTT client) provide floor request queued response notification to the MCPTT user (NOTE 1)? | - | - | 11 | P |
| 23a12 | The SS sends a Floor Granted message with no acknowledgement required | <-- | Floor Granted | - | - |
| 23a13 | Check: Does the UE (MCPTT client) provide a floor granted notification to the MCPTT user (NOTE 1)? | - | - | 11 | P |
| 23b1 | ELSE IF NOT pc_MCPTT_FloorRequestQueueing: Make the MCPTT User request to speak (e.g. pressing the PTT button). (NOTE 1) | - | - | - | - |
| 23b2 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Granted as described in TS 36.579-1 [2] Table 5.3.16.3-1? | - | - | 2 | P |
| 24-38 | Void |  |  |  |  |
| 39 | Make the MCPTT User indicate end of talking (e.g. releasing the PTT button) (NOTE 1). | - | - | - | - |
| 40 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Release – Floor Idle as described in TS 36.579-1 [2] Table 5.3.20.3-1? | - | - | 2 | P |
| 41a1-42 | Void | - | - | - | - |
| 43 | Make the MCPTT User end the on-demand group call. (NOTE 1). | - | - | - | - |
| 44 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCX CO call release as described in TS 36.579-1 [2] Table 5.3.10.3-1 to end the on-demand group call? | - | - | 3 | P |
| 45 | Void | - | - | - | - |
| 46 | Check: Is the Generic Test Procedure for MCPTT CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to establish an on-demand pre-arranged group call with automatic commencement mode and implicit floor control correctly performed? | - | - | 1 | P |
| 47a1-51 | Void | - | - | - | - |
| 52 | The SS sends a Floor Taken message with no acknowledgement required | <-- | Floor Taken | - | - |
| 53 | Check: Is the Generic Test Procedure for MCPTT CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to upgrade the On-Demand Pre-arranged Group Call to an MCPTT Emergency Group Call correctly performed? | - | - | 4 | P |
| 54A | Void | - | - | - | - |
| 54B | The SS sends a Floor Taken message | <-- | Floor Taken | - | - |
| 55 | The SS sends a Floor Idle message with no acknowledgement required | <-- | Floor Idle | - | - |
| 56 | Make the MCPTT User request to speak (e.g. pressing the PTT button) (NOTE 1). | - | - | - | - |
| 57 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Granted as described in TS 36.579-1 [2] Table 5.3.16.3-1? | - | - | 5 | P |
| 58-60 | Void | - | - | - | - |
| 61 | Make the MCPTT User indicate end of talking (e.g. releasing the PTT button) (NOTE 1). | - | - | - | - |
| 62 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Release – Floor Idle as described in TS 36.579-1 [2] Table 5.3.20.3-1? | - | - | 5 | P |
| 63a1-64 | Void | - | - | - | - |
| 65 | Check: is the Generic Test Procedure for MCPTT CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to cancel the emergency state correctly performed? | - | - | 6 | P |
| 65Aa1-66A | Void | - | - | - | - |
| 67 | The SS sends a Floor Taken message with no acknowledgement required | <-- | Floor Taken | - | - |
| 68 | The SS sends a Floor Idle message with no acknowledgement required | <-- | Floor Idle | - | - |
| 69 | Make the MCPTT User request to speak (e.g. pressing the PTT button) (NOTE 1). | - | - | - | - |
| 70 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Granted as described in TS 36.579-1 [2] Table 5.3.16.3-1? | - | - | 2 | P |
| 71-73 | Void | - | - | - | - |
| 74 | Make the MCPTT User indicate end of talking (e.g. releasing the PTT button) (NOTE 1). | - | - | - | - |
| 75 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Release – Floor Idle as described in TS 36.579-1 [2] Table 5.3.20.3-1? | - | - | 2 | P |
| 76a1-77 | Void | - | - | - | - |
| 78 | Check: Is the Generic Test Procedure for MCPTT CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to upgrade the On-Demand Pre-arranged Group Call to MCPTT Imminent Peril Group Call correctly performed? | - | - | 7 | P |
| 78Aa1-79A | Void | - | - | - | - |
| 80 | The SS sends a Floor Idle message with no acknowledgement required | <-- | Floor Idle | - | - |
| 81 | Make the MCPTT User request to speak (e.g. pressing the PTT button) (NOTE 1). | - | - | - | - |
| 82 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Granted as described in TS 36.579-1 [2] Table 5.3.16.3-1? | - | - | 8 | P |
| 83-85 | Void | - | - | - | - |
| 86 | Make the MCPTT User indicate end of talking (e.g. releasing the PTT button) (NOTE 1). | - | - | - | - |
| 87 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Release – Floor Idle as described in TS 36.579-1 [2] Table 5.3.20.3-1? | - | - | 8 | P |
| 88a1-89 | Void | - | - | - | - |
| 90 | Check: Is the Generic Test Procedure for MCPTT CT session establishment/modification without provisional responses other than 100 Trying as described in TS 36.579-1 [2] Table 5.3.4.3-1 to cancel the imminent peril state correctly performed? | - | - | 9 | P |
| 90Aa1-91A | Void | - | - | - | - |
| 92 | The SS sends a Floor Taken message with no acknowledgement required | <-- | Floor Taken | - | - |
| 93 | The SS sends a Floor Idle message with no acknowledgement required | <-- | Floor Idle | - | - |
| 94 | Make the MCPTT User request to speak (e.g. pressing the PTT button) (NOTE 1). | - | - | - | - |
| 95 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Request – Floor Granted as described in TS 36.579-1 [2] Table 5.3.16.3-1? | - | - | 9 | P |
| 96-98 | Void | - | - | - | - |
| 99 | Make the MCPTT User indicate end of talking (e.g. releasing the PTT button) (NOTE 1). | - | - | - | - |
| 100 | Check: Does the UE (MCPTT client) perform Generic Test Procedure for MCPTT Floor Release – Floor Idle as described in TS 36.579-1 [2] Table 5.3.20.3-1? | - | - | 9 | P |
| 101a1-102 | Void | - | - | - | - |
| 103 | Check: Is the Generic Test Procedure for MCX CT call release as described in TS 36.579-1 [2] Table 5.3.12.3-1 to end the On-demand Pre-arranged Group Call correctly performed? | - | - | 10 | P |
| 104 | Void | - | - | - | - |
| NOTE 1: This is expected to be done via a suitable implementation dependent MMI. |  |  |  |  |  |

Table 6.1.1.2.3.2-2: Parallel Behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | Check: Does the UE (MCPTT Client) inform the MCPTT User that the permission to send RTP media is being revoked? NOTE 1 in Table 6.1.1.2.3.2-1 | - | - | 2 | P |

6.1.1.2.3.3	Specific message contents

Table 6.1.1.2.3.3-1: SIP INVITE (steps 1, 46, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info |  |  |
| MIME part body | MCPTT Info as described in Table 6.1.1.2.3.3-1A |  |  |  |

Table 6.1.1.2.3.3-1A: MCPTT-Info in SIP INVITE (Table 6.1.1.2.3.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.2-1, condition GROUP-CALL

Table 6.1.1.2.3.3-2: Void

Table 6.1.1.2.3.3-3: SIP INVITE (step 53, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 condition EMERGENCY-CALL, re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info | TS 24.379 [9] clause F.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.1.2.3.3-4 |  |  |  |

Table 6.1.1.2.3.3-4: MCPTT-Info in SIP INVITE (Table 6.1.1.2.3.3-3)

Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 condition GROUP-CALL, EMERGENCY-CALL

Table 6.1.1.2.3.3-5: SIP INVITE (step 65, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 condition re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info | TS 24.379 [9] clause F.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.1.2.3.3-6 |  |  |  |

Table 6.1.1.2.3.3-6: MCPTT-Info in SIP INVITE (Table 6.1.1.2.3.3-5)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| emergency-ind | "false" |  |  |  |
| alert-ind | "false" |  |  |  |

Table 6.1.1.2.3.3-7: SIP INVITE (step 78, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 condition GROUP-CALL, IMMPERIL-CALL and re_INVITE

Table 6.1.1.2.3.3-8: SIP INVITE (step 90, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.4.3-1)

| Derivation Path: TS 36.579-1 [2], Table 5.5.2.5.2-1 condition re_INVITE |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| Message-body |  |  |  |  |
| MIME body part |  | MCPTT Info | TS 24.379 [9] clause F.1 |  |
| MIME-part-body | MCPTT-Info as described in Table 6.1.1.2.3.3-9 |  |  |  |

Table 6.1.1.2.3.3-9: MCPTT-Info in SIP INVITE (Table 6.1.1.2.3.3-8)

| Derivation Path: TS 36.579-1 [2], Table 5.5.3.2.1-1 condition GROUP-CALL |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcpttinfo |  |  |  |  |
| mcptt-Params |  |  |  |  |
| imminentperil-ind | "false" |  |  |  |

Table 6.1.1.2.3.3-10: Void

Table 6.1.1.2.3.3-11: Floor Taken (steps 7, 17, 23a8, 52, 67, 92, Table 6.1.1.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3.21.3-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.7-1 condition ON-NETWORK

Table 6.1.1.2.3.3-11A: Floor Taken (step 54B Table 6.1.1.2.3.2-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.7-1 condition ON-NETWORK, EMERGENCY-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-12: Floor Idle (steps 40, 68, 75, 93, 100, Table 6.1.1.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3.19.3-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.6-1

Table 6.1.1.2.3.3-13: Floor Idle (steps 55, 62, Table 6.1.1.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3.19.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.6-1 condition EMERGENCY-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-14: Floor Idle (steps 80, 87, Table 6.1.1.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3.19.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.6-1 condition IMMPERIL-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-15: Floor Request (steps 11, 20, 23a2, 23a10, 70, 95, Table 6.1.1.2.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.14.3-1; step 1, TS 36.579-1 [2] Table 5.3.16.3-1; step 1, TS 36.579-1 [2] Table 5.3.17.3-1; step 1, TS 36.579-1 [2] Table 5.3.18.3-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.2-1 condition ON-NETWORK

Table 6.1.1.2.3.3-16: Floor Request (step 57, Table 6.1.1.2.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.14.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.2-1 condition ON-NETWORK, EMERGENCY-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-17: Floor Request (step 82, Table 6.1.1.2.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.14.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.2-1 condition ON-NETWORK, IMMPERIL-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-18: Floor Granted (step 11, 70, 95, Table 6.1.1.2.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.14.3-1))

Derivation Path: 36.579-1 [2], Table 5.5.6.3-1 condition ON-NETWORK, ACK

Table 6.1.1.2.3.3-19: Floor Granted (steps 23a12,23b2, Table 6.1.1.2.3.2-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.3-1 condition ON-NETWORK |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-20: Floor Granted (step 57, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.14.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.3-1 condition ON-NETWORK, EMERGENCY-CALL, ACK |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-21: Floor Granted (step 82, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.14.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.3-1 condition ON-NETWORK, IMMPERIL-CALL, ACK |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-22: Floor Ack (steps 11, 57, 70, 82, 95, Table 6.1.1.2.3.2-1; step 3, TS 36.579-1 [2] Table 5.3.14.3-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.11-1, condition UPLINK

Table 6.1.1.2.3.3-23: Floor Revoke (step 16, Table 6.1.1.2.3.2-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.8-1

Table 6.1.1.2.3.3-24: Floor Release (steps 16, 23a8, 40, 75, 100, Table 6.1.1.2.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.20.3-1; step 1, TS 36.579-1 [2] Table 5.3.19.3-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.5-1 condition ON-NETWORK

Table 6.1.1.2.3.3-25: Floor Release (step 62, Table 6.1.1.2.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.19.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.5-1 condition ON-NETWORK, EMERGENCY-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-26: Floor Release (step 87, Table 6.1.1.2.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.19.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.5-1 condition ON-NETWORK, IMMPERIL-CALL |  |  |  |
| --- | --- | --- | --- |

Table 6.1.1.2.3.3-27: Floor Deny (step 21, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.18.3-1)

| Derivation Path: 36.579-1 [2], Table 5.5.6.4-1 condition ON-NETWORK |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| Reject Cause |  |  |  |
| Reject Cause | "255" | Cause #255 - Other reason |  |
| Reject Phrase | "Other reason" |  |  |

Table 6.1.1.2.3.3-28: Floor Queue Position Info (steps 23a2, 28, 34, Table 6.1.1.2.3.2-1; step 2, TS 36.579-1 [2] Table 5.3.16.3-1; step 2, TS 36.579-1 [2] Table 5.3.17.3-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.10-1 condition ON-NETWORK

Table 6.1.1.2.3.3-29: Floor Queue Position Request (step 23a5, Table 6.1.1.2.3.2-1; step 1, TS 36.579-1 [2] Table 5.3.17.3-1)

Derivation Path: 36.579-1 [2], Table 5.5.6.9-1 condition ON-NETWORK