3GPP TSG-RAN5 Meeting #93-e			R5-217991

# Electronic Meeting, 08th Nov– 19th Nov 2021

Title:	LS on Duplicate Measurements when SCell is a Neighbor Cell

Source:	TSG RAN WG5

To:	TSG RAN WG2

Contact Person:

#### Name:	Taran Dhuppad

E-mail Address:	tdhuppad@qti.qualcomm.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

1. Overall Description:

RAN5 thanks RAN2 for the LS response in [1]. However, RAN5 couldn’t reach consensus on whether signalling conformance test cases should mandate SCell measurements to be duplicated in both measResultServingCellMOList and measResultNeighCells, hence would like to reconfirm core spec intention.

TS 38.331 clause 5.5.5.1 contains the following text:

1>	for each serving cell configured with servingCellMO:

2>	if the reportConfig associated with the measId that triggered the measurement reporting includes rsType:

3>	if the serving cell measurements based on the rsType included in the reportConfig that triggered the measurement report are available:

4>	set the measResultServingCell within measResultServingMOList to include RSRP, RSRQ and the available SINR of the serving cell, derived based on the rsType included in the reportConfig that triggered the measurement report;

The above mentioned core spec requirement and the LS response in [1] give the understanding SCell measurements need to be duplicated in both measResultServingCellMOList and measResultNeighCells.

However, views have been expressed in RAN5  that SCell measurements included in the measResultNeighCells are sufficient for event A3/A5, which is primarily used for handover evaluation. Hence signalling conformance test cases covering event A3/A5 shouldn’t mandate duplicate measurements and accept UE implementation including SCell measurements only in measResultNeighCells.

2. Actions:

To RAN WG2 group.

ACTION: RAN5 respectfully asks RAN2 to reconfirm core spec intention is to mandate SCell measurements to be duplicated in both measResultServingCellMOList and measResultNeighCells.

3. Date of Next TSG-RAN WG5 Meetings:

TSG-RAN5 Meeting#94   21st Feb – 4th Mar 2022	Electronic Meeting

TSG-RAN5 Meeting#95   16th – 27th May 2022	Electronic Meeting

4. References

[1] R2-2111473, Reply LS on association between serving cell and measurement object, WG2 Meeting #116