3GPP TSG-RAN5 Meeting #93-e	R5-218261

# Electronic Meeting, 08th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1480 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to LTE SA RRM TCs - NR inter-RAT accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Both test requirements under normal conditions and extreme conditions are given in TC 8.5.2.X. However, in initial condition, only Test enviroment "NC" is given. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Extreme test environments are added to initial test enviroments |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Confusion may be caused. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.2.1.1, 8.5.2.1.2, 8.5.2.2.1, 8.5.2.2.2, 8.5.2.3.1, 8.5.2.3.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from: R5-216867r1. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

8.5.2.1.1	SS-RSRP with NR FR1 target cell

8.5.2.1.1.1	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR1 neighbour cell

8.5.2.1.1.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRP absolute measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.1.1.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements.

8.5.2.1.1.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.1.1.1.

8.5.2.1.1.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.1.1.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.1.1.1.4.1-1.

Table 8.5.2.1.1.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.1.1.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.1.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.1.1.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.1.1.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.1.1.1.4.1-2.

Table 8.5.2.1.1.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | Without faders |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.1.1.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.1.1.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

8.5.2.1.1.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.1.1.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in clause 8.5.2.1.1.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.1.1.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.1.1.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.1.1.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.1.1.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-rsrp | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.1.1.1.5	Test requirement

Table 8.5.2.1.1.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 8.5.2.1.1.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.1.1.1.5-2 for test configurations 1, 2, 4 and 5, and the corresponding absolute accuracy requirements in Table 8.5.2.1.1.1.5-3 for test configurations 3 and 6.

Table 8.5.2.1.1.1.5-1: SS-RSRP inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -94.65 |  | -117 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/15kHz | -96.00 |  | -117 + ΔBG_offset |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -94.65 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -93.00 |  | -114 + ΔBG_offset |  |
|  |  |  | dB | 10 |  | -3.2 |  |
|  |  |  | dB | 10 |  | -3.2 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -84.65 |  | -120.2 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -83.00 |  | -117.2 + ΔBG_offset |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -56.28 |  | -87.35 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -51.53 |  | -81.25 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |

Table 8.5.2.1.1.1.5-2: SS-RSRP Inter RAT absolute accuracy requirements for the reported values for test configurations 1, 2, 4 and 5

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 62 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 30 |
|  |  | Bands NR_FDD_FR1_B | 31 |
|  |  | Bands NR_TDD_FR1_C | 31 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 32 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 32 |
|  |  | Bands NR_FDD_FR1_F | 33 |
|  |  | Bands NR_FDD_FR1_G | 33 |
|  |  | Bands NR_FDD_FR1_H | 34 |
| Highest reported value (Cell 2) | 82 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 43 |
|  |  | Bands NR_FDD_FR1_B | 43 |
|  |  | Bands NR_TDD_FR1_C | 44 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 44 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 45 |
|  |  | Bands NR_FDD_FR1_F | 45 |
|  |  | Bands NR_FDD_FR1_G | 46 |
|  |  | Bands NR_FDD_FR1_H | 46 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 59 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 26 |
|  |  | Bands NR_FDD_FR1_B | 26 |
|  |  | Bands NR_TDD_FR1_C | 27 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 27 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 28 |
|  |  | Bands NR_FDD_FR1_F | 28 |
|  |  | Bands NR_FDD_FR1_G | 29 |
|  |  | Bands NR_FDD_FR1_H | 29 |
| Highest reported value (Cell 2) | 85 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 47 |
|  |  | Bands NR_FDD_FR1_B | 48 |
|  |  | Bands NR_TDD_FR1_C | 48 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 49 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 49 |
|  |  | Bands NR_FDD_FR1_F | 50 |
|  |  | Bands NR_FDD_FR1_G | 50 |
|  |  | Bands NR_FDD_FR1_H | 51 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2. |  |  |  |

Table 8.5.2.1.1.1.5-3: SS-RSRP Inter RAT absolute accuracy requirements for the reported values for test configurations 3 and 6

| Normal Conditions | Test 1 All bands | Test 2 |  |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 64 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 33 |
|  |  | Bands NR_FDD_FR1_B | 34 |
|  |  | Bands NR_TDD_FR1_C | 34 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 35 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 35 |
|  |  | Bands NR_FDD_FR1_F | 36 |
|  |  | Bands NR_FDD_FR1_G | 36 |
|  |  | Bands NR_FDD_FR1_H | 37 |
| Highest reported value (Cell 2) | 83 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 46 |
|  |  | Bands NR_FDD_FR1_B | 46 |
|  |  | Bands NR_TDD_FR1_C | 47 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 47 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 48 |
|  |  | Bands NR_FDD_FR1_F | 48 |
|  |  | Bands NR_FDD_FR1_G | 49 |
|  |  | Bands NR_FDD_FR1_H | 49 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 61 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 29 |
|  |  | Bands NR_FDD_FR1_B | 29 |
|  |  | Bands NR_TDD_FR1_C | 30 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 30 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 31 |
|  |  | Bands NR_FDD_FR1_F | 31 |
|  |  | Bands NR_FDD_FR1_G | 32 |
|  |  | Bands NR_FDD_FR1_H | 32 |
| Highest reported value (Cell 2) | 86 | Bands NR_FDD_FR1_A, NR_TDD_FR1_A | 50 |
|  |  | Bands NR_FDD_FR1_B | 51 |
|  |  | Bands NR_TDD_FR1_C | 51 |
|  |  | Bands NR_FDD_FR1_D, NR_TDD_FR1_D | 52 |
|  |  | Bands NR_FDD_FR1_E, NR_TDD_FR1_E | 52 |
|  |  | Bands NR_FDD_FR1_F | 53 |
|  |  | Bands NR_FDD_FR1_G | 53 |
|  |  | Bands NR_FDD_FR1_H | 54 |
| Note 1: NR operating band groups are defined in clause 3A.4, Table 3A.4.1-2 |  |  |  |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

8.5.2.1.1.2	Void

8.5.2.1.2	E-UTRA SS-RSRP absolute measurement accuracy of a NR FR2 neighbour cell

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

8.5.2.1.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRP measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.1.2.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements.

8.5.2.1.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.1.2.

8.5.2.1.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.1.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.1.2.4.1-1.

Table 8.5.2.1.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.1.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.1.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.1.2.4.1-2.

Table 8.5.2.1.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.1.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.1.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.1.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.1.2.5-1 and Table 8.5.2.1.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRP reported values in the periodic MeasurementReport. The SS-RSRP value of Cell 2 reported by the UE is compared to the expected SS-RSRP. If the value is outside the limits in clause 8.5.2.1.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.1.2.5-1 and 8.5.2.1.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.1.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.1.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.1.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| rsrp | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.1.2.5	Test requirement

Table 8.5.2.1.2.5-1 defines the general test parameters and Table 8.5.2.1.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRP measurement report for each of the tests in Table 8.5.2.1.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.1.2.5-3

Table 8.5.2.1.2.5-1: SS-RSRP Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Void Note 3: Void Note 4: Void |  |  |  |

Table 8.5.2.1.2.5-2: SS-RSRP Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 according to A.3.15.1 | Setup 1 according to A.3.15.1 |
| Assumption for UE beamsNote 9 |  | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105.1 | N/A |
| Note1 | dBm/SCS Note4 | -96.07 | N/A |
|  | dB | 11 | N/A |
| Es | dBm/SCS Note 4 |  | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +6.65dB) |
| SSB_RPNote2 | dBm/SCS Note4 | -85.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +6.65dB) |
| BB Note 7 | dB | 9.95 | 1.84 |
| IoNote2 | dBm/95.04 MHz Note4 | -55.75 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +35.65dB) |
| Note 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone Note 5: Void Note 6: Void Note 7: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [3], and an allowance of 1dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [3] Table 6.2.1.3-4. Note 8: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |

Table 8.5.2.1.2.5-3: evaluation limits for the reported values

| UE power class 3 |  |  |  |
| --- | --- | --- | --- |
| Normal Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 48 | n257, n258, n261 | 29 |
|  |  | n260 | 32 |
|  |  | n259 | 33 |
| Highest reported value (Cell 2) | 105 | n257, n258, n261 | 87 |
|  |  | n260 | 89 |
|  |  | n259 | 90 |
| Extreme Conditions | Test 1 All bands | Test 2 |  |
| Lowest reported value (Cell 2) | 45 | n257, n258, n261 | 26 |
|  |  | n260 | 29 |
|  |  | n259 | 30 |
| Highest reported value (Cell 2) | 108 | n257, n258, n261 | 90 |
|  |  | n260 | 92 |
|  |  | n259 | 93 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

<End of modified section 1>

<Start of modified section 2>

8.5.2.2.1	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR1 neighbour cell

8.5.2.2.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRQ measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.2.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR measurements.

8.5.2.2.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.2.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.2.1.

8.5.2.2.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.2.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.2.1.4.1-1.

Table 8.5.2.2.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.2.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.2.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.2.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.2.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.2.1.4.1-2.

Table 8.5.2.2.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.2.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.2.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

8.5.2.2.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.2.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in clause 8.5.2.2.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.2.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.2.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.2.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.2.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-rsrq | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.2.1.5	Test requirement

Table 8.5.2.2.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.2.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.2.1.5-2.

Table 8.5.2.2.1.5-1: SS-RSRQ inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -81.68 |  | -106 |  | -116 + ΔBG_offset |  |
|  | Config 3,6 |  |  | -87.80 |  | -113 |  | Same as Noc for 15kHz for Config 1,2,4,5 |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -81.68 |  | -106 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -84.80 |  | -110 |  | -113 + ΔBG_offset |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
|  |  |  | dB | -1.75 |  | -1.75 |  | -1.75 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -81.93 |  | -107.75 |  | -117.75 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -85.02 |  | -111.75 |  | -114.75 + ΔBG_offset |  |
| SS-RSRQ Note3 |  |  | dB | -14.77 |  | -14.76 |  | -14.76 |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -50 |  | -75.83 |  | -85.83 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -50 |  | -76.73 |  | -79.73 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

Table 8.5.2.2.1.5-2: SS-RSRQ accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_51 | SS-RSRQ_51 | SS-RSRQ_51 |
| Highest reported value (Cell 2) | SS-RSRQ_63 | SS-RSRQ_63 | SS-RSRQ_63 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-RSRQ_48 | SS-RSRQ_48 | SS-RSRQ_48 |
| Highest reported value (Cell 2) | SS-RSRQ_66 | SS-RSRQ_66 | SS-RSRQ_66 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

8.5.2.2.2	E-UTRA SS-RSRQ absolute measurement accuracy of a NR FR2 neighbour cell

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

8.5.2.2.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-RSRQ measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.2.2.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards and capable of NR FR2 measurements.

8.5.2.2.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.1.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.2.2.

8.5.2.2.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.2.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.2.2.4.1-1.

Table 8.5.2.2.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.2.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.2.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.2.2.4.1-2.

Table 8.5.2.2.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.2.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.2.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.2.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.2.2.5-1 and Table 8.5.2.2.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-RSRQ reported values in the periodic MeasurementReport. The SS-RSRQ value of Cell 2 reported by the UE is compared to the expected SS-RSRQ. If the value is outside the limits in clause 8.5.2.2.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.2.2.5-1 and 8.5.2.2.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.2.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.2.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| rsrq | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.2.2.5	Test requirement

Table 8.5.2.2.2.5-1 defines the general test parameters and Table 8.5.2.2.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.2.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.2.2.5-3

Table 8.5.2.2.2.5-1: SS-RSRQ Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 |
| Duplex mode |  | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |
| DRX cycle configuration | ms | Not applicable |  |
| PDSCH Reference measurement channel |  | - | - |
| RMSI CORESET Reference Channel |  | - | - |
| OCNG Patterns |  | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |

Table 8.5.2.2.2.5-2: SS-RSRQ Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 | Setup 1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak -5dB) (Note 7) |
| Note1 | dBm/SCS Note4 | -95.97 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +4dB) (Note 7) |
|  | dB | -0.5 | 3.55 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.47 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +7.55dB) (Note 8) |
| SS-RSRQNote2 | dB | -14.06 | -12.38 |
| Note2 | dB | -0.5 | 3.55 |
| IoNote2 | dBm/95.04 MHz Note4 | -64.21 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +43.16dB) (Note 9) |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, SS-RSRQ, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Void Note 7: Noc for SCS 15kHz is applied at -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Noc for SCS 120kHz is applied at 4dB above the minimum level specified in Table B.2.3-2 for beam peak. Note 8: SSB_RP is applied at 2.25dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 9: Io is applied at 10log10(792)+6.22dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 10: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation. |  |  |  |

Table 8.5.2.2.2.5-3: evaluation limits for the reported values

| Normal Conditions | Test 1 | Test 2 |
| --- | --- | --- |
| Lowest reported value (Cell 2) | 23 | 26 |
| Highest reported value (Cell 2) | 95 | 98 |
| Extreme Conditions | Test 1 | Test 2 |
| Lowest reported value (Cell 2) | 20 | 23 |
| Highest reported value (Cell 2) | 98 | 101 |

<End of modified section 2>

<Start of modified section 3>

8.5.2.3.1	E-UTRA SS-SINR absolute measurement accuracy of a NR FR1 neighbour cell

8.5.2.3.1.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-SINR measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR1.

8.5.2.3.1.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards supporting ss-SINR-Meas and capable of NR measurements.

8.5.2.3.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.3.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.3.1.

8.5.2.3.1.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR1 neighbour cell.

8.5.2.3.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.3.1.4.1-1.

Table 8.5.2.3.1.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.3.1-1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE FDD |
| 8.5.2.3.1-2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.1-3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.1-4 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode, LTE TDD |
| 8.5.2.3.1-5 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode, LTE TDD |
| 8.5.2.3.1-6 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.3.1.4.1-2.

Table 8.5.2.3.1.4.1-2: Initial conditions

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the selected test configuration. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.1 |
| Connection Diagram | TE Part 2Rx | A.3.1.7.2 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.7.3 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	The general test parameter settings are set up according to Table 8.5.2.3.1.4.1-3.

2.	Message contents are defined in clause 8.5.2.3.1.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

8.5.2.3.1.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.3.1.5-1 as appropriate.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the expected SS- SINR. If the value is outside the limits in clause 8.5.2.3.1.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Table 8.5.2.3.1.5-1 as appropriate and repeat steps 5-7.

8.5.2.3.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.3.1.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.3.1.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| ss-sinr | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.3.1.5	Test requirement

Table 8.5.2.3.1.5-1 defines the primary level settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.3.1.5-1 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.3.1.5-2.

Table 8.5.2.3.1.5-1: SS-SINR inter-RAT test parameters

| Parameter |  |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Cell 2 |  | Cell 2 |  | Cell 2 |  |
| SSB ARFCN |  |  |  | freq1 |  | freq1 |  | freq1 |  |
| Duplex mode |  | Config 1,4 |  | FDD |  |  |  |  |  |
|  |  | Config 2,3,5,6 |  | TDD |  |  |  |  |  |
| TDD configuration |  | Config 1,4 |  | Not Applicable |  |  |  |  |  |
|  |  | Config 2,5 |  | TDDConf.1.1 |  |  |  |  |  |
|  |  | Config 3,6 |  | TDDConf.2.1 |  |  |  |  |  |
| Downlink initial BWP configuration |  |  |  | DLBWP.0.1 |  |  |  |  |  |
| Uplink initial BWP configuration |  |  |  | ULBWP.0.1 |  |  |  |  |  |
| DRX Cycle configuration |  |  | ms | Not Applicable |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| RMSI CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| Dedicated CORESET Reference Channel |  | Config 1,4 |  | - |  | - |  | - |  |
|  |  | Config 2,5 |  |  |  |  |  |  |  |
|  |  | Config 3,6 |  |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |  |  |
| SS-RSSI-Measurement |  |  |  | Not Applicable |  |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  | Config 1,2,4,5 |  | SSB.1 FR1 |  |  |  |  |  |
|  |  | Config 3,6 |  | SSB.2 FR1 |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2,4,5 | kHz | 15 |  |  |  |  |  |
|  |  | Config 3,6 |  | 30 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2,4,5 | Depending on band group | dBm/15kHz | -88 |  | -108.5 |  | -119.5 + ΔBG_offset |  |
| Note2 | Config 1,2,4,5 |  | dBm/SCS | -88 |  | -108.5 |  | Same as Noc for 15kHz |  |
|  | Config 3,6 | Depending on band group |  | -85 |  | -105.5 |  | -116.5 + ΔBG_offset |  |
|  |  |  | dB | -1.75 |  | 20 |  | -3.2 |  |
|  |  |  | dB | -1.75 |  | 20 |  | -3.2 |  |
| SS-RSRPNote3 | Config 1,2,4,5 | Depending on band group | dBm/SCS | -89.75 |  | -88.5 |  | -122.7 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group |  | -86.75 |  | -85.5 |  | -119.7 + ΔBG_offset |  |
| SS-SINR Note3 |  |  | dB | -1.75 |  | 20 |  | -4.0 |  |
| IoNote3 | Config 1,2,4,5 | Depending on band group | dBm/ 9.36MHz | -57.83 |  | -60.5 |  | -89.85 + ΔBG_offset |  |
|  | Config 3,6 | Depending on band group | dBm/ 38.16MHz | -51.73 |  | -54.41 |  | -83.75 + ΔBG_offset |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  |  |
| Antenna configuration |  |  | - | 1x2 |  |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR, SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: NR operating band groups are as defined in clause 3.5.2. Note 6: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |  |  |  |  |

Table 8.5.2.3.1.5-2: SS-SINR accuracy requirements for the reported values

|  | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
|  | All bands | All bands | All bands |
| Normal Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_35 | SS-SINR_79 | SS-SINR_32 |
| Highest reported value (Cell 2) | SS-SINR_51 | SS-SINR_94 | SS-SINR_49 |
| Extreme Conditions |  |  |  |
| Lowest reported value (Cell 2) | SS-SINR_33 | SS-SINR_77 | SS-SINR_31 |
| Highest reported value (Cell 2) | SS-SINR_53 | SS-SINR_96 | SS-SINR_50 |

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

8.5.2.3.2	E-UTRA SS-SINR absolute measurement accuracy of a NR FR2 neighbour cell

Editor’s Note: This test case has been completed for the following configurations:

- Test frequency f ≤ 40.8 GHz

- UE PC3

- Normal conditions

- The test is incomplete for UE power classes other than PC3

- The test is incomplete for test frequencies > 40.8 GHz

- The test is incomplete for extreme conditions

8.5.2.3.2.1	Test purpose

The purpose of this test is to verify that the inter-RAT SS-SINR measurement accuracy is within the specified limits for all bands, when the serving cell is E-UTRA and the target cell is NR FR2.

8.5.2.3.2.2	Test applicability

This test applies to all E-UTRA UE release 15 onwards supporting ss-SINR-Meas and capable of NR FR2 measurements.

8.5.2.3.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 8.5.2.3.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.8.5.2.3.2.

8.5.2.3.2.4	Test description

Two cells are configured in this test: E-UTRA Cell 1 is the E-UTRAN PCell and Cell 2 is the inter-RAT NR FR2 neighbour cell.

8.5.2.3.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 8.5.2.3.2.4.1-1.

Table 8.5.2.3.2.4.1-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 8.5.2.3.2-1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE FDD |
| 8.5.2.3.2-2 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode, LTE TDD |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Configure the test equipment and the DUT according to the parameters in Table 8.5.2.3.2.4.1-2.

Table 8.5.2.3.2.4.1-2: Initial conditions

| Parameter | Value | Comment |
| --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH | As specified in TS 36.508 [25] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.6-2 and TS 38.508-1 [14] clause 4.3.1. |  |
| Channel bandwidth | As specified by the selected test configuration. |  |
| Propagation conditions | AWGN | As specified in Annex C.2.1 |
| Connection Diagram | TE Part: A.3.3.3.1 | As specified in TS 38.508-1 [14] Annex A. |
|  | UE Part: A.3.4.1.1 |  |
| Exceptions to connection diagram | N/A |  |

1.	The general test parameter settings are set up according to Table 8.5.2.3.2.4.1-3.

2.	Message contents are defined in clause 8.5.2.3.2.4.3.

3.	There are two carriers and two cells specified in the test, where E-UTRA Cell 1 is the E-UTRA PCell on the E-UTRA carrier and Cell 2 is the NR neighbour cell on the NR carrier. E-UTRA Cell 1 is configured according to TS 36.521-3 [26] Annex C.1.0 and C.1.1.

4.	The UE Rx beam peak direction has been obtained previously using one of the Rx beam peak search procedures as described in Annex I.

8.5.2.3.2.4.2	Test procedure

1.	Ensure the UE is in State 3A-RF according to TS 36.508 [25] clause 7.2A.3.

2.	Set the parameters according to Table 8.5.2.3.2.5-1 and Table 8.5.2.3.2.5-2 as appropriate. The TE shall ensure that the NR FR2 cell will be received by the UE from the Rx beam peak direction.

3.	The SS shall transmit an RRCConnectionReconfiguration message on Cell 1 to configure inter-RAT NR neighbour cell periodical measurements.

4.	The UE shall transmit an RRCConnectionReconfigurationComplete message.

5.	The UE shall transmit periodically MeasurementReport messages.

6.	After 10s wait from Step 3, the SS shall check the SS-SINR reported values in the periodic MeasurementReport. The SS- SINR value of Cell 2 reported by the UE is compared to the expected SS- SINR. If the value is outside the limits in clause 8.5.2.3.2.5 or the UE fails to report the measurement value for Cell 2, the number of failed iterations is increased by one. Otherwise, the number of passed iterations is increased by one.

7.	The SS shall continue checking the MeasurementReport messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

8.	Set the parameters according to each sub-test in Tables 8.5.2.3.2.5-1 and 8.5.2.3.2.5-2 as appropriate and repeat steps 5-7.

8.5.2.3.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 8.5.2.3.2.4.3-1: Common Exception messages

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.4-7 Table 4.6.6-2B in TS 36.508 [25] |

Table 8.5.2.3.2.4.3-2: ReportConfigInterRAT-DEFAULT(Periodical)

| Derivation Path: 36.508 [25] Table 4.6.6-9 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ReportConfigNR::= SEQUENCE { |  |  |  |
| reportType CHOICE { |  |  |  |
| periodical SEQUENCE { |  |  |  |
| ReportQuantityNR-r15 SEQUENCE { | PRESENT |  |  |
| sinr | true |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

8.5.2.3.2.5	Test requirement

Table 8.5.2.3.2.5-1 defines the general test parameters and Table 8.5.2.3.2.5-2 defines the primary level OTA settings including test tolerances for all tests.

Each SS-RSRQ measurement report for each of the tests in Table 8.5.2.3.2.5-2 shall meet the corresponding absolute accuracy requirements in Table 8.5.2.3.2.5-3.

Table 8.5.2.3.2.5-1: SS-SINR Inter-RAT general test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| SSB ARFCN |  | Freq1 | freq1 | freq1 |
| Duplex mode |  | TDD | TDD | TDD |
| TDD configuration |  | TDDConf.3.1 | TDDConf.3.1 | TDDConf.3.1 |
| BWchannel | MHz | 100: NRB,c = 66 | 100: NRB,c = 66 | 100: NRB,c = 66 |
| Downlink initial BWP configuration |  | DLBWP.0.1 |  |  |
| Uplink initial BWP configuration |  | ULBWP.0.1 |  |  |
| DRX cycle configuration | ms | Not applicable |  |  |
| PDSCH Reference measurement channel |  | - | - | - |
| RMSI CORESET Reference Channel |  | - | - | - |
| OCNG Patterns |  | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  | SMTC.1 | SMTC.1 | SMTC.1 |
| SSB configuration |  | SSB.3 FR2 | SSB.3 FR2 | SSB.3 FR2 |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 | 120 | 120 |
| EPRE ratio of PSS to SSS | dB | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-SINR, SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-SINR and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |

Table 8.5.2.3.2.5-2: SS-SINR Inter-RAT OTA related test parameters

| Parameter | Unit | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- | --- |
|  |  | Cell 2 | Cell 2 | Cell 2 |
| Angle of arrival configuration |  | Setup 1 | Setup 1 | Setup 1 |
| Assumption for UE beamsNote 10 |  | Rough | Rough | Rough |
| Note1 | dBm/15kHz Note4 | -105 | -105.1 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak -5dB) (Note 7) |
| Note1 | dBm/SCS Note4 | -95.97 | -96.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +4dB) (Note 7) |
|  | dB | -0.5 | 11 | 3.7 |
| SSB_RPNote2 | dBm/SCS Note4 | -96.47 | -85.07 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +7.6dB) (Note 8) |
| SS-SINRNote2 | dB | -0.5 | 11 | 3.6 |
| Note2 | dB | -0.5 | 11 | 3.6 |
| IoNote2 | dBm/95.04 MHz Note4 | -64.21 | -55.75 | (Table B.2.3-2 of TS 38.133 [6] Rx Beam Peak +43.19dB) (Note 9) |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SSB_RP, SS-SINR, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: Void Note 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone. Note 5: Void Note 6: Void Note 7: Noc for SCS 15kHz is applied at -10log10(8)+4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Noc for SCS 120kHz is applied at 4dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 8: SSB_RP is applied at 3dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 9: Io is applied at level 10log10(792)+6.54dB above the minimum level specified in Table B.2.3-2 of TS 38.133 [6] for beam peak. Note 10: Information about types of UE beam is given in B.2.1.3 of TS 38.133 [6], and does not limit UE implementation or test system implementation. |  |  |  |  |

Table 8.5.2.3.2.5-3: evaluation limits for the reported values

| Normal Conditions | Test 1 | Test 2 | Test 3 |
| --- | --- | --- | --- |
| Lowest reported value (Cell 2) | 9 | 32 | 17 |
| Highest reported value (Cell 2) | 82 | 105 | 90 |
| Extreme Conditions | Test 1 | Test 2 | Test 3 |
| Lowest reported value (Cell 2) | 7 | 30 | 15 |
| Highest reported value (Cell 2) | 84 | 107 | 92 |

<End of modified section 3>