3GPP TSG-RAN5 Meeting #93-e	R5-218317

# Electronic Meeting, 08th – 19th Nov. 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1525 | rev | 1 | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 6.6.8.2 NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The test case 6.6.8.2 was missing in TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding the test case 6.6.8.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The WP would not be completed. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.6.8.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 CR 1523 1528 1530 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-217209r1 |  |  |  |  |  |  |  |  |

<Start of Changes>

6.6.8.2	NR SA FR1 SSB based CMR and dedicated IMR L1-SINR measurement in non-DRX

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test requirement is FFS.

-	MU/TT is FFS.

-	Connection Diagram is TBD since the antenna configuration is not determined in TS 38.133.

6.6.8.2.1	Test purpose

To verify that the UE makes correct reporting of L1-SINR measurement in non-DRX within L1-SINR measurement requirements in TS 38.133 [6] clause 9.8.4.2.

6.6.8.2.2	Test applicability

This test applies to all types of NR UE release 16 and forward.

6.6.8.2.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 6.6.8.0.2.

The normative reference for this requirement is TS 38.133 [6] clause A.6.6.8.2.

6.6.8.2.4	Test description

6.6.8.2.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 6.6.8.2.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 6.6.8.2.4.1-2. Test environment parameters are given in Table 6.6.8.2.4.1-3.

Table 6.6.8.2.4.1-1: NR SA SSB based CMR and dedicated IMR L1-SINR measurement supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table 6.6.8.2.4.1-2: General test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB GSCN | 1~3 |  | freq1 |
| Duplex mode | 1 |  | FDD |
|  | 2 |  | TDD |
|  | 3 |  | TDD |
| TDD Configuration | 1 |  | N/A |
|  | 2 |  | TDDConf.1.1 |
|  | 3 |  | TDDConf.2.1 |
| BWchannel | 1 | MHz | 10: NRB,c = 52 |
|  | 2 |  | 10: NRB,c = 52 |
|  | 3 |  | 40: NRB,c = 106 |
| PDSCH Reference measurement channel | 1 |  | SR.1.1 FDD |
|  | 2 |  | SR.1.1 TDD |
|  | 3 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.1.1 FDD |
|  | 2 |  | CR.1.1 TDD |
|  | 3 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.1.1 FDD |
|  | 2 |  | CCR.1.1 TDD |
|  | 3 |  | CCR.2.1 TDD |
| SSB configuration | c |  | SSB.3 FR1 |
|  | 2 |  | SSB.3 FR1 |
|  | 3 |  | SSB.4 FR1 |
| CSI-RS configuration | 1 |  | CSI-RS 1.1A FDD |
|  | 2 |  | CSI-RS 1.1A TDD |
|  | 3 |  | CSI-RS 2.1A TDD |
| OCNG Patterns | 1~3 |  | OP.1 |
| Initial BWP Configuration | 1~3 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1~3 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1~3 |  | SMTC.1 |
| TRS Configuration | 1 |  | TRS.1.1 FDD |
|  | 2 |  | TRS.1.1 TDD |
|  | 3 |  | TRS.1.2 TDD |
| DRX configuration | 1~3 |  | off |
| reportConfigType | 1~3 |  | periodic |
| reportQuantity-r16 | 1~3 |  | ssb-Index-SINR-r16 |
| Number of reported RS | 1~3 |  | 2 |
| L1-SINR reporting period | 1~3 | slot | 80 |
| T1 | 1~3 | s | 5 |
| T2 | 1~3 | s | 1 |
| EPRE ratio of PSS to SSS | 1~3 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1~3 |  | AWGN |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |

Table 6.6.8.2.4.1-3: Test Environment parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1 and 4.4.2. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 6.6.8.2.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part | TBD | As specified in TS 38.508-1 [14] Annex A. |
|  | DUT Part | TBD |  |
| Exceptions to connection diagram | For 4Rx capable UEs without any 2 Rx RF bands use A.3.2.5.2 for DUT part and A.3.1.8.4 for TE Part |  |  |

1.	Message contents are defined in clause 6.6.8.2.4.3.

2.	Single Cell is used, which is NR FR1 Pcell. The connection setup is done according to the settings in Annex C.1.2 and C.1.3. The test parameters are given in tables 6.6.8.2.4.1-2, 6.6.8.2.5-1 and 6.6.8.2.5-2. UE is configured to perform RLM and BFD based on the SSBs.

6.6.8.2.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above Table 6.6.8.2.4.1-2. Before the test, UE is configured to perform RLM, BFD and L1-SINR measurement based on the SSBs.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 6.6.8.2.4.1-2.

2.	Set the parameters according to T1 in Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2. T1 starts. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

3.	The UE shall be transmitting CSI on PUCCH with a periodicity of 80 slots.

4.	When T1 expires, the SS shall set the parameters according to T2 in Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2. T2 starts. SS transmits CSI-RS as IMR with a periodicity of 20 slots.

5.	If the UE sends L1-SINR reports meeting the corresponding absolute accuracy requirements in Table 6.6.8.2.5-3 for test configurations 1 and 2 the corresponding absolute accuracy requirements in Table 6.6.8.2.5-4 for test configurations 3 and the corresponding relative accuracy requirements in Table 6.6.8.2.5-5 for all test configurations every 80 slots from no later than 720 ms for configuration 1 and 2 and no later than 680 ms for configuration 3 from the beginning of time period T2 until the end of time period T2, the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

6.	The SS waits until T2 expires.

7.	The SS shall transmit RRCRelease message to release the RRC connection which includes the release of the established radio bearers as well as all radio resources.

8. After the RRC connection release, the SS:

- transmits in Cell 1 a Paging message (including PagingRecord with ue-Identity) for the UE and ensures the UE in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5. (if the paging fails, switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.),
or:
- switches off and on the UE and ensures the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity NR SA, Connected without release On and Test Mode On according to TS 38.508-1 [14] clause 4.5.

9.	Repeat steps 2-8 until the confidence level according to Tables G.2.3-1 in Annex G clause G.2 is achieved.

6.6.8.2.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 6.6.8.2.4.3-1: Common Exception messages NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions PERIODIC and SS-SINR and CSI-RS_IMR Table H.3.6A-2 with conditions SSB and PERIODIC Table H.3.6A-3 with condition PERIODIC |

Table 6.6.8.2.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| detectionResource CHOICE { |  |  |  |
| ssb-Index | 0 |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

6.6.8.2.5	Test requirement

Table 6.6.8.2.5-1 and Table 6.6.8.2.5-2 define the primary level settings including test tolerances for all tests.

Table 6.6.8.2.5-1: SSB specific test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | SSB#0 |  | SSB#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65+TT |  |  |  |
| Note2 | 1,2 | dBm/SSB SCS | -94.65+TT |  |  |  |
|  | 3 |  | -91.65+TT |  |  |  |
|  | 1~3 | dB | 0+TT | 0+TT | -Infinity | 3+TT |
| SSB RSRP Note3 | 1,2 | dBm/SSB SCS | -94.65+TT | -94.65+TT | -Infinity | -91.65+TT |
|  | 3 |  | -91.65+TT | -91.65+TT | -Infinity | -88.65+TT |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69+TT | -63.69+TT | -66.70+TT | -61.93+TT |
|  | 3 | dBm/38.16 MHz | -57.59+TT | -57.59+TT | -60.61+TT | -55.84+TT |
|  | 1~3 | dB | 0+TT | 0+TT | -Infinity | 3+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

Table 6.6.8.2.5-2: CSI-RS specific test parameters for NR SA SSB based CMR and dedicated IMR L1-SINR measurement

| Parameter | Config | Unit | CSI-RS#0 |  | CSI-RS#1 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| Note2 | 1~3 | dBm/15kHz | -94.65+TT |  |  |  |
| Note2 | 1,2 | dBm/CSI-RS SCS | -94.65+TT |  |  |  |
|  | 3 |  | -91.65+TT |  |  |  |
|  | 1~3 | dB | 0+TT | 0+TT | -Infinity | 3+TT |
| CSI-RS RSRP Note3 | 1,2 | dBm/CSI-RS SCS | -94.65+TT | -94.65+TT | -Infinity | -91.65+TT |
|  | 3 |  | -91.65+TT | -91.65+TT | -Infinity | -88.65+TT |
| Io Note3 | 1,2 | dBm/9.36 MHz | -63.69+TT | -63.69+TT | -66.70+TT | -61.93+TT |
|  | 3 | dBm/38.16 MHz | -57.59+TT | -57.59+TT | -60.61+TT | -55.84+TT |
|  | 1~3 | dB | 0+TT | 0+TT | -Infinity | 3+TT |
| Note 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |

The UE shall send L1-SINR report every 80 slots. No later than 640ms plus 80 slots from the beginning of time period T2, UE shall send L1-SINR report including results of both SSB#0+CSI-RS#0 and SSB#1+CSI-RS#1. Each L1-SINR measurement report shall meet the corresponding absolute accuracy requirements in Table 6.6.8.2.5-3 for test configurations 1 and 2 and the corresponding absolute accuracy requirements in Table 6.6.8.2.5-4 for test configuration 3 and the corresponding relative accuracy requirements in Table 6.6.8.2.5-4 for all test configurations.

Table 6.6.8.2.5-3: L1-SINR absolute accuracy requirements for the reported values for test configurations 1 and 2

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | TBD |
| Highest reported value (SSB#1) | - | TBD |

Table 6.6.8.2.5-4: L1-SINR absolute accuracy requirements for the reported values for test configurations 3

| Normal Conditions | T1 | T2 |
| --- | --- | --- |
| Lowest reported value (SSB#1) | - | TBD |
| Highest reported value (SSB#1) | - | TBD |

Table 6.6.8.2.5-5: L1-SINR relative accuracy requirements for the reported values for all test configurations

|  | T1 | T2 |
| --- | --- | --- |
| Lowest DIFF SINR reported (SSB#0) | - | TBD |
| Highest DIFF SINR reported (SSB#0) | - | TBD |

The rate of correct events observed during repeated tests shall be at least 90% with a confidence level of 95%.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

<End of Changes>