3GPP TSG-RAN5 Meeting #93-e	R5-218378

# Electronic meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.905 | CR | 0243 | rev | 1 | Current version: | 16.7.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of A-MPR TP analysis for NS_56 (B24) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_B24_mod-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The A-MPR test point analysis for NS_56 is missing in TR 36.905 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the attached zip file “TpAnalysisAMPR_6.2.4(NS_56_PC3)’’ to TR 36.905 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The analysis to derive the Test point for NS_56 not finalised |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Added .zip file into TR 36.905 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision (include change to spec and impcated clause in the coversheet) |  |  |  |  |  |  |  |  |

-------- Start of changes --------

### 4.1.1	A-MPR test case for single carrier

This section contains information on test point selection for test case 6.2.4 in [2], Additional Maximum Power Reduction (A-MPR).

Test points in this test were added in the past, and no selection information is therefore available prior the NS values in table 4.1.1-1.

Selection of test points should include some possible worst combinations based on the A-MPR characteristics specified for each NS value and these shall be selected so that they match with corresponding spectrum emission requirements test points. The number of test points should be realistic.

Table 4.1.1-1: NS value specific test points for A-MPR

| NS value | Justification | Comments |
| --- | --- | --- |
| NS_05 | See attachment “TpAnalysisAMPR(NS_05)_6.2.4_v2.zip” | Added at RAN5#73 |
| NS_24 | See attachment “TpAnalysisAMPR_6.2.4(NS_24 and NS_25).zip” | Added at RAN5#72 |
| NS_25 | See attachment “TpAnalysisAMPR_6.2.4(NS_24 and NS_25).zip” | Added at RAN5#72 |
| NS_27 | See attachment “TpAnalysisAMPR_6.2.4(NS_27)_v3.zip” | Added at RAN5#80 |
| NS_04 (Power Class 2) | See attachment “TpAnalysisAMPR_6.2.4_1(NS_04_PC2).zip” | Added at RAN5#74 |
| NS_04 (Power Class 3) | See attachment “TpAnalysisAMPR_6.2.4(NS_04_PC3).zip” | Added at RAN5#75 |
| NS_22 | See attachment “TpAnalysisAMPR_6.2.4(NS_22 and NS_23).zip” | Added at RAN5#75 |
| NS_23 | See attachment “TpAnalysisAMPR_6.2.4(NS_22 and NS_23).zip” | Added at RAN5#75 |
| NS_56 | See attachment “TpAnalysisAMPR_6.2.4(NS_56_PC3).zip” | Added at RAN5#93-e |

The analyses for UE category M1 and category M2 are performed per NS-value in table 4.1.1-2 for category M1, table 4.1.1-2a for category M1 with subPRB allocation and in table 4.1.1-3 for category M2. The general principles for selection of test points are:

-	For Additional spurious emissions, requirements are the same for all bandwidths. Since channel bandwidth is not judged to have any impact on the UE transmit signal for category M1, only one bandwidth needs to be tested. The lowest bandwidth with emissions requirement has lowest guard band and is therefore the most stringent. For category M2 the same applies, with the exception that channel bandwidth 5 MHz is always additionally selected, even if this is not the smallest bandwidth. Because maximum RB allocation of 24 resource blocks requires at least 5 MHz channel bandwidth.

-	For NS-values with Additional SEM requirement and without any allowed A-MPR, same test points as in SEM test case 6.6.2.1EA for category M1 and same test points as in SEM test case 6.6.2.1EC for category M2 can be used.

-	For NS-values with Additional Spurious requirement and without any allowed A-MPR, same test points as in spurious test case 6.6.3EA.1 for category M1 and same test points as in spurious test case 6.6.3EC.1 for category M2 can be used

-	For Additional SEM, test frequency is selected as in SEM test case.

-	For Additional spurious emissions, test frequency selection depends on if protected range is above or below the UE transmit signal in frequency. If protected range is above UE transmit frequency, then High frequency range is selected. If protected range is below UE transmit frequency, then Low frequency range is selected.

-------- End of changes --------

## Add the following attached .zip file to TR 36.905:

## “TpAnalysisAMPR_6.2.4(NS_56_PC3)”