3GPP TSG-RAN5 Meeting #93-e	R5-218404

# Electronic Meeting, November 8 – November 19, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1396 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | A-MPR updates for Table 6.2.3.3.1-1 for new NS value for 3450-3550 MHz frequency range of n77 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RF_FR1-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-11 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | A-MPR updates for Table 6.2.3.3.1-1 for new NS value for 3450-3550 MHz frequency range of n77 |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated general section of minimum conformance requirements for A-MPR. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | NS_55 not mapped correctly |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.3.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1 WIC is corrected to NR_RF_FR1-UEConTest |  |  |  |  |  |  |  |  |

### <Start of Changes>

### 6.2.3	UE additional maximum output power reduction

Editor’s note: The following aspects are either missing or not yet determined:

- Tests for network signalling values NS_07, NS_40, NS_09, NS_56 not complete.

- PC1 requirements for NR operating bands other than Band n14 are not fully defined in RAN4 Rel-15 and Rel-16 specifications due to the lack of MPR requirements.

- The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_44, NS_46, NS_47, NS_48, and NS_49 to all types of NR Power Class 2 and 3 UE release 16 forward, and UE release 15 if the corresponding channel bandwidths are supported.

#### 6.2.3.1	Test purpose

Additional emission requirements can be signalled by the network. Each additional emission requirement is associated with a unique network signalling (NS) value indicated in RRC signalling by an NR frequency band number of the applicable operating band and an associated value in the field additionalSpectrumEmission. Throughout this specification, the notion of indication or signalling of an NS value refers to the corresponding indication of an NR frequency band number of the applicable operating band, the IE field freqBandIndicatorNR and an associated value of additionalSpectrumEmission in the relevant RRC information elements [6].

To meet the additional requirements, additional maximum power reduction (A-MPR) is allowed for the maximum output power as specified in Table 6.2.1.3-1. Unless stated otherwise, the total reduction to UE maximum output power is max(MPR, A-MPR) where MPR is defined in clause 6.2.2. Outer and inner allocation notation used in clause 6.2.3 is defined in clause 6.2.2. In absence of modulation and waveform types the A-MPR applies to all modulation and waveform types.

#### 6.2.3.2	Test applicability

The requirements of this test apply in test case 6.5.2.3 Additional Spectrum Emission mask for network signalling values NS_03, NS_03U, NS_04, NS_06, and NS_35 to all types of NR UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

The requirements of this test apply in test case 6.5.2.4 Adjacent channel leakage ratio for network signalling values NS_03U, NS_05U, NS_43U and NS_100 to all types of NR Power Class 2 and 3 UE release 15 and forward.

The requirements of this test apply in test case 6.5.3.3 Additional Spurious Emissions for network signalling values NS_04, NS_05, NS_05U, NS_12, NS_13, NS_14, NS_15, NS_17, NS_18, NS_21, NS_37, NS_38, NS_39, NS_40, NS_41, NS_42, NS_43, NS_43U, NS_44, NS_45, NS_48, NS_49, NS_50 and NS_56 to all types of NR Power Class 2 and 3 UE release 15 and forward.

#### 6.2.3.3	Minimum conformance requirements

##### 6.2.3.3.1	General

Table 6.2.3.3.1-1 specifies the additional requirements with their associated network signalling values and the allowed A-MPR and applicable operating band(s) for each NS value. In case of a power class 3 UE, when IE powerBoostPi2BPSK is set to 1, power class 2 A-MPR values apply. The mapping of NR frequency band numbers and values of the additionalSpectrumEmission to network signalling labels is specified in Table 6.2.3.3.1-1A.

For almost contiguous allocations in CP-OFDM waveforms in power class 3, the allowed A-MPR defined in clause 6.2.3 is increased by CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB, where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs, and the parameter LCRB is replaced by NRB_alloc + NRB_gap in specifying the RB allocation regions.

Unless otherwise specified, pi/2 BPSK in following A-MPR tables refers to both variants of pi/2 BPSK referenced in 6.2.2 Table 6.2.2.3-1.

Table 6.2.3.3.1-1: Additional maximum power reduction (A-MPR)

| Network signalling label | Requirements (subclause) | NR Band | Channel bandwidth (MHz) | Resources blocks (NRB) | A-MPR (dB) |
| --- | --- | --- | --- | --- | --- |
| NS_01 |  | Table 5.2-1 | 5, 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 | Table 5.3.2-1 | N/A |
| NS_03 | 6.5.2.3.3.3 | n2, n25, n66, n70, n86 |  |  | Clause 6.2.3.3.7 |
| NS_03U | 6.5.2.3.3.3, 6.5.2.4.2.3 | n2, n25, n66, n86 |  |  | Clause 6.2.3.3.7 |
| NS_04 | 6.5.2.3.3.2, 6.5.3.3.3.1 | n41 | 10, 15, 20, 30, 40, 50, 60, 70, 80, 90, 100 |  | Clause 6.2.3.3.2 |
| NS_05 | 6.5.3.3.3.4 | n1, n65, n84 | 5, 10, 15, 20 (Note 2) |  | Clause 6.2.3.3.4 |
| NS_05U | 6.5.3.3.3.4, 6.5.2.4.2.3 | n1, n65, n84 | 5, 10, 15, 20 |  | Clause 6.2.3.3.4 |
| NS_06 | 6.5.2.3.3.4 | n12 | 5, 10, 15 |  | N/A |
|  |  | n14 | 5, 10 |  |  |
| NS_10 |  | n20 | 15, 20 | Table 6.2.3.3.3-1 | Table 6.2.3.3.3-1 |
| NS_12 | 6.5.3.3.17 | n26 | 5, 10 | Table 6.2.3.3.21-1 | Table 6.2.3.3.21-2 |
| NS_13 | 6.5.3.3.18 | n26 | 5 | Table 6.2.3.3.22-1 | Table 6.2.3.3.22-2 |
| NS_14 | 6.5.3.3.19 | n26 | 10, 15, 20 | Table 6.2.3.3.23-1 | Table 6.2.3.3.23-2 |
| NS_15 | 6.5.3.3.20 | n26 | 5, 10, 15, 20 | Table 6.2.3.3.24-1 | Table 6.2.3.3.24-2 |
| NS_17 | 6.5.3.3.3.2 | n28, n83 | 5, 10 | Table 5.3.2-1 | N/A |
| NS_18 | 6.5.3.3.3.3 | n28, n83 | 5 |  | Table 6.2.3.3.13-1, A1 |
|  |  |  | 10, 15, 20 |  | Table 6.2.3.3.13-1, A2 |
|  |  |  | 30 |  | Table 6.2.3.3.13-1, A3, A4, A5 |
| NS_24 | 6.5.3.3.3.13 | n65 (Note 4) | 5, 10, 15, 20 | Table 6.2.3.3.15-1 | Subclause 6.2.3.3.15 |
| NS_27 | 6.5.2.3.3.8 6.5.3.3.3.14 | n48 | 5, 10, 15, 20, 40 | Table 6.2.3.3.16-1 | Table 6.2.3.3.16-2 |
| NS_35 | 6.5.2.3.3.1 | n71 | 5, 10, 15, 20 | Table 5.3.2-1 | N/A |
| NS_37 | 6.5.3.3.3.6 | n74 (Note 3) | 10, 15 | Table 6.2.3.3.8-1 | Table 6.2.3.3.8-1 |
| NS_38 | 6.5.3.3.3.7 | n74 | 5, 10, 15, 20 | Table 6.2.3.3.9-1 | Table 6.2.3.3.9-1 |
| NS_39 | 6.5.3.3.3.8 | n74 | 10, 15, 20 | Table 6.2.3.3.10-1 | Table 6.2.3.3.10-1 |
| NS_40 | 6.5.3.3.3.9 | n51 | 5 |  | Table 6.2.3.3.5-1 |
| NS_41 | 6.5.3.3.3.10 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.11-1 |
| NS_42 | 6.5.3.3.3.11 | n50 | 5, 10, 15, 20, 40, 50, 60 |  | Table 6.2.3.3.12-1 |
| NS_43 | 6.5.3.3.3.5 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_43U | 6.5.3.3.3.5, 6.5.2.4.2.3 | n8, n81 | 5, 10, 15 |  | Clause 6.2.3.3.6 |
| NS_44 | 6.5.3.3.24 | n38 | 25,30,40 | Table 6.2.3.3.20-1 | Table 6.2.3.3.20-1 |
| NS_45 | 6.5.3.3.3.21 | n53 | 5, 10 |  | Clause 6.2.3.3.25 |
| NS_46 | 6.5.3.3.3.25 | n7 | 25, 30, 40, 50 | Table 6.2.3.3.17-1 | Table 6.2.3.3.17-2 |
| NS_47 | 6.5.3.3.3.15 | n41 (Note 5) | 30 | Table 6.2.3.3.18-1 | Table 6.2.3.3.18-2 |
| NS_48 | 6.5.3.3.3.22 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.26-1 | Table 6.2.3.3.26-1 |
| NS_49 | 6.5.3.3.3.23 | n1 and n84 | 25, 30, 40, 50 | Table 6.2.3.3.27-1 | Table 6.2.3.3.27-1 |
| NS_50 | 6.5.3.3.3.16 | n39 | 25, 30, 40 |  | Clause 6.2.3.3.19 |
| NS_55 | NOTE 6 | n77 | 10, 15, 20, 25, 30, 40, 50, 60, 70, 80, 90, 100 |  | N/A |
| NS_56 | 6.5.3.3.3.25 | n24, n99 | 5, 10 | Table 6.2.3.3.28 | Clause 6.2.3.3.28 |
| NS_100 | 6.5.2.4.2.3 | n1, n2, n3, n5, n8, n25, n26, n66, n80, n81, n84, n86 (Note 1) |  |  | Table 6.2.3.3.1-2 |
| NOTE 1: This NS can be signalled for NR bands that have UTRA services deployed. NOTE 2: No A-MPR is applied for 5 MHz BWchannel where the lower channel edge is ≥1930 MHz,10 MHz BWchannel where the lower channel edge is ≥1950 MHz and 15 MHz BWchannel where the lower channel edge is ≥1955 MHz. NOTE 3: Applicable when the NR carrier is within 1447.9 – 1462.9 MHz. NOTE 4: Applicable when the upper edge of the channel bandwidth frequency is greater than 1980 MHz. NOTE 5: Applicable when the NR carrier is within 2545 – 2575 MHz. NOTE 6: This NS value is applicable for cells in the range 3450 – 3550 MHz for operations in the USA. This NS value does not indicate any additional spurious emission and maximum output power reduction requirements. |  |  |  |  |  |

Table 6.2.3.3.1-1A: Mapping of Network Signalling label

| NR band | Value of additionalSpectrumEmission |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| n1 | NS_01 | NS_100 | NS_05 | NS_05U | NS_48 | NS_49 |  |  |
| n2 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n3 | NS_01 | NS_100 |  |  |  |  |  |  |
| n5 | NS_01 | NS_100 |  |  |  |  |  |  |
| n7 | NS_01 | NS_46 |  |  |  |  |  |  |
| n8 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n12 | NS_01 | NS_06 |  |  |  |  |  |  |
| n14 | NS_01 | NS_06 |  |  |  |  |  |  |
| n20 | NS_01 | Void | NS_10 |  |  |  |  |  |
| n24 | NS_01 | NS_56 |  |  |  |  |  |  |
| n25 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n26 | NS_01 | NS_100 | NS_12 | NS_13 | NS_14 | NS_15 |  |  |
| n28 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n30 | NS_01 | NS_21 |  |  |  |  |  |  |
| n34 | NS_01 |  |  |  |  |  |  |  |
| n38 | NS_01 | NS_44 |  |  |  |  |  |  |
| n39 | NS_01 | NS_50 |  |  |  |  |  |  |
| n40 | NS_01 |  |  |  |  |  |  |  |
| n41 | NS_01 | NS_04 | NS_47 |  |  |  |  |  |
| n48 | NS_01 | NS_27 |  |  |  |  |  |  |
| n50 | NS_01 | NS_41 | NS_42 |  |  |  |  |  |
| n51 | NS_01 | NS_40 |  |  |  |  |  |  |
| n53 | NS_01 | NS_45 |  |  |  |  |  |  |
| n65 | NS_01 | NS_24 | NS_100 | NS_05 | NS_05U |  |  |  |
| n66 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n70 | NS_01 | NS_03 |  |  |  |  |  |  |
| n71 | NS_01 | NS_35 |  |  |  |  |  |  |
| n74 | NS_01 | NS_37 | NS_38 | NS_39 |  |  |  |  |
| n77 | NS_01 | NS_55 |  |  |  |  |  |  |
| n78 | NS_01 |  |  |  |  |  |  |  |
| n79 | NS_01 |  |  |  |  |  |  |  |
| n80 | NS_01 | NS_100 |  |  |  |  |  |  |
| n81 | NS_01 | NS_100 | NS_43 | NS_43U |  |  |  |  |
| n82 | NS_01 | Void |  |  |  |  |  |  |
| n83 | NS_01 | NS_17 | NS_18 |  |  |  |  |  |
| n84 | NS_01 | NS_100 | NS_05 | NS_05U |  |  |  |  |
| n86 | NS_01 | NS_100 | NS_03 | NS_03U |  |  |  |  |
| n95 | NS_01 |  |  |  |  |  |  |  |
| n99 | NS_01 | NS_56 |  |  |  |  |  |  |
| NOTE: additionalSpectrumEmission corresponds to an information element of the same name defined in clause 6.3.2 of TS 38.331 [6]. |  |  |  |  |  |  |  |  |

Table 6.2.3.3.1-2: A-MPR for NS_100 (UTRA protection)

| Modulation/Waveform |  | Outer (dB) |
| --- | --- | --- |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 2 |
|  | QPSK | ≤ 2 |
|  | 16 QAM | ≤ 2.5 |
|  | 64 QAM | ≤ 3 |
|  | 256 QAM | ≤ 4.5 |
| CP-OFDM | QPSK | ≤ 4 |
|  | 16 QAM | ≤ 4 |
|  | 64 QAM | ≤ 4 |
|  | 256 QAM | ≤ 6.5 |

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.3.1.

### <End of Changes>