3GPP TSG-RAN5 Meeting #93-e	R5-218414

# Electronic Meeting, 8th – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2166 | rev | 1 | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to demod parameter CSI-RS-ResourceMapping to add 1Tx condition |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_L1enh_URLLC -UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Demod test cases with 1Tx are specified by RAN4, while the default configuration in CSI-RS-ResourceMapping is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding default configuration for 1Tx in CSI-RS-ResourceMapping. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | 1Tx test cases can’t be performed appropriately |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.4.2.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revised from R5-218206 |  |  |  |  |  |  |  |  |

<Unchanged Text Skipped>

5.4.2.4	Message contents for CQI reporting requirements

<Unchanged Text Skipped>

NZP CSI-RS for CSI Acquisition

CSI-RS-ResourceMapping

Table 5.4.2.4-2: CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 001000 | K0 = 6, row3, | NOT 1Tx test cases |
| row1 | 0001 | K0 = 0, row1 | 1Tx test cases |
| } |  |  |  |
| nrofPorts | P2 |  | NOT 1Tx test cases |
|  | P1 |  | 1Tx test cases |
| firstOFDMSymbolInTimeDomain | 13 | I0 = 13 |  |
| cdm-Type | fd-CDM2 |  | NOT 1Tx test cases |
|  | noCDM |  | 1Tx test cases |
| density CHOICE { |  |  |  |
| one | NULL |  | NOT 1Tx test cases |
| three | NULL |  | 1Tx test cases |
| } |  |  |  |
| freqBand | CSI-FrequencyOccupation |  |  |
| } |  |  |  |

CSI-ResourcePeriodicityAndOffset

Table 5.4.2.4-2a: NZP CSI-ResourcePeriodicityAndOffset

| Derivation Path: Table 4.6.3-43 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-ResourcePeriodicityAndOffset ::= CHOICE { |  |  |  |
| slots5 | 1 |  | FR1_FDD |
| slot10 | 1 |  | FR1_TDD |
| slot8 | 1 |  | FR2 |
| } |  |  |  |

ZP CSI-RS for CSI Acquisition

CSI-RS-ResourceMapping

Table 5.4.2.4-3: ZP CSI-RS-ResourceMapping

| Derivation Path: Table 4.6.3-45 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| CSI-RS-ResourceMapping ::= SEQUENCE { |  |  |  |
| frequencyDomainAllocation CHOICE { |  |  |  |
| other | 000100 | K0 = 4 | FR1 |
| row4 | 100 | K0=8 | FR2 |
| } |  |  |  |
| nrofPorts | P4 |  |  |
| firstOFDMSymbolInTimeDomain | 9 | I0 = 9 | FR1 |
|  | 13 | l0 = 13 | FR2 |
| cdm-Type | fd-CDM2 |  |  |
| density CHOICE { |  |  |  |
| one | NULL |  |  |
| } |  |  |  |
| freqBand | ZP CSI-FrequencyOccupation |  |  |
| } |  |  |  |

<End of Changes >