3GPP TSG-RAN5 Meeting #93-e	R5-218421

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1465 | rev | 1 | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of A-MPR NS_04 for band n41 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | In the current version of the test configuration table for NS_04, certain test frequencies are defined by an equation. This equation is difficult to handle and increases the risk of misunderstandings, especially when adjusting the test frequency to the channel raster. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The equations have been replaced by additional test frequencies. Furthermore, the Inner-Full RB allocation has been replaced by Edge_1RB_Left to better align with other A-MPR test cases. The corresponding test requirements have been updated accordingly. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test frequencies for NS_04 can still be misinterpreted. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.3.4.1, 6.2.3.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Test point analysis in R5-218487 (CR 0529). |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is an update of R5-217625 and the outcome of 4 revisions to this document. |  |  |  |  |  |  |  |  |

## [Start of changes]

6.2.3.4	Test description

6.2.3.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in tables 6.2.3.4.1-1 to 6.2.3.4.1-26. The details of the uplink reference measurement channels (RMCs) are specified in Annex A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2

Network signalling value NS_10 is not tested. The relaxation specified under NS_10 is for the purpose to control / mitigate self-desensitization of the UE’s own receiver, but no corresponding REFSENS requirements are specified. There is no need to test A-MPR requirements for its own.

Table 6.2.3.4.1-1: Test Configuration table for NS_06 and NS_35

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  | Normal |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  | Lowest, Highest |  |
| A-MPR test parameters for NS_35 |  |  |  |  |  |  |  |
|  |  |  |  | Downlink Configuration | Uplink Configuration |  |  |
| Test ID | Freq | ChBw | SCS | N/A for A-MPR testing. | Modulation (NOTE 2) |  | RB allocation (NOTE 1) |
| 1 | Low | Default | Default |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Left |
| 2 | High |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Edge_1RB_Right |
| 3 | Default |  |  |  | DFT-s-OFDM PI/2 BPSK |  | Outer Full |
| 4 | Low |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Left |
| 5 | High |  |  |  | DFT-s-OFDM QPSK |  | Edge_1RB_Right |
| 6 | Default |  |  |  | DFT-s-OFDM QPSK |  | Outer Full |
| 7 | Low |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Left |
| 8 | High |  |  |  | DFT-s-OFDM 16 QAM |  | Edge_1RB_Right |
| 9 | Default |  |  |  | DFT-s-OFDM 16 QAM |  | Outer Full |
| 10 | Low |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Left |
| 11 | High |  |  |  | DFT-s-OFDM 64 QAM |  | Edge_1RB_Right |
| 12 | Default |  |  |  | DFT-s-OFDM 64 QAM |  | Outer Full |
| 13 | Low |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Left |
| 14 | High |  |  |  | DFT-s-OFDM 256 QAM |  | Edge_1RB_Right |
| 15 | Default |  |  |  | DFT-s-OFDM 256 QAM |  | Outer Full |
| 16 | Low |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Left |
| 17 | High |  |  |  | CP-OFDM QPSK |  | Edge_1RB_Right |
| 18 | Default |  |  |  | CP-OFDM QPSK |  | Outer Full |
| 19 | Low |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Left |
| 20 | High |  |  |  | CP-OFDM 16 QAM |  | Edge_1RB_Right |
| 21 | Default |  |  |  | CP-OFDM 16 QAM |  | Outer Full |
| 22 | Low |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Left |
| 23 | High |  |  |  | CP-OFDM 64 QAM |  | Edge_1RB_Right |
| 24 | Default |  |  |  | CP-OFDM 64 QAM |  | Outer Full |
| 25 | Low |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Left |
| 26 | High |  |  |  | CP-OFDM 256 QAM |  | Edge_1RB_Right |
| 27 | Default |  |  |  | CP-OFDM 256 QAM |  | Outer Full |
| 283, 4 | Low |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Edge_1RB_Left |
| 293, 4 | High |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Edge_1RB_Right |
| 303, 4 | Default |  |  |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS |  | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UEs supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. NOTE 4: This configuration is only applicable for NS_06 power class 1 UE. |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2: Test Configuration table for NS_04

Table 6.2.3.4.1-2: Test Configuration table for NS_04

| Initial Conditions |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  |  |  |  |  |  | Normal |  |  |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  | (See Freq column) |  |  |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  |  |  |  |  |  | Lowest, Highest |  |  |  |
| A-MPR test parameters for NS_04 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  |  |  |  |  |  |  | Downlink Configuration |  |  | Uplink Configuration |  |  |  |  |
| Test ID |  | Freq |  |  |  |  |  | N/A for A-MPR testing |  |  | Modulation (NOTE 2) |  |  | RB allocation (NOTE 1) |  |
| 1 (Note 3) |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |  |
| 2 (Note 3, 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |  |
| 3 (Note 3) |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |  |
| 4 (Note 3) |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer Full |  |
| 5 (Note 3) |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Right |  |
| 6 (Note 3) |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer Full |  |
| 7 (Note 4) |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |  |
| 8 (Note 4, 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |  |
| 9 (Note 4) |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Left |  |
| 10 (Note 4) |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer Full |  |
| 11 (Note 4) |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Edge_1RB_Right |  |
| 12 (Note 4) |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM PI/2 BPSK |  |  | Outer Full |  |
| 13 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edge_1RB_Left |  |
| 14 (Note 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edge_1RB_Left |  |
| 15 |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edge_1RB_Left |  |
| 16 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Outer Full |  |
| 17 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Edge_1RB_Right |  |
| 18 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM QPSK |  |  | Outer Full |  |
| 19 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Left |  |
| 20 (Note 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Left |  |
| 21 |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Left |  |
| 22 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Outer Full |  |
| 23 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Edge_1RB_Right |  |
| 24 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 16 QAM |  |  | Outer Full |  |
| 25 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Edge_1RB_Left |  |
| 26 (Note 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Edge_1RB_Left |  |
| 27 |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Outer Full |  |
| 28 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Outer Full |  |
| 29 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Edge_1RB_Right |  |
| 30 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 64 QAM |  |  | Outer Full |  |
| 31 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Left |  |
| 32 (Note 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Left |  |
| 33 |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Left |  |
| 34 |  | Low |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Outer Full |  |
| 35 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Edge_1RB_Right |  |
| 36 |  | High |  |  |  |  |  |  |  |  | DFT-s-OFDM 256 QAM |  |  | Outer Full |  |
| 37 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Left |  |
| 38 (Note 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Left |  |
| 39 |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Left |  |
| 40 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | Outer Full |  |
| 41 |  | High |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | Edge_1RB_Right |  |
| 42 |  | High |  |  |  |  |  |  |  |  | CP-OFDM QPSK |  |  | Outer Full |  |
| 43 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Left |  |
| 44 (Note 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Left |  |
| 45 |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Left |  |
| 46 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Outer Full |  |
| 47 |  | High |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Edge_1RB_Right |  |
| 48 |  | High |  |  |  |  |  |  |  |  | CP-OFDM 16 QAM |  |  | Outer Full |  |
| 49 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Left |  |
| 50 (Note 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Left |  |
| 51 |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Left |  |
| 52 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Outer Full |  |
| 53 |  | High |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Edge_1RB_Right |  |
| 54 |  | High |  |  |  |  |  |  |  |  | CP-OFDM 64 QAM |  |  | Outer Full |  |
| 55 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Edge_1RB_Left |  |
| 56 (Note 5) |  | Table 6.2.3.4.1-2a - Table 6.2.3.4.1-2c |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Edge_1RB_Left |  |
| 57) |  | Table 6.2.3.4.1-2d - Table 6.2.3.4.1-2f |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Outer Full |  |
| 58 |  | Low |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Outer Full |  |
| 59 |  | High |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Edge_1RB_Right |  |
| 60 |  | High |  |  |  |  |  |  |  |  | CP-OFDM 256 QAM |  |  | Outer Full |  |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for band n41. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n41, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n41. NOTE 5: Only applicable for 10 MHz and 15 MHz channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.4.1-2a: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 |  |  | 2505 | 501000 | 2500.32 | 500064 | 0 | 15 | 6255 | 500430 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2512.5 | 502500 | 2505.39 | 501078 | 0 | 15 | 6270 | 501630 | 4 | 1 | 2 (4) | 5 |

Table 6.2.3.4.1-2b: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2505 | 501000 | 2500.68 | 500136 | 0 | 30 | 6261 | 500910 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2512.5 | 502500 | 2505.66 | 501132 | 0 | 30 | 6273 | 501870 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2c: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2505 | 501000 | 2501.04 | 500208 | 0 | 15 | - | 500568 |
| 15 | 18 |  |  | 2512.5 | 502500 | 2506.02 | 501204 | 0 | 15 | - | 501564 |

Table 6.2.3.4.1-2d: Additional test frequencies for NS_04 (SCS=15 kHz, ΔFRaster = 15 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 52 |  |  | 2511 | 502200 | 2506.32 | 501264 | 0 | 15 | 6270 | 501630 | 2 | 0 | 0 (0) | 0 |
| 15 | 79 |  |  | 2513.505 | 502701 | 2506.395 | 501279 | 0 | 15 | 6273 | 501870 | 5 | 2 | 2 (4) | 6 |
| 20 | 106 |  |  | 2520 | 504000 | 2510.46 | 502092 | 0 | 15 | 6282 | 502590 | 10 | 1 | 1 (2) | 3 |
| 30 | 160 |  |  | 2535 | 507000 | 2520.6 | 504120 | 0 | 15 | 6306 | 504510 | 10 | 0 | 0 (0) | 0 |
| 40 | 216 |  |  | 2550 | 510000 | 2530.56 | 506112 | 0 | 15 | 6333 | 506670 | 6 | 1 | 2 (4) | 5 |
| 50 | 270 |  |  | 2565 | 513000 | 2540.7 | 508140 | 0 | 15 | 6357 | 508590 | 6 | 0 | 1 (2) | 2 |

Table 6.2.3.4.1-2e: Additional test frequencies for NS_04 (SCS=30 kHz, ΔFRaster = 30 kHz)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |  | Offset Carrier CORESET#0 [RBs] Note 2 | CORESET#0 Index (Offset [RBs]) Note 1 | offsetToPointA (SIB1) [PRBs] Note 1 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 24 |  |  | 2511 | 502200 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 15 | 38 |  |  | 2513.52 | 502704 | 2506.68 | 501336 | 0 | 30 | 6276 | 502110 | 18 | 0 | 0 (0) | 0 |
| 20 | 51 |  |  | 2520 | 504000 | 2510.82 | 502164 | 0 | 30 | 6288 | 503070 | 14 | 0 | 2 (2) | 4 |
| 30 | 78 |  |  | 2535 | 507000 | 2520.96 | 504192 | 0 | 30 | 6312 | 504990 | 2 | 0 | 1 (1) | 2 |
| 40 | 106 |  |  | 2550 | 510000 | 2530.92 | 506184 | 0 | 30 | 6336 | 506910 | 2 | 0 | 0 (0) | 0 |
| 50 | 133 |  |  | 2565 | 513000 | 2541.06 | 508212 | 0 | 30 | 6363 | 509070 | 22 | 0 | 1 (1) | 2 |
| 60 | 162 |  |  | 2580 | 516000 | 2550.84 | 510168 | 0 | 30 | 6387 | 510990 | 10 | 0 | 1 (1) | 2 |
| 70 | 189 |  |  | 2595 | 519000 | 2560.98 | 512196 | 0 | 30 | 6414 | 513150 | 6 | 0 | 3 (3) | 6 |
| 80 | 217 |  |  | 2610 | 522000 | 2570.94 | 514188 | 0 | 30 | 6438 | 515070 | 6 | 0 | 2 (2) | 4 |
| 90 | 245 |  |  | 2625 | 525000 | 2580.9 | 516180 | 0 | 30 | 6462 | 516990 | 6 | 0 | 1 (1) | 2 |
| 100 | 273 |  |  | 2640 | 528000 | 2590.86 | 518172 | 0 | 30 | 6486 | 518910 | 6 | 0 | 0 (0) | 0 |

Table 6.2.3.4.1-2f: Additional test frequencies for NS_04 (SCS=60 kHz, ΔFRaster = 15 kHz, without CORESET#0)

| CBW [MHz] | carrierBandwidth [PRBs] | Range |  | Carrier centre [MHz] | Carrier centre [ARFCN] | point A [MHz] | absoluteFrequencyPointA [ARFCN] | offsetToCarrier [Carrier PRBs] | SS block SCS [kHz] | GSCN | absoluteFrequencySSB [ARFCN] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 10 | 11 |  |  | 2511 | 502200 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 15 | 18 |  |  | 2513.52 | 502704 | 2507.04 | 501408 | 0 | 15 | - | 501768 |
| 20 | 24 |  |  | 2520 | 504000 | 2511.36 | 502272 | 0 | 15 | - | 502632 |
| 30 | 38 |  |  | 2535 | 507000 | 2521.32 | 504264 | 0 | 15 | - | 504624 |
| 40 | 51 |  |  | 2550 | 510000 | 2531.64 | 506328 | 0 | 15 | - | 506688 |
| 50 | 65 |  |  | 2550 | 510000 | 2526.6 | 505320 | 0 | 15 | - | 505680 |
| 60 | 79 |  |  | 2580 | 516000 | 2551.56 | 510312 | 0 | 15 | - | 510672 |
| 70 | 93 |  |  | 2595 | 519000 | 2561.52 | 512304 | 0 | 15 | - | 512664 |
| 80 | 107 |  |  | 2610 | 522000 | 2571.48 | 514296 | 0 | 15 | - | 514656 |
| 90 | 121 |  |  | 2625 | 525000 | 2581.44 | 516288 | 0 | 15 | - | 516648 |
| 100 | 135 |  |  | 2640 | 528000 | 2591.4 | 518280 | 0 | 15 | - | 518640 |

Table 6.2.3.4.1-3: Test Configuration table for NS_03, NS_03U and NS_100

| Initial Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  |  |  |  |  | Normal |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Low range, High range |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  |  |  |  |  | Lowest, Highest |
| Test SCS as specified in Table 5.3.5-1 |  |  |  |  |  |  | Lowest, Highest |
| A-MPR test parameters for NS_03, NS_03U and NS_100 |  |  |  |  |  |  |  |
| Test ID | Freq | ChBw | SCS | Downlink Configuration | Uplink Configuration |  |  |
|  |  |  |  |  | Modulation (Note 2) |  | RB allocation (Note 1) |
|  |  |  |  | N/A for A-MPR test cases | DFT-s OFDM |  |  |
| 1 | Low | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Left |
| 2 | High | Default | Default |  |  | PI/2 BPSK | Edge_1RB_Right |
| 3 | Default | Default | Default |  |  | PI/2 BPSK | Outer_Full |
| 4 | Low | Default | Default |  |  | QPSK | Edge_1RB_Left |
| 5 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 6 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 7 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 8 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 9 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 10 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 11 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 12 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 13 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 14 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 15 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| 16 | Low | Default | Default |  | CP-s OFDM | QPSK | Edge_1RB_Left |
| 17 | High | Default | Default |  |  | QPSK | Edge_1RB_Right |
| 18 | Default | Default | Default |  |  | QPSK | Outer_Full |
| 19 | Low | Default | Default |  |  | 16 QAM | Edge_1RB_Left |
| 20 | High | Default | Default |  |  | 16 QAM | Edge_1RB_Right |
| 21 | Default | Default | Default |  |  | 16 QAM | Outer_Full |
| 22 | Low | Default | Default |  |  | 64 QAM | Edge_1RB_Left |
| 23 | High | Default | Default |  |  | 64 QAM | Edge_1RB_Right |
| 24 | Default | Default | Default |  |  | 64 QAM | Outer_Full |
| 25 | Low | Default | Default |  |  | 256 QAM | Edge_1RB_Left |
| 26 | High | Default | Default |  |  | 256 QAM | Edge_1RB_Right |
| 27 | Default | Default | Default |  |  | 256 QAM | Outer_Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1 unless otherwise stated in this table. NOTE 2: DFT-s-OFDM PI/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: Void. NOTE 4: Void |  |  |  |  |  |  |  |

## [Several Tables and Clauses skipped]

6.2.3.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in the applicable table from table 6.2.3.5-1. The allowed A-MPR values specified in table 6.2.3.3-1 are in addition to the allowed MPR requirements specified in clause 6.2.2. For the UE maximum output power modified by MPR and/or A-MPR, the power limits specified in table 6.2.1.3-1 apply.

Table 6.2.3.5-0: Test Tolerance (UE additional maximum output power reduction)

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.3.5-1: UE Power Class 3 test requirements (NS_35) for band n71

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 2 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 3 | 23 | 0.5 | 0 | 0 | 22.5 | 2 | 2.5 | 25+TT | 20-TT |
| 4 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 5 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 6 | 23 | 1 | 0 | 0 | 22 | 2 | 2.5 | 25+TT | 19.5-TT |
| 7 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 8 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 9 | 23 | 2 | 0 | 0 | 21 | 2 | 2.5 | 25+TT | 18.5-TT |
| 10 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 11 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 12 | 23 | 2.5 | 0 | 0 | 20.5 | 2.5 | 2.5 | 25+TT | 18-TT |
| 13 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 14 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 15 | 23 | 4.5 | 0 | 0 | 18.5 | 4 | 2.5 | 25+TT | 14.5-TT |
| 16 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 17 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 18 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 19 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 20 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 21 | 23 | 3 | 0 | 0 | 20 | 2.5 | 2.5 | 25+TT | 17.5-TT |
| 22 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 23 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 24 | 23 | 3.5 | 0 | 0 | 19.5 | 3.5 | 2.5 | 25+TT | 16-TT |
| 25 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 26 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| 27 | 23 | 6.5 | 0 | 0 | 16.5 | 5 | 2.5 | 25+TT | 11.5-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.3.5-0. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-2: UE Power Class 2 test requirements (NS_04) for band n41 (Step 3)

| Test ID | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (Note 2) (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 8 | 26 | 3,5 | 5.5 | 1.5 | 19 | 3.5 | 2 | 28+TT | 15.5-TT |
| 9 | 26 | 3,5 | 3.5 | 1.5 | 21 | 2.0 | 2 | 28+TT | 19-TT |
| 10 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 11 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 2 | 28+TT | 23.5-TT |
| 12 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 13 | 26 | 0.5 | 0 | 0 | 25.5 | 2.0 | 2 | 28+TT | 23.5-TT |
| 14 | 26 | 3,5 | 6 | 1.5 | 18.5 | 4.0 | 2 | 28+TT | 14.5-TT |
| 15 | 26 | 3,5 | 4.5 | 1.5 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 16 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 17 | 26 | 1 | 0 | 0 | 25 | 2.0 | 2 | 28+TT | 23-TT |
| 18 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 19 | 26 | 1 | 0 | 0 | 25 | 2.0 | 2 | 28+TT | 23-TT |
| 20 | 26 | 3,5 | 6 | 1.5 | 18.5 | 4.0 | 2 | 28+TT | 14.5-TT |
| 21 | 26 | 3,5 | 5 | 1.5 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 22 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 23 | 26 | 2 | 0 | 0 | 24 | 2.0 | 2 | 28+TT | 22-TT |
| 24 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 25 | 26 | 2 | 0 | 0 | 24 | 2.0 | 2 | 28+TT | 22-TT |
| 26 | 26 | 3,5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 27 | 26 | 3,5 | 5 | 1.5 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 28 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 29 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 2 | 28+TT | 21.5-TT |
| 30 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 31 | 26 | 2.5 | 0 | 0 | 23.5 | 2.0 | 2 | 28+TT | 21.5-TT |
| 32 | 26 | 4.5 | 8 | 1.5 | 16.5 | 5.0 | 2 | 28+TT | 11.5-TT |
| 33 | 26 | 4.5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 34 | 26 | 4.5 | 0 | 1.5 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 35 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 2 | 28+TT | 19.5-TT |
| 36 | 26 | 4.5 | 0 | 1.5 | 20 | 2.5 | 2 | 28+TT | 17.5-TT |
| 37 | 26 | 4.5 | 0 | 0 | 21.5 | 2.0 | 2 | 28+TT | 19.5-TT |
| 38 | 26 | 3,5 | 7.5 | 1.5 | 17 | 5.0 | 2 | 28+TT | 12-TT |
| 39 | 26 | 3,5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 40 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 41 | 26 | 3 | 0 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 42 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 43 | 26 | 3 | 0 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 44 | 26 | 3,5 | 7.5 | 1.5 | 17 | 5.0 | 2 | 28+TT | 12-TT |
| 45 | 26 | 3,5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 46 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 47 | 26 | 3 | 0 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 48 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 49 | 26 | 3 | 0 | 0 | 23 | 2.0 | 2 | 28+TT | 21-TT |
| 50 | 26 | 3,5 | 7.5 | 1.5 | 17 | 5.0 | 2 | 28+TT | 12-TT |
| 51 | 26 | 3,5 | 6.5 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 52 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 53 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 54 | 26 | 3,5 | 0 | 1.5 | 24.5 | 2.0 | 2 | 28+TT | 22.5-TT |
| 55 | 26 | 3.5 | 0 | 0 | 22.5 | 2.0 | 2 | 28+TT | 20.5-TT |
| 56 | 26 | 6.5 | 10 | 1.5 | 14.5 | 5.0 | 2 | 28+TT | 9.5-TT |
| 57 | 26 | 6.5 | 7.5 | 1.5 | 17 | 5.0 | 2 | 28+TT | 12-TT |
| 58 | 26 | 6.5 | 0 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| 59 | 26 | 6.5 | 0 | 0 | 19.5 | 3.5 | 2 | 28+TT | 16-TT |
| 60 | 26 | 6.5 | 0 | 1.5 | 18 | 4.0 | 2 | 28+TT | 14-TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-3: UE Power Class 3 test requirements (NS_04) for band n41 (Step 3) and UE Power Class 1.5 and Power Class 2 test requirements (NS_04) for band n41 (Step 4)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX,c (dBm) |  | T(PCMAX_L,c) (dB) |  | TL,c (dB) | Upper limit (dBm) |  |  | Lower limit (Note 2) (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 3.5 | 3.5 | 1.5 | 21 |  | 2.0 |  | 2 | 28+TT |  |  | 19-TT |  |
| 2 | 23 | -3 | 3.5 | 3.5 | 1.5 | 21 |  | 2.0 |  | 2 | 28+TT |  |  | 19-TT |  |
| 3 | 23 | -3 | 3.5 | 0 | 1.5 | 21 |  | 2.0 |  | 2 | 28+TT |  |  | 19-TT |  |
| 4 | 23 | -3 | 1.2 | 0 | 0 | 24.8 |  | 2.0 |  | 2 | 28+TT |  |  | 22.8-TT |  |
| 5 | 23 | -3 | 3.5 | 0 | 1.5 | 21 |  | 2.0 |  | 2 | 28+TT |  |  | 19-TT |  |
| 6 | 23 | -3 | 1.2 | 0 | 0 | 24.8 |  | 2.0 |  | 2 | 28+TT |  |  | 22.8-TT |  |
| 7 | 23 | 0 | 0.5 | 3.5 | 1.5 | 18 |  | 4.0 |  | 2 | 25+TT |  |  | 14-TT |  |
| 8 | 23 | 0 | 0.5 | 3.5 | 1.5 | 18 |  | 4.0 |  | 2 | 25+TT |  |  | 14-TT |  |
| 9 | 23 | 0 | 0.5 | 0 | 1.5 | 21 |  | 2.0 |  | 2 | 25+TT |  |  | 19-TT |  |
| 10 | 23 | 0 | 0.5 | 0 | 0 | 22.5 |  | 2.0 |  | 2 | 25+TT |  |  | 20.5-TT |  |
| 11 | 23 | 0 | 0.5 | 0 | 1.5 | 21 |  | 2.0 |  | 2 | 25+TT |  |  | 19-TT |  |
| 12 | 23 | 0 | 0.5 | 0 | 0 | 22.5 |  | 2.0 |  | 2 | 25+TT |  |  | 20.5-TT |  |
| 13 | 23 | 0 | 1 | 4 | 1.5 | 17.5 |  | 5.0 |  | 2 | 25+TT |  |  | 12.5-TT |  |
| 14 | 23 | 0 | 1 | 4 | 1.5 | 17.5 |  | 5.0 |  | 2 | 25+TT |  |  | 12.5-TT |  |
| 15 | 23 | 0 | 1 | 0 | 1.5 | 20.5 |  | 2.5 |  | 2 | 25+TT |  |  | 18-TT |  |
| 16 | 23 | 0 | 1 | 0 | 0 | 22 |  | 2.0 |  | 2 | 25+TT |  |  | 20-TT |  |
| 17 | 23 | 0 | 1 | 0 | 1.5 | 20.5 |  | 2.5 |  | 2 | 25+TT |  |  | 18-TT |  |
| 18 | 23 | 0 | 1 | 0 | 0 | 22 |  | 2.0 |  | 2 | 25+TT |  |  | 20-TT |  |
| 19 | 23 | 0 | 2 | 4 | 1.5 | 17.5 |  | 5.0 |  | 2 | 25+TT |  |  | 12.5-TT |  |
| 20 | 23 | 0 | 2 | 4 | 1.5 | 17.5 |  | 5.0 |  | 2 | 25+TT |  |  | 12.5-TT |  |
| 21 | 23 | 0 | 2 | 0 | 1.5 | 19.5 |  | 3.5 |  | 2 | 25+TT |  |  | 16-TT |  |
| 22 | 23 | 0 | 2 | 0 | 0 | 21 |  | 2.0 |  | 2 | 25+TT |  |  | 19-TT |  |
| 23 | 23 | 0 | 2 | 0 | 1.5 | 19.5 |  | 3.5 |  | 2 | 25+TT |  |  | 16-TT |  |
| 24 | 23 | 0 | 2 | 0 | 0 | 21 |  | 2.0 |  | 2 | 25+TT |  |  | 19-TT |  |
| 25 | 23 | 0 | 2.5 | 4.5 | 1.5 | 17 |  | 5.0 |  | 2 | 25+TT |  |  | 12-TT |  |
| 26 | 23 | 0 | 2.5 | 4 | 1.5 | 17.5 |  | 5.0 |  | 2 | 25+TT |  |  | 12.5-TT |  |
| 27 | 23 | 0 | 2.5 | 0 | 1.5 | 19 |  | 3.5 |  | 2 | 25+TT |  |  | 15.5-TT |  |
| 28 | 23 | 0 | 2.5 | 0 | 0 | 20.5 |  | 2.5 |  | 2 | 25+TT |  |  | 18-TT |  |
| 29 | 23 | 0 | 2.5 | 0 | 1.5 | 19 |  | 3.5 |  | 2 | 25+TT |  |  | 15.5-TT |  |
| 30 | 23 | 0 | 2.5 | 0 | 0 | 20.5 |  | 2.5 |  | 2 | 25+TT |  |  | 18-TT |  |
| 31 | 23 | 0 | 4.5 | 6 | 1.5 | 15.5 |  | 5.0 |  | 2 | 25+TT |  |  | 10.5-TT |  |
| 32 | 23 | 0 | 4.5 | 4.5 | 1.5 | 17 |  | 5.0 |  | 2 | 25+TT |  |  | 12-TT |  |
| 33 | 23 | 0 | 4.5 | 0 | 1.5 | 17 |  | 5.0 |  | 2 | 25+TT |  |  | 12-TT |  |
| 34 | 23 | 0 | 4.5 | 0 | 0 | 18.5 |  | 4.0 |  | 2 | 25+TT |  |  | 14.5-TT |  |
| 35 | 23 | 0 | 4.5 | 0 | 1.5 | 17 |  | 5.0 |  | 2 | 25+TT |  |  | 12-TT |  |
| 36 | 23 | 0 | 4.5 | 0 | 0 | 18.5 |  | 4.0 |  | 2 | 25+TT |  |  | 14.5-TT |  |
| 37 | 23 | 0 | 3 | 5.5 | 1.5 | 16 |  | 5.0 |  | 2 | 25+TT |  |  | 11-TT |  |
| 38 | 23 | 0 | 3 | 5.5 | 1.5 | 16 |  | 5.0 |  | 2 | 25+TT |  |  | 11-TT |  |
| 39 | 23 | 0 | 3 | 0 | 1.5 | 18.5 |  | 4.0 |  | 2 | 25+TT |  |  | 14.5-TT |  |
| 40 | 23 | 0 | 3 | 0 | 0 | 20 |  | 2.5 |  | 2 | 25+TT |  |  | 17.5-TT |  |
| 41 | 23 | 0 | 3 | 0 | 1.5 | 18.5 |  | 4.0 |  | 2 | 25+TT |  |  | 14.5-TT |  |
| 42 | 23 | 0 | 3 | 0 | 0 | 20 |  | 2.5 |  | 2 | 25+TT |  |  | 17.5-TT |  |
| 43 | 23 | 0 | 3 | 5.5 | 1.5 | 16 |  | 5.0 |  | 2 | 25+TT |  |  | 11-TT |  |
| 44 | 23 | 0 | 3 | 5.5 | 1.5 | 16 |  | 5.0 |  | 2 | 25+TT |  |  | 11-TT |  |
| 45 | 23 | 0 | 3 | 0 | 1.5 | 18.5 |  | 4.0 |  | 2 | 25+TT |  |  | 14.5-TT |  |
| 46 | 23 | 0 | 3 | 0 | 0 | 20 |  | 2.5 |  | 2 | 25+TT |  |  | 17.5-TT |  |
| 47 | 23 | 0 | 3 | 0 | 1.5 | 18.5 |  | 4.0 |  | 2 | 25+TT |  |  | 14.5-TT |  |
| 48 | 23 | 0 | 3 | 0 | 0 | 20 |  | 2.5 |  | 2 | 25+TT |  |  | 17.5-TT |  |
| 49 | 23 | 0 | 3.5 | 5.5 | 1.5 | 16 |  | 5.0 |  | 2 | 25+TT |  |  | 11-TT |  |
| 50 | 23 | 0 | 3.5 | 5.5 | 1.5 | 16 |  | 5.0 |  | 2 | 25+TT |  |  | 11-TT |  |
| 51 | 23 | 0 | 3.5 | 0 | 1.5 | 18 |  | 4.0 |  | 2 | 25+TT |  |  | 14-TT |  |
| 52 | 23 | 0 | 3.5 | 0 | 0 | 19.5 |  | 3.5 |  | 2 | 25+TT |  |  | 16-TT |  |
| 53 | 23 | 0 | 3.5 | 0 | 1.5 | 18 |  | 4.0 |  | 2 | 25+TT |  |  | 14-TT |  |
| 54 | 23 | 0 | 3.5 | 0 | 0 | 19.5 |  | 3.5 |  | 2 | 25+TT |  |  | 16-TT |  |
| 55 | 23 | 0 | 6.5 | 8 | 1.5 | 13.5 |  | 5.0 |  | 2 | 25+TT |  |  | 8.5-TT |  |
| 56 | 23 | 0 | 6.5 | 6.5 | 1.5 | 15 |  | 5.0 |  | 2 | 25+TT |  |  | 10-TT |  |
| 57 | 23 | 0 | 6.5 | 0 | 1.5 | 15 |  | 5.0 |  | 2 | 25+TT |  |  | 10-TT |  |
| 58 | 23 | 0 | 6.5 | 0 | 0 | 16.5 |  | 5.0 |  | 2 | 25+TT |  |  | 11.5-TT |  |
| 59 | 23 | 0 | 6.5 | 0 | 1.5 | 15 |  | 5.0 |  | 2 | 25+TT |  |  | 10-TT |  |
| 60 | 23 | 0 | 6.5 | 0 | 0 | 16.5 |  | 5.0 |  | 2 | 25+TT |  |  | 11.5-TT |  |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the lower limit shall be decreased by 1.0 dB for CP-OFDM 256 QAM and decreased by 1.5 dB for other modulations. NOTE 3: TT=0.7 dB for BWchannel ≤ 40 MHz; TT=1.0 dB for 40 MHz < BWchannel ≤ 100 MHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.3.5-4: UE Power Class 3 test requirements (NS_03 and NS_03U) for band n25, n66, n70

| Test ID | Network signalling label | PPowerClass (dBm) | MPR (dB) | A-MPR (dB) | ΔTC,c (dB) | PCMAX_L,c (dBm) | T(PCMAX_L,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1, 2 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 3 | NS_03 | 23 | 0.5 | 1.5 | 0 | 21.5 | 2 | 2 | 25+TT | 19.5-TT |
|  | NS_03U | 23 | 0.5 | 2 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 4, 5 | NS_03, NS_03U | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 6 | NS_03, NS_03U | 23 | 1 | 1 | 0 | 21 | 2 | 2 | 25+TT | 19-TT |
| 7, 8 | NS_03, NS_03U | 23 | 2 | 2.5 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 9 | NS_03, NS_03U | 23 | 2 | 2.5 | 0 | 20 | 2.5 | 2 | 25+TT | 17.5-TT |
| 10, 11 | NS_03, NS_03U | 23 | 2.5 | 3 | 0 | 19.5 | 3.5 | 2 | 25+TT | 16-TT |

## [End of changes]