3GPP TSG-RAN5 Meeting #93-e	R5-216422r2

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1379 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update of 6.3.3.4 PRACH time mask |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MediaTek Beijing Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The current PRACH time mask doesn't define ss-PBCH-BlockPower value for different SCS, instead to use default value 32 for all condition, which made UE transmit PRACH with incorrect target power in condition SCS30K and SCS60K. The test result will be -3dB power offset in SCS30K and -6dB in SCS60K condition. For example UE transmit PRACH in condition SCS15/30/60k on format 0 : PRACH POWER = PrachReceivedTargetPower +（ss-pbch-blockpower - RSRP） 15K: PRACH POWER =-118+(32-(-85))=-1dBm 30K: PRACH POWER =-118+(32-(-82))=-4dBm 60K: PRACH POWER =-118+(32-(-79))=-7dBm |  |  |  |  |  |  |  |  |
| Summary of change: |  | Specified ss-PBCH-BlockPower Value for different SCS_15K/30K/60K |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Made UE to transmit PRACH with incorrect target power in condition SCS30K and SCS60K |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.3.4.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Updated content in coversheet and Table 6.3.3.4.4.3-3 |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision (add the missing information of source to TSG R5 to solve the 3GU issue) 2nd revision (remove 60kHz ss-PBCH-BlockPower change in Table 6.3.3.4.4.3-3, since is not applicable for PRACH) |  |  |  |  |  |  |  |  |

[Start of change]

6.3.3.4.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6.3 with the following exceptions:

Table 6.3.3.4.4.3-1: RACH-ConfigCommon: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-128 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommon::= SEQUENCE { |  |  |  |
| rach-ConfigGeneric | RACH-ConfigGeneric |  |  |
| totalNumberOfRA-Preambles | Not present |  |  |
| ssb-perRACH-OccasionAndCB-PreamblesPerSSB CHOICE { |  |  |  |
| one | n8 |  | FR1 |
| } |  |  |  |
| groupBconfigured | Not present |  |  |
| ra-ContentionResolutionTimer | sf64 |  |  |
| rsrp-ThresholdSSB | RSRP-Range |  |  |
| rsrp-ThresholdSSB-SUL | Not present |  |  |
|  | RSRP-Range |  | SUL |
| prach-RootSequenceIndex CHOICE { |  |  |  |
| l139 | Set according to table 4.4.2-2 for the NR Cell. |  | PRACH Format A3 |
| l839 | 0 | NR Cell 1 | PRACH Format 0 |
|  | TBD | Other than NR Cell 1 | PRACH Format 0 |
| } |  |  |  |
| msg1-SubcarrierSpacing | SubcarrierSpacing |  |  |
| restrictedSetConfig | unrestrictedSet |  |  |
| msg3-transformPrecoder | Not present | transform precoding is disabled for Msg3 PUSCH transmission and any PUSCH transmission scheduled with DCI format 0_0 |  |
| } |  |  |  |

Table 6.3.3.4.4.3-2: RACH-ConfigGeneric: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-130 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGeneric ::= SEQUENCE { |  |  |  |
| prach-ConfigurationIndex | 4 | Paired Spectrum | PRACH Format 0 |
|  | 160 | Paired Spectrum | PRACH Format A3 |
|  | 12 | Unpaired Spectrum | PRACH Format 0 |
|  | 123 | Unpaired Spectrum | PRACH Format A3 |
| msg1-FDM | four |  | FR1 |
|  | one |  | FR1 5MHz PRACH Format A3 for SCS 15 kHz OR FR1 10MHz PRACH Format A3 for SCS 30 kHz OR FR1 10MHz PRACH Format A3 for SCS 60 kHz |
| msg1-FrequencyStart | 0 |  |  |
| zeroCorrelationZoneConfig | 15 |  |  |
| preambleReceivedTargetPower | -118 |  | PRACH Format 0 |
|  | -122 |  | PRACH Format A3 for SCS 15 kHz |
|  | -124 |  | PRACH Format A3 for SCS 30 kHz |
|  | -128 |  | PRACH Format A3 for SCS 60kHz |
| preambleTransMax | n7 |  |  |
| powerRampingStep | dB0 |  |  |
| ra-ResponseWindow | sl20 |  |  |
| } |  |  |  |

Table 6.3.3.4.4.3-3: ServingCellConfigCommonSIB: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-169 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfigCommonSIB ::= SEQUENCE { |  |  |  |
| downlinkConfigCommon | DownlinkConfigCommonSIB |  |  |
| uplinkConfigCommon | UplinkConfigCommonSIB |  |  |
| supplementaryUplink | Not present |  |  |
|  | UplinkConfigCommonSIB |  | SUL |
| n-TimingAdvanceOffset | Not present |  |  |
| ssb-PositionsInBurst SEQUENCE { |  |  |  |
| inOneGroup | ’0100 0000’B | When carrier frequency is smaller than or equal to 3 GHz, only the 4 leftmost bits are valid; |  |
| groupPresence | Not present |  |  |
| } |  |  |  |
| ssb-PeriodicityServingCell | ms20 |  |  |
| tdd-UL-DL-ConfigurationCommon | TDD-UL-DL-ConfigCommon |  | FR1_TDD |
| ss-PBCH-BlockPower | 32 |  | SCS_15kHz |
|  | 35 |  | SCS_30kHz |
| } |  |  |  |

Table 6.3.3.4.4.3-4: PUSCH-TimeDomainResourceAllocationList: PRACH measurement

| Derivation Path: TS 38.508-1[5], Table 4.6.3-122 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-TimeDomainResourceAllocationList ::= SEQUENCE (SIZE(1..maxNrofUL-Allocations)) OF PUSCH-TimeDomainResourceAllocation { | 2 entries |  |  |
| PUSCH-TimeDomainResourceAllocation[2] SEQUENCE { |  | entry 2 addressed by Msg3 PUSCH time resource allocation field of the Random Access Response acc. to TS 38.213 [22] Table 8.2-1. |  |
| k2 | 6 | K2+ Δ=8 acc. to TS 38.214 [21] Table 6.1.2.1.1-5 | Unpaired Spectrum for SCS15kHz and PRACH Format 0 |
| } |  |  |  |
| } |  |  |  |

[End of change]