3GPP TSG-RAN5 Meeting #93-e	R5-216437r1

# Electronic Meeting, 8th Nov 2021 – 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2042 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update RRCReconfiguration |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5GS_NR_LTE-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | To add connectivity NE-DC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The RRCReconfiguration has been updated - NE-DC has been added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The connectivity NE-DC will not be finalized. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.6.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision R5-216437r1. Some NE-DC conditions added. |  |  |  |  |  |  |  |  |

<Start of first modified section>

#### –	RRCReconfiguration

Table 4.6.1-13: RRCReconfiguration

| Derivation Path: TS 38.331 [6], clause 6.2.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element |  | Value/remark | Comment | Condition |
| RRCReconfiguration ::= SEQUENCE { |  |  |  |  |
| rrc-TransactionIdentifier |  | RRC-TransactionIdentifier |  |  |
| criticalExtensions CHOICE { |  |  |  |  |
| rrcReconfiguration SEQUENCE { |  |  |  |  |
| radioBearerConfig |  | Not present |  |  |
|  |  | RadioBearerConfig with conditions SRB2 and DRB1 |  | NR, NR-DC |
|  |  | RadioBearerConfig with conditions SRB2 and REEST |  | REEST |
| secondaryCellGroup |  | CellGroupConfig | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC |
|  |  | CellGroupConfig with condition NR-DC_SCG | OCTET STRING (CONTAINING CellGroupConfig) | NR-DC_SCG |
|  |  | CellGroupConfig with conditions EN-DC and PSCell_change | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_HO |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_MEAS |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | EN-DC_SCell_add |
|  |  | Not present |  |  |
| measConfig |  | Not present |  |  |
|  |  | MeasConfig | Measurements configuration | NR_MEAS, EN-DC_MEAS |
| lateNonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR, SCell_add, NR_MEAS, NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC, REEST |
| masterCellGroup |  | CellGroupConfig with condition SRB2_DRB1 | OCTET STRING (CONTAINING CellGroupConfig) | NR, NR-DC |
|  |  | CellGroupConfig with condition SCell_add | OCTET STRING (CONTAINING CellGroupConfig) | SCell_add |
|  |  | CellGroupConfig with condition MEAS | OCTET STRING (CONTAINING CellGroupConfig) | NR_MEAS |
|  |  | CellGroupConfig with condition REEST | OCTET STRING (CONTAINING CellGroupConfig) | REEST |
| fullConfig |  | Not present |  |  |
| dedicatedNAS-MessageList |  | Not present |  |  |
| dedicatedNAS-MessageList SEQUENCE (SIZE(1..maxDRB)) OF DedicatedNAS-Message { |  | 1 entry |  | NR |
| DedicatedNAS-Message[1] |  | DedicatedNAS-Message | entry 1 A sequence of OCTET STRING (s) containing one or more DedicatedNAS-Message(s) |  |
| } |  |  |  |  |
| masterKeyUpdate |  | Not present |  |  |
| masterKeyUpdate SEQUENCE { |  |  |  | MasterKeyChange |
| keySetChangeIndicator |  | true |  |  |
| nextHopChainingCount |  | NextHopChainingCount |  |  |
| nas-Container |  | Not present |  |  |
|  |  | OCTET STRING including the 10 Octets value generated according to TS 24.501 [28] clause 9.11.2.9 |  | Inter_Sys_HO |
| } |  |  |  |  |
| dedicatedSIB1-Delivery |  | Not present |  |  |
| dedicatedSystemInformationDelivery |  | Not present |  |  |
| otherConfig |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | NR-DC, SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC, NE-DC |
| otherConfig-v1540 |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  |  |
| mrdc-SecondaryCellGroupConfig |  | Not present |  |  |
| mrdc-SecondaryCellGroupConfig CHOICE { |  |  |  | NR-DC, NE-DC |
| setup SEQUENCE { |  |  |  |  |
| mrdc-ReleaseAndAdd |  | Not present |  |  |
| mrdc-SecondaryCellGroup CHOICE { |  |  |  |  |
| nr-SCG |  | RRCReconfiguration with condition NR-DC_SCG | OCTET STRING (CONTAINING RRCReconfiguration) | NR-DC |
| eutra-SCG |  | RRCConnectionReconfiguration according TS 36.508 [2], table 4.6.1-8 with condition NE-DC | OCTET STRING (CONTAINING RRCConnectionReconfiguration) | NE-DC |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| radioBearerConfig2 |  | Not present |  | NE-DC |
|  |  | RadioBearerConfig with condition DRBn and SecondaryKeys | OCTET STRING (CONTAINING RadioBearerConfig) | NR-DC |
| sk-Counter |  | SK-Counter |  |  |
| nonCriticalExtension |  | Not present |  |  |
| nonCriticalExtension SEQUENCE { |  |  |  | SIDELINK, DAPS_HO_ReleaseSource, CHO, CPC |
| otherConfig-v1610 |  | Not present |  |  |
| bap-Config-r16 |  | Not present |  |  |
| iab-IP-AddressConfigurationList-r16 |  | Not present |  |  |
| conditionalReconfiguration-r16 |  | Not present |  |  |
|  |  | ConditionalReconfiguration |  | CHO, CPC |
| daps-SourceRelease-r16 |  | Not present |  |  |
|  |  | true |  | DAPS_HO_ReleaseSource |
| t316-r16 |  | Not present |  |  |
| needForGapsConfigNR-r16 |  | Not present |  |  |
| onDemandSIB-Request-r16 |  | Not present |  |  |
| dedicatedPosSysInfoDelivery-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 |  | Not present |  |  |
| sl-ConfigDedicatedNR-r16 CHOICE { |  |  |  | SIDELINK |
| setup |  | SL-ConfigDedicatedNR-r16 |  |  |
| } |  |  |  |  |
| sl-ConfigDedicatedEUTRA-Info-r16 |  | Not present |  |  |
| smtc-r16 |  | Not present |  |  |
| nonCriticalExtension |  | Not present |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |
| } |  |  |  |  |

| Condition | Explanation |
| --- | --- |
| EN-DC | E-UTRA-NR Dual Connectivity |
| EN-DC_MEAS | An EN-DC measurement is configured |
| NR_MEAS | A NR measurement is configured |
| NR | NG-RAN NR Radio Access |
| NR-DC | NR-NR Dual Connectivity is configured |
| NR-DC_SCG | Add SCG side configuration(NR-DC) |
| EN-DC_HO | NR PSCell change (EN-DC) |
| SCell_add | Add SCell |
| EN-DC_SCell_add | Add Scell (EN-DC) |
| MasterKeyChange | MasterKeyUpdate when peforming ReconfigurationWithSync and indicating a change of the AS security algorithms associated to the master key |
| Inter_Sys_HO | Used during inter-system handover to NR procedure |
| REEST | The first RRCReconfiguration message after successful completion of the RRC re-establishment procedure |
| SIDELINK | For NR sidelink dedicated configuration |
| DAPS_HO_ReleaseSource | The source cell part of DAPS operation is to be stopped and the source cell part of DAPS configuration is to be released. |
| CHO | Conditional handover |
| CPC | Conditional PSCell change |
| NE-DC | NR E-UTRA Dual Connectivity is configured |

<End of modified section>