3GPP TSG-RAN WG5 Meeting #93-e	R5-216469r1

Electronic Meeting, November 8 – 19, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1382 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Introduction of PC1.5 n79 MOP for UL MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_UE_PC1_5_n79-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of PC1.5 n79 MOP for UL MIMO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | PC1.5 n79 MOP for UL MIMO has been added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | PC1.5 n79 MOP UE conformance test can not be implemented. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2D.1.3, 6.2D.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | r1: current version has been changed from 17.2.0 into 17.2.1. |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

6.2D.1.3	Minimum conformance requirements

For UE with two transmit antenna connectors in closed-loop spatial multiplexing scheme, the maximum output power for any transmission bandwidth within the channel bandwidth is specified in Table 6.2D.1.3-1. The requirements shall be met with the UL MIMO configurations specified in Table 6.2D.1.3-2. For UE supporting UL MIMO, the maximum output power is defined as the sum of the maximum output power from both UE antenna connectors. The period of measurement shall be at least one sub frame (1ms).

The requirements shall be met with the UL MIMO configurations of using 2-layer UL MIMO transmission with codebook of. DCI Format for UE configured in PUSCH transmission mode for uplink single-user MIMO shall be used.

Table 6.2D.1.3-1: UE Power Class for UL MIMO in closed loop spatial multiplexing scheme

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n2 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n3 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n7 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n25 |  |  |  |  |  |  | 23 | +2/-31 |  |  |
| n30 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n34 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n38 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n39 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n40 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n41 |  |  | 29 | +2/-31 | 26 | +2/-31 | 23 | +2/-31 |  |  |
| n48 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n66 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n70 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n71 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| n77 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n78 |  |  |  |  | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n79 |  |  | 29 | +2/-3 | 26 | +2/-3 | 23 | +2/-3 |  |  |
| n97 |  |  |  |  |  |  | 23 | +2/-3 |  |  |
| NOTE 1: 1 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB. NOTE 2: Power class 3 is the default power class unless otherwise stated. |  |  |  |  |  |  |  |  |  |  |

<< UNCHANGED PARTS SKIPPED >>

6.2D.1.5	Test requirement

The maximum output power, derived in step 3 or step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2D.1.5-1.

Table 6.2D.1.5-1: UE Power Class

| NR band | Class 1 (dBm) | Tolerance (dB) | Class 1.5 (dBm) | Tolerance (dB) | Class 2 (dBm) | Tolerance (dB) | Class 3 (dBm) | Tolerance (dB) | Class 4 (dBm) | Tolerance (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| n1 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n2 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n3 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n7 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n25 |  |  |  |  |  |  | 23 | +2+TT/-31-TT |  |  |
| n30 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n34 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n38 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n39 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n40 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n41 |  |  | 29 | _+2+TT/-31-TT | 26 | +2+TT/-31-TT | 23 | +2+TT/-31-TT |  |  |
| n48 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n66 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n70 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n71 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| n77 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n78 |  |  |  |  | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n79 |  |  | 29 | +2+TT/-3-TT | 26 | +2+TT/-3-TT | 23 | +2+TT/-3-TT |  |  |
| n97 |  |  |  |  |  |  | 23 | +2+TT/-3-TT |  |  |
| NOTE 1: 1 refers to the transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high, the maximum output power requirement is relaxed by reducing the lower tolerance limit by 1.5 dB NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2D.1.5-2 |  |  |  |  |  |  |  |  |  |  |

<< END OF CHANGES >>