3GPP TSG-RAN5 Meeting #93-e	R5-216522r1

# Electronic Meeting, November 8 – November 19, 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1394 | rev |  | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Adding Power Class 1.5 for n77 n78 MPR |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Verizon, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | HPUE_PC1_5_n77_n78-UEConTest |  |  |  |  |  | Date: |  |  | 2021-11-17 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Addition of PC1.5 for n77 n78 to Maximum Power Reduction testing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated test applicability, minimum conformance and test requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Devices transmitting 29dBm will not be tested for conformance to MPR requirements. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2.2.3, 6.2.2.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Include Editor’s notes for test requirement |  |  |  |  |  |  |  |  |

### <Start of Changes>

### 6.2.2	UE maximum output power reduction

Editor’s Note: The following aspects are either missing or not yet determined:

- PC1 requirements for NR operating bands other than Band n14 are not defined in RAN4 Rel-15 and Rel-16 specifications.

- Seeking some clarification from RAN4 on how ΔTIB,c in TS 38.101-1 [2] subclause 6.2A.4.2/6.2C.2 and TS 38.101-3 [4] subclause 6.2A.4.2/6.2B.4.2 shall be applied for applicable bands.

- The test requirements need an update for a PC1.5 UE indicating modifiedMPR-Behavior

6.2.2.1	Test purpose

The number of RB identified in Table 6.2.2.3-1 is based on meeting the requirements for adjacent channel leakage ratio and the maximum power reduction (MPR) due to Cubic Metric (CM).

6.2.2.2	Test applicability

The requirements of this test apply to all types of NR Power Class 1.5, Power Class 2 and 3 UE release 15 and forward and NR Power Class 1 UE release 15 and forward in NR Band n14.

NOTE:	Test execution is not necessary if TS 38.521-1 6.5.2.4.1 is executed.

6.2.2.3	Minimum conformance requirements

UE is allowed to reduce the maximum output power due to higher order modulations and transmit bandwidth configurations. For UE power class 2 and 3 and UE power class 1 in n14, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-2, Table 6.2.2.3-1 and Table 6.2.2.3-5, respectively for channel bandwidths  ≤ 100 MHz. For UE power class 1.5, the allowed maximum power reduction (MPR) is defined in Table 6.2.2.3-4 and Table 6.2.2.3-4a in accordance with the indicated modifiedMPR-Behavior specified in Table L.1-1 of 38.101-1 [2] for channel bandwidths ≤ 100 MHz.

If the relative channel bandwidth ≤ 4% for TDD bands or ≤ 3% for FDD band, the ∆MPR is set to zero.

If the relative channel bandwidth is larger than 4% for TDD bands or 3% for FDD bands, the ∆MPR is defined in Table 6.2.2.3-3.

Where relative channel bandwidth = 2*BWChannel / (FUL_low + FUL_high).

The allowed MPR for SRS, PUCCH formats 0, 1, 3 and 4, and PRACH shall be as specified for QPSK modulated DFT-s-OFDM of equivalent RB allocation. The allowed MPR for PUCCH format 2 shall be as specified for QPSK modulated CP-OFDM of equivalent RB allocation.

Table 6.2.2.3-1: Maximum power reduction (MPR) for power class 3

| Modulation |  | MPR (dB) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Edge RB allocations |  | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.51 |  | ≤ 1.21 | ≤ 0.21 |
|  |  | ≤ 0.52 |  |  | 02 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.52 | 02 |  |  |
|  | QPSK | ≤ 1 |  |  | 0 |
|  | 16 QAM | ≤ 2 |  |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |  |
| CP-OFDM | QPSK | ≤ 3 |  |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |  |
| NOTE 1: Applicable for UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and if the IE powerBoostPi2BPSK is set to 1 and 40 % or less slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. The reference power of 0dB MPR is 26dBm. NOTE 2: Applicable for UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79 with Pi/2 BPSK modulation and if the IE powerBoostPi2BPSK is set to 0 and if more than 40% of slots in radio frame are used for UL transmission for bands n40, n41, n77, n78 and n79. |  |  |  |  |  |

Table 6.2.2.3-2: Maximum power reduction (MPR) for power class 2

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 3.5 | ≤ 0.5 | 0 |
|  | QPSK | ≤ 3.5 | ≤ 1 | 0 |
|  | 16 QAM | ≤ 3.5 | ≤ 2 | ≤ 1 |
|  | 64 QAM | ≤ 3.5 | ≤ 2.5 |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3.5 | ≤ 3 | ≤ 1.5 |
|  | 16 QAM | ≤ 3.5 | ≤ 3 | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Table 6.2.2.3-3: ∆MPR

| NR Band | Power class | Channel bandwidth | ∆MPR (dB) |
| --- | --- | --- | --- |
| n28 and n83 | Power class 3 | 30 MHz | 0.5 |

Table 6.2.2.3-4: Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6.5 | ≤ 3.5 | ≤ 1.5 |
|  | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 5 | ≤ 2.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 6 | ≤ 3 |
|  | 16 QAM | ≤ 6.5 | ≤ 6 | ≤ 3.5 |
|  | 64 QAM | ≤ 6.5 | ≤ 6.5 | ≤ 5 |
|  | 256 QAM | ≤ 9.5 | ≤ 9.5 | ≤ 9.5 |

Table 6.2.2.3-4a Maximum power reduction (MPR) for power class 1.5 with dual Tx

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 6 | ≤ 1.5 | ≤ 0 |
|  | QPSK | ≤ 6.5 | ≤ 2 | ≤ 0 |
|  | 16 QAM | ≤ 6.5 | ≤ 3 | ≤ 1 |
|  | 64 QAM | ≤ 6.5 | ≤ 3.5 | ≤ 3 |
|  | 256 QAM | ≤ 6.5 | ≤ 5.5 | ≤ 5.5 |
| CP-OFDM | QPSK | ≤ 6.5 | ≤ 4 | ≤ 1.5 |
|  | 16 QAM | ≤ 6.5 | ≤ 4 | ≤ 2 |
|  | 64 QAM | ≤ 6.5 | ≤ 4.5 | ≤ 4 |
|  | 256 QAM | ≤ 7.5 | ≤ 7.5 | ≤ 7.5 |
| NOTE 1: This table is targeted to large FWA form factor with 20 dB or above antenna isolation. |  |  |  |  |

Table 6.2.2-5: Maximum power reduction (MPR) for power class 1 for Band n14

| Modulation |  | MPR (dB) |  |  |
| --- | --- | --- | --- | --- |
|  |  | Edge RB allocations | Outer RB allocations | Inner RB allocations |
| DFT-s-OFDM | Pi/2 BPSK | ≤ 0.5 | ≤ 0.5 | 0 |
|  | Pi/2 BPSK w Pi/2 BPSK DMRS | ≤ 0.5 | ≤ 0 | 0 |
|  | QPSK | ≤ 1 |  | 0 |
|  | 16 QAM | ≤ 2 |  | ≤ 1 |
|  | 64 QAM | ≤ 2.5 |  |  |
|  | 256 QAM | ≤ 4.5 |  |  |
| CP-OFDM | QPSK | ≤ 3 |  | ≤ 1.5 |
|  | 16 QAM | ≤ 3 |  | ≤ 2 |
|  | 64 QAM | ≤ 3.5 |  |  |
|  | 256 QAM | ≤ 6.5 |  |  |

Where the following parameters are defined to specify valid RB allocation ranges for Outer and Inner RB allocations:

NRB is the maximum number of RBs for a given Channel bandwidth and sub-carrier spacing defined in Table 5.3.2-1.

RBStart,Low = max(1, floor(LCRB/2))

where max() indicates the largest value of all arguments and floor(x) is the greatest integer less than or equal to x.

RBStart,High = NRB – RBStart,Low – LCRB

The RB allocation is an Inner RB allocation if the following conditions are met:

RBStart,Low ≤ RBStart ≤ RBStart,High, and

LCRB ≤ ceil(NRB/2)

where ceil(x) is the smallest integer greater than or equal to x.

An Edge RB allocation is the one for which the RB(s) is (are) allocated at the lowermost or uppermost edge of the channel with LCRB ≤ 2 RBs.

The RB allocation is an Outer RB allocation for all other allocations which are not an Inner RB allocation or Edge RB allocation.

If CP-OFDM allocation satisfies following conditions, it is considered as almost contiguous allocation

NRB_gap / (NRB_alloc + NRB_gap) ≤ 0.25

and NRB_alloc + NRB_gap is larger than 106, 51 or 24 RBs for 15 kHz, 30 kHz or 60 kHz respectively where NRB_gap is the total number of unallocated RBs between allocated RBs and NRB_alloc is the total number of allocated RBs. The size and location of allocated and unallocated RBs are restricted by RBG parameters specified in sub-clause 6.1.2.2 of TS 38.214 [12]. For these almost contiguous signals in power class 2 and 3, the allowed maximum power reduction defined in Table 6.2.2.3-1 is increased by

CEIL{ 10 log10(1 + NRB_gap / NRB_alloc), 0.5 } dB,

where CEIL{x,0.5} means x rounding upwards to closest 0.5dB. The parameters of RBStart,Low and RBStart,High to specify valid RB allocation ranges for Outer and Inner RB allocations are defined as following:

RBStart,Low = max(1, floor((NRB_alloc + NRB_gap)/2))

RBStart,High = NRB – RBStart,Low – NRB_alloc –NRB_gap

For the UE maximum output power modified by MPR, the power limits specified in subclause 6.2.4 apply.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.2.2.

6.2.2.4	Test description

6.2.2.4.1	Initial conditions

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, channel bandwidths and sub-carrier spacing based on NR operating bands specified in table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2b and Table 6.2.2.4.1-3. The details of the uplink reference measurement channels (RMCs) are specified in Annexes A.2. Configurations of PDSCH and PDCCH before measurement are specified in Annex C.2.

Table 6.2.2.4.1-1: Test Configuration Table for power class 3 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 13 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 23 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 33 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 43 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 54 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 64 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 74 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 84 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 10 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 13 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 17 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 18 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 19 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 20 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 21 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 22 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 23 | Default |  | CP-OFDM QPSK | Inner Full |
| 24 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 25 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM QPSK | Outer Full |
| 27 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 28 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 29 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 30 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 31 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 32 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 33 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 374,5 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 384,5 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 394,5 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. NOTE 3: UE operating in TDD mode with Pi/2 BPSK modulation and UE indicates support for UE capability powerBoosting-pi2BPSK and the IE powerBoostPi2BPSK is set to 1 for bands n40, n41, n77, n78 and n79. NOTE 4: UE operating in FDD mode, or in TDD mode in bands other than n40, n41, n77, n78 and n79, or in TDD mode the IE powerBoostPi2BPSK is set to 0 for bands n40, n41, n77, n78 and n79. NOTE 5: Applicable to UEs indicating support for UE capability lowPAPR-DMRS-PUSCHwithPrecoding-r16.. |  |  |  |  |

Table 6.2.2.4.1-1a: Void

Table 6.2.2.4.1-2: Test Configuration Table for power class 2 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |

Table 6.2.2.4.1-2a: Test Configuration Table for power class 1 for Band n14 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default |  | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 14 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 15 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 16 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 17 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 18 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 19 | Default |  | CP-OFDM QPSK | Inner Full |
| 20 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 21 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 22 | Default |  | CP-OFDM QPSK | Outer Full |
| 23 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 24 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 25 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 26 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 27 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 28 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 29 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 30 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 256 QAM | Outer Full |
| 333 | Low |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Left |
| 343 | High |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Edge_1RB_Right |
| 353 | Default |  | DFT-s-OFDM Pi/2 BPSK w Pi/2 BPSK DMRS | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for Ues which supports half Pi BPSK in FR1. NOTE 3: Ues supporting pi/2 BPSK DMRS and the corresponding IE [DMRSPi2BPSK] is set to 1. |  |  |  |  |

Table 6.2.2.4.1-2b: Test Configuration Table for power class 1.5 (contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Lowest, Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation (NOTE 2) | RB allocation (NOTE 1) |
| 1 | Default | Reduction (MPR) test case | DFT-s-OFDM Pi/2 BPSK | Inner Full |
| 2 | Low |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Left |
| 3 | High |  | DFT-s-OFDM Pi/2 BPSK | Edge_1RB_Right |
| 4 | Default |  | DFT-s-OFDM Pi/2 BPSK | Outer Full |
| 5 | Default |  | DFT-s-OFDM QPSK | Inner Full |
| 6 | Low |  | DFT-s-OFDM QPSK | Edge_1RB_Left |
| 7 | High |  | DFT-s-OFDM QPSK | Edge_1RB_Right |
| 8 | Default |  | DFT-s-OFDM QPSK | Outer Full |
| 9 | Default |  | DFT-s-OFDM 16 QAM | Inner Full |
| 10 | Low |  | DFT-s-OFDM 16 QAM | Edge_1RB_Left |
| 11 | High |  | DFT-s-OFDM 16 QAM | Edge_1RB_Right |
| 12 | Default |  | DFT-s-OFDM 16 QAM | Outer Full |
| 13 | Default |  | DFT-s-OFDM_64_QAM | Inner Full |
| 14 | Low |  | DFT-s-OFDM 64 QAM | Edge_1RB_Left |
| 15 | High |  | DFT-s-OFDM 64 QAM | Edge_1RB_Right |
| 16 | Default |  | DFT-s-OFDM 64 QAM | Outer Full |
| 17 | Default |  | DFT-s-OFDM_256_QAM | Inner Full |
| 18 | Low |  | DFT-s-OFDM 256 QAM | Edge_1RB_Left |
| 19 | High |  | DFT-s-OFDM 256 QAM | Edge_1RB_Right |
| 20 | Default |  | DFT-s-OFDM 256 QAM | Outer Full |
| 21 | Default |  | CP-OFDM QPSK | Inner Full |
| 22 | Low |  | CP-OFDM QPSK | Edge_1RB_Left |
| 23 | High |  | CP-OFDM QPSK | Edge_1RB_Right |
| 24 | Default |  | CP-OFDM QPSK | Outer Full |
| 25 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 26 | Low |  | CP-OFDM 16 QAM | Edge_1RB_Left |
| 27 | High |  | CP-OFDM 16 QAM | Edge_1RB_Right |
| 28 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 29 | Default |  | CP-OFDM 64 QAM | Inner Full |
| 30 | Low |  | CP-OFDM 64 QAM | Edge_1RB_Left |
| 31 | High |  | CP-OFDM 64 QAM | Edge_1RB_Right |
| 32 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 33 | Default |  | CP-OFDM 256 QAM | Inner Full |
| 34 | Low |  | CP-OFDM 256 QAM | Edge_1RB_Left |
| 35 | High |  | CP-OFDM 256 QAM | Edge_1RB_Right |
| 36 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.1-1. NOTE 2: DFT-s-OFDM Pi/2 BPSK test applies only for UEs which supports half Pi BPSK in FR1. |  |  |  |  |

Table 6.2.2.4.1-3: Test Configuration Table for power class 2&3 (almost contiguous allocation)

| Initial Conditions |  |  |  |  |
| --- | --- | --- | --- | --- |
| Test Environment as specified in TS 38.508-1 [5] subclause 4.1 |  |  | Normal, TL/VL, TL/VH, TH/VL, TH/VH |  |
| Test Frequencies as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Low range, High range |  |
| Test Channel Bandwidths as specified in TS 38.508-1 [5] subclause 4.3.1 |  |  | Highest |  |
| Test SCS as specified in Table 5.3.5-1 |  |  | Lowest, Highest |  |
| Test Parameters for Channel Bandwidths |  |  |  |  |
| Test ID | Freq | Downlink Configuration | Uplink Configuration |  |
|  |  | N/A for Maximum Power | Modulation | RB allocation (NOTE 1) |
| 1 | Default |  | CP-OFDM QPSK | Inner Full |
| 2 | Default |  | CP-OFDM QPSK | Outer Full |
| 3 | Default |  | CP-OFDM 16 QAM | Inner Full |
| 4 | Default |  | CP-OFDM 16 QAM | Outer Full |
| 5 | Default |  | CP-OFDM 64 QAM | Outer Full |
| 6 | Default |  | CP-OFDM 256 QAM | Outer Full |
| NOTE 1: The specific configuration of each RB allocation is defined in Table 6.2.2.4.1-4. NOTE 2: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |

Table 6.2.2.4.1-4: Uplink configuration for almost contiguous allocation

| Channel Bandwidth（MHz） | SCS(kHz) | OFDM | Outer Full |  | Inner Full |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cluster1 RB allocations | Cluster2 RB allocations | Cluster1 RB allocations | Cluster2 RB allocations |
|  |  |  | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) | (LCRB @ RBstart) |
| 25 | 15 | CP | 48@0 | 53@80 | N/A | N/A |
|  | 30 | CP | 24@0 | 25@40 | N/A | N/A |
|  | 60 | CP | 12@0 | 13@18 | N/A | N/A |
| 30 | 15 | CP | 64@0 | 64@96 | N/A | N/A |
|  | 30 | CP | 32@0 | 30@48 | N/A | N/A |
|  | 60 | CP | 16@0 | 14@24 | N/A | N/A |
| 40 | 15 | CP | 80@0 | 88@128 | N/A | N/A |
|  | 30 | CP | 40@0 | 42@64 | N/A | N/A |
|  | 60 | CP | 20@0 | 19@32 | 12@12 | 8@28 |
| 50 | 15 | CP | 96@0 | 110@160 | 48@64 | 48@144 |
|  | 30 | CP | 48@0 | 53@80 | 24@32 | 24@72 |
|  | 60 | CP | 24@0 | 25@40 | 12@16 | 12@36 |
| 60 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 64@0 | 66@96 | 32@32 | 16@80 |
|  | 60 | CP | 32@0 | 31@48 | 16@16 | 8@40 |
| 80 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 80@0 | 89@128 | 32@32 | 16@80 |
|  | 60 | CP | 40@0 | 43@64 | 16@16 | 8@40 |
| 90 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 96@0 | 101@144 | 32@32 | 16@80 |
|  | 60 | CP | 48@0 | 49@72 | 16@16 | 8@40 |
| 100 | 15 | CP | N/A | N/A | N/A | N/A |
|  | 30 | CP | 112@0 | 97@176 | 48@64 | 48@144 |
|  | 60 | CP | 48@0 | 55@80 | 24@32 | 24@72 |

1.	Connect the SS to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure A.3.1.2.1 for TE diagram and section A.3.2 for UE diagram.

2.	The parameter settings for the cell are set up according to TS 38.508-1 [5] subclause 4.4.3.

3.	Downlink signals are initially set up according to Annex C.0, C.1, C.2, and uplink signals according to Annex G.0, G.1, G.2, G.3.0.

4.	The UL Reference Measurement Channel is set according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a and Table 6.2.2.4.1-3.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in State RRC_CONNECTED with generic procedure parameters Connectivity NR, Connected without release On, Test Mode On and Test Loop Function On according to TS 38.508-1 [5] clause 4.5. Message contents are defined in clause 6.2.2.4.3.

6.2.2.4.2	Test procedure

1.	SS sends uplink scheduling information for each UL HARQ process via PDCCH DCI format 0_1 for C_RNTI to schedule the UL RMC according to Table 6.2.2.4.1-1, Table 6.2.2.4.1-2, Table 6.2.2.4.1-2a, Table 6.2.2.4.1-2b and Table 6.2.2.4.1-3. Since the UE has no payload and no loopback data to send the UE sends uplink MAC padding bits on the UL RMC.

2.	Send continuously uplink power control "up" commands in every uplink scheduling information to the UE; allow at least 200ms for the UE to reach PUMAX level.

3.	Measure the mean power of the UE in the channel bandwidth of the radio access mode. For PC1.5 the measured power is the sum of the two ports. The period of measurement shall be at least the continuous duration of 1ms over consecutive active uplink slots. For TDD, only slots consisting of only UL symbols are under test.

4.	For UEs supporting Power Class 1 in Band n14, Power Class 1.5 and Power Class 2, repeat steps 1~3 for Test ID 22 and 36 in Table 6.2.2.4.1-1 on the applicable bands with message exception of P-Max defined in Table 6.2.2.4.3-2.

5.	For UEs supporting Power Class 1.5, repeat steps 1~3 for Test ID 18 and 32 in Table 6.2.2.4.1-2 on the applicable bands with message exception of P-Max defined in Table 6.2.2.4.3-2a.

NOTE 1: When switching to DFT-s-OFDM waveform, as specified in the test configuration Table 6.2.2.4.1-1, Table 6.2.2.4.1-2 and Table 6.2.2.4.1-2b, send an NR RRCReconfiguration message according to TS 38.508-1 [5] clause 4.6.3 Table 4.6.3-118 PUSCH-Config with TRANSFORM_PRECODER_ENABLED condition.

6.2.2.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

Table 6.2.2.4.3-1: PUSCH-Config

| Derivation Path: TS 38.508-1 [5] subclause 4.6.3 Table 4.6.3-118 PUSCH-Config |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| PUSCH-Config ::= SEQUENCE { |  |  |  |
| resourceAllocation | resourceAllocationType0 |  | Almost contiguous allocation |
|  | resourceAllocationType1 |  | Contiguous allocation |
| } |  |  |  |

Table 6.2.2.4.3-2: P-Max (Step 4)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 23 |  | PC2 UE or PC1.5 UE or PC1 UE |

Table 6.2.2.4.3-2a: P-Max (Step 5)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-89 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| P-Max | 26 |  | PC1.5 UE |

Table 6.2.2.4.3-3: DMRS-UplinkConfig (Test ID 37 - 39 in Table 6.2.2.4.1-1)

| Derivation Path: TS 38.508-1 [5], Table 4.6.3-51 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| DMRS-UplinkConfig ::= SEQUENCE { |  |  |  |
| transformPrecodingEnabled SEQUENCE { |  |  |  |
| dmrs-UplinkTransformPrecoding-r16 SEQUENCE { |  |  |  |
| pi2BPSK-ScramblingID0 | Not present |  |  |
| pi2BPSK-ScramblingID1 | Not present |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

Table 6.2.2.4.3-4: ServingCellConfig

| Derivation Path: TS 38.508-1 [5] Table 4.6.3-167 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| ServingCellConfig ::= SEQUENCE { |  |  |  |
| uplinkConfig SEQUENCE { |  |  |  |
| powerBoostPi2BPSK | 1 |  | Test IDs where NOTE 3 in Table 6.2.2.4.1-1 applies. |
|  | 0 |  | Test IDs where NOTE 4 in Table 6.2.2.4.1-1 applies. |
| } |  |  |  |
| } |  |  |  |

6.2.2.5	Test requirement

The maximum output power, derived in step 3 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 to Table 6.2.2.5-9a.

The maximum output power, derived in step 4 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-1 and Table 6.2.2.5-3.

The maximum output power, derived in step 5 shall be within the range prescribed by the nominal maximum output power and tolerance in Table 6.2.2.5-4

Table 6.2.2.5-1a: UE Power Class test requirements Band 24 for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | （4.52） | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 25.0 + TT | 20.5 - TT | （16.5 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | （4.52） | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | （4.52） | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | （4.52） | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | （4.52） | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 3 | （4.52） | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | （4.52） | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | （4.52） | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | （4.52） | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | （4.52） | 25.0 + TT | 17.5 – TT | （14.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | （4.52） | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 3 | （4.52） | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | （4.52） | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | （4.52） | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 3 | （4.52） | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | （4.52） | 25.0 + TT | 18.5 - TT | （15.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | （4.52） | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | （4.52） | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | （4.52） | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 3 | （4.52） | 25.0 + TT | 18.0 - TT | （15.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | （4.52） | 25.0 + TT | 17.0 – TT | （14.0 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | （4.52） | 25.0 + TT | 17.0 – TT | （14.0 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 3 | （4.52） | 25.0 + TT | 17.0 - TT | （14.0 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | （4.52） | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 16.0 - TT | （13.5 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 19.5 - TT | （16.5 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.0 - TT | （17.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: Transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-1: UE Power Class test requirements(for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | （1.52） | 25.8 | （24.32） | 2.0 |  | 2 | （3.52） | 28.0 + TT | 23.8 - TT | （22.3 - TT2） |
| 2 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 3 | 23 | -3 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 4 | 23 | -3 | 1.2 | 0 | （1.52） | 24.8 | （23.32） | 2.0 |  | 2 | （3.52） | 28.0 + TT | 22.8 - TT | （21.3 - TT2） |
| 5 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 6 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 7 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 8 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 9 | 23 | 0 | 0 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 10 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 11 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 12 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 13 | 23 | 0 | 1 | 0 | （1.52） | 22.0 | （20.52） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 20.0 - TT | （18.0 - TT2） |
| 14 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | （3.52） | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 15 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | （3.52） | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 16 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | （3.52） | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 17 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | （3.52） | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 18 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | （3.52） | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 19 | 23 | 0 | 2.5 | 0 | （1.52） | 20.5 | （19.02） | 2.5 | （3.52） | 2 | （3.52） | 25.0 + TT | 18.0 - TT | （15.5 - TT2） |
| 20 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | （3.52） | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 21 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | （3.52） | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 22 | 23 | 0 | 4.5 | 0 | （1.52） | 18.5 | （17.02） | 4.0 | （5.02） | 2 | （3.52） | 25.0 + TT | 14.5 - TT | （12.0 - TT2） |
| 23 | 23 | 0 | 1.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 2 | （3.52） | 25.0 + TT | 19.5 - TT | （17.5 - TT2） |
| 24 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 25 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 26 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 27 | 23 | 0 | 2 | 0 | （1.52） | 21.0 | （19.52） | 2.0 | （3.52） | 2 | （3.52） | 25.0 + TT | 19.0 - TT | （16.0 - TT2） |
| 28 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 29 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 30 | 23 | 0 | 3 | 0 | （1.52） | 20.0 | （18.52） | 2.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 17.5 - TT | （14.5 - TT2） |
| 31 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 33 | 23 | 0 | 3.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 2 | （3.52） | 25.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 34 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 35 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 36 | 23 | 0 | 6.5 | 0 | （1.52） | 16.5 | （15.02） | 5.0 |  | 2 | （3.52） | 25.0 + TT | 11.5 - TT | （10.0 - TT2） |
| 37 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 - TT | （19 - TT2） |
| 38 | 23 | 0 | 0.5 | 0 | （1.52） | 22.5 | （212） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 20.5 - TT | （19 - TT2） |
| 39 | 23 | 0 | 0 | 0 | （1.52） | 23 | （21.52） | 2.0 |  | 2 | （3.52） | 25.0 + TT | 21 - TT | （19.5 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18.0 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 2.5 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 2.5 | 25.0 + TT | 19.0 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 2.5 | 25.0 + TT | 18.5 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 2.5 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20.0 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 2.5 | 25.0 + TT | 20.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-2a: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 5 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 6 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 7 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 8 | 23 | 0 | 1 | 0 | 22 | 2 | 2.5 | 25.0 + TT | 19.5-TT |
| 9 | 23 | 0 | 0.5 | 0 | 22.5 | 2 | 2.5 | 25.0 + TT | 20-TT |
| 10 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 11 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 12 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 13 | 23 | 0 | 1.5 | 0 | 21.5 | 2 | 2.5 | 25.0 + TT | 19-TT |
| 14 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 15 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 16 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 17 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 18 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 19 | 23 | 0 | 3 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5-TT |
| 20 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 21 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 22 | 23 | 0 | 5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14-TT |
| 23 | 23 | 0 | 2 | 0 | 21 | 2 | 2.5 | 25.0 + TT | 18.5-TT |
| 24 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 25 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 26 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 27 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18-TT |
| 28 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 29 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 30 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16-TT |
| 31 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 32 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 33 | 23 | 0 | 4 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5-TT |
| 34 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 35 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 36 | 23 | 0 | 7 | 0 | 16 | 5 | 2.5 | 25.0 + TT | 11-TT |
| 37 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 1 | 0 | 22 | 2.0 | 2.5 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 2.5 | 25.0 + TT | 20 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-3: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 23 | -3 | 0.2 | 0 | 25.8 | 2.0 | 3 | 28.0 + TT | 22.8 - TT |
| 2 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 3 | 23 | -3 | 3.5 | 0 | 22.5 | 2.0 | 3 | 28.0 + TT | 19.5 - TT |
| 4 | 23 | -3 | 1.2 | 0 | 24.8 | 2.0 | 3 | 28.0 + TT | 21.8 - TT |
| 5 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 6 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 7 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 8 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 9 | 23 | 0 | 0 | 0 | 23.0 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| 10 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 11 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 12 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 13 | 23 | 0 | 1 | 0 | 22.0 | 2.0 | 3 | 25.0 + TT | 19.0 - TT |
| 14 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 15 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 16 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 17 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 18 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 19 | 23 | 0 | 2.5 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 20 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 21 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 22 | 23 | 0 | 4.5 | 0 | 18.5 | 4.0 | 3 | 25.0 + TT | 14.5 - TT |
| 23 | 23 | 0 | 1.5 | 0 | 21.5 | 2.0 | 3 | 25.0 + TT | 18.5 - TT |
| 24 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 25 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 26 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 27 | 23 | 0 | 2 | 0 | 21.0 | 2.0 | 3 | 25.0 + TT | 18.0 - TT |
| 28 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 29 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 30 | 23 | 0 | 3 | 0 | 20.0 | 2.5 | 3 | 25.0 + TT | 17.0 - TT |
| 31 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 32 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 33 | 23 | 0 | 3.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16.0 - TT |
| 34 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 35 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 36 | 23 | 0 | 6.5 | 0 | 16.5 | 5.0 | 3 | 25.0 + TT | 11.5 - TT |
| 37 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 38 | 23 | 0 | 0.5 | 0 | 22.5 | 2.0 | 3 | 25.0 + TT | 19.5 - TT |
| 39 | 23 | 0 | 0 | 0 | 23 | 2.0 | 3 | 25.0 + TT | 20.0 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4: UE Power Class test requirements (for Bands n41, n77, n78, n79) for Power Class 2 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 2 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 3 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 4 | 26 | 0 | 0.5 | 0 | （1.52） | 25.5 | （24.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 22.5 - TT | （21.0 - TT2） |
| 5 | 26 | 0 | 0 | 0 | （1.52） | 26.0 | （24.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 23.0 - TT | （21.5 - TT2） |
| 6 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 7 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 8 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 9 | 26 | 0 | 1 | 0 | （1.52） | 25.0 | （23.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 22.0 - TT | （20.5 - TT2） |
| 10 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 11 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 12 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 13 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 14 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0- TT2） |
| 15 | 26 | 0 | 2.5 | 0 | （1.52） | 23.5 | （22.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 20.5 - TT | （19.0 - TT2） |
| 16 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | （4.52） | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 17 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | （4.52） | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 18 | 26 | 0 | 4.5 | 0 | （1.52） | 21.5 | （20.02） | 2.0 | （2.52） | 3 | （4.52） | 28.0 + TT | 18.5 - TT | （17.0 - TT2） |
| 19 | 26 | 0 | 1.5 | 0 | （1.52） | 24.5 | （23.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 21.5 - TT | （20.0 - TT2） |
| 20 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 21 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 22 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 23 | 26 | 0 | 2 | 0 | （1.52） | 24.0 | （22.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 21.0 - TT | （19.5 - TT2） |
| 24 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 25 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 26 | 26 | 0 | 3 | 0 | （1.52） | 23.0 | （21.52） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 20.0 - TT | （18.5 - TT2） |
| 27 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 28 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 29 | 26 | 0 | 3.5 | 0 | （1.52） | 22.5 | （21.02） | 2.0 |  | 3 | （4.52） | 28.0 + TT | 19.5 - TT | （18.0 - TT2） |
| 30 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | （4.52） | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 31 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | （4.52） | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| 32 | 26 | 0 | 6.5 | 0 | （1.52） | 19.5 | （18.02） | 3.5 | （4.02） | 3 | （4.52） | 28.0 + TT | 16.0 - TT | （14.0 - TT2） |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4a: UE Power Class test requirements for Band n14 for Power Class 1 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 2 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 3 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 4 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 5 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| 6 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 7 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 8 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 9 | 31 | 0 | 1 | 0 | 30.0 | 2.0 | 3 | 33.0 + TT | 27.0 — TT |
| 10 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 11 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 12 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 13 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 14 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 15 | 31 | 0 | 2.5 | 0 | 28.5 | 2.0 | 3 | 33.0 + TT | 25.5 — TT |
| 16 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 17 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 18 | 31 | 0 | 4.5 | 0 | 26.5 | 2.0 | 3 | 33.0 + TT | 23.5 — TT |
| 19 | 31 | 0 | 1.5 | 0 | 29.5 | 2.0 | 3 | 33.0 + TT | 26.5 — TT |
| 20 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 21 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 22 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 23 | 31 | 0 | 2 | 0 | 29.0 | 2.0 | 3 | 33.0 + TT | 26.0 — TT |
| 24 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 25 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 26 | 31 | 0 | 3 | 0 | 28.0 | 2.0 | 3 | 33.0 + TT | 25.0 — TT |
| 27 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 28 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 29 | 31 | 0 | 3.5 | 0 | 27.5 | 2.0 | 3 | 33.0 + TT | 24.5 — TT |
| 30 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 31 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 32 | 31 | 0 | 6.5 | 0 | 24.5 | 2.0 | 3 | 33.0 + TT | 21.5 — TT |
| 33 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 34 | 31 | 0 | 0.5 | 0 | 30.5 | 2.0 | 3 | 33.0 + TT | 27.5 — TT |
| 35 | 31 | 0 | 0 | 0 | 31.0 | 2.0 | 3 | 33.0 + TT | 28.0 — TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-4b: UE Power Class test requirements (for Band n41, n77, n78) for Power Class 1.5 (contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | (26.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 24.5 - TT | (23.0 - TT²) |
| 2 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 3 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 4 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 5 | 29 | 0 | 1.5 | 0 | （1.52） | 27.5 | (26.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 24.5 - TT | (23.0 - TT²) |
| 6 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 7 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 8 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | (23.5²) | 2 |  | 3 | （4.52） | 31.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 9 | 29 | 0 | 2.5 | 0 | （1.52） | 26.5 | (25.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 23.5 - TT | (22.0 - TT²) |
| 10 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 11 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 12 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | (22.5²) | 2 |  | 3 | （4.52） | 31.0 + TT | 21.0 - TT | (19.5 - TT²) |
| 13 | 29 | 0 | 4 | 0 | （1.52） | 25.0 | (23.5²) | 2 |  | 3 | （4.52） | 31.0 + TT | 22.0 - TT | (20.5 - TT²) |
| 14 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 15 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 16 | 29 | 0 | 5.5 | 0 | （1.52） | 23.5 | (22.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 20.5 - TT | (19.0 - TT²) |
| 17 | 29 | 0 | 7.5 | 0 | （1.52） | 21.5 | (20.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 18.5 - TT | (17.0 - TT²) |
| 18 | 29 | 0 | 7.5 | 0 | （1.52） | 21.5 | (20.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 18.5 - TT | (17.0 - TT²) |
| 19 | 29 | 0 | 7.5 | 0 | （1.52） | 21.5 | (20.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 18.5 - TT | (17.0 - TT²) |
| 20 | 29 | 0 | 7.5 | 0 | （1.52） | 21.5 | (20.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 18.5 - TT | (17.0 - TT²) |
| 21 | 29 | 0 | 3 | 0 | （1.52） | 26.0 | (24.5²) | 2 |  | 3 | （4.52） | 31.0 + TT | 23.0 - TT | (21.5 - TT²) |
| 22 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 23 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 24 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 2 |  | 3 | （4.52） | 31.0 + TT | 20.0 - TT | (18.5 - TT²) |
| 25 | 29 | 0 | 3.5 | 0 | （1.52） | 25.5 | (24.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 22.5 - TT | (21.0 - TT²) |
| 26 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 27 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 28 | 29 | 0 | 6 | 0 | （1.52） | 23.0 | (21.5²) | 2 |  | 3 | （4.52） | 31.0 + TT | 20.0 - TT | (18.5 - TT²) |
| 29 | 29 | 0 | 5 | 0 | （1.52） | 24.0 | (22.5²) | 2 |  | 3 | （4.52） | 31.0 + TT | 21.0 - TT | (19.5 - TT²) |
| 30 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 31 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 32 | 29 | 0 | 6.5 | 0 | （1.52） | 22.5 | (21.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 19.5 - TT | (18.0 - TT²) |
| 33 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | (18.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 16.5 - TT | (15.0 - TT²) |
| 34 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | (18.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 16.5 - TT | (15.0 - TT²) |
| 35 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | (18.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 16.5 - TT | (15.0 - TT²) |
| 36 | 29 | 0 | 9.5 | 0 | （1.52） | 19.5 | (18.0²) | 2 |  | 3 | （4.52） | 31.0 + TT | 16.5 - TT | (15.0 - TT²) |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-5: Test Tolerance (Maximum Power Reduction (MPR))

|  | f ≤ 3.0GHz | 3.0GHz < f ≤ 4.2GHz | 4.2GHz < f ≤ 6.0GHz |
| --- | --- | --- | --- |
| BW ≤ 40MHz | 0.7 dB | 1.0 dB | 1.0 dB |
| 40MHz < BW ≤ 100MHz | 1.0 dB | 1.0 dB | 1.0 dB |

Table 6.2.2.5-6: UE Power Class test requirements (for Bands n1, n2, n3, n5, n7, n8, n12, n14, n20, n25, n26, n30, n34, n38, n39, n40, n41, n50, n51, n53, n65, n66, n70, n74) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 23 | 0 | 1.5 | 1.5 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 3.52 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 15 | 23 | 0 | 1.5 | 1 | 0 | 1.52 | 20.5 | 192 | 2.5 | 3.52 | 2 | 3.52 | 25.0 + TT | 18 - TT | 15.5 - TT2 |
| 26 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 3.52 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 27 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 3.52 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 34 | 23 | 0 | 2 | 1.5 | 0 | 1.52 | 19.5 | 182 | 3.5 | 42 | 2 | 3.52 | 25.0 + TT | 16 - TT | 14 - TT2 |
| 35 | 23 | 0 | 2 | 1 | 0 | 1.52 | 20 | 18.52 | 2.5 | 42 | 2 | 3.52 | 25.0 + TT | 17.5 - TT | 14.5 - TT2 |
| 46 | 23 | 0 | 3 | 1.5 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 3.52 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 47 | 23 | 0 | 3 | 1 | 0 | 1.52 | 19 | 17.52 | 3.5 | 52 | 2 | 3.52 | 25.0 + TT | 15.5 - TT | 12.5 - TT2 |
| 56 | 23 | 0 | 3.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 2 | 3.52 | 25.0 + TT | 14 - TT | 11.5 - TT2 |
| 57 | 23 | 0 | 3.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 2 | 3.52 | 25.0 + TT | 14.5 - TT | 12 - TT2 |
| 66 | 23 | 0 | 6.5 | 1.5 | 0 | 1.52 | 15 | 13.52 | 5 | 52 | 2 | 3.52 | 25.0 + TT | 10 - TT | 8.5- TT2 |
| 67 | 23 | 0 | 6.5 | 1 | 0 | 1.52 | 15.5 | 142 | 5 | 52 | 2 | 3.52 | 25.0 + TT | 10.5 - TT | 9 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n2, n3, n7, n8, n12, n20, n26, n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-6a: Void

Table 6.2.2.5-7: UE Power Class test requirements (for Bands n28 with channel bandwidth other than 30MHz, n71) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 2.5 | 25.0 + TT | 18 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 2.5 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 2.5 | 25.0 + TT | 17.5 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 2.5 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 2.5 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-7a: Void

Table 6.2.2.5-7b: UE Power Class test requirements (for Band n28 with 30MHz channel bandwidth) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 24 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 43 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 44 | 23 | 0 | 3.5 | 1 | 0 | 17.5 | 4 | 2.5 | 25.0 + TT | 14.5 - TT |
| 53 | 23 | 0 | 4 | 1.5 | 0 | 17.5 | 5 | 2.5 | 25.0 + TT | 12.5 - TT |
| 54 | 23 | 0 | 4 | 1 | 0 | 18 | 4 | 2.5 | 25.0 + TT | 14 - TT |
| 63 | 23 | 0 | 7 | 1.5 | 0 | 14.5 | 5 | 2.5 | 25.0 + TT | 9.5 - TT |
| 64 | 23 | 0 | 7 | 1 | 0 | 15 | 5 | 2.5 | 25.0 + TT | 10 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW=30MHz when SCS=60kHz. NOTE 4: Applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz when SCS=30kHz. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8: UE Power Class test requirements (for Bands n48, n77, n78, n79) for Power Class 3 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) | PCMAX_L,f,c (dBm) | T(PCMAX_L,f,c) (dB) | TL,c (dB) | Upper limit (dBm) | Lower limit (dBm) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 13 | 23 | 0 | 1.5 | 1.5 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 14 | 23 | 0 | 1.5 | 1 | 0 | 20.5 | 2.5 | 3 | 25.0 + TT | 17.5 - TT |
| 25 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 26 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 33 | 23 | 0 | 2 | 1.5 | 0 | 19.5 | 3.5 | 3 | 25.0 + TT | 16 - TT |
| 34 | 23 | 0 | 2 | 1 | 0 | 20 | 2.5 | 3 | 25.0 + TT | 17 - TT |
| 45 | 23 | 0 | 3 | 1.5 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 46 | 23 | 0 | 3 | 1 | 0 | 19 | 3.5 | 3 | 25.0 + TT | 15.5 - TT |
| 55 | 23 | 0 | 3.5 | 1.5 | 0 | 18 | 4 | 3 | 25.0 + TT | 14 - TT |
| 56 | 23 | 0 | 3.5 | 1 | 0 | 18.5 | 4 | 3 | 25.0 + TT | 14.5 - TT |
| 65 | 23 | 0 | 6.5 | 1.5 | 0 | 15 | 5 | 3 | 25.0 + TT | 10 - TT |
| 66 | 23 | 0 | 6.5 | 1 | 0 | 15.5 | 5 | 3 | 25.0 + TT | 10.5 - TT |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 3: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 4: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 5: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 6: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-8a: Void

Table 6.2.2.5-9: UE Power Class test requirements (for Bands n41, n77, n78, n79) for Power Class 2 (almost contiguous allocation)

| Test ID | PPowerClass (dBm) | ΔPPowerClass (dB) | MPR (dB) | MPR increase (dB) | ΔTC,c (dB) |  | PCMAX_L,f,c (dBm) |  | T(PCMAX_L,f,c) (dB) |  | TL,c (dB) |  | Upper limit (dBm) | Lower limit (dBm) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 14 | 26 | 0 | 1.5 | 1.5 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 4.52 | 28.0 + TT | 20 - TT | 17 - TT2 |
| 15 | 26 | 0 | 1.5 | 1 | 0 | 1.52 | 23.5 | 222 | 2 | 22 | 3 | 4.52 | 28.0 + TT | 20.5 - TT | 17.5 - TT2 |
| 26 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 18.5 - TT | 15.5 - TT2 |
| 27 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 19 - TT | 16 - TT2 |
| 34 | 26 | 0 | 2 | 1.5 | 0 | 1.52 | 22.5 | 212 | 2 | 22 | 3 | 4.52 | 28.0 + TT | 19.5 - TT | 16.5 - TT2 |
| 35 | 26 | 0 | 2 | 1 | 0 | 1.52 | 23 | 21.52 | 2 | 22 | 3 | 4.52 | 28.0 + TT | 20 - TT | 17 - TT2 |
| 46 | 26 | 0 | 3 | 1.5 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 18.5 - TT | 15.5 - TT2 |
| 47 | 26 | 0 | 3 | 1 | 0 | 1.52 | 22 | 20.52 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 19 - TT | 16 - TT2 |
| 56 | 26 | 0 | 3.5 | 1.5 | 0 | 1.52 | 21 | 19.52 | 2 | 3.52 | 3 | 4.52 | 28.0 + TT | 18 - TT | 15 - TT2 |
| 57 | 26 | 0 | 3.5 | 1 | 0 | 1.52 | 21.5 | 202 | 2 | 2.52 | 3 | 4.52 | 28.0 + TT | 18.5 - TT | 15.5 - TT2 |
| 66 | 26 | 0 | 6.5 | 1.5 | 0 | 1.52 | 18 | 16.52 | 4 | 52 | 3 | 4.52 | 28.0 + TT | 14 - TT | 11.5 - TT2 |
| 67 | 26 | 0 | 6.5 | 1 | 0 | 1.52 | 18.5 | 172 | 4 | 52 | 3 | 4.52 | 28.0 + TT | 14.5 - TT | 12 - TT2 |
| NOTE 1: PPowerClass is the maximum UE power specified without taking into account the tolerance. NOTE 2: For Band n41, transmission bandwidths confined within FUL_low and FUL_low + 4 MHz or FUL_high – 4 MHz and FUL_high. NOTE 3: TT for each frequency and channel bandwidth is specified in Table 6.2.2.5-5. NOTE 4: Applicable for CBW/SCS combinations other than CBW=40MHz when SCS=60kHz. NOTE 5: Only applicable for CBW 40MHz when SCS is 60kHz. NOTE 6: Applicable for CBW/SCS combinations other than CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 7: Only applicable for CBW=30MHz when SCS=15kHz and CBW=30MHz, 60MHz, 90MHz when SCS=30kHz and CBW=25MHz, 60MHz, 90MHz when SCS=60kHz. NOTE 8: Test applies only for UEs which support almost contiguous UL CP-OFDM transmissions. For PC2 UE which support almost contiguous UL CP-OFDM transmissions, test is only applicable for Release 16 and forward. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table 6.2.2.5-9a: Void

### <End of Changes>