3GPP TSG-RAN WG5 Meeting #93-e	R5-216597r1

# Electronic Meeting, 8th – 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.521-1 | CR | 1402 | rev | - | Current version: | 17.2.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update to test case 6.4E.2.4.1D In-band emissions for V2X / non-concurrent operation / SL-MIMO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MTCC, KTL |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Some information is incorrect |  |  |  |  |  |  |  |  |
| Summary of change: |  | The name of test case is incorrect |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case for 5G V2X will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.4E.2.4.1D |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 38.522 CR |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1 - Corrected editorial mistakes in Test procedure. |  |  |  |  |  |  |  |  |

<< Start of changes >>

6.4E.2.4.1D	In-band emissions for V2X / non-concurrent operation / SL-MIMO

Editor’s Note:

- The test case is not completed due to the following aspects are not yet determined:

- Uplink RMC is TBD in RAN4

- Connection diagram is TBD

- Preconfiguration is TBD in 38.508-1

- Test state and generic procedure are TBD in 38.508-1

6.4E.2.4.1D.1	Test purpose

The in-band emissions are a measure of the interference falling into the non-allocated resources blocks, this is to verify that the in-band emissions of V2X sidelink non-concurrent operation satisfy the minimum requirements.

6.4E.2.4.1D.2	Test applicability

This test case applies to all types of NR UE release 16 and forward that support NR V2X sidelink MIMO communication with non-concurrent operation.

NOTE: This test case can’t be performed due to lack of appropriate test points.

6.4E.2.4.1D.3	Minimum conformance requirements

For NR V2X UE with two transmit antenna connectors, the requirements specified for single carrier described in clause 6.4E.2.4.1.3 shall apply to each transmit antenna connector.

The normative reference for this requirement is TS 38.101-1 [2] clause 6.4E.2.4.

6.4E.2.4.1D.4	Test description

6.4E.2.4.1D.4.1	Initial condition

Initial conditions are a set of test configurations the UE needs to be tested in and the steps for the SS to take with the UE to reach the correct measurement state.

The initial test configurations consist of environmental conditions, test frequencies, test channel bandwidths and sub-carrier spacing based on NR operating bands specified in Table 5.3.5-1. All of these configurations shall be tested with applicable test parameters for each combination of test channel bandwidth and sub-carrier spacing, and are shown in table 6.4E.2.4.1D.4.1-1 and 6.4E.2.4.1D.4.1-2. The details of the V2X reference measurement channels (RMCs) are specified in Annexes TBD.

Table 6.4E.2.4.1D.4.1-1: Test Configuration Table for PSSCH and PSCCH

FFS

Table 6.4E.2.4.1D.4.1-2: Test Configuration Table for PSBCH

FFS

1.	Connect the SS and GNSS simulator to the UE antenna connectors as shown in TS 38.508-1 [5] Annex A, Figure [TBD] for TE diagram and section [TBD] for UE diagram.

2.	The parameter settings for the V2X sidelink transmission over PC5 are pre-configured according to TS 38.508-1 [5] subclause [TBD]. Message content exceptions are defined in clause 6.4E.2.4.1D.4.3.

3.	The V2X Reference Measurement Channel is set according to Table 6.4E.2.4.1D.4.1-1.

4.	The GNSS simulator is configured for Scenario #1: static in Geographical area #1, as defined in TS38.508-1 [5] Table [TBD]. Geographical area #1 is also pre-configured in the UE.

5.	Propagation conditions are set according to Annex B.0.

6.	Ensure the UE is in state [TBD].

6.4E.2.4.1D.4.2	Test procedure

Test procedure for PSCCH+PSSCH:

1.	Set the V2X sidelink communication parameters for both the V2X sidelink capable UE (hereinafter referred to as V2X UE) and SS according to SL-PreconfigurationNR in [TBD].

2.	V2X UE schedules the V2X RMC according to SL-PreconfigurationNR which is in line with the test configuration in Table 6.4E.2.4.1D.4.1-1.

3.	Measure the V2X UE output power to make sure V2X UE transmission power to be [TBD].

4.	Measure In-band emission using Global In-Channel Tx-Test (Annex E) for each of transmit antenna.

Test procedure for PSBCH:

1.	Set the V2X sidelink communication parameters for both the V2X sidelink capable UE (hereinafter referred to as V2X UE) and SS according to SL-PreconfigurationNR in [TBD].

2.	V2X UE schedules the V2X RMC according to SL-PreconfigurationNR which is in line with the test configuration in Table 6.4E.2.4.1D.4.1-2.

3.	Measure the V2X UE output power to make sure V2X UE transmission power to be [TBD].

4.	Measure In-band emission using Global In-Channel Tx-Test (Annex E) for each of transmit antenna.

6.4E.2.4.1D.4.3	Message contents

Message contents are according to TS 38.508-1 [5] subclause 4.6 and 5.4 with the following exceptions:

FFS

6.4E.2.4.1D.5	Test requirement

FFS

<< End of changes >>