3GPP TSG-RAN5 Meeting #93-e	R5-216613r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.523-1 | CR | 2499 | rev | - | Current version: | 16.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of new test case 8.1.6.4.1 for RACH logging and reporting in NR SON/MDT |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CMCC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_SON_MDT-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-26 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-16 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | Introduction of TC RACH logging and reporting in Rel-16 NR SON/MDT. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of new TC 8.1.6.4.1 SON/ RACH logging and reporting in Rel-16 NR SON/MDT. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The test purpose of the new feature could not be covered. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.1.6.4.1(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | r1: 1. Added the missing steps in test procedure. 2. Minimized Table 8.1.6.4.1.3.3-1 referring to a condition RA. 3. Added the missing IEs in message contents and updated some editorial issues. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<<< START OF CHANGES >>>

8.1.6.4	SON / RACH Optimisation

8.1.6.4.1	SON / RACH logging and reporting

8.1.6.4.1.1	Test Purpose (TP)

(1)

with { UE in NR RRC_CONNECTED state and supports delivery of rachReport and no contention resolution failure occurred for the last successfully completed random access procedure }

ensure that {
  when { UE receives UEInformationRequest message with ra-ReportReq set to true }

then { UE transmits UEInformationResponse message with RA-Report including contentionDetected set to false }

}

(2)

with { UE in NR RRC_CONNECTED state and supports delivery of rachReport and contention resolution was not successful for at least one of the transmitted preambles for the last successfully completed random access procedure }

ensure that {
  when { UE receives UEInformationRequest message with ra-ReportReq set to true }

then { UE transmits UEInformationResponse message with RA-Report including contentionDetected set to true }

}

8.1.6.4.1.2	Conformance requirements

References: The conformance requirements covered in the current TC is specified in: TS 38.331 clause 5.7.10.3, 5.7.10.4, 5.7.10.5.

[TS 38.331, clause 5.7.10.3]

Upon receiving the UEInformationRequest message, the UE shall, only after successful security activation:

…

1>	if ra-ReportReq is set to true and the UE has random access related information available in VarRA-Report and if the RPLMN is included in plmn-IdentityList stored in VarRA-Report:

2>	set the ra-ReportList in the UEInformationResponse message to the value of ra-ReportList in VarRA-Report;

2>	discard the ra-ReportList from VarRA-Report upon successful delivery of the UEInformationResponse message confirmed by lower layers;

…

[TS 38.331, clause 5.7.10.4]

Upon successfully performing 4 step random access procedure, the UE shall:

1>	if the RPLMN or the PLMN selected by upper layers (see TS24.501 [23]) from the PLMN(s) included in the plmn-IdentityList in SIB1 is not included in plmn-IdentityList stored in a non-empty VarRA-Report:

2>	clear the information included in VarRA-Report;

1>	if the number of RA-Report entries stored in the ra-ReportList in VarRA-Report is less than maxRAReport:

2>	if the number of PLMN entries in plmn-IdentityList stored in VarRA-Report is less than maxPLMN; or

2>	if the number of PLMN entries in plmn-IdentityList stored in VarRA-Report is equal to maxPLMN and the list of EPLMNs is subset of or equal to the plmn-IdentityList stored in VarRA-Report:

3>	append the following contents associated to the successfully completed random-access procedure as a new entry in the VarRA-Report:

4>	if the list of EPLMNs has been stored by the UE:

5>	set the plmn-IdentityList to include the list of EPLMNs stored by the UE (i.e. includes the RPLMN) without exceeding the limit of maxPLMN;

4>	else:

5>	set the plmn-Identity, in plmn-IdentityList, to the PLMN selected by upper layers (see TS 24.501 [23]) from the PLMN(s) included in the plmn-IdentityInfoList in SIB1;

4>	set the cellId to the global cell identity and the tracking area code, if available, otherwise to the physical cell identity and carrier frequency of the cell in which the corresponding random-access preamble was transmitted;

4>	set the raPurpose to include the purpose of triggering the random-access procedure;

4>	set the ra-InformationCommon as specified in subclause 5.7.10.5.

The UE may discard the random access report information, i.e. release the UE variable VarRA-Report, 48 hours after the last successful random access procedure related information is added to the VarRA-Report.

NOTE 1:	The UE does not log the RA information in the RA report if the triggering event of the random access is consistent UL LBT on SpCell as specified in TS 38.321 [6].

[TS 38.331, clause 5.7.10.5]

The UE shall set the content in ra-InformationCommon as follows:

1>	set the absoluteFrequencyPointA to indicate the absolute frequency of the reference resource block associated to the random-access resources used in the random-access procedure;

1>	set the locationAndBandwidth and subcarrierSpacing associated to the UL BWP of the random-access resources used in the random-access procedure;

1>	set the msg1-FrequencyStart, msg1-FDM and msg1-SubcarrierSpacing associated to the contention based random-access resources if used in the random-access procedure;

1>	set the msg1-FrequencyStartCFRA, msg1-FDMCFRA and msg1-SubcarrierSpacingCFRA associated to the contention free random-access resources if used in the random-access procedure;

1>	set the parameters associated to individual random-access attempt in the chronological order of attempts in the perRAInfoList as follows:

2>	if the random-access resource used is associated to a SS/PBCH block, set the associated random-access parameters for the successive random-access attempts associated to the same SS/PBCH block for one or more random-access attempts as follows:

3>	set the ssb-Index to include the SS/PBCH block index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnSSB to indicate the number of successive random-access attempts associated to the SS/PBCH block;

3>	for each random-access attempt performed on the random-access resource, include the following parameters in the chronological order of the random-access attempt:

4>	if the random-access attempt is performed on the contention based random-access resource and if raPurpose is not equal to 'requestForOtherSI', include contentionDetected as follows:

5>	if contention resolution was not successful as specified in TS 38.321 [6] for the transmitted preamble:

6>	set the contentionDetected to true;

5>	else:

6>	set the contentionDetected to false;

4>	if the random-access attempt is performed on the contention based random-access resource; or

4>	if the random-access attempt is performed on the contention free random-access resource and if the random-access procedure was initiated due to the PDCCH ordering:

5>	if the SS/PBCH block RSRP of the SS/PBCH block corresponding to the random-access resource used in the random-access attempt is above rsrp-ThresholdSSB:

6>	set the dlRSRPAboveThreshold to true;

5>	else:

6>	set the dlRSRPAboveThreshold to false;

2>	else if the random-access resource used is associated to a CSI-RS, set the associated random-access parameters for the successive random-access attempts associated to the same CSI-RS for one or more random-access attempts as follows:

3>	set the csi-RS-Index to include the CSI-RS index associated to the used random-access resource;

3>	set the numberOfPreamblesSentOnCSI-RS to indicate the number of successive random-access attempts associated to the CSI-RS.

NOTE 1:	Void.

8.1.6.4.1.3	Test description

8.1.6.4.1.3.1	Pre-test conditions

System Simulator:

-	NR Cell 1

Preamble:

-	The UE is in state 3N-A as defined in TS 38.508-1 [4], subclause 4.4A on NR Cell 1.

8.1.6.4.1.3.2	Test procedure sequence

Table 8.1.6.4.1.3.2-1: Main behaviour

| St | Procedure | Message Sequence |  | TP | Verdict |
| --- | --- | --- | --- | --- | --- |
|  |  | U - S | Message |  |  |
| 1 | The SS transmits a UEInformationRequest message on NR Cell 1. | <-- | NR RRC: UEInformationRequest | - | - |
| 2 | Check: Does the UE transmit a UEInformationResponse message with RA-Report? | --> | NR RRC: UEInformationResponse | 1 | P |
| 3 | SS transmits an RRCRelease message to release the RRC connection. | <-- | NR RRC:RRCRelease |  |  |
| 4 | SS waits for 5s. | - | - | - | - |
| 5 | SS is configured to transmit MAC control element with non-matched UE Contention Resolution Identity for first RACH procedure and responds with matched UE Contention Resolution from second RACH procedure | - | - | - | - |
| 6 | SS sends a Paging message to the UE on the appropriate paging block, and including the UE identity in one entry of the IE pagingRecordLists. | <-- | NR RRC: Paging | - | - |
| 7-13 | Steps 2 to 8 of the generic procedure in TS 38.508-1 [4] Table 4.5.4.2-3 are performed on NR Cell 1. | - | - | - | - |
| 14 | The SS transmits a UEInformationRequest message. | <-- | NR RRC: UEInformationRequest | - | - |
| 15 | Check: Does the UE transmit a UEInformationResponse message with RA-Report? | --> | NR RRC: UEInformationResponse | 2 | P |

8.1.6.4.1.3.3	Specific message contents

Table 8.1.6.4.1.3.3-1: UEInformationRequest (Step 1 and Step14, Table 8.1.6.4.1.3.2-1)

Derivation Path: TS 38.508-1 [4], Table 4.6.1-32A with condition RA

Table 8.1.6.4.1.3.3-2: UEInformationResponse (Step 2, Table 8.1.6.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Condition |  |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |  |  |  |  |
| criticalExtensions CHOICE { |  |  |  |  |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |  |  |  |  |
| ra-ReportList-r16 SEQUENCE (SIZE (1..maxRAReport-r16)) OF RA-Report-r16{ |  | 1 entry |  |  |  |  |  |
| RA-Report-r16[1] SEQUENCE { |  |  |  | Entry 1 |  |  |  |
| cellId-r16 CHOICE { |  |  |  |  |  |  |  |
| pci-arfcn-r16 SEQUENCE { |  |  |  |  |  |  |  |
| physCellId-r16 |  | PCI of NR Cell 1 |  |  |  |  |  |
| carrierFreq-r16 |  | NARFCN of NR Cell 1 |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| ra-InformationCommon-r16 SEQUENCE { |  |  |  |  |  |  |  |
| absoluteFrequencyPointA-r16 |  | Not checked |  |  |  |  |  |
| locationAndBandwidth-r16 |  | Not checked |  |  |  |  |  |
| subcarrierSpacing-r16 |  | Not checked |  |  |  |  |  |
| msg1-FrequencyStart-r16 |  | Not checked |  |  |  |  |  |
| msg1-FrequencyStartCFRA-r16 |  | Not checked |  |  |  |  |  |
| msg1-SubcarrierSpacing-r16 |  | Not checked |  |  |  |  |  |
| msg1-SubcarrierSpacingCFRA-r16 |  | Not checked |  |  |  |  |  |
| msg1-FDM-r16 |  | Not checked |  |  |  |  |  |
| msg1-FDMCFRA-r16 |  | Not checked |  |  |  |  |  |
| perRAInfoList-r16 SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16{ |  | 1 entry |  |  |  |  |  |
| PerRAInfo-r16[1] CHOICE { |  |  |  | Entry 1 |  |  |  |
| perRASSBInfoList-r16 SEQUENCE { |  |  |  |  |  |  |  |
| ssb-Index-r16 |  | Any allowed value |  |  |  |  |  |
| numberOfPreamblesSentOnSSB-r16 |  | Any allowed value |  |  |  |  |  |
| perRAAttemptInfoList-r16 SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16{ |  | 1 entry |  |  |  |  |  |
| PerRAAttemptInfo-r16[1] SEQUENCE { |  |  |  | Entry 1 |  |  |  |
| contentionDetected-r16 |  | false |  |  |  |  |  |
| dlRSRPAboveThreshold-r16 | Not checked |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |
| } |  |  |  |
| raPurpose-r16 |  | accessRelated |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |
| } |  |  |  |  |  |  |  |

Table 8.1.6.4.1.3.3-3: UEInformationResponse (Step 15, Table 8.1.6.4.1.3.2-1)

| Derivation Path: TS 38.508-1 [4], Table 4.6.1-32B |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| UEInformationResponse-r16 ::= SEQUENCE { |  |  |  |
| criticalExtensions CHOICE { |  |  |  |
| ueInformationResponse-r16 SEQUENCE { |  |  |  |
| ra-ReportList-r16 SEQUENCE (SIZE (1..maxRAReport-r16)) OF RA-Report-r16{ | 1 entry |  |  |
| RA-Report-r16[1] SEQUENCE { |  | Entry 1 |  |
| cellId-r16 CHOICE { |  |  |  |
| pci-arfcn-r16 SEQUENCE { |  |  |  |
| physCellId-r16 | PCI of NR Cell 1 |  |  |
| carrierFreq-r16 | NARFCN of NR Cell 1 |  |  |
| } |  |  |  |
| } |  |  |  |
| ra-InformationCommon-r16 SEQUENCE { |  |  |  |
| absoluteFrequencyPointA-r16 | Not checked |  |  |
| locationAndBandwidth-r16 | Not checked |  |  |
| subcarrierSpacing-r16 | Not checked |  |  |
| msg1-FrequencyStart-r16 | Not checked |  |  |
| msg1-FrequencyStartCFRA-r16 | Not checked |  |  |
| msg1-SubcarrierSpacing-r16 | Not checked |  |  |
| msg1-SubcarrierSpacingCFRA-r16 | Not checked |  |  |
| msg1-FDM-r16 | Not checked |  |  |
| msg1-FDMCFRA-r16 | Not checked |  |  |
| perRAInfoList-r16 SEQUENCE (SIZE (1..200)) OF PerRAInfo-r16{ | 1 entry |  |  |
| PerRAInfo-r16[1] CHOICE { |  | Entry 1 |  |
| perRASSBInfoList-r16 SEQUENCE { |  |  |  |
| ssb-Index-r16 | Any allowed value |  |  |
| numberOfPreamblesSentOnSSB-r16 | Any allowed value |  |  |
| perRAAttemptInfoList-r16 SEQUENCE (SIZE (1..200)) OF PerRAAttemptInfo-r16{ | 1 entry |  |  |
| PerRAAttemptInfo-r16[1] SEQUENCE { |  | Entry 1 |  |
| contentionDetected-r16 | true |  |  |
| dlRSRPAboveThreshold-r16 | Not checked |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| raPurpose-r16 | accessRelated |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |
| } |  |  |  |

<<< END OF CHANGES >>>