3GPP TSG-RAN WG5 Meeting #93-e	R5-216675r1

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0199 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.3.12 - Xcap-diff documents |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - Attributes for MCVideo and MCData missing - Reference RFC 5854 incorrect - RFC 5874 is correct |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Attributes for MCVideo and MCData added - Reference corrected |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incomplete test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.3.12 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision1: Reference corrected |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

#### 5.5.3.12	Xcap-diff documents

Table 5.5.3.12-1: xcap-diff document for MCX configuration

| Derivation Path: RFC 5874 [107] clause 4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff | encrypted (NOTE 5) |  |  |  |
| xcap-root attribute | tsc_MCX_CMSXCAPRootURI | same URI as <CMS-XCAP-root-URI> element of the initial UE configuration |  |  |
| document[1] |  |  |  |  |
| sel attribute | AUID1 & "/users/" & XUID & "/" & MCSUEID & "/" & UE-Config" | NOTE 1a, 2, 2A, 3 |  |  |
| new-etag | arbitrary value |  |  |  |
| previous-etag | same as new-etag |  |  |  |
| document[2] |  |  |  |  |
| sel attribute | AUID2 & "/users/" & XUID & "/" & User-Profile | NOTE 1b, 2, 2B |  |  |
| new-etag | arbitrary value (different than for document[1]) |  |  |  |
| previous-etag | same as new-etag |  |  |  |
| document[3] |  |  |  |  |
| sel attribute | AUID3 & "/global/service-config.xml" | NOTE 1c |  |  |
| new-etag | arbitrary value (different than for document[1] and [2]) |  |  |  |
| previous-etag | same as new-etag |  |  |  |
| NOTE 1a: AUID1 = "org.3gpp.mcptt.ue-config" for Condition MCPTT AUID1 = "org.3gpp.mcvideo.ue-config" for Condition MCVideo AUID1 = "org.3gpp.mcdata.ue-config" for Condition MCData NOTE 1b: AUID2 = "org.3gpp.mcptt.user-profile" for Condition MCPTT AUID2 = "org.3gpp.mcvideo.user-profile" for Condition MCVideo AUID2 = "org.3gpp.mcdata.user-profile" for Condition MCData NOTE 1c: AUID3 = "org.3gpp.mcptt.service-config" for Condition MCPTT AUID3 = "org.3gpp.mcvideo.service-config" for Condition MCVideo AUID3 = "org.3gpp.mcdata.service-config" for Condition MCData NOTE 2: XUID = "sip:" & px_MCPTT_ID_User_A for Condition MCPTT XUID = "sip:" & px_MCVideo_ID_User_A for Condition MCVideo XUID = "sip:" & px_MCData_ID_User_A for Condition MCData NOTE 2A: UE-Config = "mcptt-ue-configuration.xml" for Condition MCPTT UE-Config = "mcvideo-ue-configuration.xml" for Condition MCVideo UE-Config = "mcdata-ue-configuration.xml" for Condition MCData NOTE 2B: User-Profile = "mcptt-user-profile-" & profile-index & ".xml" for Condition MCPTT (NOTE 4) User-Profile = "mcvideo-user-profile-" & profile-index & ".xml" for Condition MCVideo (NOTE 4) User-Profile = "mcdata-user-profile-" & profile-index & ".xml" for Condition MCData (NOTE 4) NOTE 3: MCSUEID = Instance id of the UE (derived from the IMEI according to 23.003 [69] clause 13.8) NOTE 4: profile-index is the same as in the user-profile-index attribute of the corresponding document NOTE 5: The content of the root element <xcap-diff> (not including the xcap-root attribute) is encrypted as described in Table 5.5.13.2-2 |  |  |  |  |

Table 5.5.3.12-2: xcap-diff document for MCX group configuration

| Derivation Path: RFC 5874 [107] clause 4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| xcap-diff | encrypted (NOTE 1) |  |  |  |
| xcap-root | tsc_MCX_GMSXCAPRootURI | same URI as <GMS-XCAP-root-URI> element of the initial UE configuration |  |  |
| document[1] |  |  |  | GROUPCONFIG |
| sel attribute | "org.openmobilealliance.groups/global/byGroupID/" & Group-ID | NOTE 2 |  |  |
| new-etag | arbitrary value for first notification, 'incremented' value otherwise (NOTE 4) | NOTE 5 |  |  |
| previous-etag | same as new-etag for first notification, same as <new-etag> of previous notification otherwise | NOTE 5 |  |  |
| element[1] |  |  |  | GROUPKEY |
| sel attribute | “org.3gpp.MCPTT-GKTP/global/byGroupID/” & Group-ID & "/~~" & Node-Sel | NOTE 2, 3 |  |  |
| GKTPs | group key transport payloads (GKTP) document as described in Table 5.5.3.14-1 |  |  |  |
| NOTE 1: The content of the root element <xcap-diff> (not including the xcap-root attribute) is encrypted as described in Table 5.5.13.2-2 NOTE 2: Group-ID = px_MCPTT_Group_A_ID for Condition MCPTT Group-ID = px_MCVideo_Group_A_ID for Condition MCVideo Group-ID = px_MCData_Group_A_ID for Condition MCData NOTE 3: Node-Sel = “/group/list-service/mgktp:GKTPs?xmlns(mgktp=urn:3gpp:ns:mcpttGKTP:1.0)” NOTE 4: It is TTCN implementation dependent how the etag is incremented NOTE 5: Values for <new-etag> and <previous-etag> shall be different for different groups |  |  |  |  |