3GPP TSG-RAN WG5 Meeting #93-e	R5-216685r1

# Online, , 8th November 2021 - 19th November 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.579-1 | CR | 0209 | rev | - | Current version: | 15.3.1 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of clause 5.5.8 - Default MCS configuration management messages and other information elements |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | MCC TF160 |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | TEI14_Test, MCImp-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-15 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | - According to 24.483 clause 8.1 the UE initial configuration is "common to all MCS with some MCPTT specific elements, some MCData specific elements and some MCVideo specific elements" and according to 24.484 there is only one Initial UE Configuration and even though it is named mcptt-UE-initial-configuration it is common to all MCS, i.e. there are no mcvideo-UE-initial-configuration or mcdata-UE-initial-configuration documents. - PLMN attribute for HPLMN entry of the Initial UE Configuration is not precisely specified - According to the test model there is only one (H)PLMN - There is no need to distinguish any domain name for the particular services - According to 24.484 clauses 8.3.2.6, 9.3.2.6 and 10.3.2.6 the value of the "XUI-URI" attribute in the respective user-profile documents shall be the same as the XUI value of the Document URI. - According to 24.484 clauses 8.3.1, 9.3.1 and 10.3.1 the name of the respective user profile document matches the value of the <ProfileName> element in the profile document. - According to 24.484 clauses 8.3.2.6, 9.3.2.6 and 10.3.2.6 the value of the <DiscoveryGroupID> shall be 3 octets expressed in hexadecimal format and the value of the <User-Info-ID> shall be 6 octets expressed in hexadecimal format. - The <Group-KMSURI> element of the <MCDataGroupInfo> element is mandatory for the <OnNetwork> and <OffNetwork> elements of the MCData user-profile |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clause 5.5.8.1 and Table 5.5.8.1-1 made common for all services; clauses 5.5.8.5 and 5.5.8.9 removed. NOTE: there are no timers TDU1, TDU2, TFS1, TFS2 and TFS3 and no counters CFS1 and CFS2 in the MCS UE initial configuration document according to 24.484 (as listed in Table 5.5.8.9-1 for MCData) - PLMN attribute for HPLMN entry clarified. - List of <VPLMN> elements in HPLMN entry made empty - px_MCVideo_User_A_Organization, px_MCData_User_A_Organization replaced by px_MCX_DomainName_Organization_A - "XUI-URI" corrected in Table 5.5.8.3-1, 5.5.8.7-1 and 5.5.8.11-1 - <ProfileName> element corrected in Table 5.5.8.3-1, 5.5.8.7-1 and 5.5.8.11-1 - <DiscoveryGroupID> and <User-Info-ID> element corrected in Table 5.5.8.3-1, 5.5.8.7-1 and 5.5.8.11-1 - <Group-KMSURI> element added to the <MCDataGroupInfo> element in <OnNetwork> and <OffNetwork> elements of the MCData user-profile - Editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Invalid test specification |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Revision1: Editorial corrections |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

### 5.5.8	Default MCS configuration management messages and other information elements

#### 5.5.8.1	MCX Initial UE Configuration

The structure of a initial UE configuration document is specified in TS 24.484 [14] clause 7.2, single MCX group configuration parameters are defined in TS 24.483 [13] clause 8.2.

Table 5.5.8.1-1: MCX Initial UE Configuration Defaults

| Derivation Path: TS 24.484 [13], clause 7.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-UE-initial-configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| Default-user-profile | not present |  |  |  |
| on-network |  |  |  |  |
| Timers |  |  |  |  |
| T100 | "2" | Values 0-255 sec | TS 24.483 [13] clause 8.2.11 |  |
| T101 | "2" | Values 0-255 sec | TS 24.483 [13] clause 8.2.12 |  |
| T103 | "5" | Values 0-255 sec | TS 24.483 [13] clause 8.2.13 |  |
| T104 | "2" | Values 0-255 sec | TS 24.483 [13] clause 8.2.14 |  |
| T132 | "3" | Values 0-255 sec | TS 24.483 [13] clause 8.2.15 |  |
| HPLMN |  |  |  |  |
| PLMN attribute | PLMN-Id = MCC ¦¦ MNC with MCC and MNC being the same as stored in EFIMSI on the test SIM card according to clause 4.9.2 in TS 36.508 [6] | PLMN on which the UE is allowed for MCX services. NOTE: Same PLMN as of the Cell on which the UE is camped during testing. | TS 23.003 [69] clause 12.1 TS 24.483 [13] clause 8.2.16 |  |
| service |  | MCX related services on a per HPLMN basis |  |  |
| MCPTT-to-con-ref | px_MCX_APN | configuration parameter for establishment of the PDN connection for the MCX service | TS 24.483 [13] clause 8.2.21 |  |
| MC-common-core-to-con-ref | px_MCX_APN | configuration parameter for establishment of the PDN connection for the MC common core service | TS 24.483 [13] clause 8.2.24 |  |
| MC-ID-to-con-ref | px_MCX_APN | configuration parameter for establishment of the PDN connection for the MC identity management service | TS 24.483 [13] clause 8.2.27 |  |
| VPLM[1] | empty list |  |  |  |
| App-Server-Info |  |  |  |  |
| idms-auth-endpoint | "https://" & px_MCX_IdMS_auth_IPAddress & ":" & px_MCX_IdMS_auth_Port & tsc_MCX_IdMS_auth_UriPath | Identity management server authorisation endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41 | IPv4 |
|  | "https://[" & px_MCX_IdMS_auth_IPAddress & "]:" & px_MCX_IdMS_auth_Port & tsc_MCX_IdMS_auth_UriPath | Identity management server authorisation endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41 | IPv6 |
| idms-token-endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| http-proxy | "https://" & px_MCX_HTTP_Proxy_IPAddress & ":" & px_MCX_HTTP_Proxy_Port | IP address and port used by the UE for the HTTP TCP connection | TS 23.003 [69] TS 24.483 [13] clause 8.2.41B | IPv4 |
|  | "https://[" & px_MCX_HTTP_Proxy_IPAddress & "]:" & px_MCX_HTTP_Proxy_Port | IP address and port used by the UE for the HTTP TCP connection | TS 23.003 [69] TS 24.483 [13] clause 8.2.41B | IPv6 |
| gms | tsc_MCX_GMS_Hostname | Indicates the group management server identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.42 |  |
| cms | tsc_MCX_CMS_Hostname | Indicates the configuration management server identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.43 |  |
| kms | tsc_MCX_KMS_Hostname | Indicates the key management server identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.44 |  |
| tls-tunnel-auth-method |  |  |  |  |
| mutual-authentication | "false" | Indicates whether mutual authentication is used for the TLS tunnel authentication false=one-way authentication based on the server certificate is used | TS 24.483 [13] clause 8.2.44B |  |
| x509 | Not present | the X.509 certificate for mutual authentication for the TLS tunnel authentication | TS 24.483 [13] clause 8.2.44C |  |
| key | Not present | pre-shared key for mutual authentication for the TLS tunnel authentication | TS 24.483 [13] clause 8.2.44D |  |
| GMS-URI | tsc_MCX_GMSURI | The group management service URI information which contains the public service identity for performing subscription proxy function of the GMS | TS 23.003 [69] TS 24.483 [13] clause 8.2.9 |  |
| group-creation-XUI | px_MCX_GroupCreationXUI | Indicates the group creation XUI information for creation of groups | TS 23.003 [69] TS 24.483 [13] clause 8.2.9A |  |
| GMS-XCAP-root-URI | tsc_MCX_GMSXCAPRootURI | Indicates the group management server XCAP Root URI information | TS 23.003 [69] TS 24.483 [13] clause 8.2.9B |  |
| CMS-XCAP-root-URI | tsc_MCX_CMSXCAPRootURI | Indicates the configuration management server XCAP Root URI information | TS 23.003 [69] TS 24.483 [13] clause 8.2.9C |  |
| integrity-protection-enabled | "true" | Indicates whether integrity protection is enabled | TS 24.483 [13] clause 8.2.44E |  |
| confidentiality-protection-enabled | "true" | Indicates whether integrity protection is enabled | TS 24.483 [13] clause 8.2.44F |  |
| off-network |  |  |  |  |
| Timers |  |  |  |  |
| TFG1 | "150" | Indicates the timer for wait for call announcement; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.47 |  |
| TFG2 | "2000" | Indicates the timer for call announcement; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.48 |  |
| TFG3 | "40" | Indicates the timer for call probe retransmission; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.49 |  |
| TFG4 | "20" | Indicates the timer for waiting for the MCX user; Values: 0-60 s | TS 24.483 [13] clause 8.2.50 |  |
| TFG5 | "2" | Indicates the timer for not present incoming call announcements; Values: 0-255 s | TS 24.483 [13] clause 8.2.51 |  |
| TFG11 | "3000" | Indicates the timer for MCX emergency end retransmission; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.52 |  |
| TFG12 | "3000" | Indicates the timer for MCX imminent peril end retransmission; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.53 |  |
| TFG13 | "1" | Indicates the timer for implicit priority downgrade; Values: 0-255 s | TS 24.483 [13] clause 8.2.54 |  |
| TFG14 | "1" | Indicates the MCX timer for implicit priority downgrade (imminent peril); Values: 0-255 s | TS 24.483 [13] clause 8.2.54A |  |
| TFP1 | "2000" | Indicates the timer for private call request retransmission; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.55 |  |
| TFP2 | "50" | Indicates the timer for waiting for call response message; Values: 0-60 s | TS 24.483 [13] clause 8.2.56 |  |
| TFP3 | "2000" | Indicates the timer for private call release retransmission; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.57 |  |
| TFP4 | "5000" | Indicates the timer for private call release retransmission; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.58 |  |
| TFP5 | "30" | Indicates the timer for call release; Values: 0-600 s | TS 24.483 [13] clause 8.2.59 |  |
| TFP6 | "3000" | Indicates the timer for MCX emergency private call cancel retransmission; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.60 |  |
| TFP7 | "6" | Indicates the timer for waiting for any message with same call identifier; Values: 0-255 s | TS 24.483 [13] clause 8.2.61 |  |
| TFB1 | "300" | Indicates the timer for max duration; Values: 0-600 s | TS 24.483 [13] clause 8.2.62 |  |
| TFB2 | "10" | Indicates the timer for max duration; Values: 0-10 s | TS 24.483 [13] clause 8.2.63 |  |
| TFB3 | "20" | Indicates the timer for waiting for the MCX user; Values: 0-60 s | TS 24.483 [13] clause 8.2.64 |  |
| T201 | "1000" | Indicates the timer for floor request; Values: 0-65535 ms | TS 24.483 [13] clause 8.2.65 |  |
| T203 | "5" | Indicates the timer for end of RTP media; Values: 0-255 s | TS 24.483 [13] clause 8.2.66 |  |
| T204 | "5" | Indicates the timer for floor queue position request; Values: 0-255 s | TS 24.483 [13] clause 8.2.67 |  |
| T205 | "1" | Indicates the timer for floor granted request; Values: 0-255 s | TS 24.483 [13] clause 8.2.68 |  |
| T230 | "10" | Indicates the timer for inactivity; Values: 0-255 s | TS 24.380 [10] TS 24.581 [88] |  |
| T233 | "10" | Indicates the timer for pending user action; Values: 0-255 s | TS 24.483 [13] clause 8.2.70 |  |
| TFE1 | "30" | Indicates the timer for MCX emergency alert; Values: 0-65535 s | TS 24.483 [13] clause 8.2.71 |  |
| TFE2 | "10" | Indicates the timer for MCXemergency alert re-transmission; Values: 0-10 s | TS 24.483 [13] clause 8.2.72 |  |
| Counters |  |  |  |  |
| CFP1 | "3" | Indicates the counter for private call request retransmission | TS 24.483 [13] clause 8.2.74 |  |
| CFP3 | "5" | Indicates the counter for private call release retransmission | TS 24.483 [13] clause 8.2.75 |  |
| CFP4 | "2" | Indicates the counter for private call accept retransmission | TS 24.483 [13] clause 8.2.76 |  |
| CFP6 | "2" | Indicates the counter for private call accept retransmission | TS 24.483 [13] clause 8.2.77 |  |
| CFP11 | "2" | Indicates the counter for MCX group call emergency end retransmission | TS 24.483 [13] clause 8.2.78 |  |
| CFP12 | "2" | Indicates the counter for MCX imminent peril call emergency end retransmission | TS 24.483 [13] clause 8.2.79 |  |
| C201 | "3" | Indicates the counter for floor request | TS 24.483 [13] clause 8.2.80 |  |
| C204 | "2" | Indicates the counter for floor queue position request | TS 24.483 [13] clause 8.2.81 |  |
| C205 | "4" | Indicates the counter for floor granted request | TS 24.483 [13] clause 8.2.82 |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | IP address is IPv4 address |
| IPv6 | IP address is IPv6 address |

#### 5.5.8.2	MCPTT UE Configuration

The structure of a group configuration document is specified in TS 24.484 [14] clause 8.2, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 4.2.

Table 5.5.8.2-1: MCPTT UE Configuration Defaults

| Derivation Path: TS 24.484 [14] clause 8.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-UE-configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| common |  |  |  |  |
| private-call |  |  |  |  |
| Max-Simul-Call-N10 | "2" | Indicates the maximum number of private calls | TS 24.483 [13] clause 4.2.7 |  |
| MCPTT-Group-Call |  |  |  |  |
| Max-Simul-Call-N4 | "3" | Indicates the maximum number of simultaneous group calls | TS 24.483 [13] clause 4.2.9 |  |
| Max-Simul-Trans-N5 | "5" | Indicates the maximum number of transmissions in a group | TS 24.483 [13] clause 4.2.10 |  |
| Prioritized-MCPTT-Group |  |  |  |  |
| MCPTT-Group-Priority[1] |  |  |  |  |
| MCPTT-Group-ID | px_MCPTT_Group_A_ID | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. | TS 24.483 [13] clause 4.2.13 |  |
| group-priority-hierarchy | "7" | Indicates the requested presentation priority of group call; Values: 0-7 "7"=the top priority among groups | TS 24.483 [13] clause 4.2.14 |  |
| on-network |  |  |  |  |
| IPv6Preferred | "false" | Indicates whether IPv6 is preferred over IPv4 for on-network operation when the MCPTT UE has both IPv4 and IPv6 host configuration. | TS 24.483 [13] clause 4.2.17 |  |
| Relay-Service | "true" | Indicates the authorisation to use a relay service | TS 24.483 [13] clause 4.2.16 |  |
| Relayed-MCPTT-Group[1] |  |  |  |  |
| MCPTT-Group-ID | px_MCPTT_Group_A_ID | One allowed relayed MCPTT group | TS 24.483 [13] clause 4.2.20 |  |
| Relay-Service-Code | "123456" | Identifies a connectivity service the ProSe UE-to-Network Relay provides to Public Safety applications; 24-bit value | TS 23.303 [68] TS 24.483 [13] clause 4.2.21 |  |

#### 5.5.8.3	MCPTT User Profile

The structure of a user profile document is specified in TS 24.484 [14] clause 8.3, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 5.2.

The structure of the configuration document is based on the XML Schema in clause 8.3.2.3 of TS 24.484 [14] and XML "ruleset" schema according to IETF RFC 4745 [103]. To distinguish the schemas the prefix "cp" ("common policy") is used for the ruleset.

Table 5.5.8.3-1: MCPTT User Profile Defaults

| Derivation Path: TS 24.484 [14] clause 8.3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| mcptt-user-profile |  |  |  |  |  |  |  |  |  |
| XUI-URI attribute |  | "sip:" & px_MCPTT_ID_User_A |  | same as the XUI value of the Document URI |  |  |  |  |  |
| user-profile-index attribute |  | "49" |  | value arbitrarily selected |  |  |  |  |  |
| Status |  | true |  | MCPTT user profile is enabled |  |  |  |  |  |
| ProfileName |  | "mcptt-user-profile-" & user-profile-index & ".xml" |  | name of the user profile document; user-profile-index is the value of the user-profile-index attribute |  | TS 24.483 [13] clause 5.2.7B |  |  |  |
| Common |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  | Index for the particular MCPTT user profile |  |  |  |  |  |
| MCPTTUserID |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_A |  | MCPTT user identity (MCPTT ID) which is a globally unique identifier within the MCPTT service that represents the MCPTT user |  | TS 24.483 [13] clause 5.2.7 |  |  |  |
| UserAlias |  |  |  | Alphanumeric aliases of MCPTT user |  | TS 24.483 [13] clause 5.2.8 |  |  |  |
| alias-entry |  | px_MCPTT_User_A_Alias |  |  |  |  |  |  |  |
| ParticipantType |  | px_MCPTT_User_A_ParticipantType |  | Participant type of the MCPTT user |  | TS 24.483 [13] clause 5.2.10 |  |  |  |
| MissionCriticalOrganization |  | px_MCX_DomainName_Organization_A |  | Indicates the organization an MCPTT user belongs to |  | TS 24.483 [13] clause 5.2.11 |  |  |  |
| PrivateCall |  |  |  |  |  |  |  |  |  |
| PrivateCallList |  |  |  |  |  |  |  |  |  |
| PrivateCallURI[1] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_B |  | MCPTT user(s) who can be called in a MCPTT private call |  | TS 24.483 [13] clause 5.2.17 |  |  |  |
| display-name |  | "User B Name" |  | a human readable name for this User |  | TS 24.483 [13] clause 5.2.18 |  |  |  |
| PrivateCallURI[2] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "1" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_C |  | MCPTT user(s) who can be called in a MCPTT private call |  | TS 24.483 [13] clause 5.2.17 |  |  |  |
| display-name |  | "User C Name" |  | a human readable name for this User |  | TS 24.483 [13] clause 5.2.18 |  |  |  |
| PrivateCallProSeUser[1] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| DiscoveryGroupID |  | '123456'O |  | Discovery group ID in the ProSe discovery procedures |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.19 |  |  |  |
| User-Info-ID |  | '555555555555'O |  | Prose user Info ID in the ProSe discovery procedures |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.19A |  |  |  |
| PrivateCallProSeUser[2] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "1" |  |  |  |  |  |  |  |
| DiscoveryGroupID |  | '123456'O |  | Discovery group ID in the ProSe discovery procedures |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.19 |  |  |  |
| User-Info-ID |  | '666666666666'O |  | Prose user Info ID in the ProSe discovery procedures |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.19A |  |  |  |
| EmergencyCall |  |  |  |  |  |  |  |  |  |
| MCPTTPrivateRecipient |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UsePreConfigured" |  | Indicates the criteria to determine when initiation of an MCPTT emergency private call uses the MCPTT private recipient ID. |  | TS 24.483 [13] clause 5.2.29F |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_B |  | The MCPTT private recipient for an MCPTT emergency private call |  | TS 24.483 [13] clause 5.2.29B |  |  |  |
| display-name |  | "User B Name" |  | a human readable name for this User |  | TS 24.483 [13] clause 5.2.29E |  |  |  |
| ProSeUserID-entry |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| DiscoveryGroupID |  | '123456'O |  | Discovery group ID in the ProSe discovery procedures |  | TS 24.483 [13] clause 5.2.29C |  |  |  |
| User-Info-ID |  | '555555555555'O |  | ProSe user Info ID in the ProSe discovery procedures |  | TS 24.483 [13] clause 5.2.29D |  |  |  |
| MCPTT-group-call |  |  |  |  |  |  |  |  |  |
| MaxSimultaneousCallsN6 |  | "3" |  | Indicates the maximum number of simultaneously received MCPTT group calls |  | TS 24.483 [13] clause 5.2.31 |  |  |  |
| EmergencyCall |  |  |  |  |  |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  | Use currently selected MCPTT group for an on-network MCPTT emergency group call |  | TS 24.483 [13] clause 5.2.34D |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | The group used upon certain criteria on initiation of an MCPTT emergency group call |  | TS 24.483 [13] clause 5.2.34B |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | The display name for group used for emergency |  | TS 24.483 [13] clause 5.2.34C |  |  |  |
| ImminentPerilCall |  |  |  |  |  |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  | Use currently selected MCPTT group for an on-network MCPTT imminent peril group call |  | TS 24.483 [13] clause 5.2.39D |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | the group used on initiation of an MCPTT imminent peril group call. |  | TS 24.483 [13] clause 5.2.39B |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | display name for group used for the imminent peril call |  | TS 24.483 [13] clause 5.2.39C |  |  |  |
| EmergencyAlert |  |  |  |  |  |  |  |  |  |
| MCPTTGroupInitiation |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| entry-info attribute |  | "UseCurrentlySelectedGroup" |  | Use currently selected MCPTT group for emergency alert |  | TS 24.483 [13] clause 5.2.43E |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | Indicates the MCPTT group used upon certain criteria on initiation of an MCPTT emergency alert. |  | TS 24.483 [13] clause 5.2.43B |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | Optional; name of emergency alert group |  | TS 24.483 [13] clause 5.2.43D |  |  |  |
| Priority |  | "10" |  | Indicates the priority of the MCPTT group calls, 0-255 |  | TS 24.483 [13] clause 5.2.43F |  |  |  |
| OffNetwork |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| MCPTTGroupInfo |  |  |  |  |  |  |  |  |  |
| entry[1] |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | Indicates an off-network MCPTT group for use by an MCPTT user |  | TS 24.483 [13] clause 5.2.53 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | The display name corresponding to off-network group id |  | TS 24.483 [13] clause 5.2.53A |  |  |  |
| User-Info-ID |  | '555555555555'O |  | ProSe user info ID |  | TS 23.303 [68] TS 24.483 [13] clause 5.2.58 |  |  |  |
| OnNetwork |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| MCPTTGroupInfo |  |  |  |  |  |  |  |  |  |
| entry[1] |  |  |  | Group 1 the MCPTT user is allowed to affiliate to |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | The MCPTT group ID for the on-network MCPTT group that the MCPTT user is allowed to affiliate to. |  | TS 24.483 [13] clause 5.2.48B4 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | The display name for the group |  | TS 24.483 [13] clause 5.2.48B5 |  |  |  |
| MaxAffiliationsN2 |  | 20 |  |  |  |  |  |  |  |
| MaxSimultaneousTransmissionsN7 |  | 20 |  |  |  |  |  |  |  |
| ImplicitAffiliations |  |  |  | Group 1 the MCPTT user is implicitly affiliated to |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_Group_A_ID |  | indicates a MCPTT group ID to which the MCPTT user is implicitly affiliated to |  | TS 24.483 [13] clause 5.2.48C4 |  |  |  |
| display-name |  | px_MCPTT_Group_A_Name |  | display name for implicitly affiliated group |  | TS 24.483 [13] clause 5.2.48C5 |  |  |  |
| PrivateEmergencyAlert |  |  |  |  |  |  |  |  |  |
| entry |  |  |  |  |  |  |  |  |  |
| entry-info attribute |  | "UsePreConfigured" |  | Indicates the criteria to determine when initiation of an MCPTT emergency private call uses the MCPTT private recipient ID. |  | TS 24.483 [13] clause 5.2.48O |  |  |  |
| index attribute |  | "0" |  |  |  |  |  |  |  |
| uri-entry |  | px_MCPTT_ID_User_B |  | Indicates the default MCPTT user ID to be used upon certain criteria on initiation of an MCPTT private emergency alert for on-network |  | TS 24.483 [13] clause 5.2.48M |  |  |  |
| display-name |  | px_MCPTT_User_A_Alias |  | The display name corresponding to private emergency call id |  | TS 24.483 [13] clause 5.2.48N |  |  |  |
| anyExt |  |  |  |  |  |  |  |  |  |
| FunctionalAliasList |  |  |  |  |  | TS 24.483 [13] clause 5.2.48W6 |  |  |  |
| entry |  | px_MCPTT_ID_FA_A |  |  |  |  |  |  |  |
| cp:ruleset |  |  |  |  |  |  |  |  |  |
| cp:rule |  |  |  |  |  |  |  |  |  |
| cp:id attribute |  | "rule1" |  |  |  |  |  |  |  |
| cp:actions |  |  |  |  |  |  |  |  |  |
| allow-create-delete-user-alias |  | "true" |  | Indicates authorisation to create and delete aliases of other MCPTT users |  | TS 24.483 [13] clause 5.2.9 |  |  |  |
| allow-private-call |  | "true" |  | Indicates the authorisation to make a MCPTT private call |  | TS 24.483 [13] clause 5.2.13 |  |  |  |
| allow-private-call-to-any-user |  | "true" |  | indicates the authorisation to make a MCPTT private call to any MCPTT user |  | TS 24.483 [13] clause 5.2.14 |  |  |  |
| allow-manual-commencement |  | "true" |  | Indicates the authorisation to make a MCPTT private call with manual commencement |  | TS 24.483 [13] clause 5.2.20 |  |  |  |
| allow-automatic-commencement |  | "true" |  | Indicates the authorisation to make a MCPTT private call with automatic commencement |  | TS 24.483 [13] clause 5.2.21 |  |  |  |
| allow-force-auto-answer |  | "true" |  | Indicates the authorisation of MCPTT user to force automatic answer for a MCPTT private call |  | TS 24.483 [13] clause 5.2.22 |  |  |  |
| allow-failure-restriction |  | "false" |  | Indicates the authorisation to restrict the provision of a notification of call failure reason for a MCPTT private call |  | TS 24.483 [13] clause 5.2.23 |  |  |  |
| allow-private-call-media-protection |  | "true" |  | Indicates authorisation to protect confidentiality and integrity of media for MCPTT private calls |  | TS 24.483 [13] clause 5.2.24 |  |  |  |
| allow-private-call-floor-control-protection |  | "true" |  | Indicates authorisation to protect confidentiality and integrity of floor control signalling for MCPTT private calls. |  | TS 24.483 [13] clause 5.2.25 |  |  |  |
| allow-emergency-private-call |  | "true" |  | Indicates the authorisation to make an MCPTT emergency private call. |  | TS 24.483 [13] clause 5.2.27 |  |  |  |
| allow-cancel-private-emergency-call |  | "true" |  | Indicates the authorisation to cancel emergency priority in an MCPTT emergency private call by an authorised MCPTT user |  | TS 24.483 [13] clause 5.2.28 |  |  |  |
| allow-emergency-group-call |  | "true" |  | Indicates the authorisation to make an MCPTT emergency group call functionality enabled for MCPTT user |  | TS 24.483 [13] clause 5.2.33 |  |  |  |
| allow-cancel-group-emergency |  | "true" |  | Indicates the authorisation to cancel an in progress MCPTT emergency call associated with a group. |  | TS 24.483 [13] clause 5.2.35 |  |  |  |
| allow-imminent-peril-call |  | "true" |  | Indicates the authorisation to make an Imminent Peril group call |  | TS 24.483 [13] clause 5.2.37 |  |  |  |
| allow-cancel-imminent-peril |  | "true" |  | Indicates the authorisation for in-progress MCPTT imminent peril cancelation |  | TS 24.483 [13] clause 5.2.38 |  |  |  |
| allow-activate-emergency-alert |  | "true" |  | Indicates the authorisation to activate an MCPTT emergency alert |  | TS 24.483 [13] clause 5.2.41 |  |  |  |
| allow-cancel-emergency-alert |  | "true" |  | Indicates the authorisation to cancel an MCPTT emergency alert |  | TS 24.483 [13] clause 5.2.42 |  |  |  |
| allow-create-group-broadcast-group |  | "true" |  | Indicates the authorisation to create a group-broadcast group. |  | TS 24.483 [13] clause 5.2.46 |  |  |  |
| allow-create-user-broadcast-group |  | "true" |  | Indicates the authorisation to create a user-broadcast group |  | TS 24.483 [13] clause 5.2.48 |  |  |  |
| allow-offnetwork |  | "true" |  | Indicates the authorisation for off-network services |  | TS 24.483 [13] clause 5.2.50 |  |  |  |
| allow-listen-both-overriding-and-overridden |  | "false" |  | Indicates whether the MCPTT user is allowed to listen both overriding and override |  | TS 24.483 [13] clause 5.2.54 |  |  |  |
| allow-transmit-during-override |  | "false" |  | Indicates whether the MCPTT user is allowed to transmit in case of override (overriding and/or overridden) |  | TS 24.483 [13] clause 5.2.55 |  |  |  |
| allow-off-network-group-call-change-to-emergency |  | "true" |  | Indicates the authorisation for a participant to change an off-network group call in-progress to an off-network MCPTT emergency group call |  | TS 24.483 [13] clause 5.2.56 |  |  |  |
| allow-imminent-peril-change |  | "true" |  | Indicates the authorisation for a participant to change an off-network group call in-progress to an off-network MCPTT imminent peril group call |  | TS 24.483 [13] clause 5.2.57 |  |  |  |
| allow-regroup |  | "true" |  | Indicates whether the MCPTT user is authorised to perform dynamic regrouping operations |  | TS 24.483 [13] clause 5.2.48D |  |  |  |
| allow-presence-status |  | "true" |  | Indicates the presence status on the network of this MCPTT user is available |  | TS 24.483 [13] clause 5.2.48E |  |  |  |
| allow-request-presence |  | "true" |  | Indicates whether the MCPTT user is authorised to obtain whether a particular MCPTT User is present on the network |  | TS 24.483 [13] clause 5.2.48F |  |  |  |
| allow-private-call-participation |  | "true" |  | Indicates whether the MCPTT user is allowed to participate in MCPTT private calls that they are invited to |  | TS 24.483 [13] clause 5.2.48G |  |  |  |
| allow-override-of-transmission |  | "true" |  | Indicates whether the MCPTT user is authorised to override transmission in a MCPTT private call |  | TS 24.483 [13] clause 5.2.48H |  |  |  |
| allow-manual-off-network-switch |  | "true" |  | Indicates whether the MCPTT user is authorised to manually switch to off-network operation while in on-network operation |  | TS 24.483 [13] clause 5.2.48I |  |  |  |
| anyExt |  |  |  |  |  |  |  |  |  |
| allow-request-first-to-answer-call |  | "true" |  | Indicates whether the MCPTT user is authorised to request a first to answer call |  | TS 24.483 [13] clause 5.2.48T |  |  |  |
| allow-request-remote-init-private-call |  | "true" |  | Indicates whether the MCPTT user is authorised to request remotely initiated private calls |  | TS 24.483 [13] clause 5.2.48W1 |  |  |  |
| allow-query-functional-alias-other-user |  | "true" |  | Indicates whether the MCPTT user is authorised to query the functional alias(es) activated by another MCPTT user |  | TS 24.483 [13] clause 5.2.48W8 |  |  |  |
| allow-takeover-functional-alias-other-user |  | "true" |  | Indicates whether he MCPTT user is authorised to take over the functional alias(es) previously activated by another MCPTT user |  | TS 24.483 [13] clause 5.2.48W9 |  |  |  |
| allow-location-info-when-talking |  | "false" |  | When set to "true" the MCPTT user is authorised to send its location information when it is requesting the floor. When set to "false" the MCPTT user is not authorised to send its location information when it is requesting the floor. |  | TS 24.483 [13] clause 5.2.48W10 |  |  |  |

#### 5.5.8.4	MCPTT Service Configuration

The structure of a user profile document is specified in TS 24.484 [14] clause 8.4, single MCPTT group configuration parameters are defined in TS 24.483 [13] clause 7.2.

Table 5.5.8.4-1: MCPTT Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 8.4 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Information Element |  | Value/remark |  | Comment |  | Reference |  | Condition |  |
| service configuration |  |  |  |  |  |  |  |  |  |
| domain attribute |  | px_MCX_DomainName_Organization_A |  | Mandatory attribute: domain name of the mission critical organization |  |  |  |  |  |
| common |  |  |  |  |  |  |  |  |  |
| min-length-alias |  | "2" |  | Indicates minimum length of an alphanumeric identifier (i.e., alias) |  | TS 24.483 [13] clause 7.2.9 |  |  |  |
| broadcast-group |  |  |  |  |  |  |  |  |  |
| num-levels-group-hierarchy |  | "1" |  | Indicates the number of levels of group hierarchy for group-broadcast groups |  | TS 24.483 [13] clause 7.2.7 |  |  |  |
| num-levels-user-hierarchy |  | "1" |  | Indicates the number of levels of user hierarchy for user-broadcast groups |  | TS 24.483 [13] clause 7.2.8 |  |  |  |
| on-network |  |  |  |  |  |  |  |  |  |
| emergency-call |  |  |  |  |  |  |  |  |  |
| private-cancel-timeout |  | "PT30M" |  | 30 minutes |  |  |  |  |  |
| group-time-limit |  | "PT20M" |  | 20 minutes |  |  |  |  |  |
| private-call |  |  |  |  |  |  |  |  |  |
| hang-time |  | "PT30S" |  | 30 seconds |  |  |  |  |  |
| max-duration-with-floor-control |  | "PT30S" |  | 30 seconds |  |  |  |  |  |
| max-duration-without-floor-control |  | "PT20M" |  | 20 minutes |  |  |  |  |  |
| num-levels-priority-hierarchy |  | 10 |  |  |  |  |  |  |  |
| transmit-time |  |  |  |  |  |  |  |  |  |
| time-limit |  | "PT30S" |  | 30 seconds |  |  |  |  |  |
| time-warning |  | "PT20M" |  | 20 minutes |  |  |  |  |  |
| hang-time-warning |  | "PT20M" |  | 20 minutes |  |  |  |  |  |
| floor-control-queue |  |  |  |  |  |  |  |  |  |
| depth |  | 5 |  |  |  |  |  |  |  |
| max-user-request-time |  | "PT20M" |  | 20 minutes |  |  |  |  |  |
| fc-timers-counters |  |  |  |  |  |  |  |  |  |
| T1-end-of-rtp-media |  | "PT4S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T3-stop-talking-grace |  | "PT3S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T7-floor-idle |  | "PT2S" |  | Depends on the characteristic of the radio access network |  | TS 24.380 [10] clause 11 |  |  |  |
| T8-floor-revoke |  | "PT1S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T11-end-of-RTP-dual |  | "PT4S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T12-stop-talking-dual |  | "PT30S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T15-conversation |  | "PT30S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T16-map-group-to-bearer |  | "PT0.5S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T17-unmap-group-to-bearer |  | "PT0.2S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T20-floor-granted |  | "PT1S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T55-connect |  | "PT2S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| T56-disconnect |  | "PT2S" |  | Default value Value in seconds |  | TS 24.380 [10] clause 11 |  |  |  |
| C7-floor-idle |  | 10 |  | Default value |  | TS 24.380 [10] clause 11 |  |  |  |
| C17-unmap-group-to-bearer |  | 3 |  | Default value |  | TS 24.380 [10] clause 11 |  |  |  |
| C20-floor-granted |  | 3 |  | Default value |  | TS 24.380 [10] clause 11 |  |  |  |
| C55-connect |  | 3 |  | Default value |  | TS 24.380 [10] clause 11 |  |  |  |
| C56-disconnect |  | 3 |  | Default value |  | TS 24.380 [10] clause 11 |  |  |  |
| signalling-protection |  |  |  |  |  |  |  |  |  |
| confidentiality-protection |  | true |  |  |  |  |  |  |  |
| integrity-protection |  | true |  |  |  |  |  |  |  |
| protection-between-mcptt-servers |  |  |  |  |  |  |  |  |  |
| allow-signalling-protection |  | true |  |  |  |  |  |  |  |
| allow-floor-control-protection |  | true |  |  |  |  |  |  |  |
| emergency-resource-priority |  |  |  |  |  |  |  |  |  |
| resource-priority-namespace |  | "mcpttp" |  |  |  | RFC 8101 |  |  |  |
| resource-priority-priority |  | "8" |  |  |  | RFC 8101 |  |  |  |
| imminent-peril-resource-priority |  |  |  |  |  |  |  |  |  |
| resource-priority-namespace |  | "mcpttp" |  |  |  | RFC 8101 |  |  |  |
| resource-priority-priority |  | "5" |  |  |  | RFC 8101 |  |  |  |
| normal-resource-priority |  |  |  |  |  |  |  |  |  |
| resource-priority-namespace |  | "mcpttp" |  |  |  | RFC 8101 |  |  |  |
| resource-priority-priority |  | "1" |  |  |  | RFC 8101 |  |  |  |
| anyExt |  |  |  |  |  |  |  |  |  |
| functional-alias-list |  |  |  |  |  |  |  |  |  |
| functional-alias |  | px_MCPTT_ID_FA_A |  |  |  |  |  |  |  |
| max-simultaneous-activations |  | "1" |  |  |  |  |  |  |  |
| allow-takeover |  | "true" |  |  |  |  |  |  |  |
| mcptt-user-list |  |  |  |  |  |  |  |  |  |
| entry |  | px_MCPTT_ID_User_A |  |  |  |  |  |  |  |
| off-network |  |  |  |  |  |  |  |  |  |
| emergency-call |  |  |  |  |  |  |  |  |  |
| private-cancel-timeout |  | "PT5S" |  | 5 seconds; Indicates timeout value for the cancellation of an in progress emergency for an MCPTT private call. Values: : 0-65535 s |  | TS 24.483 [13] clause 7.2.14 |  |  |  |
| group-time-limit |  | "PT5S" |  | 5 seconds; Indicates time limit for an in progress MCPTT emergency call related to an MCPTT group. Values: 0-65535 s |  | TS 24.483 [13] clause 7.2.16 |  |  |  |
| private-call |  |  |  |  |  |  |  |  |  |
| hang-time |  | "PT5S" |  | 5 seconds; Indicates hang timer for private calls (with floor control). Values: 0-65535 s |  | TS 24.483 [13] clause 7.2.13 |  |  |  |
| max-duration-with-floor-control |  | "PT60S" |  | 60 seconds; Indicates max private call (with floor control) duration. Values: 0-65535 s |  | TS 24.483 [13] clause 7.2.12 |  |  |  |
| num-levels-priority-hierarchy |  | "4" |  | Indicates the number of levels of hierarchy for floor control override in off-network. Values: 4-256 |  | TS 24.483 [13] clause 7.2.17 |  |  |  |
| transmit-time |  |  |  |  |  |  |  |  |  |
| time-limit |  | "PT60S" |  | 60 seconds; Indicates transmit time limit from a single request to transmit in a group or private call. Values: 0-65535 s |  | TS 24.483 [13] clause 7.2.18 |  |  |  |
| time-warning |  | "PT50S" |  | 50 seconds; Indicates configuration of warning time before time limit of transmission is reached (off-network). Values: 0-255 s |  | TS 24.483 [13] clause 7.2.19 |  |  |  |
| hang-time-warning |  | "PT4S" |  | 4 seconds; Indicates configuration of warning time before hang time is reached (off-network). Values: Values: 0-255 s |  | TS 24.483 [13] clause 7.2.20 |  |  |  |
| default-prose-per-packet-priority |  |  |  |  |  |  |  |  |  |
| mcptt-private-call-signalling |  | "1" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 23.303 [68] TS 24.483 [13] clause 7.2.22 |  |  |  |
| mcptt-private-call-media |  | "1" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 23.303 [68] TS 24.483 [13] clause 7.2.23 |  |  |  |
| mcptt-emergency-private-call-signalling |  | "8" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 23.303 [68] TS 24.483 [13] clause 7.2.24 |  |  |  |
| mcptt-emergency-private-call-media |  | "8" |  | Indicates the default ProSe Per-Packet Priority (PPPP) value |  | TS 23.303 [68] TS 24.483 [13] clause 7.2.25 |  |  |  |
| allow-log-metadata |  | "true" |  | Indicates whether an MCPTT emergency group call is permitted on the MCPTT group |  | TS 24.483 [13] clause 7.2.26 |  |  |  |

#### 5.5.8.5	Void

#### 5.5.8.6	MCVideo UE Configuration

The structure of a UE configuration document is specified in TS 24.484 [14] clause 9.2. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 12.2.

Table 5.5.8.6-1: MCVideo UE Configuration Defaults

| Derivation Path: TS 24.484 [14] clause 9.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcvideo-UE-configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| common |  |  |  |  |
| Mcvideo-private-call |  |  |  |  |
| Max-Simul-Call-N10 | "2" | Indicates the maximum number of private calls |  |  |
| MCVideo-Group-Call |  |  |  |  |
| Max-Simul-Call-Nc4 | "3" | Indicates the maximum number of simultaneous group calls |  |  |
| Max-Simul-Trans-Nc5 | "5" | Indicates the maximum number of transmissions in a group |  |  |
| Prioritized-MCVideo-Group |  |  |  |  |
| MCVideo-Group-Priority[1] |  |  |  |  |
| MCVideo-Group-ID | px_MCVideo_Group_A_ID | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. |  |  |
| group-priority-hierarchy | "7" | Indicates the requested presentation priority of group call; Values: 0-7 "7"=the top priority among groups |  |  |
| on-network |  |  |  |  |
| IPv6Preferred | "false" | Indicates whether IPv6 is preferred over IPv4 for on-network operation when the MCPTT UE has both IPv4 and IPv6 host configuration. |  |  |
| Relay-Service | "true" | Indicates the authorisation to use a relay service |  |  |
| Relayed-MCVideo-Group[1] |  |  |  |  |
| MCVideo-Group-ID | px_MCVideo_Group_A_ID | One allowed relayed MCPTT group |  |  |
| Relay-Service-Code | "123456" | Identifies a connectivity service the ProSe UE-to-Network Relay provides to Public Safety applications; 24-bit value | TS 23.303 [68] |  |

#### 5.5.8.7	MCVideo User Profile

The structure of a user profile document is specified in TS 24.484 [14] clause 9.3. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 13.2.

Table 5.5.8.7-1: MCVideo User Profile Defaults

| Derivation Path: TS 24.24.484, clause 9.3 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcptt-user-profile |  |  |  |  |
| XUI-URI attribute | "sip:" & px_MCVideo_ID_User_A | same as the XUI value of the Document URI |  |  |
| user-profile-index attribute | "42" | value arbitrarily selected |  |  |
| Status | “true” | MCVideo user profile is enabled |  |  |
| ProfileName | "mcvideo-user-profile-" & user-profile-index & ".xml" | name of the user profile document; user-profile-index is the value of the user-profile-index attribute | TS 24.483 [13] clause 13.2.3; |  |
| Common |  |  |  |  |
| index attribute | "0" | Index for the particular MCVideo user profile |  |  |
| MCVideoUserID |  | Indicates an MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user | TS 24.483 [13] clause 13.2.7 |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCVideo_ID_User_A | MCVideo user identity (MCVideo ID) which is a globally unique identifier within the MCVideo service that represents the MCVideo user |  |  |
| UserAlias | px_MCVideo_User_A_Alias | Alphanumeric aliases of MCVideo user | TS 24.483 [13] clause 13.2.11 |  |
| ParticipantType | px_MCVideo_User_A_ParticipantType | The functional category of the participant (e.g., first responder, second responder, dispatch, dispatch supervisor), typically defined by the MCVideo administrators. | TS 24.483 [13] clause 13.2.15 |  |
| MissionCriticalOrganization | px_MCX_DomainName_Organization_A | Indicates the organization an MCVideo user belongs to | TS 24.483 [13] clause 13.2.16 |  |
| NotifyList |  |  |  |  |
| index attribute | "0" |  |  |  |
| uri-entry | px_MCVideo_ID_User_B |  |  |  |
| CatList |  |  |  |  |
| catentry | "1" |  | TS 24.483 [13] clause 13.2.38 |  |
| ReceptionPriority | "1" |  |  |  |
| OnNetwork |  |  |  |  |
| index | "1" |  |  |  |
| MCVideoGroupInfo |  |  |  |  |
| MCVideo-Group-ID | px_MCVideo_Group_A_ID |  |  |  |
| GMS-App-Serv-Id | tsc_MCX_GMS_Hostname |  |  |  |
| IdMS-Token-Endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| RelativePresentationPriority | "7" |  | TS 24.483 [13] clause 13.2.51 |  |
| MaxAffiliationsNc2 | "10" |  | TS 24.483 [13] clause 13.2.67 |  |
| MaxTimeSingleTransmit | "600" | Value in seconds | TS 24.483 [13] clause 13.2.87 |  |
| OffNetwork |  |  |  |  |
| index | "1" |  |  |  |
| MCVideoGroupInfo |  |  |  |  |
| MCVideo-Group-ID | px_MCVideo_Group_A_ID |  |  |  |
| GMS-App-Serv-Id | tsc_MCX_GMS_Hostname |  |  |  |
| IdMS-Token-Endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| RelativePresentationPriority | "7" |  | TS 24.483 [13] clause 13.2.51 |  |
| User-Info-Id | '555555555555'O |  | TS 24.483 [13] clause 13.2.102 |  |
| cp:ruleset |  |  |  |  |
| cp:rule |  |  |  |  |
| cp:id attribute | "rule1" |  |  |  |
| cp:actions |  |  |  |  |
| allow-create-delete-user-alias | "true" |  |  |  |
| allow-create-group-broadcast- group | "true" |  |  |  |
| allow-create-user-broadcast-group | "true" |  |  |  |
| allow-modify-video | "true" |  |  |  |
| allow-renegotiate-codec | "true" |  |  |  |
| allow-camera-control | "true" |  |  |  |
| allow-remote-control | "true" |  |  |  |
| allow-display-remote-ue | "true" |  |  |  |
| allow-remote-camera | "true" |  |  |  |
| allow-push-video | "true" |  |  |  |
| allow-auto-send-notify | "true" |  |  |  |
| allow-request-affiliated-groups | "true" |  |  |  |
| allow-request-to-affiliate-other-users | "true" |  |  |  |
| allow-recommend-to-affiliate-other-users | "true" |  |  |  |
| allow-regroup | "true" |  |  |  |
| allow-presence-status | "true" |  |  |  |
| allow-request-presence | "true" |  |  |  |
| allow-activate-emergency-alert | "true" |  |  |  |
| allow-cancel-emergency-alert | "true" |  |  |  |
| allow-cancel-emergency-alert-any-user | "true" |  |  |  |
| allow-enable-disable-user | "true" |  |  |  |
| allow-enable-disable-UE | "true" |  |  |  |
| allow-off-network-manual-switch | "true" |  |  |  |
| allow-unlimited-video-streams | "true" |  |  |  |
| allow-auto-recv | "true" |  |  |  |
| allow-auto-recv-emergency | "true" |  |  |  |
| allow-auto-recv-imminent-peril | "true" |  |  |  |
| allow-request-override | "true" |  |  |  |
| allow-select-override | "true" |  |  |  |
| allow-override-group-call | "true" |  |  |  |
| allow-off-network | "true" |  |  |  |
| allow-private-call | "true" |  |  |  |
| allow-manual-commencement | "true" |  |  |  |
| allow-automatic-commencement | "true" |  |  |  |
| allow-failure-restriction | "true" |  |  |  |
| allow-emergency-group-call | "true" |  |  |  |
| allow-emergency-private-call | "true" |  |  |  |
| allow-cancel-group-emergency | "true" |  |  |  |
| allow-imminent-peril-call | "true" |  |  |  |
| allow-cancel-imminent-peril | "true" |  |  |  |
| allow-off-network-group-call-change-to-emergency | "true" |  |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | IP address is IPv4 address |
| IPv6 | IP address is IPv6 address |

#### 5.5.8.8	MCVideo Service Configuration

The structure of a service configuration document is specified in TS 24.484 [14] clause 8.4. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 14.2.

Table 5.5.8.8-1: MCVideo Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 9.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| service configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| Common |  |  |  |  |
| min-length-alias | "2" | Indicates minimum length of an alphanumeric identifier (i.e., alias) |  |  |
| broadcast-group |  |  |  |  |
| num-levels-group-hierarchy | "1" | Indicates the number of levels of group hierarchy for group-broadcast groups |  |  |
| num-levels-user-hierarchy | "1" | Indicates the number of levels of user hierarchy for user-broadcast groups |  |  |
| on-network |  |  |  |  |
| signalling-protection |  |  |  |  |
| confidentiality-protection | "true" |  |  |  |
| integrity-protection | "true" |  |  |  |
| protection-between-mcvideo-servers |  |  |  |  |
| allow-signalling-protection | "true" |  |  |  |
| allow-transmission-control-protection | "true" |  |  |  |
| off-network |  |  |  |  |
| default-prose-per-packet-priority |  | - |  |  |
| mcvideo-private-call-signalling | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-private-call-media | "1" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-emergency-private-call-signalling | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| mcvideo-emergency-private-call-media | "8" | Indicates the default ProSe Per-Packet Priority (PPPP) value |  |  |
| private-call |  |  |  |  |
| mcvideo-max-duration | "600" | Value in seconds | TS 24.483 [13] clause 14.2.17 |  |
| num-levels-priority-hierarchy | "4" |  | TS 24.483 [13] clause 14.2.18 |  |

#### 5.5.8.9	Void

#### 5.5.8.10	MCDATA UE Configuration

The structure of a UE configuration document is specified in TS 24.484 [14] clause 10.2. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 9.2.

Table 5.5.8.10-1: MCDATA UE Configuration Defaults

| Derivation Path: TS 24.484 [14] clause 10.2 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-UE-configuration |  |  |  |  |
| domain attribute | px_MCX_DomainName_Organization_A | Mandatory attribute: domain name of the mission critical organization |  |  |
| common |  |  |  |  |
| short-data-service |  | Contains an integer indicating the maximum number of simultaneous SDS transactions (Nc4) allowed for an MCData UE for on-network or off-network group SDS | TS 24.483 clause 9.2.8 |  |
| Max-Simul-SDS-Txns-Nc4 | "2" | Indicates the maximum number of SDS Transactions | TS 24.483 [13] clause 10.2 |  |
| SDS-Presentation-Priority |  |  | TS 24.483 clause 9.2.8 |  |
| MCDATA -Group-Priority |  |  |  |  |
| MCDATA-Group-ID | px_MCData_Group_A_ID | Value is a "uri" attribute specified in OMA OMA-TS-XDM_Group-V1_1 that indicates the group id. | TS 24.483 [13] clause 10.2 |  |
| group-priority-hierarchy | "7" | Indicates the requested presentation priority of group call; Values: 0-7 "7"=the top priority among groups | TS 24.483 [13] clause 9.2.11, 10.2 |  |
| File distribution |  |  |  |  |
| Max-Simul-FD-Txns-Nc4 | "4" | Contains an integer indicating the maximum number of simultaneous FD transactions (Nc4) allowed for an MCData UE for on-network or off-network group FD | TS 24.483 clause 9.2.12 |  |
| FD-Presentation-Priority |  | contains a list of <MCData-Group-Priority> elements that contains the following elements shown below. | TS 24.483 clause 9.2.13 |  |
| MCDATA-Group-Priority |  |  |  |  |
| MCDATA-Group-ID | px_MCData_Group_A_ID | Identifies a MCData group | TS 24.483 clause 9.2.15 |  |
| group-priority-hierarchy | "7" | Contains an integer that identifies the relative priority level of that MCData group with 0 being the lowest priority and 255 being the highest priority | TS 24.483 [13] clause 9.2.16, 10.2 |  |
| conversation-management |  |  |  |  |
| Conversation-Presentation-Priority |  |  |  |  |
| MCData-Group-Priority |  |  |  |  |
| MCData-Group-ID | px_MCData_Group_A_ID | Identifies a MCData group | TS 24.483 clause 9.2.15 |  |
| group-priority-hierarchy | "7" | Indicates the requested presentation priority of conversation management transactions | TS 24.483 clause 9.2.16 |  |
| transmission-control |  |  |  |  |
| Max-Simul-Data-Transmissions-Nc4 | "3" | Indicates the maximum number of simultaneous data transmissions. | TS 24.483 clause 9.2.21 |  |
| Max-Data-Transmissions-In-Group-Nc5 | "3" | Indicates the maximum number of simultaneous data transmissions. | TS 24.483 clause 9.2.22 |  |
| Data-Presentation-Priority |  | Iindicates the requested presentation priority of data received. | TS 24.483 clause 9.2.23 |  |
| MCData-Group-Priority |  |  |  |  |
| MCData-Group-ID | px_MCData_Group_A_ID |  |  |  |
| group-priority-hierarchy | "7" | Indicates the requested presentation priority of data received. | TS 24.483 clause 9.2.26 |  |
| reception-control |  |  |  |  |
| Max-Simul-Data_Reception-Nc4 | "3" | Indicates the maximum number of simultaneous data receptions. |  |  |
| Max-Simul-Data_Receptions-In-Group-Nc5 | "5" | Indicates the maximum number of data receptions in a group. |  |  |
| on-network |  |  |  |  |
| IPv6Preferred | "false" | Indicates whether IPv6 is preferred over IPv4 for on-network operation when the MCDATA UE has both IPv4 and IPv6 host configuration. | TS 24.483 [13] clause 9.2.31, 10.2 |  |
| Relay-Service | "true" | Indicates the authorisation to use a relay service. NOTE: When the <Relay-Service> element is set to "false" a list of <Relayed-MCData-Group> elements is not needed. | TS 24.483 [13] clause 9.2.32, 10.2 |  |

#### 5.5.8.11	MCDATA User Profile

The structure of a user profile document is specified in TS 24.484 [14] clause 10.3.2.1. Single MCDATA configuration parameters are defined in TS 24.483 [13] clause 10.2.

Table 5.5.8.11-1: MCDATA User Profile Defaults

| Derivation Path: TS 24.484, clause 10.3.2.1 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| mcdata-user-profile |  |  |  |  |
| XUI-URI attribute | "sip:" & px_MCData_ID_User_A | same as the XUI value of the Document URI |  |  |
| user-profile-index attribute | "49" | value arbitrarily selected |  |  |
| Status | “true” | MCData user profile is enabled |  |  |
| ProfileName | "mcdata-user-profile-" & user-profile-index & ".xml" | name of the user profile document; user-profile-index is the value of the user-profile-index attribute | TS 24.483 [13] clause 5.2.7B |  |
| Common |  |  |  |  |
| index attribute | "0" | Index for the particular MCDATA user profile | TS 24.483 [13] clause 10.2.6 |  |
| UserAlias |  |  |  |  |
| alias-entry | px_MCData_User_A_Alias | Alphanumeric aliases of MCDATA user | TS 24.483 [13] clause 10.2.11 |  |
| MCDATAUserID |  |  |  |  |
| entry | px_MCData_ID_User_A |  |  |  |
| MissionCriticalOrganization | px_MCX_DomainName_Organization_A | Indicates the organization an MCData user belongs to | TS 24.483 [13] clause 10.2.16 |  |
| FileDistribution |  |  |  |  |
| FD-cancel-List-Entry |  |  |  |  |
| MCData-ID | px_MCData_ID_User_A | Contains the MCData user identity (MCData ID) of an MCData user that the configured MCData user is authorised to initiate a one-to-one communication, and corresponds to the "MCDataID" element of clause 10.2.16E in 3GPP TS 24.483 [4]; | TS 24.483 clause 10.2.21A |  |
| MCData_ID_KMSURI | tsc_MCX_KMS_Hostname | Contains the KMS URI for the security domain of the MCData user identity (MCData ID) of the MCData user and corresponds to the "MCDataUserIDKMSURI" element of clause 10.2.9A in 3GPP TS 24.483 [4]. If this parameter is absent, the KMS URI is identified by the <kms-sec> element of the <App-Server-Info> of the MCS UE initial configuration document as specified in clause 7.2.2.1 | TS 24.483 [13] clause 10.2.21A |  |
| TxRxControl |  |  |  |  |
| MaxData1To1 | “65535” | Indicates the maximum amount of data (in megabytes) that an MCData user can transmit in a single request during one-to-one communication. | TS 24.483 [13] clause 10.2.25 |  |
| MaxTime1to1 | “65535” | Indicates the maximum amount of time that an MCData user can transmit for in a single request during one-to-one communication. | TS 24.483 [13] clause 10.2.26 |  |
| TxReleaseList | px_MCData_ID_User_A | Indicates an MCData ID of an MCData user that this MCData user is allowed to request release of an ongoing transmission | TS 24.483 [13] clause 10.2.30 |  |
| GroupEmergencyAlert |  | Indicates the MCData group recipient for an MCData emergency Alert | TS 24.483 [13] clause 10.2.38 |  |
| entry | px_MCData_ID_User_A |  |  |  |
| OnNetwork |  |  |  |  |
| index attribute | "0" | Is of type "token" and is included within some elements for uniqueness purposes, and does not appear in the user profile configuration managed object specified in 3GPP TS 24.483 [4]. |  |  |
| MCDataGroupInfo |  |  |  |  |
| MCData-Group-ID | px_MCData_Group_A_ID | Indicates the MCData group ID for the on-network MCData group that the MCData user is allowed to use. | TS 24.483 clause 10.2.47 |  |
| GMS-App-Serv-ID |  |  |  |  |
| entry | tsc_MCX_GMS_Hostname | Placeholder for one or more Group Management Server configurations. |  |  |
| IdMS-Token-Endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| Group-KMSURI | tsc_MCX_KMS_Hostname |  | TS 24.483 [13] clause 10.2.54A |  |
| Relativepresentation Priority | "7" |  |  |  |
| MaxAffiliations | “10” | contains an integer value between 0 and 255 indicating the presentation priority of the off-network group relative to other off-network groups and off-network users | TS 24.483 clause 10.2.71 |  |
| One-To-One-EmergencyAlert |  | Indicates the MCData user recipient for an on-network MCData emergency one-to-one alert | TS 24.483 clause 10.2.91 |  |
| entry | px_MCData_ID_User_A | Indicates the name of the MCData user recipient for an on-network MCData emergency one-to-one alert | TS 24.43 clause 10.2.92 |  |
| OffNetwork |  |  |  |  |
| index attribute | "0" |  |  |  |
| MCDataGroupInfo |  |  |  |  |
| MCData-Group-ID | px_MCData_Group_A_ID | Indicates the MCData group ID for the off-network MCData group that the MCData user is allowed to use. | TS 24.483 [13] clause 10.2.103 |  |
| GMS-App-Serv-Id | tsc_MCX_GMS_Hostname |  |  |  |
| IdMS-Token-Endpoint | "https://" & px_MCX_IdMS_token_IPAddress & ":" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv4 |
|  | "https://[" & px_MCX_IdMS_token_IPAddress & "]:" & px_MCX_IdMS_token_Port & tsc_MCX_IdMS_token_UriPath | Identity management server token endpoint identity information | TS 23.003 [69] TS 24.483 [13] clause 8.2.41A | IPv6 |
| Group-KMSURI | tsc_MCX_KMS_Hostname |  | TS 24.483 [13] clause 10.2.110A |  |
| RelativePresentationPriority | "7" | When it appears in: - the <MCDataGroupInfo> element of the <OnNetwork> element, contains an integer value between 0 and 255 indicating the presentation priority of the on-network group relative to other on-network groups and on-network users, and corresponds to the "PresentationPriority" element of clause 10.2.55 in 3GPP TS 24.483 [4]; and - the <MCDataGroupInfo> element of the <OffNetwork> element, contains an integer value between 0 and 255 indicating the presentation priority of the off-network group relative to other off-network groups and off-network users, and corresponds to the "PresentationPriority" element of clause 10.2.111 in 3GPP TS 24.483 [4]; |  |  |
| User-Info-Id | '555555555555'O |  |  |  |
| ruleset |  |  |  |  |
| rule |  |  |  |  |
| actions |  |  |  |  |
| allow-create-delete-user-alias | "true" |  |  |  |
| allow-create-group-broadcast-group | "true" |  |  |  |
| allow-create-user-broadcast-group | "true" |  |  |  |
| allow-transmit-data | "true" |  |  |  |
| allow-request-affiliated-groups | "true" |  |  |  |
| allow-request-to-affiliate-other-users | "true" |  |  |  |
| allow-recommend-to-affiliate-other-users | "true" |  |  |  |
| allow-regroup | "true" |  |  |  |
| allow-presence-status | "true" |  |  |  |
| allow-request-presence | "true" |  |  |  |
| allow-activate-emergency-alert | "true" |  |  |  |
| allow-cancel-emergency-alert | "true" |  |  |  |
| allow-cancel-emergency-alert-any-user | "true" |  |  |  |
| allow-enable-disable-user | "true" |  |  |  |
| allow-enable-disable-UE | "true" |  |  |  |
| allow-off-network-manual-switch | "true" |  |  |  |
| allow-off-network | "true" |  |  |  |

| Condition | Explanation |
| --- | --- |
| IPv4 | IP address is IPv4 address |
| IPv6 | IP address is IPv6 address |

#### 5.5.8.12	MCDATA Service Configuration

The structure of a service configuration document is specified in TS 24.484 [14] clause 10.4.2.1. Single MCVideo group configuration parameters are defined in TS 24.483 [13] clause 11.2.

Table 5.5.8.12-1: MCDATA Service Configuration Defaults

| Derivation Path: TS 24.484 [14], clause 10.4 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Reference | Condition |
| service configuration |  |  |  |  |
| domain attribute | px_MCData_User_A_Organization | Mandatory attribute: domain name of the mission critical organization |  |  |
| on-network |  |  |  |  |
| tx-and-rx-control |  |  |  |  |
| max-data-size-sds-bytes | "10000000" | The maximum data that the originating client can send in an SDS message |  |  |
| max-payload-size-sds-cplane-bytes | "1000" | The maximum payload data that the originating client can send in an SDS message over C-plane |  |  |
| max-data-size-fd-bytes | "100000000" | The maximum data that the originating client can send in an FD message |  |  |
| max-data-size-auto-recv-bytes | "10000000" | The maximum data that the server can send to the terminating client without requesting the user to indicate a present need for the data |  |  |
| signalling-protection |  |  |  |  |
| confidentiality-protection | "true" | Indicating whether confidentiality protection of MCData signalling is enabled or disabled between the MCData client and MCData server |  |  |
| integrity-protection | "true" | Indicating whether integrity protection of MCData signalling is enabled or disabled between the MCData client and MCData server |  |  |
| protection-between-mcdata-servers |  |  |  |  |
| allow-signalling-protection | :true" | Indicating whether protection of MCData signalling is enabled between MCData servers |  |  |
| file-availability |  |  |  |  |
| default-file-availability | "10000000" | The default time for which a file is available on the server for download, if a explicit time period is not requested by the originating client |  |  |
| max-file-availability | "10000000" | The maximum time for which a file can be made available on the server for download |  |  |
| off-network |  |  |  |  |
| default-prose-per-packet-priority |  |  |  |  |
| mcdata-one-to-one-call-signalling | "1" |  | TS 24.483 [13] clause 11.2.11 |  |
| mcdata-one-to-one-call-media | "1" |  | TS 24.483 [13] clause 11.2.12 |  |