# Title:	Test Tolerances Methodology for Intra-frequency conditional handover from FR1 to FR1 tests

# Source:	Huawei, Hisilicon

# 1.	Introduction

The purpose of this document is to present the methodology and principles used for the development of Test Tolerances for RRM tests 6.3.3.1.

The test cases in question, from TS 38.533, are:

- 6.3.3.1, Intra-frequency conditional handover from FR1 to FR1

This document describes the process to derive the Test Tolerances. The calculations are provided in the accompanying spreadsheet.

# 2. 	Test Tolerance Methodology for 6.3.3.1 in TS 38.133

The test conditions are defined in the following extracts from TS 38.133 v16.9.0.

#### A.6.3.3.1	Intra-frequency conditional handover from FR1 to FR1

##### A.6.3.3.1.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR1-NR FR1 intra frequency conditional handover requirements specified in clause 6.1.4.2.

##### A.6.3.3.1.2	Test Parameters

Supported test configurations are shown in table A.6.3.3.1.2-1. Both conditional handover delay and interruption length are tested by using the parameters in table A.6.3.3.1.2-2, and A.6.3.3.1.2-3.

The test consists of two successive time periods, with time durations of T1 and  T2 respectively. At the start of time duration T1, the UE may not have any timing information of cell 2.

NR shall configure a condition implying handover to cell 2 during T1, at a time earlier than TRRC before the beginning of T2.

Table A.6.3.3.1.2-1: Intra-frequency conditional handover from FR1 to FR1 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | Source cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode Target cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | Source cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode Target cell: NR 30 kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.3.3.1.2-2: General test parameters Intra-frequency conditional handover from FR1 to FR1

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| A3-Offset in condition |  | dB | 0 |  |
| Hysteresis |  | dB | 0 |  |
| Time To Trigger |  | s | 0 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| PRACH configuration index |  |  | FR1 PRACH configuration 1 | As specified in table Table 6.3.3.2-3 in TS 38.211 [6] |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 2 |  |

Table A.6.3.3.1.2-3: Cell specific test parameters for NR FR1-FR1 Intra frequency conditional handover test case

| Parameter |  |  | Unit | Cell 1 |  |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 |  | T1 |  | T2 |
| NR RF Channel Number |  |  |  | 1 |  |  |  | 1 |  |  |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration |  | Config 1 |  | Not Applicable |  |  |  |  |  |  |
|  |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| BWP BW |  | Config 1 | MHz | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 2 |  | 10: NRB,c = 52 |  |  |  |  |  |  |
|  |  | Config 3 |  | 40: NRB,c = 106 |  |  |  |  |  |  |
| DRx Cycle |  |  | ms | Not Applicable |  |  |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | SR2.1 TDD |  |  |  |  |  |  |
| CORESET Reference Channel |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | CR2.1 TDD |  |  |  |  |  |  |
| TRS configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| OCNG Patterns |  |  |  | OCNG pattern 1 |  |  |  |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  | Config 1,2 | kHz | 15 kHz |  |  |  |  |  |  |
|  |  | Config 3 |  | 30 kHz |  |  |  |  |  |  |
| PRACH configuration |  |  |  | FR1 PRACH configuration 1 |  |  |  |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15kHz | -98 |  |  |  |  |  |  |
| Note2 | Config 1,2 |  | dBm/SCS | -98 |  |  |  |  |  |  |
|  | Config 3 |  |  | -95 |  |  |  |  |  |  |
|  |  |  | dB | 8 | -3.3 |  | -Infinity |  | 2.36 |  |
|  |  |  | dB | 8 | 8 |  | -Infinity |  | 11 |  |
| SSB_RP | Config 1,2 |  | dBm/SCS | -90 | -90 |  | -Infinity |  | -87 |  |
|  | Config 3 |  | dBm/SCS | -87 | -87 |  | -Infinity |  | -84 |  |
| IoNote3 | Config 1,2 |  | dBm/ 9.36MHz | -61.41 | -57.06 |  | -61.41 |  | -57.06 |  |
|  | Config 3 |  | dBm/ 38.16MHz | -55.31 | -50.96 |  | -55.31 |  | -50.96 |  |
| Propagation condition |  |  | - | AWGN |  |  |  |  | AWGN |  |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |  |

##### A.6.3.3.1.3 Test Requirements

TRRC + TEvent_DU occurs during T1 as the handover condition becomes satisfied at the start of T2. The test shall verify that there are no interruptions during T1.

The UE shall start to transmit the PRACH to Cell 2 less than Tmeasure + Tinterrupt + TCHO_execution = 800 +62 +10=872ms from the start of T2 and the interruption during T2 shall not exceeed Tinterrupt=Tprocessing + TIU + T∆ + Tmargin =40+20+2 = 62ms

# 3. 	Discussion

Both test cases consist of 3 test configurations. For test 6.3.3.1 each test configuration has 2 time periods: T1 and T2. In this test case, the distribution of power is identical for all 3 test configurations.

During T1 only NR Cell 1 is present and the nominal Ês/Iot value will be +8dB.

During T2 the SS-RSRP of NR Cell 2 is the 3dB larger than that of NR Cell 2, with a nominal Ês/Iot value of +11dB. The “Cond Event A3 (Conditional reconfiguration candidate becomes (0dB) offset better than PCell/PSCell)” condition is contained in clause 5.5.4.4 of TS 38.331 and table A.6.3.3.1.2-2. With nominal signal levels the measurement Cond Event A3 is met, but the effect of uncertainties on the test verdict needs to be checked. A decision can then be made on whether Test Tolerances need to be applied.

# 4. 	Calculation of Test Tolerances

General approach

The general approach is given in the steps below:

- Copy the originally specified key parameters from the core requirements

- Where relevant, calculate derived parameters from the core requirements

- Define uncertainties for a minimum set of parameters

- Define controlled parameters (critical to the test verdict), calculate sensitivity factors and uncertainty

- Determine which original or derived parameters to offset (apply Test Tolerances to) and by how much

- Recalculate original or derived parameters including Test Tolerances

- Check that the controlled parameters meet requirements to get the correct test verdict

Each step is explained below, and the calculations are given in the accompanying spreadsheet.

a) Original specified key parameters

The key parameters are copied from Table A.6.3.3.1-3 in TS 38.133 for test 6.3.3.1. The key parameters are selected as the minimum set to define the cell power levels, and which are subject to a test system uncertainty, which may affect the verdict of the test.

b) Derived parameters

A number of derived parameters are calculated, using the base information in a). The reason for deriving each additional parameter is given in the “Comment” column of section b) in the accompanying spreadsheet.

At the start of T2, NR Cell 2 becomes detectable and the UE is expected to detect and evaluate CHO execution condition on Cell 2. The criterion is CondEvent A3 (Conditional reconfiguration candidate becomes amount of offset better than PCell/PSCell).

The UE makes a measurement of NR Cell 2 (neighbour) relative to NR Cell 1 (PCell), and compares it to the signalled A3 offset. A further step is done to calculate RSRP power difference:

- (NR Cell 2 RSRP – NR Cell 1 RSRP – A3 offset)

c) Uncertainties

The SS provides AWGN and 2 NR intra-frequency cells to the UE. We propose to control the following parameters:

- AWGN Absolute power, Noc ±1.5 dB averaged over BWConfig

- AWGN Absolute power, Noc ±1.5 dB for PRBs #RBJ-RBJ+19

- Ratio of NR cell 1 signal to noise, Ês1 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR cell 1 signal to noise, Ês1 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

- Ratio of NR cell 2 signal to noise, Ês2 / Noc ±0.3 dB averaged over BWConfig

- Ratio of NR cell 2 signal to noise, Ês2 / Noc ±0.3 dB for PRBs #RBJ-RBJ+19

In this test the UE measures the SS-RSRP over specific Physical Resource Block (PRB) numbers # RBJ to # RBJ+19. In addition, this test has separate constraints on the overall power in the configured bandwidth Io.

This choice forms a minimum set, so the superposition principle can be applied if necessary.

In addition, the UE RSRP relative measurement accuracy needs to be considered, because when the UE makes the (NR Cell 2 RSRP – NR Cell 1 RSRP) comparison with the A3 offset, it has some inaccuracy which could affect the test verdict. The UE measurement accuracy is defined in the following extract from TS 38.133:

##### 10.1.2.1.2	Relative SS-RSRP Accuracy

The relative accuracy of SS-RSRP is defined as the SS-RSRP measured from one cell compared to the SS-RSRP measured from another cell on the same frequency, or between any two SS-RSRP levels measured on the same cell in FR1. The accuracy requirements in this clause are also applicable when highSpeedMeasFlag-r16 is configured.

The accuracy requirements in Table 10.1.2.1.2-1 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for intra-frequency measurements are fulfilled according to Annex B.2.2 for a corresponding Band for each relevant SSB.

Table 10.1.2.1.2-1: SS-RSRP Intra frequency relative accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB Ês/Iot Note 2 | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 4 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| 2 | 3 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| 3 | 3 | -6 | Note 3 | Note 3 | Note 3 | N/A | Note 3 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: The parameter SSB Ês/Iot is the minimum SSB Ês/Iot of the pair of cells to which the requirement applies. NOTE 3: The same bands and the same Io conditions for each band apply for this requirement as for the corresponding highest accuracy requirement. NOTE 4: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

To ensure this relative SS-RSRP accuracy, the conditions of Io, SSB Es/Iot in Table 10.1.2.1.2-1, and conditions of SSB_RP and SSB Es/Iot in section Annex B.2.2 need to be fulfilled.

## B.2.2	Conditions for NR intra-frequency measurements

This clause defines the following conditions for NR intra-frequency measurements and corresponding procedures performed based on SSBs: SSB_RP and SSB Ês/Iot, applicable for a corresponding operating band.

The conditions are defined in Table B.2.2-1 for FR1 NR cells.

The conditions are defined in Table B.2.2-2 for FR2 NR cells.

Table B.2.2-1: Conditions for intra-frequency measurements in FR1

| Parameter | NR operating band groups Note1 | Minimum SSB_RP |  | SSB Ês/Iot |
| --- | --- | --- | --- | --- |
|  |  | dBm / SCSSSB |  | dB |
|  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |
| Conditions | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -127 | -124 | -6 |
|  | NR_FDD_FR1_B | -126.5 | -123.5 |  |
|  | NR_TDD_FR1_C | -126 | -123 |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D | -125.5 | -122.5 |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E | -125 | -122 |  |
|  | NR_FDD_FR1_F | -124.5 | -121.5 |  |
|  | NR_FDD_FR1_G | -124 | -121 |  |
|  | NR_FDD_FR1_H | -123.5 | -120.5 |  |
| NOTE 1: NR operating band groups are defined in clause 3.5.2. |  |  |  |  |

In addition, the NR Cell 2 needs to be detectable so that the requirement of conditional handover can apply.

##### 6.1.4.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4.

When TTT or L3 filtering is used an additional delay can be expected.

A cell is detectable only if at least one SSB measured from the cell being configured remains detectable during the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover. If a cell which has been detectable at least for the time period Tidentify_intra_without_index or Tidentify_intra_with_index for intra-frequency handover or Tidentify_inter_without_index for inter-frequency handover becomes undetectable for a period and then the cell becomes detectable again and triggers a handover, the measurement time delay shall be less than TSSB_measurement_period_intra or TSSB_measurement_period_inter provided the timing to that cell has not changed more than ± 3200 Tc while the measurement gap has not been available and the L3 filter has not been used. When L3 filtering is used, an additional delay can be expected.

The condition that an intra-frequency cell is detectable is defined in TS 38.133 9.2.2.

### 9.2.2	Requirements applicability

The requirements in clause 9.2 apply, provided:

-	The cell being identified or measured is detectable.

An intra-frequency cell shall be considered detectable when for each relevant SSB:

-	SS-RSRP related side conditions given in clauses 10.1.2 and 10.1.3 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-RSRQ related side conditions given in clauses 10.1.7 and 10.1.8 for FR1 and FR2, respectively, for a corresponding Band,

-	SS-SINR related side conditions given in clauses 10.1.12 and 10.1.13 for FR1 and FR2, respectively, for a corresponding Band,

-	SSB_RP and SSB Ês/Iot according to Annex B.2.2 for a corresponding Band.

All the side conditions mentioned above refer to Annex B.2.2, which is already considered.

In summary the SSB Es/Iot of both NR Cell 1 and Cell 2 shall be above -3dB, the tightest requirement for Io of RF channel 1 shall be between -117.5dBm/15kHz (-89.5dBm/9.36MHz) or -114.5dBm/30kHz (-83.5dBm/38.16MHz) and -50dBm/BWchannel for band group NR_FDD_FR1_H, and the tightest requirement for SSB_RP shall be above -123.5dBm/15kHz or -120.5/30kHz for band group NR_FDD_FR1_H.

d) Controlled parameters critical to verdict

In many RRM test cases, and particularly intra-frequency multi-cell tests, there is not a simple one-to-one relationship between the parameters that can be set by the test equipment, and their effect on parameters determining the test verdict.

It is therefore essential to identify those parameters determining the test verdict. These may be obvious, such as the difference in SS-RSRP between NR Cell 1 and NR Cell 2 which the UE uses to compare with the A3 offset and trigger the reporting. The 6 controlled parameters listed in the accompanying spreadsheet have been derived by study of the test case and by careful reading of the relevant clauses in TS 38.133. The reason for each parameter being critical to the test verdict is given briefly in the “Comment” column of section d) in the accompanying spreadsheet. Information about the value to be achieved is given later in the “Source and Reference” column of section g) in the spreadsheet.

In general all the values are listed for each time interval. When a cell is not present during a time interval, the corresponding entries are greyed out.

Having identified the parameters critical to the test verdict which need to be controlled, we now need to consider how they are affected by the parameters which can be set by the test equipment. This is done by working out “sensitivity factors”. A sensitivity factor is just the ratio (effect on a critical parameter y / a test equipment uncertainty x) and is usually in dB/dB. Often it can be derived by inspection as one or zero. For example, an error of 1dB in the absolute level Noc would cause 1dB error in the Io on RF Channel 1, so the sensitivity factor is 1.000 during each time interval. However, the same error of 1dB in the absolute level Noc would cause no change to NR Cell 1 Es/Iot, because all other powers are specified relative to Noc, so the sensitivity factor is zero.

In some cases, the sensitivity factor is an intermediate value. For example, the NR Cell 2 Es/Noc has an effect on NR Cell 1 Es/Iot which depends on ratios of the powers making up the total. In such cases a sensitivity factor value between 0 and 1 results. It is important to calculate these correctly to obtain the overall uncertainty.

These factors can also be derived intuitively, by looking at the pie charts presented in step a). A change in the power of NR Cell 2 Es/Noc alone is slightly diluted in the overall interference. It would be wrong to approximate because any change in geometry factors could make the approximation invalid.

Having filled in the matrix of sensitivity factors, the accompanying spreadsheet calculates the overall uncertainty for each controlled parameter, considering the uncertainties and sensitivity factors for each parameter that can be set by the test equipment and the UE RSRP relative measurement accuracy where relevant. This process follows the superposition principle. More details and explanation can be found in section 4 of TS 38.903. Uncertainties are calculated separately for each time interval.

For the test system uncertainties, the normal procedure of combining uncorrelated uncertainties root-sum-square is followed.

When the test system uncertainties and the UE RSRP reporting accuracy are combined, the (root-sum square of test system uncertainties) is added arithmetically to the UE RSRP reporting accuracy. If all the uncertainties were combined root-sum square, the resulting smaller test limits could case a conformant test system to fail a conformant UE, which would be unacceptable.

e) Determine parameters to offset

During T1 the UE is required to select NR Cell 1 as PSCell. The uncertainties do not take NR Cell 1 Ês/Iot or SSB_RP power outside the allowed range, therefore no offsets are required.

During T2, the NR Cell 2 becomes detected. UE is expected to perform intra-frequency measurement. The nominal conditions in TS 38.133 Table A.6.3.3.1-3 gives RSRP values, as measured by the UE, which are 3 dB apart nominally. More details are available in the attached spreadsheet.

The Event A3 measurement report trigger requirement in TS 38.331 clause 5.5.4.4 is “Event A3 (Neighbour becomes offset better than SpCell), inequality A3-1 (Entering condition) is: Mn + Ofn + Ocn – Hys > Mp + Ofp + Ocp + Off”.

The values in this equation are derived as follows:

- Mn is evaluated for NR Cell 2 at T2 and its calculation is available in the attached spreadsheet.

- Ofn is the measurement object specific offset offsetMO of the reference signal of the neighbour cell. It’s default value is specified as 0dB in TS 38.508-1 Table 4.6.3-56.

- Ocn is the cell specific offset cellIndividualOffset of the neighbour cell and set to zero if not configured for the neighbour cell.

- Hys is the hysteresis parameter for this event, specified as 0dB in TS 38.133 Table A.6.3.3.1-2.

- Ms is evaluated for NR Cell 1 at T2 and its calculation is available in the attached spreadsheet.

- Ofp is the measurement object specific offset offsetMO of the SpCell. Its default value is specified as 0dB in TS 38.508-1 Table 4.6.3-56.

- Ocp is the cell specific offset cellIndividualOffset of the serving cell and set to zero if not configured for the neighbour cell.

- Off is the offset parameter for this event (i.e. A3-Offset), specified as 0dB in TS 38.133 Table A.6.3.3.1.2-2.

The equation therefore reduces to:

Mn > Mp + Off

After updating the values in the spreadsheet, it can be seen that for test configuration 6.3.3.1-1, during T2 the Io maximum exceed the upper limit of -50.00 dBm / 40MHz. In order to reduce the maximum Io, the Es/Noc for Cell 2 during T2 is applied an offset of -1dB.

Accounting for this offset, the inequality defined above does not ensure that Cell 2 SS-RSRP – Cell 1 SS-RSRP > A3 offset. Therefore a -1dB offset is required to be applied to the A3-offset so the inequality is satisfied.

The check that all other controlled parameters meet their required range is done in step g). In theory it is possible for steps e) to g) to be iterative, or possibly even steps c) to g) to be iterative, but this test case can be done without such iteration.

f) Parameters modified by Test Tolerances

Based on the decision in e), the set of parameters in a) and b) is reproduced in section f) of the accompanying spreadsheet, but this time modified by the Test Tolerances (applied offsets).

Re-derived parameters are calculated using the same methods as were used in step b). In general, all the values are listed for each time interval. Whenever a cell is not present during a time interval, the corresponding entries are greyed out (the formulae are left in place for use by future test cases).

g) Check controlled parameters Min/Max

Using a format like that in step d), the nominal value of each controlled parameter is recalculated.

The minimum and maximum values, due to variability from uncertainties, of controlled parameters is then calculated and compared against the requirements (RSRP difference, Es/Iot, SSB_RP and Io). It is not necessary to calculate all parameters during each time interval, so a selection is made of those critical to the test verdict. The critical requirement for each parameter is given briefly in the “Comment” column of section g) in the accompanying spreadsheet. The cases closest to limit are identified by turquoise cells in the spreadsheet. If all requirements are met, then the exercise is complete.

With the uncertainty values and Test Tolerances proposed, all the requirements are met.

# 5. Treatment of other configurations within the same test case

TS 38.533 test case 6.3.3.1 also contains configurations with different subcarrier spacings and different channel bandwidths. As the calculations use different values, a separate spreadsheet tab is provided. The following parameters are changed:

- Subcarrier spacing, section a)

- Channel bandwidth, section a)

- RBs in Channel bandwidth, section a) and associated comment

- Cell 1 and Cell 2 SSB_RP minimum requirement, section g) and associated Source/Reference

- Io minimum requirement, section g) and associated Source/Reference

After updating the values in the spreadsheet, it can be seen that for test configuration 6.3.3.1-3, during T2 the Io maximum exceed the upper limit of -50.00 dBm / 40MHz. In order to reduce the maximum Io, the Es/Noc for Cell 2 during T2 is applied an offset of -1dB.

Accounting for this offset, the inequality defined above does not ensure that Cell 2 SS-RSRP – Cell 1 SS-RSRP > A3 offset. Therefore a -1dB offset is required to be applied to the A3-offset so the inequality is satisfied.