3GPP TSG-RAN5 Meeting #93-e	R5-217022r2

# Electronic Meeting, 08th Nov– 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2099 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of default configuration of DCI and SCI for NR SL test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | 5G_V2X_NRSL_eV2XARC-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-29 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | 1, To add default configuration of DCI and SCI for NR SL test |  |  |  |  |  |  |  |  |
| Summary of change: |  | 1, Default configuration of DCI and SCI for NR SL test is added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Work plan is incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.3.6.1.4(new), 4.3.6.2(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | 1st revision: Editorial change according to comments received: - "1_A", "2_A", "2_B" is changed to "1-A", "2-A", "2-B". - Wrong table number is corrected. - Wording is refined. 2nd Revision: - Name of the field in DCI format 3_0 is corrected. |  |  |  |  |  |  |  |  |

<Start of modified section 1>

4.3.6	Physical layer parameters

4.3.6.1	Downlink physical layer parameters

4.3.6.1.1	Physical layer parameters for scheduling of PUSCH

4.3.6.1.1.1	Physical layer parameters for DCI format 0_0

DCI format 0_0 is used for the scheduling of PUSCH in one cell.

Default physical layer parameters for DCI format 0_0 are specified in table 4.3.6.1.1.1-1.

Table 4.3.6.1.1.1-1: Physical layer parameters for DCI format 0_0

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Identifier for DCI formats | Indicating an UL DCI format | "0" |
| Frequency domain resource assignment | Dependent on test parameters | - |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | "0000" |
| Frequency hopping flag | w/o hopping | "0" |
| Modulation and coding scheme | Dependent on test parameters |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |
| Redundancy version | Dependent on test parameters | - |
| HARQ process number | Dependent on test parameters | - |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 | "01" |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |

4.3.6.1.1.2	Physical layer parameters for DCI format 0_1

DCI format 0_1 is used for the scheduling of PUSCH in one cell.

Default physical layer parameters for DCI format 0_1 are specified in table 4.3.6.1.1.2-1.

Table 4.3.6.1.1.2-1: Physical layer parameters for DCI format 0_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| UL/SUL indicator | Not present (0 bit for UEs not configured with SUL in the cell) | - |  |
|  | 1 | 1 | SUL |
| Identifier for DCI formats | Indicating an UL DCI format | “0” |  |
| Bandwidth part indicator | Not present (indicating active BWP, not present in case of only one BWP-Id as per Table 4.6.3-8) | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PUSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| Frequency hopping flag | Not present | - |  |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission / retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| 1st downlink assignment index | Dependent on test parameters |  |  |
| 2nd downlink assignment index | Not present (0 bit if one HARQ-ACK sub-codebook) | - |  |
| TPC command for scheduled PUSCH | 0 dB (accumulated TPC) as per Table 7.1.1-1 in TS 38.213 [22] | “01” |  |
| SRS resource indicator | Not present | - |  |
| Precoding information and number of layers | Not present (0 bits for 1 antenna port and if the higher layer parameter txConfig = Codebook as per clause 7.3.1.1.2 TS 38.212 [27]) | - |  |
|  | 2 | “10” | 2TX_UL_MIMO or ULFPTx_Mode1 |
|  | Dependent on UE reported full power PMI | 1 bit as per Table 7.3.1.1.2-5 in TS 38.212 | ULFPTx_Mode2 |
|  | 0 or 1 | 1 bit | ULFPTx_ModeFull |
| Antenna ports | Port 0 (NOTE 2) | “000” |  |
|  |  | “00” | TRANSFORM _PRECODER _ENABLED |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 (no SUL configured) | “00” |  |
| CSI request | Not present | - |  |
| CBG transmission information | Not present | - |  |
| PTRS-DMRS association | DMRS port 0 | “00” | PTRS_UL_CONFIG |
|  | Not present | - |  |
| beta_offset indicator | Not present (0 bit if the higher layer parameter dynamic in uci-on-PUSCH is not configured) | - |  |
| DMRS sequence initialization | = 0 (ScramblingID0 is not present as per Table 4.6.3-50) | “0” |  |
|  | Not present | - | TRANSFORM _PRECODER _ENABLED |
| UL-SCH indicator | Dependent on test parameters 1 bit. A value of "1" indicates UL-SCH shall be transmitted on the PUSCH and a value of "0" indicates UL-SCH shall not be transmitted on the PUSCH. | - |  |
| NOTE 1: codebookSubset = nonCoherent, 2 layers,TPMI = 0 as specified in TS 38.212 [27] Table 7.3.1.1.2-4 NOTE 2: Bitsize depends on transform precoder being enabled/disabled (PUSCH_Config, Table 4.6.3-118) and on dmrs-Type and maxLength (DMRS-UplinkConfig, Table 4.6.3-51); 3 bits (transform precoder disabled) or 2 bits (transform precoder enabled) for DMRS type 1 and len1 |  |  |  |

| Condition | Explanation |
| --- | --- |
| 2TX_UL_MIMO | UL-MIMO test cases with 2 Tx antenna ports |
| PTRS_UL_CONFIG | When PTRS Uplink is configured |
| TRANSFORM_PRECODER_ENABLED | Transform precoding is enabled (PUSCH_Config, Table 4.6.3-118) |
| SUL | On the SUL carrier when supplementary carrier is configured |
| ULFPTx_Mode1 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-1 |
| ULFPTx_Mode2 | UL-MIMO test cases with UEs supporting UL full power transmission Mode-2 |
| ULFPTx_ModeFull | UL-MIMO test cases with UEs supporting UL full power transmission Mode-full power |

4.3.6.1.2	Physical layer parameters for scheduling of PDSCH

4.3.6.1.2.1	Physical layer parameters for DCI format 1_0

DCI format 1_0 is used for the scheduling of PDSCH in one cell.

Default physical layer parameters for DCI format 1_0 are specified in table 4.3.6.1.2.1-1 to 4.3.6.1.2.1-4.

Table 4.3.6.1.2.1-1: Physical layer parameters for DCI format 1_0

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Identifier for DCI formats | Indicating a DL DCI format | “01” |
| Frequency domain resource assignment | Dependent on test parameters | - |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |
| VRB-to-PRB mapping | Non-interleaved | “0” |
| Modulation and coding scheme | Dependent on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |
| Redundancy version | Dependent on test parameters | - |
| HARQ process number | Dependent on test parameters | - |
| Downlink assignment index | Dependent on test parameters | - |
| TPC command for scheduled PUCCH | 0 dB (accumulated TPC) as per Table 7.2.1-1 in TS 38.213 | “01” |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213) | “110” |
| PDSCH-to-HARQ_feedback timing indicator | K1 slots as specified in 9.2.3 in TS 38.213 |  |
|  | µ=0 (SCS=15kHz): K1=7 | '110'B |
|  | µ=1 (SCS=30kHz): K1=5 | '100'B |
|  | µ=3 (SCS=120kHz): K1=8 | '111'B |

Table 4.3.6.1.2.1-2: Physical layer parameters for DCI format 1_0 for paging

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Short Messages Indicator | Only scheduling information for Paging is present in the DCI | “01” |
| Short Messages | Reserved | - |
| Frequency domain resource assignment | Dependent on test parameters | - |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |
| VRB-to-PRB mapping | Non-interleaved | “0” |
| Modulation and coding scheme | Dependent on test parameters | - |
| TB scaling | Scaling factor S=1 as defined in Table 5.1.3.2-2 in TS 38.214) | “00” |
| Reserved bits | Reserved 6 bits | - |

Table 4.3.6.1.2.1-3: Physical layer parameters for DCI format 1_0 for SI

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Frequency domain resource assignment | Dependent on test parameters | - | - |
| Time domain resource assignment | Indicating the first entry of Table 5.1.2.1.1-2 in TS 38.214 [21] to be used | “0000” | SIB1 |
|  | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” | SI |
| VRB-to-PRB mapping | Non-interleaved | “0” | - |
| Modulation and coding scheme | Dependent on test parameters | - | - |
| Redundancy version | Dependent on test parameters | - | - |
| System information indicator | SIB1 | “0” | SIB1 |
|  | SI message | “1” | SI |
| Reserved bits | Reserved 15 bits | - | - |

| Condition | Explanation |
| --- | --- |
| SIB1 | Used for DCI format 1_0 for SIB1 |
| SI | Used for DCI format 1_0 for SI |

Table 4.3.6.1.2.1-4: Physical layer parameters for DCI format 1_0 for random access

| Parameter | Value | Value in binary |
| --- | --- | --- |
| Frequency domain resource assignment | Dependent on test parameters | - |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |
| VRB-to-PRB mapping | Non-interleaved | “0” |
| Modulation and coding scheme | Dependent on test parameters | - |
| Redundancy version | Dependent on test parameters | - |
| TB scaling | Scaling factor S=[1] as defined in Table 5.1.3.2-2 in TS 38.214) | “00” |
| Reserved bits | Reserved 16 bits | - |

4.3.6.1.2.2	Physical layer parameters for DCI format 1_1

DCI format 1_1 is used for the scheduling of PDSCH in one cell.

Default physical layer parameters for DCI format 1_1 are specified in table 4.3.6.1.2.2-1.

Table 4.3.6.1.2.2-1: Physical layer parameters for DCI format 1_1

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Carrier indicator | Not present | - |  |
| Identifier for DCI formats | Indicating a DL DCI format | “1” |  |
| Bandwidth part indicator | Not present | - |  |
| Frequency domain resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | Indicating the first entry of PDSCH-TimeDomainResourceAllocationList to be used | “0000” |  |
| VRB-to-PRB mapping | Non-interleaved | “0” |  |
| PRB bundling size indicator | Not present (semi-static PRB_bundling) | - |  |
| Rate matching indicator | Not present | - |  |
| ZP CSI-RS trigger | Not present | - |  |
| Modulation and coding scheme (TB1) | Dependent on test parameters | - |  |
| New data indicator (TB1) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version (TB1) | Dependent on test parameters | - |  |
| Modulation and coding scheme (TB2) | Dependent on test parameters | - |  |
| New data indicator (TB2) | Set for every data transmission/retransmission according to the rules specified in TS 38.321 [20] | - |  |
| Redundancy version (TB2) | Dependent on test parameters | - |  |
| HARQ process number | Dependent on test parameters | - |  |
| Downlink assignment index | Dependent on test parameters | - |  |
| TPC command for scheduled PUCCH | 0 dB (accumulated TPC) as per Table 7.2.1-1 in TS 38.213 [22] | “01” |  |
| PUCCH resource indicator | PUCCH-ResourceId[7] = 6 in pucch-ResourceSetID[1] or PUCCH-ResourceId[7] = 14 in pucch-ResourceSetID[2] as defined in Table 4.6.3-112 (Mapping as per Table 9.2.3-2 in TS 38.213 [22]) | “110” |  |
| PDSCH-to-HARQ_feedback timing indicator | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 |  |  |
|  | µ=0 (SCS=15kHz): K1=7 | “101” |  |
|  | µ=1 (SCS=30kHz): K1=5 | “011” |  |
|  | µ=3 (SCS=120kHz): K1=8 | “110” |  |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For 60KHz SCS K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,39} For 120KHz SCS K1 = 4 if mod(i,5) = 0 K1 =3 if mod(i,5) = 1 K1 =7 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,79} |  | RF_FR2_DL |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For TDD K1 = 4 if mod(i,5) = 0 K1 = 3 if mod(i,5) = 1 K1 = 2 if mod(i,5) = 2 where i is slot index per frame; i = {0,…,9} For FDD K1 = 2 |  | RF_FR1_15kHz |
|  | For TDD corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 K1 = 8 if mod(i,10) = 0 K1 = 7 if mod(i,10) = 1 K1 = 6 if mod(i,10) = 2 K1 = 5 if mod(i,10) = 3 K1 = 4 if mod(i,10) = 4 K1 = 3 if mod(i,10) = 5 K1 = 2 if mod(i,10) = 6 where i is slot index per frame; i = {0,…,19} For FDD K1 = 2 |  | RF_FR1_30kHz |
|  | corresponding to K1 slots as per Table 9.2.3-1 in TS 38.213 [22] and dl-DataToUL-ACK in Table 4.6.3-112 For TDD K1 = 13 if mod(i,20) = 2 K1 = 12 if mod(i,20) = 3 K1 = 11 if mod(i,20) = 4 K1 = 10 if mod(i,20) = 5 K1 = 9 if mod(i,20) = 6 K1 = 8 if mod(i,20) = 7 K1 = 7 if mod(i,20) = 8 K1 = 6 if mod(i,20) = 9 K1 = 6 if mod(i,20) = 10 K1 = 6 if mod(i,20) = 11 K1 = 6 if mod(i,20) = 12 K1 = 6 if mod(i,20) = 13 where i is slot index per frame; i = {0,…,39} For FDD K1 = 2 |  | RF_FR1_60kHz |
| Antenna port(s) | DMRS port 0 as per Table 7.3.1.2.2-1 in TS 38.212 [27] (dmrs-Type = DMRS type 1 and maxLength = len1 as per Table 4.6.3-50) | “0000” |  |
| Transmission configuration indication | Not present (0 bits, tci-PresentInDCI = Not present as per Table 4.6.3-28) | - |  |
| SRS request | No aperiodic SRS resource set triggered as per Table 7.3.1.1.2-24 in TS 38.212 [27] (no SUL configured) | “00” |  |
| CBG transmission information | Not present | - |  |
| CBG flushing out information | Not present | - |  |
| DMRS sequence initialization | fix length of 1 bit; '0'B for DMRS-DownlinkConfig.scramblingID0 (or physCellId if scramblingID0 is not present); see Table 4.6.3-50 | “0” |  |

| Condition | Explanation |
| --- | --- |
| RF_FR1_15kHz | RF testing in TS 38.521-x for FR1. SCS is set to 15kHz. |
| RF_FR1_30kHz | RF testing in TS 38.521-x for FR1. SCS is set to 30kHz. |
| RF_FR1_60kHz | RF testing in TS 38.521-x for FR1. SCS is set to 60kHz. |
| RF_FR2_DL | RF testing in TS 38.521-x for FR2. SCS is set to 60kHz or 120kHz |

4.3.6.1.3	Void

4.3.6.1.4	Physical layer parameters for scheduling of PSCCH/PSSCH

4.3.6.1.4.1	Physical layer parameters for DCI format 3_0

DCI format 3_0 is used to schedule PSCCH/PSSCH transmission on NR sidelink.

Default physical layer parameters for DCI format 3_0 are specified in Table 4.3.6.1.4.1-1.

Table 4.3.6.1.4.1-1: Physical layer parameters for DCI format 3_0

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Resource pool index | Not present | - |  |
| Time gap | Indicating the first entry of sl-DCI-ToSL-Trans to be used | “000”B |  |
| HARQ process number | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321[20] | - |  |
| Lowest index of the subchannel allocation to the initial transmission | Dependent on test parameters | - |  |
| Frequency resource assignment | Dependent on test parameters | - |  |
| Time resource assignment | only one PSCCH/PSSCH occasion is scheduled per grant | "00000"B |  |
| PSFCH-to-HARQ feedback timing indicator | Indicating the first entry of sl-PSFCH-ToPUCCH to be used | "000"B |  |
| PUCCH resource indicator | Indicating the first entry of sl-PUCCH-Config to be used | "000"B |  |
| Configuration index | Not present | - | SL-RNTI |
|  | Indicating the first entry of sl-ConfiguredGrantConfigList to be used | "000" | SL-CS-RNTI |
| Counter sidelink assignment index | PDCCH monitoring occasions in which DCI format 3_0 scheduling PSSCH transmissions with corresponding PSFCH reception occasions is present, denoted as Y, satisfies (Y-1) mod 4 =0, as specified in 38.213 Table 16.5.2.1-1 | "00" |  |

| Condition | Explanation |
| --- | --- |
| SL-RNTI | UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-RNTI, i.e. dymanic sidelink grant |
| SL-CS-RNTI | UE is configured to monitor DCI format 3_0 with CRC scrambled by SL-CS-RNTI, i.e. configured sidelink grant |

4.3.6.1.4.2	Physical layer parameters for DCI format 3_1

DCI format 3_1 is used to schedule PSCCH/PSSCH transmission on E-UTRA V2X sidelink.

FFS

4.3.6.2	Sidelink physical layer parameters

4.3.6.2.1	Physical layer parameters for scheduling of PSSCH on PSCCH

4.3.6.2.1.1	Physical layer parameters for SCI format 1-A

SCI format 1-A is used to schedule PSSCH transmission.

Default physical layer parameters for SCI format 1-A are specified in Table 4.3.6.2.1.1-1.

Table 4.3.6.2.1.1-1: Physical layer parameters for SCI format 1-A

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| Priority | Dependent on test parameters | - |  |
| Frequency resource assignment | Dependent on test parameters | - |  |
| Time domain resource assignment | only one PSCCH/PSSCH occasion is scheduled per grant | "00000"B |  |
| Resource reservation period | Not present | - |  |
| DMRS pattern | Dependent on test parameters | - |  |
| 2nd-stage SCI format | SCI format 2-A | “00”B |  |
|  | SCI format 2-B | “01”B | DISTANCE-BASED HARQ |
| Beta_offset indicator | Indicating the first entry of sl-BetaOffsets2ndSCI to be used | "00"B |  |
| Number of DMRS port | Single port p = 1000 is used | "0"B |  |
|  | Two ports p = 1000 and 1001 are used | "1"B | SL MIMO |
| Modulation and coding scheme | Dependent on test parameters | - |  |
| Additional MCS table indicator | Not present | - |  |
| PSFCH overhead indication | Dependent on test parameters | - |  |
| Reserved | Number of reserved bits is given by sl-NumReservedBits-r16 | "00" |  |

| Condition | Explanation |
| --- | --- |
| DISTANCE-BASED HARQ | For distance-based HARQ test |
| SL MIMO | For SL MIMO test |

4.3.6.2.2	Physical layer parameters for scheduling on PSSCH

4.3.6.2.2.1	Physical layer parameters for SCI format 2-A

SCI format 2-A is used to provide HARQ information, to provide Souce/Destination L1 ID or to trigger CSI reporting.

Default physical layer parameters for SCI format 2-A are specified in Table 4.3.6.2.2.1-1.

Table 4.3.6.2.2.1-1: Physical layer parameters for SCI format 2-A

| Parameter | Value | Value in binary | Condition |
| --- | --- | --- | --- |
| HARQ process number | Dependent on test parameters | - |  |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |  |
| Redundancy version | Dependent on test parameters | - |  |
| Source ID | Dependent on test parameters | - |  |
| Destination ID | Dependent on test parameters | - |  |
| HARQ feedback enabled/disabled indicator | Dependent on test parameters | - |  |
| Cast type indicator | broadcast | "00"B | BROADCAST |
|  | groupcast with ACK/NACK HARQ | "01"B | GROUPCAST_ACK_NACK |
|  | unicast | "10"B | UNICAST |
|  | groupcast with NACK-only HARQ | "11"B | GROUPCAST_NACK_ONLY |
| CSI request | Not ask for SL CSI report | "0"B |  |
|  | Ask for SL CSI report | "1"B | SL_CSI |

| Condition | Explanation |
| --- | --- |
| BROADCAST | For broadcast SL communication test |
| GROUPCAST_ACK_NACK | For groupcast SL communication with ACK/NACK HARQ feedback test |
| GROUPCAST_NACK_ONLY | For groupcast SL communication with NACK-only HARQ feedback test |
| UNICAST | For unicast SL communication test |
| SL_CSI | For SL CSI reporting test |

4.3.6.2.2.2	Physical layer parameters for SCI format 2-B

SCI format 2-B is used to provide HARQ information, Souce/Destination L1 ID or distance-based HARQ related parameters.

Default physical layer parameters for SCI format 2-B are specified in Table 4.3.6.2.2.2-1.

Table 4.3.6.2.2.2-1: Physical layer parameters for SCI format 2-B

| Parameter | Value | Value in binary |
| --- | --- | --- |
| HARQ process number | Dependent on test parameters | - |
| New data indicator | Set for every data transmission/retransmission according to the rules specified in TS 38.321 | - |
| Redundancy version | Dependent on test parameters | - |
| Source ID | Dependent on test parameters | - |
| Destination ID | Dependent on test parameters | - |
| HARQ feedback enabled/disabled indicator | Dependent on test parameters | - |
| Zone ID | Dependent on test parameters | - |
| Communication range requirement | Indicating the first entry of sl-ZoneConfigMCR-List to be used | "0000"B |

<End of modified section 1>