3GPP TSG-RAN5 Meeting #93-e	R5-217068r1

# Online, , 8th Nov 2021 - 19th Nov 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.508-1 | CR | 2124 | rev | - | Current version: | 17.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Update Radio resource control information elements for NR 2-step RACH test cases |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_2step_RACH-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-28 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | The default contents of Radio resource control information elements should be introduced for NR 2-step RACH test cases. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Updated message contents of MsgA-ConfigCommon, MsgA-PUSCH-Config RACH-ConfigGenericTwoStepRA and RACH-ConfigGenericTwoStepRA. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The default contents of Radio resource control information elements for NR 2-step RACH test cases are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Table 4.6.3-81A, Table 4.6.3-81B, Table 4.6.3-128A, Table 4.6.3-130A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | Revision 1: Added some missing field. |  |  |  |  |  |  |  |  |

#### –	MsgA-ConfigCommon

Table 4.6.3-81A: MsgA-ConfigCommon

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-ConfigCommonL-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigCommonTwoStepRA-r16 | RACH-ConfigCommonTwoStepRA |  |  |
| msgA-PUSCH-Config-r16 | MsgA-PUSCH-Config |  |  |
| } |  |  |  |

#### –	MsgA-PUSCH-Config

Table 4.6.3-81B: MsgA-PUSCH-Config

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| MsgA-PUSCH-Config-r16 ::= SEQUENCE { |  |  |  |
| MsgA-PUSCH-Resource-r16 SEQUENCE { |  |  |  |
| msgA-MCS-r16 | 1 |  |  |
| nrofSlotsMsgA-PUSCH-r16 | 1 |  |  |
| nrofMsgA-PO-PerSlot-r16 | one |  |  |
| msgA-PUSCH-TimeDomainOffset-r16 | 4 |  |  |
| msgA-PUSCH-TimeDomainAllocation-r16 | Not present |  |  |
| startSymbolAndLengthMsgA-PO-r16 | 32 |  |  |
| mappingTypeMsgA-PUSCH-r16 | typeA |  |  |
| guardPeriodMsgA-PUSCH-r16 | 0 |  |  |
| guardBandMsgA-PUSCH-r16 | 0 |  |  |
| frequencyStartMsgA-PUSCH-r16 | 1 |  |  |
| nrofPRBs-PerMsgA-PO-r16 | 1 |  |  |
| nrofMsgA-PO-FDM-r16 | one |  |  |
| msgA-IntraSlotFrequencyHopping-r16 | Not present |  |  |
| msgA-HoppingBits-r16 | Not present |  |  |
| MsgA-DMRS-Config-r16 SEQUENCE { |  |  |  |
| msgA-DMRS-AdditionalPosition-r16 | pos0 |  |  |
| msgA-MaxLength-r16 | Len2 |  |  |
| msgA-PUSCH-DMRS-CDM-Group-r16 | 0 |  |  |
| msgA-PUSCH-NrofPorts-r16 | 0 |  |  |
| msgA-ScramblingID0-r16 | Not present |  |  |
| msgA-ScramblingID1-r16 | Not present |  |  |
| } |  |  |  |
| nrofDMRS-Sequences-r16 | 1 |  |  |
| msgA-Alpha-r16 | alpha0 |  |  |
| interlaceIndexFirstPO-MsgA-PUSCH-r16 | 1 |  |  |
| nrofInterlacesPerMsgA-PO-r16 | 1 |  |  |
| } |  |  |  |
| msgA-PUSCH-ResourceGroupB-r16 | Not present |  |  |
| msgA-TransformPrecoder-r16 | enabled |  |  |
| msgA-DataScramblingIndex-r16 | Not present |  |  |
| msgA-DeltaPreamble-r16 | 1 |  |  |
| } |  |  |  |

< Unchanged sections omitted >

#### –	RACH-ConfigCommonTwoStepRA

Table 4.6.3-128A: RACH-ConfigCommonTwoStepRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigCommonTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| rach-ConfigGenericTwoStepRA-r16 | RACH-ConfigGenericTwoStepRA |  |  |
| msgA-TotalNumberOfRA-Preambles-r16 | Not present |  |  |
| msgA-SSB-PerRACH-OccasionAndCB-PreamblesPerSSB-r16 CHOICE { | Not present |  |  |
| one | n8 |  | FR1 |
|  | n4 |  | FR2 |
| } |  |  |  |
| msgA-CB-PreamblesPerSSB-PerSharedRO-r16 | Not present |  |  |
| msgA-SSB-SharedRO-MaskIndex-r16 | Not present |  |  |
| groupB-ConfiguredTwoStepRA-r16 | Not present |  |  |
| msgA-PRACH-RootSequenceIndex-r16 CHOICE {} | Not present |  |  |
| msgA-TransMax-r16 | N8 |  |  |
| msgA-RSRP-Threshold-r16 | RSRP-Range |  |  |
| msgA-RSRP-ThresholdSSB-r16 | RSRP-Range |  |  |
| msgA-SubcarrierSpacing-r16 | Not present |  |  |
| msgA-RestrictedSetConfig-r16 | Not present |  |  |
| ra-PrioritizationForAccessIdentityTwoStep-r16 SEQUENCE { |  |  |  |
| ra-Prioritization-r16 | RA-Prioritization |  |  |
| ra-PrioritizationForAI-r16 | '10'B |  |  |
| } |  |  |  |
| ra-ContentionResolutionTimer-r16 | sf32 |  |  |
| } |  |  |  |

< Unchanged sections omitted >

#### –	RACH-ConfigGenericTwoStepRA

Table 4.6.3-130A: RACH-ConfigGenericTwoStepRA

| Derivation Path: TS 38.331 [6], clause 6.3.2 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RACH-ConfigGenericTwoStepRA-r16 ::= SEQUENCE { |  |  |  |
| msgA-PRACH-ConfigurationIndex-r16 | 120 |  |  |
| msgA-RO-FDM-r16 | four |  |  |
| msgA-RO-FrequencyStart-r16 | 0 |  |  |
| msgA-ZeroCorrelationZoneConfig-r16 | 15 |  |  |
| msgA-PreamblePowerRampingStep-r16 | dB0 |  |  |
| msgA-PreambleReceivedTargetPower-r16 | -118 |  |  |
| msgB-ResponseWindow-r16 | sl2 |  |  |
| preambleTransMax-r16 | n10 |  |  |
| } |  |  |  |