3GPP TSG-RAN5 Meeting #93-e	R5-217222r1

# Electronic Meeting, 16th – 27th August 2021

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.533 | CR | 1533 | rev | - | Current version: | 17.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Addition of 4.7.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Sporton |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R5 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_eMIMO-UEConTest |  |  |  |  |  | Date: |  |  | 2021-10-22 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RRM Test Case 4.7.7.1 EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy in RAN4 [6] core-spec, but not added in RAN5 TS 38.533. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Addition of Test Case 4.7.7.1 & relevant minimum conformance requirement |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test Case 4.7.7.1 will be still missed |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.7.7.1 (New) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R1: - Test procedure is modifyied. - “TT” is added in test requirment. |  |  |  |  |  |  |  |  |

## << Unchanged sections omitted >>

4.7.7	L1-SINR measurement for beam reporting

4.7.7.0	Minimum conformance requirements

4.7.7.0.1	Minimum conformance requirements for CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off

The UE shall be capable of performing L1-SINR measurements with the CSI-RS configured as CMR and no dedicated resource configured as IMR for L1-SINR computation, and the UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_Only is defined in Table 4.7.7.0.1-1 for FR1 and in Table 4.7.7.0.1-2 for FR2, where

For the value of M,

-	For periodic and semi-persistent CSI-RS resources as CMR, M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise;

-	For aperiodic CSI-RS resources as CMR, M=1.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requriements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 of TS 38.133 [6] if number of resources in the resource set is smaller than maxNumberRxBeam. The requriements apply provided qcl-info is configured for all resources in the resource set.

For the value of P in FR1,

-	P=11-TCSI-RSMRGP, when in the monitored cell there are measurement gaps configured for intra-frequency, inter-frequency or inter-RAT measurements, which are overlapping with some but not all occasions of the CSI-RS; and

-	P=1 when in the monitored cell there are no measurement gaps overlapping with any occasion of the CSI-RS.

For the value of P in FR2,

-	P=1, when CSI-RS is not overlapped with measurement gap and also not overlapped with SMTC occasion.

-	P=11-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is not overlapped with SMTC occasion (TCSI-RS < MGRP)

-	P=11-TCSI-RSTSMTCperiod, when CSI-RS is not overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod).

-	P=3, when CSI-RS is not overlapped with measurement gap and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod).

-	P= 11-TCSI-RSMRGP-TCSI-RSTSMTCperiod, when CSI-RS is partially overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and

-	TSMTCperiod ≠ MGRP or

-	TSMTCperiod = MGRP and TCSI-RS < 0.5*TSMTCperiod

-	P=31-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is not overlapped with measurement gap and TSMTCperiod = MGRP and TCSI-RS = 0.5*TSMTCperiod

-	P= 11-TCSI-RSmin⁡(TSMTCperiod,MGRP), when CSI-RS is partially overlapped with measurement gap (TCSI-RS < MGRP) and CSI-RS is partially overlapped with SMTC occasion (TCSI-RS < TSMTCperiod) and SMTC occasion is partially or fully overlapped with measurement gap.

-	P=31-TCSI-RSMRGP, when CSI-RS is partially overlapped with measurement gap and CSI-RS is fully overlapped with SMTC occasion (TCSI-RS = TSMTCperiod) and SMTC occasion is partially overlapped with measurement gap (TSMTCperiod < MGRP)

Where:

TSMTCperiod = the configured SMTC1 period or SMTC2 period if configured.

TCSI-RS = the periodicity of CSI-RS configured for L1-SINR measurement

If the high layer in TS 38.331 [2] signaling of smtc2 is configured, TSMTCperiod corresponds to the value of higher layer parameter smtc2; Otherwise TSMTCperiod corresponds to the value of higher layer parameter smtc1.

Note: The overlap between CSI-RS for L1-SINR measurement and SMTC means that CSI-RS for L1-SINR measurement is within the SMTC window duration.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

Table 4.7.7.0.1-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

Table 4.7.7.0.1-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_Only for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_Only (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. |  |

The accuracy requirements in Table 4.7.7.0.1-3 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.1 for a corresponding Band for each relevant CSI-RS based CMR.

-	The bandwidth of CSI-RS as CMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR is equal to or better than the accuracy requirements in Table 4.7.7.0.1-3.

Table 4.7.7.0.1-3: L1-SINR absolute accuracy for CSI-RS based CMR only in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±5.5 | ±6.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.1 and 10.1.27.1.

4.7.7.0.2	Minimum conformance requirements for SSB based CMR and dedicated IMR

The UE shall be capable of performing L1-SINR measurements with the SSB configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to SSB configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_SSB_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as SSB configured as CMR.

The value of TL1-SINR_Measurement_Period_SSB_CMR_IMR is defined in Table 4.7.7.0.2-1 for FR1 and in Table 4.7.7.0.2-2 for FR2, where

For the value of M

-	For periodic or semi-persistent NZP CSI-RS or CSI-IM resource as dedicated IMR, M=1 if the higher layer parameters timeRestrictionForChannelMeasurements and/or timeRestrictionForInterferenceMeasurements are configured, and M=3 otherwise;

For the value of N in FR2

-	N = 8.

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	the value of PCMR shall be derived in the same way as the value of P used for SSB based L1-RSRP measurement in clause 9.5.4.1 of TS 38.133 [6], in which the occasions and period of the SSB for CMR shall be used instead.

-	the value of PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2 of TS 38.133 [6], in which the occasions and period of the NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead.

Longer evaluation period would be expected if the combination of SSB, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For L1-SINR measurement with SSB as CMR and CSI-RS or CSI-IM as IMR, the requirement shall apply if the CSI-RS is configured as IMR with repetition field as “repetition = OFF” or CSI-IM is configured as IMR.

For L1-SINR measurement with SSB as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if SSB occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap

Table 4.7.7.0.2-1: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR channel measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

Table 4.7.7.0.2-2: Measurement period TL1-SINR_Measurement_Period_SSB_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_SSB_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(1.5*M*P*N)*TDRX |
| Note 1: TSSB = ssb-periodicityServingCell is the periodicity of the SSB-Index configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to SSB configured for channel measurement, with the same periodicity. |  |

The accuracy requirements in Tables 4.7.7.0.2-3 and 4.7.7.0.2-4 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.2 for a corresponding Band for each relevant SSB based CMR and IMR.

-	The bandwidth of NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 4.7.7.0.2-3 and 4.7.7.0.2-4.

Table 4.7.7.0.2-3: L1-SINR absolute accuracy for SSB based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | NZP-IMR Ês/Iot |  | Io Note 1 range |  |  |  |  |
|  |  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±4.0 | ±5.0 | 0 |  | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 4.7.7.0.2-4: L1-SINR absolute accuracy for SSB based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | SSB- CMR Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.2 and 10.1.27.2.

4.7.7.0.3	Minimum conformance requirements for CSI-RS based CMR and dedicated IMR

The UE shall be capable of performing L1-SINR measurements with the CSI-RS resource configured as CMR and dedicated resource configured as IMR for L1-SINR computation, in which the NZP-CSI-RS or CSI-IM resource configured as dedicated IMR shall be 1-to-1 mapped to CSI-RS resource configured as CMR, with the same periodicity. The UE physical layer shall be capable of reporting L1-SINR measured over the measurement period of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR.

The requirements in this clause are not applicable if NZP-CSI-RS or CSI-IM resource configured as dedicated IMR is scheduled with different periodicity as CSI-RS resource configured as CMR.

The value of TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR is defined in Table 4.7.7.0.3-1 for FR1 and in Table 4.7.7.0.2-2 for FR2, where

For the value of M,

-	M=1 shall be applied if

-	aperiodic NZP-CSI-RS as CMR or dedicated IMR, or

-	aperiodic CSI-IMR as dedicated IMR, or

-	periodic and semi-persistent NZP-CSI-RS as CMR or dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured, or

-	periodic and semi-persistent CSI-IM as dedicated IMR and the higher layer parameters timeRestrictionForChannelMeasurement and/or timeRestrictionForInterferenceMeasurements are configured;

-	M=3 otherwise.

For the value of N in FR2

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply if qcl-InfoPeriodicCSI-RS is configured for all the resources in the resource set and for each resource one RS has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For periodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided qcl-InfoPeriodicCSI-RS is configured for all resources in the resource set.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For semi-persistent CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=ceil(maxNumberRxBeam / Nres_per_set), where Nres_per_set is number of resources in the resource set. The requirements apply provided TCI state is provided for all resources in the resource set in the MAC CE activating the resource set.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to OFF, N=1. The requriements apply provided qcl-info is configured for all resources in the resource set and for each resource has QCL-TypeD with

-	SSB for L1-RSRP or L1-SINR measurement, or

-	another CSI-RS in resource set configured with repetition ON.

-	For aperiodic CSI-RS resources as CMR in a resource set configured with higher layer parameter repetition set to ON, N=1. UE is not required to meet the accuracy requirements in clause 10.1.28.1 and 10.1.28.3 of TS 38.133 [6] if number of resources in the resource set is smaller than maxNumberRxBeam. The requriements apply provided qcl-info is configured for all resources in the resource set.

P is defined as the maximum value between PCMR and PIMR, i.e., P = max(PCMR, PIMR), where

-	The value of PCMR and PIMR shall be derived in the same way as the value of P used for CSI-RS based L1-RSRP measurement in clause 9.5.4.2 of TS 38.133 [6], in which the occasions and period of the CSI-RS for CMR and NZP CSI-RS for NZP-IMR or CSI-IM for ZP-IMR shall be used instead respectively.

Longer evaluation period would be expected if the combination of CSI-RS, SMTC occasion and measurement gap configurations does not meet pervious conditions.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS as IMR, the requirement shall apply only if CSI-RS resources as CMR and IMR are configured with the same repetition field and the number of CSI-RS resources in the resource sets for CMR and IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-IM as IMR, the requirement shall apply only if the number of CSI-RS resources in the resource set for CMR and the number of CSI-IM resources in the resource set for IMR are same.

For L1-SINR measurement with CSI-RS as CMR and CSI-RS/CSI-IM as IMR, no requirement shall apply if CSI-RS occasions for CMR or CSI-RS/CSI-IM occasions for IMR are fully overlapped with the configured measurement gap.

Table 4.7.7.0.3-1: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR1

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

Table 4.7.7.0.3-2: Measurement period TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR for FR2

| Configuration | TL1-SINR_Measurement_Period_CSI-RS_CMR_IMR (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P*N)*TCSI-RS) |
| DRX cycle ≤ 320ms | max(TReport, ceil(1.5*M*P*N)*max(TDRX,TCSI-RS)) |
| DRX cycle > 320ms | ceil(M*P*N)*TDRX |
| Note 1: TCSI-RS is the periodicity of CSI-RS configured for L1-SINR measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: the requirements are applicable provided that the CSI-RS resource configured for L1-SINR measurement is transmitted with Density = 3. Note 3: The requirements are applicable provided that the CSI-RS resource configured for interference measurement shall be 1-to-1 mapped to CSI-RS configured for channel measurement, with the same periodicity. |  |

The accuracy requirements in Tables 4.7.7.0.3-3 and 4.7.7.0.3-4 are valid under the following conditions:

-	Conditions defined in clause 7.3 of TS 38.101-1 [18] for reference sensitivity are fulfilled.

-	Conditions for L1-SINR measurements are fulfilled according to Annex B.2.8.3 for a corresponding Band for each relevant CSI-RS based CMR and IMR.

-	The bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is 48 PRBs and the density is 3.

-	AWGN radio propagation conditions.

The performance with larger bandwidth of CSI-RS as CMR, NZP-IMR and ZP-IMR is equal to or better than the accuracy requirements in Tables 4.7.7.0.3-3 and 4.7.7.0.3-4.

Table 4.7.7.0.3-3: L1-SINR absolute accuracy for CSI-RS based CMR and NZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | NZP-IMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.0 | ±5.0 | 0 | 0 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |  |

Table 4.7.7.0.3-4: L1-SINR absolute accuracy for CSI-RS based CMR and ZP-IMR in FR1

| Accuracy |  | Conditions |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | CSI-RS CMR Ês/Iot | Io Note 1 range |  |  |  |  |  |
|  |  |  | NR operating band groups Note 2 | Minimum Io |  |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSCSI-RS |  |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSCSI-RS = 15 kHz | SCSCSI-RS = 30 kHz | SCSCSI-RS = 60 kHz |  |  |
|  |  |  | NR_FDD_FR1_A, NR_TDD_FR1_A, NR_SDL_FR1_A | -121 | -118 | -115 | N/A | -50 |
|  |  |  | NR_FDD_FR1_B | -120.5 | -117.5 | -114.5 | N/A | -50 |
|  |  |  | NR_TDD_FR1_C | -120 | -117 | -114 | N/A | -50 |
| ±4.5 | ±5.5 | -3 | NR_FDD_FR1_D, NR_TDD_FR1_D | -119.5 | -116.5 | -113.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_E, NR_TDD_FR1_E | -119 | -116 | -113 | N/A | -50 |
|  |  |  | NR_FDD_FR1_F | -118.5 | -115.5 | -112.5 | N/A | -50 |
|  |  |  | NR_FDD_FR1_G | -118 | -115 | -112 | N/A | -50 |
|  |  |  | NR_FDD_FR1_H | -117.5 | -114.5 | -111.5 | N/A | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. NOTE 2: NR operating band groups in FR1 are as defined in clause 3.5.2. |  |  |  |  |  |  |  |  |

The normative reference for this requirement is TS 38.133 [6] clauses 9.8.4.3 and 10.1.27.3.

4.7.7.1	EN-DC FR1 CSI-RS based CMR and no dedicated IMR configured and CSI-RS resource set with repetition off L1-SINR measurement accuracy

Editor’s note: This test case is incomplete. The following aspects are either missing or not yet determined:

-	The test requirement is FFS.

-	MU/TT is FFS.

-	Test applicability is incomplete in TS38.522

4.7.7.1.1	Test purpose

The purpose of this test is to verify that the L1-SINR measurement accuracy is within the specified limits.

4.7.7.1.2	Test applicability

This test applies to all types of NR UE supporting E-UTRA and EN-DC from Release 16 onwards.

4.7.7.1.3	Minimum conformance requirements

The minimum conformance requirements are specified in clause 4.7.7.0.1.

The normative reference for this requirement is TS 38.133 [6] clause A.4.7.7.1.

4.7.7.1.4	Test description

4.7.7.1.4.1	Initial conditions

This test shall be tested using any of the test configurations in Table 4.7.7.1.4.1-1. Configure the test equipment and the DUT according to the parameters in Table 4.7.7.1.4.1-2.

Table 4.7.7.1.4.1-1: EN-DC CSI-RS based CMR without dedicated IMR L1-SINR measurement supported test configurations

| Config | Description |
| --- | --- |
| 1 | LTE FDD, NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | LTE FDD, NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, FDD duplex mode |
| 5 | LTE TDD, NR 15 kHz CSI-RS SCS, 10 MHz bandwidth, TDD duplex mode |
| 6 | LTE TDD, NR 30kHz CSI-RS SCS, 40 MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to be tested in one of the supported test configurations in each supported band |  |

Table 4.7.7.1.4.1-2: Test Environment parameters for EN-DC CSI-RS based CMR without dedicated IMR L1-SINR measurement

| Parameter | Value |  | Comment |
| --- | --- | --- | --- |
| Test environment | NC, TL/VL, TL/VH, TH/VL, TH/VH |  | As specified in TS 38.508-1 [14] clause 4.1. |
| Test frequencies | As specified in Annex E, Table E.2-1 and TS 38.508-1 [14] clause 4.3.1. |  |  |
| Channel bandwidth | As specified by the test configuration selected from Table 4.7.7.1.4.1-1. |  |  |
| Propagation conditions | AWGN |  | As specified in Annex C.2.2. |
| Connection Diagram | TE Part 2Rx | A.3.1.8.2 with n = 1 | As specified in TS 38.508-1 [14] Annex A. |
|  | TE Part 4Rx | A.3.1.8.5 with n = 1 |  |
|  | DUT Part 2Rx | A.3.2.3.4 |  |
|  | DUT Part 4Rx | A.3.2.5.2 |  |
| Exceptions to connection diagram | N/A |  |  |

1.	Message contents are defined in clause 4.7.7.1.4.3.

2.	Cell 1 is the E-UTRA serving cell (PCell) for the EN-DC setup. The power levels and settings for Cell 1 are set according to Annex A.6. Cell 2 is the NR FR1 cell (PSCell). Cell 2 is the target for CSI-RS-based L1-SINR measurements. Before the test, UE is configured to perform RLM and BFD measurement based on the SSBs. The connection setup is done according to the settings in Annex C.1.1.

4.7.7.1.4.2	Test procedure

Prior to the start of the time duration T1, the UE shall be fully synchronized to PSCell. The UE shall be configured for periodic CSI reporting in PUCCH [format 2] with a reporting periodicity as mentioned in the above table 4.7.7.1.4.1-2.

1.	Ensure the UE is in state RRC_CONNECTED with generic procedure parameters Connectivity EN-DC, DC bearer MCG and SCG, Connected without release On and Test Mode On, according to TS 38.508-1 [14] clause 4.5 and general test parameters set according to Table 4.7.7.1.4.1-2.

2.	Set the parameters according to T1 in Table 4.7.7.2.1.5-1.

3.	The UE shall start sending L1-SINR report including results of both CSI-RS#0 and CSI-RS #1 every 80 slots.

4.	The SS shall check the L1-SINR reported values of CSI-RS #0 and CSI-RS #1 in the periodic L1-SINR reports. If the value for both CSI-RSs is within the limits in Table 4.7.4.2.1.5-2 or Table 4.7.4.2.1.5-3 (depending on the test configuration), the number of passed iterations is increased by one, otherwise the number of failed iterations is increased by one.

5.	The SS shall continue checking the L1-SINR report messages transmitted by the UE until the confidence level according to Table G.2.3-1 in Annex G is achieved.

6. Set the parameters according to each sub-test in Table 4.7.4.2.1.5-1 as appropriate and repeat steps 3-5.

4.7.7.1.4.3	Message contents

Message contents are according to TS 38.508-1 [14] clause 7.3 with the following exceptions:

Table 4.7.7.1.4.3-1: Common Exception messages EN-DC CSI-RS based CMR without dedicated IMR L1-SINR measurement

| Default Message Contents |  |
| --- | --- |
| Common contents of system information blocks exceptions |  |
| Default RRC messages and information elements contents exceptions | Table H.3.6A-1 with conditions APERIODIC and CSI-SINR Table H.3.6A-2 with conditions CSI-RS and APERIODIC |

Table 4.7.7.1.4.3-2: RadioLinkMonitoringConfig

| Derivation Path: TS 38.508-1 [14], Table 4.6.3-133 |  |  |  |
| --- | --- | --- | --- |
| Information Element | Value/remark | Comment | Condition |
| RadioLinkMonitoringConfig ::= SEQUENCE { |  |  |  |
| failureDetectionResourcesToAddModList SEQUENCE (SIZE(1..maxNrofFailureDetectionResources)) OF SEQUENCE { | 1 entry |  |  |
| purpose | both | UE is configured to perform RLM and BFD based on the SSBs. |  |
| } |  |  |  |
| } |  |  |  |

4.7.7.1.5	Test requirement

Table 4.7.7.1.5-1: FR1 CSI-RS based L1-SINR test parameters

| Parameter |  | Config | Unit | Test 1 | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| SSB GSCN |  | 1~6 |  | freq1 | freq1 |  |
| Duplex mode |  | 1,4 |  | FDD | FDD |  |
|  |  | 2,5 |  | TDD | TDD |  |
|  |  | 3,6 |  | TDD | TDD |  |
| TDD Configuration |  | 1,4 |  | N/A | N/A |  |
|  |  | 2,5 |  | TDDConf.1.1 | TDDConf.1.1 |  |
|  |  | 3,6 |  | TDDConf.2.1 | TDDConf.2.1 |  |
| BWchannel |  | 1,4 | MHz | 10: NRB,c = 52 | 10: NRB,c = 52 |  |
|  |  | 2,5 |  | 10: NRB,c = 52 | 10: NRB,c = 52 |  |
|  |  | 3,6 |  | 40: NRB,c = 106 | 40: NRB,c = 106 |  |
| PDSCH Reference measurement channel |  | 1,4 |  | SR.1.1 FDD | SR.1.1 FDD |  |
|  |  | 2,5 |  | SR.1.1 TDD | SR.1.1 TDD |  |
|  |  | 3,6 |  | SR.2.1 TDD | SR.2.1 TDD |  |
| RMSI CORESET Reference Channel |  | 1,4 |  | CR.1.1 FDD | CR.1.1 FDD |  |
|  |  | 2,5 |  | CR.1.1 TDD | CR.1.1 TDD |  |
|  |  | 3,6 |  | CR.2.1 TDD | CR.2.1 TDD |  |
| Dedicated CORESET Reference Channel |  | 1,4 |  | CCR.1.1 FDD | CCR.1.1 FDD |  |
|  |  | 2,5 |  | CCR.1.1 TDD | CCR.1.1 TDD |  |
|  |  | 3,6 |  | CCR.2.1 TDD | CCR.2.1 TDD |  |
| SSB configuration |  | 1,4 |  | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | 2,5 |  | SSB.1 FR1 | SSB.1 FR1 |  |
|  |  | 3,6 |  | SSB.2 FR1 | SSB.2 FR1 |  |
| OCNG Patterns |  | 1~6 |  | OP.1 | OP.1 |  |
| TRS configuration |  | 1,4 |  | TRS.1.1 FDD | TRS.1.1 FDD |  |
|  |  | 2,5 |  | TRS.1.1 TDD | TRS.1.1 TDD |  |
|  |  | 3,6 |  | TRS.1.2 TDD | TRS.1.2 TDD |  |
| Initial BWP Configuration |  | 1~6 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |  |
| Dedicated BWP configuration |  | 1~6 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |  |
| SMTC configuration |  | 1~6 |  | SMTC.1 | SMTC.1 |  |
| CSI-RS |  | 1,4 |  | CSI-RS 1.2 FDD | CSI-RS 1.2 FDD |  |
|  |  | 2,5 |  | CSI-RS 1.2 TDD | CSI-RS 1.2 TDD |  |
|  |  | 3,6 |  | CSI-RS 2.2 TDD | CSI-RS 2.2 FDD |  |
| reportConfigType |  | 1~6 |  | periodic | periodic |  |
| reportQuantity-r16 |  | 1~6 |  | cri-SINR-r16 | cri-SINR-r16 |  |
| nrofReportedRS |  | 1~6 |  | 2 | 2 |  |
| L1-RSRP reporting period |  | 1~6 |  | slot80 | slot80 |  |
| EPRE ratio of PSS to SSS |  | 1~6 | dB | 0 | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1~6 | dBm/15kHz | -94.65+TT | -117+TT |  |
|  | NR_FDD_FR1_B |  |  |  | -116.5+TT |  |
|  | NR_TDD_FR1_C |  |  |  | -116+TT |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5+TT |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115+TT |  |
|  | NR_FDD_FR1_F |  |  |  | -114.5+TT |  |
|  | NR_FDD_FR1_G |  |  |  | -114+TT |  |
|  | NR_FDD_FR1_H |  |  |  | -113.5+TT |  |
| Note2 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2,4,5 | dBm/CSI-RS SCS | -94.65+TT | -117+TT |  |
|  | NR_FDD_FR1_B |  |  |  | -116.5+TT |  |
|  | NR_TDD_FR1_C |  |  |  | -116+TT |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5+TT |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115+TT |  |
|  | NR_FDD_FR1_F |  |  |  | -114.5+TT |  |
|  | NR_FDD_FR1_G |  |  |  | -114+TT |  |
|  | NR_FDD_FR1_H |  |  |  | -113.5+TT |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3,6 |  | -91.65+TT | -114+TT |  |
|  | NR_FDD_FR1_B |  |  |  | -113.5+TT |  |
|  | NR_TDD_FR1_C |  |  |  | -114+TT |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -112.5+TT |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -112+TT |  |
|  | NR_FDD_FR1_F |  |  |  | -111.5+TT |  |
|  | NR_FDD_FR1_G |  |  |  | -111+TT |  |
|  | NR_FDD_FR1_H |  |  |  | -110.5+TT |  |
|  |  | 1~6 | dB | 10+TT | -3+TT |  |
| CSI-RS RSRP Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2,4,5 | dBm/CSI-RS SCS | -84.65+TT | -120+TT |  |
|  | NR_FDD_FR1_B |  |  |  | -119.5+TT |  |
|  | NR_TDD_FR1_C |  |  |  | -119+TT |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -118.5+TT |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -118+TT |  |
|  | NR_FDD_FR1_F |  |  |  | -117.5+TT |  |
|  | NR_FDD_FR1_G |  |  |  | -117+TT |  |
|  | NR_FDD_FR1_H |  |  |  | -116.5+TT |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3,6 |  | -81.65+TT | -117+TT |  |
|  | NR_FDD_FR1_B |  |  |  | -116.5+TT |  |
|  | NR_TDD_FR1_C |  |  |  | -116+TT |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -115.5+TT |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -115+TT |  |
|  | NR_FDD_FR1_F |  |  |  | -114.5+TT |  |
|  | NR_FDD_FR1_G |  |  |  | -114+TT |  |
|  | NR_FDD_FR1_H |  |  |  | -113.5+TT |  |
| Io Note3 | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 1,2,4,5 | dBm/9.36 MHz | -56.28+TT | -87.28+TT |  |
|  | NR_FDD_FR1_B |  |  |  | -86.78+TT |  |
|  | NR_TDD_FR1_C |  |  |  | -86.28+TT |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -85.78+TT |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -85.28+TT |  |
|  | NR_FDD_FR1_F |  |  |  | -84.78+TT |  |
|  | NR_FDD_FR1_G |  |  |  | -84.28+TT |  |
|  | NR_FDD_FR1_H |  |  |  | -83.78+TT |  |
|  | NR_FDD_FR1_A, NR_TDD_FR1_A NOTE 5 | 3,6 | dBm/38.16 MHz | -50.19+TT | -81.19+TT |  |
|  | NR_FDD_FR1_B |  |  |  | -80.69+TT |  |
|  | NR_TDD_FR1_C |  |  |  | -80.19+TT |  |
|  | NR_FDD_FR1_D, NR_TDD_FR1_D |  |  |  | -79.69+TT |  |
|  | NR_FDD_FR1_E, NR_TDD_FR1_E |  |  |  | -79.19+TT |  |
|  | NR_FDD_FR1_F |  |  |  | -78.69+TT |  |
|  | NR_FDD_FR1_G |  |  |  | -78.19+TT |  |
|  | NR_FDD_FR1_H |  |  |  | -77.69+TT |  |
|  |  | 1~6 | dB | 10+TT | -3+TT |  |
| Propagation condition |  | 1~6 |  | AWGN | AWGN |  |
| Antenna configuration |  | 1~6 |  | 1x2 | 1x2 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification |  |  |  |  |  |  |

Table 4.7.7.1.5-2: Same as Table 4.7.4.1.1.5-2

Table 4.7.7.1.5-3: Same as Table 4.7.4.1.1.5-3

For the test to pass, the ratio of successful reported values in each test shall be more than 90% with a confidence level of 95%.

## << End of changes >>